3GPP TSG RAN WG1 Meeting #96	　	                                 R1-1902837

Athens, Greece, 25th Feb – 1st March. 2019

Source:	Mitsubishi Electric

Title:	Performance evaluations for NR positioning

Agenda Item:	7.2.10.1.4 	System-level Performance Evaluation for RAT-Dependent Positioning Techniques

Document for: 	Discussion/Decision

Introduction

In this contribution, we demonstrate performance evaluations results for the DL and UL based NR positioning technique. Both SSB and CSI-RS are used in the positioning technique considered in this contribution.

Evaluation results: ECID based approach

Positioning methodology

In this section, positioning accuracy is evaluated by simulations. For ECID based approach in FR2, we have implemented both SSB-only (1 step) and SSB and CSI-RS (2-step) beam sweeping to estimate the direction of UE by AoD. For SSB beam sweeping, 64 directions were considered. In the 2-step beam sweeping, following SSB beam sweeping, CSI-RS beam sweeping was implemented to perform targeted beam sweep based on the SSB result. An illustrative example of the implemented beam sweeping is shown in Figure 1. Beam alignment was also implemented in the simulation. Beams were swept in both azimuth and zenith directions. No modifications to the channel model, other than the procedures detailed in TR 38.901 [2] were applied in the evaluation. Both received reference signal power and beam spot based method were implemented to estimate the distance between UE and BS.

Figure 1 Illustrative examples of beam sweeping using SSB and CSI-RS (2-step beam sweeping)

Table 1 Simulation parameters (DL & UL)

| Parameter | [Mitsubishi Electric, Indoor office, FR2] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Indoor office from TR 38.901 |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SSB, CSI-RS as defined in TS 38.211 |
| Reference signal (type of sequence, number of ports, …) | SSB, CSI-RS as defined in TS 38.211 |
| Number of sites | 1 |
| Number of symbols used per occasion | 11 symbols reserved for positioning (first 3 symbols reserved for PDCCH) |
| number of occasions used per positioning estimate | 64 |
| Power-boosting level | 0dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Distance estimated by power attenuation (Method 1) or beam spot based distance estimation (Method 2), angle of UE estimated by AoD from gNB |
| Network synchronization assumptions | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping is conducted and alignment is established between gNB TX and UE RX beam |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | None |
| Additional notes, if any | -No PDSCH are transmitted during positioning process in the evaluation -Antenna parameters follow the assumptions in TR 38.855 |

Simulation assumptions are shown in Table 1. Indoor hotspot in FR2 was assumed in the simulation and 2-step estimation shown in Figure 1 was evaluated. Two methods were used to estimate the distance between gNB and UE. Firstly, measurement on attenuation of transmitted power was used to estimate the distance. Alternatively, for beam spot based method, based on the zenith angle of selected beam at gNB and height difference between UE and gNB, distance of between UE and gNB can be estimated. An illustration of the proposed method for distance estimation is shown in Appendix. Performance of the method based on power attenuation is affected greatly by multipath effect.

The evaluation results are presented in Figure 2. From the figure it is clear that power based method yields inferior results compared to beam spot based estimation of the distance. In addition, angle estimation is improved by the 2-step approach, using both SSB and CSI-RS.

Figure 2 CDF of UE position estimation error

In this contribution, the following observations are made.

Observation 1 : Beam-spot based distance estimation is more accurate compared to power attenuation based distance estimation

Observation 2 : Using both CSI-RS and SSB, angle estimation can be improved compared to the method using SSB only.

Conclusion

In this contribution we made the following observation.

Observation 1 : Beam-spot based distance estimation is more accurate compared to power attenuation based distance estimation

Observation 2 : Using both CSI-RS and SSB, angle estimation can be improved

Appendix

In this section, distance between US and BS was estimated using the height difference between UE and gNB and zenith angle of the selected beam. Calculation of the distance is illustrated in Figure 3.

Figure 3 Distance estimation between gNB and UE using selected beam spot

- References

- [1]	RAN1 Chairman’s Note, RAN1 AH, Taipei, Taiwan, Jan. 2019.

- [2]	TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, v14.3.0, Dec. 2017.