3GPP TSG-RAN WG1 Meeting #96	R1-1903044

Athens, Greece, February 25th – March 1st, 2019

Agenda item:	7.2.8.3

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Multi-beam Operation

Document for:	Discussion/Decision

# Introduction

The work item on Rel-16 MIMO enhancements has been specified [1].

MIMO WI from RP-182067

The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows. - Extend specification support in the following areas [RAN1] - (A) Enhancements on multi-beam operation, primarily targeting FR2 operation: - (B) Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - (C) Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - (D) Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR

Agreement In Rel-16, an identifier (ID) that can be used at least for indicating panel-specific UL transmission is supported, where detailed usages for the panel-specific UL transmission are FFS - The ID should be defined considering the possibility to reuse/modification of Rel-15 specification support or introducing new ID - Note: RAN1 to avoid unnecessary specification support requiring UE to explicitly disclose its UL antenna panel implementation - FFS: Whether UE capability signalling is introduced for panel-specific UL transmission Agreement RAN1 has identified the following scenarios to be important for SCell BFR - Scenario 1: SCell with both uplink and downlink - Scenario 2: SCell with downlink only - PCell can be in FR1 or FR2 for scenarios above Agreement - Support L1-SINR measured from - For signal part, SSB and/or NZP CSI-RS - FFS: For interference part - Companies are encouraged to provide simulation results on how to measure/define L1-SINR, e.g. whether interference is measured from dedicated IMR - For example, take Rel-15 L1-RSRP and/or SINR specified in 38.215 as a comparative reference for evaluation purposes Agreement: Decide (agree on) either one of the followings in RAN1 NR-AH 1901: - Alt.1: Support sub-time unit for beam management RS shorter than 1 OFDM symbol in a reference numerology. - No new RS for beam management is introduced in Rel-16. - FFS: details including IFDMA-based, DFT-based, larger subcarrier spacing based, etc, or limited to only for P-3. - Alt.2: No support of sub-time unit for beam management RS shorter than 1 OFDM symbol. Agreement For interference part, down-select at least one from the following alternative: - Alt 1: Dedicated resource(s) for interference measurement - FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214 - FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both - FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement - Alt 2: The same reference signal as signal part as specified in 38.215 - Alt 3: Alt1 when SSB is used for signal part, Alt2 when CSI-RS is used for signal part - Companies are encouraged to provide simulation results for down-selection Agreement - For L1-SINR, interference can be measured based on dedicated resource(s) for interference measurement. - FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214 - FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both - FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement #RAN1 94: Companies are encouraged to evaluate and study further mechanism(s) to support multiple UL Tx panel/beam indication for PUSCH - FFS on how to support the indication, e.g., by multiple SRI fields, an extension of the existing SRI field, an indication of selected panel(s), single SRS resource transmitted associated with multiple panels etc. - FFS on relation to TPMI/TRI fields, PC/TA mechanisms - Companies to consider specification implications of multiple UL Tx panel/beam support Agreement - L1-SINR is supported. L1-RSRQ is not supported. - Companies to study and provide definition of L1-SINR - Study the reporting content, e.g. - Whether CRI/SSBRI is reported - Whether differential group/non-group reporting is applied - Whether L1-RSRP is reported - Study the interference measurement mechanism

The WI for Rel-16 covers several key features aimed at enhancing multibeam operation which we have categorized into features (A), (B), (C) and (D) as shown above. In this contribution we discuss some potential enhancements for beam management for Rel-16. Section 2 discusses feature (D) on beam failure recovery for SCell. Section 3 discusses feature (C) covering UL Tx panel indication. Sections 5, 7, 8 and 9 focus on feature (B), and Section 6 discusses feature (A) focussing on robustness improvement for multi-beam operation.

# SCell Recovery

The following agreements have been made in last meeting.

Agreement Down-select at least one of the following alternatives: - Alt 1: For SCell BFR, BFRQ can be transmitted if UE declares beam failure and identifies a new candidate beam. - UE reports new beam information by or after BFRQ - Alt 2: For SCell BFR, BFRQ can be transmitted if UE declares beam failure. - UE only indicates beam failure happens by BFRQ - Note: new beam identification can be done by using DL BM procedure - Alt 3: For SCell BFR, BFRQ can be transmitted if UE declares beam failure - UE may report new beam information during BFR procedure - FFS: impact of new beam identification threshold - Note: It is up to UE whether to do beam failure detection and new beam identification in parallel or not - For Alt1 and Alt3, reference signals for new candidate DL beam(s) are configured, which are based on CSI-RS and/or SSB. - FFS: whether the CSI-RS and/or SSB can be in another CC FFS: signalling details, e.g. RRC and/or MAC CE Agreement For SCell BFR - Decide BFRQ solution for BFR on SCell with DL only first, PCell in FR1+FR2 - Above is to facilitate RAN1 discussion but not to prioritize certain scenarios

# General BFRQ mechanism for BFR on SCell

In this section we discuss feature (D) of the approved work item, for specifying SCell BFR procedure. For intra-band CA operation where PCell and SCell may share the same QCL, beam failure recovery for SCell can be derived from the PCell BFR in Rel-15. For FR1+FR2 operation where for example PCell is FR1 and SCell is FR2, it may not be possible to determine the quality of SCell link from the PCell beams. This serves as a good use case for supporting and specifying SCell beam failure recovery. In last meeting, two scenarios have been identified as important for SCell BFR, Scenario 1 ( SCell with both UL and DL) and Scenario 2 (SCell with DL only).

SCell beam failure detection and recovery could be derived from existing Rel-15 and performed in implementation specific manners for inter-band CA operations. SCell beam failure detection (BFD) could be based either on the absence of ACK/NACK feedback for the scheduled DL transmission in SCell or depending on CQI report in SCell. These, however, incur latency and overhead. Once beam failure is detected, BFR could be attempted directly on SCell recovery resources as in Rel-15.

As a baseline option, a standalone SCell-based beam failure recovery may be defined in Rel-16, like the BFR procedure in Rel-15 for SpCell. This does not consider the advantage of having an active connection to the gNB and control channel via the PCell, and results in latencies incurred in typical beam recovery procedure. Additionally, recovery resources need to be explicitly configured for the SCell. The overheads involved include dedicated RACH resources for SCell BFR, identification of candidate beam by the UE and RACH procedure-based recovery attempts on SCell without any PCell assistance. More importantly, PCell uplink is needed at least for the case of SCell with DL only. Therefore, PCell assisted SCell BFR is indispensable given the situation that the solution for SCell with DL only will be decided first.

We prefer to use PCell resources to indicate SCell beam failure to the gNB, thereby reducing overhead and efficiently utilizing channel resources. Natural choices of UL resources are SR (PUCCH)/PUSCH/RACH. PUSCH or MAC-CE requires an UL grant, which may not be available all the time. RACH is probably expensive resource given that PCell is operational. Hence our preference is to use SR (PUCCH) which has very low resource utilization.

The proposal includes first detecting SCell failure by monitoring the BFD RS in the SCell. Once the beam failure is detected, the UE sends uplink signal such a usual SR or a dedicated SR to the gNB in the PCell. In response, the gNB may request for beam report in the PCell. Once the UE sends the corresponding L1 report regarding the SCell beams in the PCell, gNB activates/pairs the (new) CORESETs. The high-level call flow is summarized in Figure 1.

Detection of SCell Failure

R16 SCell failure report via PCell

SCell beam activation/pairing by gNB

Figure 1: Rel-16 SCell Beam Failure Recovery

For SCell failure report via PCell, two key proposals for Rel-16 are

- Dedicated SR in PCell for SCell failure indication, in response to which gNB may request L1 report and UE reports the preferred candidate beam of SCell

- Reusing existing SR in PCell for SCell failure indication. This requires defining additional triggering conditions for transmitting SR and new MAC-CE definition along with PUSCH that contains SCell recovery information.

Proposal 1: UE signalling of SCell beam failure can be based on dedicated SR in PCell or reusing an existing PCell SR mechanism to additionally report SCell beam failure.

In case that SCell has DL only, further procedure for SCell beam activation has to be performed via the PCell.

In case that SCell has both DL and UL, further procedure for SCell beam activation/pairing depends on whether beam correspondence is assumed or not.

When beam correspondence is available

- Based on the L1 report/MAC-CE information from the UE about the SCell, the gNB performs SCell CORESET TCI state activation via the PCell.

Figure 2 shows the proposed solution for Rel-16 that leverages PCell connection for performing SCell recovery under beam correspondence.

SCell

Beam Failure Detection

PCell activates SCell CORESET TCI states

DL/UL

Recovered

SCell

RS

gNB

UE

Sending Recovery to PCell via Dedicated SR

PCell Requests

Beam Index report

UE Sends

L1 Report

DL/UL

Recovered

Figure 2: PCell-aided SCell Recovery for Rel-16 under beam correspondence

In the absence of beam correspondence, we need RACH procedure to enable UL beam pairing. Leveraging the active PCell connection, the proposal is as follows:

- PCell triggers dynamic RACH resources associated with SSB index derived from UE SCell recovery report.

A high-level solution/proposal is shown in Figure 3.

SCell

Beam Failure Detection

PCell Triggers dynamic RACH for SCell Recovery

Beam Pairing

DL/UL

Recovered

SCell

RS

gNB

UE

Sending Recovery to PCell via Dedicated SR

PCell Requests

Beam Index report

UE Sends

L1 Report

DL/UL

Recovered

Figure 3: SCell Recovery with dynamic RACH for Rel-16 in the absence of beam correspondence

In last meeting, it has been agreed to down select one alternative on the candidate beam RS, BFRQ trigger, and new beam report.

Agreement Down-select at least one of the following alternatives: - Alt 1: For SCell BFR, BFRQ can be transmitted if UE declares beam failure and identifies a new candidate beam. - UE reports new beam information by or after BFRQ - Alt 2: For SCell BFR, BFRQ can be transmitted if UE declares beam failure. - UE only indicates beam failure happens by BFRQ - Note: new beam identification can be done by using DL BM procedure - Alt 3: For SCell BFR, BFRQ can be transmitted if UE declares beam failure - UE may report new beam information during BFR procedure - FFS: impact of new beam identification threshold - Note: It is up to UE whether to do beam failure detection and new beam identification in parallel or not - For Alt1 and Alt3, reference signals for new candidate DL beam(s) are configured, which are based on CSI-RS and/or SSB. - FFS: whether the CSI-RS and/or SSB can be in another CC - FFS: signalling details, e.g. RRC and/or MAC CE

In Alt. 1, UE sends BFRQ only when beam failure is detected, and new candidate beam is identified. Otherwise, no BFRQ is sent. In Alt. 2, candidate beam RS is not configured before UE sends BFRQ. This may delay new beam identification. Alt. 3 does not have above issues. It does not mandate the identification of a qualified candidate beam as a condition to transmit BFRQ. Candidate beam RS is also configured to reduce new beam identification latency. On the FFS, we support that if a new beam is detected, it is always reported together with its RSRP in or after BFRQ, regardless its RSRP is above or below identification threshold. Otherwise, UE should indicate no new beam is detected in or after BFRQ.

Proposal 2: Support the following alternative

- For SCell BFR, BFRQ can be transmitted if UE declares beam failure

  - UE may report new beam information during BFR procedure

  - On the impact of new beam identification threshold

    - If a new beam is detected, it should always be reported together with its RSRP in or after BFRQ, regardless its RSRP is above or below identification threshold

    - Otherwise, UE should indicate no new beam is detected in or after BFRQ

# BFRQ mechanism for BFR on SCell with DL only

In this section, we explain our view regarding the BFR mechanism for DL-only SCell. Our proposal for DL only SCell is in line with earlier proposal to leverage PCell connection for SCell recovery. On detecting SCell beam failure, the UE sends BFRQ to gNB via PCell, along with/or subsequently the candidate beam information. The activation of the new TCI state in the SCell can also come via PCell. Illustration is shown in Figure 4.

One potential issue while doing this is regarding how the UE confirms that BFRQ has been received by the gNB and whether the beam index report request is in response to the BFRQ or due to existing BM mechanism. This may be addressed by having some distinguishing mechanism in the PDCCH, thereby also enabling faster confirmation.

SCell

Beam Failure Detection

PCell activates SCell CORESET TCI states

DL

Recovered

SCell

RS

gNB

UE

Send Beam failure indication to gNB via PCell

PCell Requests

Beam Index report

UE Sends

L1 Report

DL

Recovered

Figure 4: BFR mechanism for DL-only SCell with PCell assistance

Proposal 3: Support BFRQ transmission and candidate beam reporting from UE in the PCell for DL-only SCell beam reconfiguration

# Latency and Overhead comparison

For different parameter combinations, the following table looks at the latency numbers for supporting the baseline, standalone SCell-based beam failure recovery and the PCell-aided SCell recovery proposal.

Table 1: Latency comparison between baseline scheme and the proposed SCell recovery mechanism

| Parameter Combinations (Value in Slots for 120Khz numerology) |  |  |  |  |  |  |  |  |  |  | Average Latency (#slots) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # RS occasions for averaging | RS periodicity | SR/PUCCH periodicity | SR to PDCCH triggering L1 Report | RACH occasion periodicity | Dynamic RACH periodicity | # RACH ReTx | RAR Delay | Delay to Activate dynamic RACH | MAC-CE Delay after CORESET activation | Baseline option |  | PCell aided SCell recovery |
| 1 | 16 | 1 | 1 | 80 | 40 | 1 | 8 | 8 | 24 | 80 |  | 70 |
| 1 | 16 | 1 | 1 | 80 | 80 | 1 | 8 | 8 | 24 | 80 |  | 90 |
| 2 | 40 | 40 | 1 | 40 | 40 | 4 | 16 | 16 | 24 | 240 |  | 258 |
| 1 | 160 | 80 | 1 | 160 | 80 | 8 | 80 | 8 | 24 | 1384 |  | 794 |
| 2 | 80 | 160 | 1 | 320 | 160 | 2 | 8 | 8 | 24 | 632 |  | 402 |
| 4 | 8 | 1 | 1 | 80 | 40 | 4 | 4 | 8 | 24 | 336 |  | 206 |
| 2 | 160 | 640 | 1 | 40 | 40 | 1 | 2 | 8 | 24 | 366 |  | 376 |

Observation 1: Considerable latency benefits when the dynamic RACH periodicity is smaller than regular RACH occasion periodicity.

Even when dynamic RACH periodicity is assumed to be same as the regular RACH periodicity, the increase in latency is marginal using the proposed PCell-aided SCell recovery.

The amount of channel resources needed for the baseline scheme and the proposed PCell-aided SCell recovery is shown below:

Table 2: Resource overhead between baseline scheme and the proposed SCell recovery mechanism

| Averaging duration (ms) | RS periodicity (#slots) | SCell failure probability | #RACH resources per 5ms | # SR resources | Average resource overhead per 5 ms |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Baseline option | PCell aided SCell recovery |
| 100 | 16 | 1% | 8 | 1 | 8 | 0.1 |
| 1000 | 16 | 2% | 16 | 1 | 16 | 2 |
| 100 | 8 | 2% | 8 | 1 | 8 | 0.4 |
| 100 | 8 | 5% | 16 | 1 | 16 | 1 |
| 1000 | 16 | 5% | 16 | 1 | 16 | 5 |

Observation 2: Significant reduction in overhead and efficient utilization of resources can be obtained with the proposed PCell aided SCell recovery since the RACH resources are activated only after SCell BFD is indicated to the gNB via the PCell.

When existing SR resource is utilized for indicating SCell beam failure on the PCell, there is no extra resource overhead. If dedicated SR is used for this purpose then additional overhead is required for the proposed scheme, which however, is marginal compared to the gain from having dynamic RACH resources.

A summary comparison between extending the existing Rel-15 BFR for SCell recovery in Rel-16 v/s using the proposed procedure is shown below.

|  | Duplicating PCell Recovery in SCell | SCell Recovery with PCell assistance |
| --- | --- | --- |
| Spec impact | Minimal change | Change to enable dynamic RACH resources |
| Resources | Dedicated RACH resource needed per candidate beam per UE | Lower resource usage due to dynamic RACH |
| Latency | High; function of RACH periodicity | Lower, since the RACH periodicity could be smaller due to the on-demand nature |

Based on the latency and overhead benefits we have the following proposal.

Proposal 4: Support PCell triggered dynamic RACH resources on SCell for SCell beam failure recovery.

# Scell BFD RS configuration

In the previous meeting, the following agreements were made for Scell BFD RS.

Agreement - SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state. - Down-select one of the following alternatives in RAN1#96: - Alt 1: SCell BFD RS is in current CC - Alt 2: SCell BFD RS is in current CC for explicit configuration and can be in current CC or another CC for implicit configuration - Alt 3: SCell BFD RS can be in current CC or another CC for both explicit and implicit configuration - SCell BFD is measured based on hypothetical BLER

In this section, we discuss our view on the CC configuration of BFD RS for Scell. For Alt 1, restricting the BFD RS in the current CC disallows 2 cross-carried QCLed CCs sharing the same monitoring RS, which may introduce a higher level of system overhead. Alt 2 allows the BFD RS configured in another CC but only in an implicit manner, e.g. via TCI states QCLed to PDCCH. As far as we are concerned, only having the implicit configuration method may suffer from a lack of flexibility. Because BFD RS must be periodic, while RS in TCI state may be SP/AP. The explicitly configured BFD RS can provide periodic BFD RS in that case. Therefore, we prefer Alt 3, which allows the configuration in the current or anther CC for both explicit and implicit configuration.

Proposal 5: Support both explicit and implicit configuration for BFD RS in another CC.

# Signalling of failed CC index

The following agreement was made regarding the support for deriving failed CC index in Scell BFR procedure.

Agreement Specification support will be provided for gNB to derive at least the failed CC index during SCell BFR procedure - FFS: Whether the information is implicitly derived or explicitly conveyed by the UE - FFS: Whether new beam information should be included - FFS: Details on triggering for transmitting BFRQ

To begin with, the additional signalling to report failed CC index may not be needed in some cases. For instance, when all the SCells can be assumed to be QCLed, e.g. all SCells in same band on FR2 + PCell on FR1, the SCells will most likely fail at the same time. Therefore, in this case, the BFRQ of one SCell indicates the failure of the others, and the failed CC indices can be derived from a regular BFRQ directly.

Observation 3: There is no need to report failed CC index, if all the SCells are cross-carrier QCLed, e.g. all SCells in same band on FR2 + PCell on FR1. Namely, in this case, the failed CC indices can be derived implicitly through a regular BFRQ without additional indication of the failed CC index.

In the other case, when QCL between all SCells cannot be assumed, the failed CC index may need to be individually conveyed. To implicitly convey the failed CC index, a dedicated resource for each CC, e.g. using different time-frequency allocations or scrambling sequences, can be reserved for BFRQ signalling to indicate the CC index. To reduce the overhead of reserving dedicated resources for each CC, one could use a common SR as BFRQ, and request a subsequent uplink report in PUCCH/ PUSCH, which contains an explicit indication of failed CC index as well as other information for BFR, e.g. L1 RSRP report of new beams, as shown in Figure 5. Therefore, to reduce overhead, we propose to support the explicit report of the failed CC index triggered by a common BFRQ signalling, when the index report is needed.

SCell

Beam Failure Detection

BFR for failed CC

SCell

RS

gNB

UE

Sending BFRQ via a SR shared between CCs

PDCCH ordering uplink report when needed

Sending

report with failed CC index

Figure 5: Explicit reporting failed CC index in Scell BFR

Proposal 6: Support explicit report of failed CC index when not all SCells are cross-carrier QCLed.

# Multiple UL Tx-Panel Transmission & UL Beam Selection

The following agreement was made in last meeting to down select the panel ID definition among 4 options.

Agreement An identifier (ID), agreed in RAN1#95, that can be used at least for indicating panel-specific UL transmission is to be down-selected or merged from the following alternatives in next RAN1 meeting: - Alt.1: an SRS resource set ID, where FFS on further association to other RS (if needed) - Alt.2: an ID, which is directly associated to a reference RS resource and/or resource set - Alt.3: an ID, which can be assigned for a target RS resource or resource set - Alt.4: an ID which is additionally configured in spatial relation info

In this section we discuss feature (C) of the work item. Multiple UL Tx-Panel indication was also discussed in the last meeting. A key motivation for conveying panel information is for improved robustness. In R15 it is not explicit if the reference signals are received in the same panel or different panels. Different panels based on panel location direction, and separation may provide robustness to blocking effects, such as finger blocking or human body blocking. Grouping of RS based on panels, can provide better information to the network to identify robust beams for transmission and reception. Another aspect is power saving and latency. UE may have only one panel active at a time, to save power.  Panel switching may lead to increased latency, and hence for instance switching from PUSCH beam transmitted on one panel, to a PUSCH beam that can be transmitted on a different panel may require additional time to activate the new panel. Panel information can enable efficient power and resource utilization. Simultaneous PUSCH transmission/selection across multiple panels can also improve robustness of PUSCH transmission.

In current R15 for non-codebook-based transmission gNB can signal UE to transmit across SRS resources associated with different DL reference signals. If the reference signals were received from different panels, then UE can transmit using multi-panels. However, for the code-book based transmission there is only limited support for multi-panel transmission, as the SRI indicates only one SRS resource, that has only one spatial QCL source.

Below, we highlight some further issues related to enabling both codebook and non-codebook based multi-panel PUSCH transmission.

- Antenna port mapping:   In R15, there is no clear distinction between antenna ports mapped to a single panel vs antenna ports mapped to multiple panels.  To enable transmission of multiple antenna ports out of multiple panels, a distinction in mapping can be signalled. One approach is to associate a panel index with an SRS resource set.

- spatialRelationInfo:  SRS-Resource currently specifies only one reference signal in spatialRelationInfo.  Different panels in the UE may be QCLed with different DL RSs, depending on the panel location and channel conditions.  For example, transmissions from UE panels 1 & 2 based on panel location in the device, may be QCLed with SSB-Index i and SSB-Index j, respectively. Multiple spatialRelationInfo is needed to enable transmission over multi-panel transmissions, which is not possible for Codebook based PUSCH transmission. A natural way is to map a SRS resource set to a panel index, and UE can transmit using SRS resources from two separate SRS resource sets, wherein each SRS resource in the set is mapped to a different panel.

- Panel-level TA: Signal propagation paths from different panels will have different distances (different propagation delays).   If timing advance is based on one particular signal path corresponding to one particular UE panel, a signal transmitted from a different panel traveling on a different path will arrive at the gNb before/past the allotted reception window, which is undesirable.

Based on our observations above, we make the following proposals:

Proposal 7: NR shall enable mechanism to associate each SRS resource set with a panel for both ‘codebook’ and ‘noncodebook’ based PUSCH transmission.

Proposal 8: SRI field in the DCI can be expanded to select multiple SRS resources belonging to multiple SRS resource sets, where each set is associated with a panel.  SRI to SRS resource table mapping shall be extended to include SRS resources across SRS resource sets.

Proposal 9: Study whether panel-level TA is needed for multi-panel UL transmissions.

In addition, we make the following proposal in regard to the UE assisted UL beam selection.  The motivations for UE aided UL beam selection are:

- Based on UE architecture and power consumption considerations, certain UL beam may be preferable for UE.

- To meet the maximum possible emission (MPE) requirement and still use the strongest beam for DL, the UE may prefer a different UL beam (not necessarily associated with same DL beam) to steer transmission away from human body.

Proposal 10: UL Beam Selection: SRI field in the DCI can be used to indicate multiple SRS resources associated with different SRS resource sets for UL transmission, and the UE may select for UL transmission a subset of the indicated SRS resources with corresponding UL beams that point away from human body.

This allows the UE to select a preferential UL beam among many feasible beams indicated by gNb.

In RAN1 AdHoc 1901, it has been agreed to down select the choice of panel ID within 4 options. But the definition of panel is unspecified. If SRS resource set ID is used to identify panel, it may imply that any combination of panels can be used for simultaneous transmissions, since SRS resources in different SRS resource sets for BM can be transmitted simultaneously. However, UL BM is optional. For other use cases, e.g. PUCCH/PUSCH/SRS for non-BM, allowing simultaneous transmissions from any combinations of panels will impose significant constraint on UE complexity. Therefore, we would like to first clarify the definition of panel as below.

Proposal 11: At least for the use cases other than simultaneous Tx of SRS resources for BM, the ID for indicating panel-specific UL transmission is clarified as below

  - Spatial Tx filters associated with same ID cannot be transmitted simultaneously.

  - Spatial Tx filters associated with different IDs may or may not be transmitted simultaneously, depending on UE capability.

    - UE may indicate max # or combinations of IDs whose spatial Tx filters can be transmitted simultaneously.

  - Maximum # of supported spatial layers associated with each ID depends on UE capability

To support panel-specific UL transmission, UE needs to provide panel related info either explicitly or implicitly, e.g. # of panels, # of beams per panel, max # of spatial layers per panel, etc. In addition, the info may vary for different panels. To avoid exposing any panel related info, UE can choose not to support the use of panel ID and hence the panel-specific UL transmission.

Proposal 12: The use of ID for indicating panel-specific is optional.

# Enhancements to multi-beam operation for robustness

In this section we consider feature (A) of the approved WI looking at enhancements to multi-beam operation. Signals in NR systems may suffer from a sudden dramatic RSRP loss due to blockage effects, e.g. from a passing-by human body or a moving vehicle near a UE device. One effective approach to overcome such blockage effects, is to leverage macro-diversity, by receiving from multiple beams of different angular directions. In this section, we discuss enhancements to multi-beam receptions of control channels.

In Figure 6, we illustrate the diversity gain in PDCCH reception from multiple beams using CDL-B channel model and Blockage Model A defined in 3GPP TR 38.901. The simulation assumptions are described in the Appendix. Numerical results indicate that in the presence of blockage effects, it is essential to exploit angular diversity from beams in different directions, which significantly outperforms using a higher aggregation level on a single beam in the PER curves. In the simulation, the multiple beams for a PDCCH are assumed to come from a single TRP. Of course, in the case where the channel from a single TRP does not offer sufficient angular diversity, PDCCH reception from multiple TRPs becomes a natural choice of solution. It, however, can be made transparent to UE whether the multiple beams are from a single or multiple TRPs. Therefore, we have the following proposal.

Proposal 13: For enhanced reliability and robustness, support single DCI transmission over multiple TCI states

In general, soft-combining of the received PDCCHs from multiple beam is not required to achieve the angular diversity gain. For instance, as a more direct method, gNB can simply send multiple PDCCHs scheduling the same PDSCH assignment without informing UE the existence of the repetition; at UE side, it suffices to decode at least one PDCCH. Compared with the method that requires soft-combining, the direct method saves the overhead to convey the PDCCH repetition to UE.

Improving DL control channel robustness/reliability alone is not sufficient, since for closed loop communication, both the UL as well as the DL control channel should be reliable. Hence enhancements to UL control channel for robustness/reliability should be specified for Rel-16. Similar to PDCCH transmission via multiple beams, PUCCH can also be repeated across multiple beams for improved UL reliability. Another UL power efficient approach is to allow PUCCH resource selection at the UE. The UE can be configured with multiple PUCCH resources, each associated with different TCI states and the UE may select one or more of the resources to transmit PUCCH. The resource selection can be up to UE implementation.

Proposal 14: Study and specify PUCCH repetition/ selection across multiple beams for enhanced reliability and robustness

Figure 6: Comparison of PER performance between single beam and multi-beam PDCCH

# Fast Beam Selection via L1- Event Trigger

In this section we consider feature (B) of the approved WI looking at beam selection enhancements for lower latency and overhead reduction. One approach for beam selection in R15 baseline is enabling periodic beam report. Periodic beam report can increase overhead, and power consumption of the UE.  To reduce overhead and power consumption, UE triggered beam report based on L1-Event can be supported. In this way, UE reports for instance the top N beams only, when the current set of top N beams change. L1 events can be specified similar L3 events, and triggering conditions and thresholds can be configured.  A low overhead UL signal such as an SR can be sent in response to an L1-Event trigger.

Another example of L1-Event trigger is partial BFR. To maintain beams proactively and to support L1-event based beam switching ahead of full beam failure, Rel-15 implementation may be leveraged to switch away from failing beams. UE triggered L1-reporting for switching beams and proactive beam failure monitoring can be specified, thereby reducing latency.

An example of L1-event trigger applied to enhancing BFR procedure is shown below. The main motivation in this proposal is that instead of waiting till all the beams in the failure detection set fall below a threshold, we look at early indication to the gNB based on L1 events in the monitored beams. This can enable the gNB to proactively switch to better TCI states ahead of a full beam failure. Additionally, this can be event triggered from the UE side rather than from the gNB side which might ensue higher latency. Using L1-event triggered beam switching, the issue of limited link utilization during the time-consuming Rel-15 BFR procedure can be mitigated, as illustrated in Figure 7.

We studied the impact of blockages on the top four beams over time to see if L1 event triggered reporting arises in practical using Blockage Model A in TS38.901, with blocker elevation width of 20 degrees. We considered UMi scenario with spatial correlation distance dcorr = 10 m for the four non-self-blockers. Figures 7 and 8, show the non-uniform variations in the indices of the top four beams over the simulation interval due to blockers. The non-uniform variations can be exploited to reduce overhead/power consumption by triggering L1 report only when needed. The latency can also be improved at low overhead, by configuring an smaller periodicity SR to indicate the L1-Event trigger to the gNB.

Proposal 15: Study and specify L1 event trigger-based report for fast beam selection.

Beams 1 failed

RS

RS

RS

RS

Both beams failed

RS

RS

Set q1

RS

RS

RS

RS

Beam Failure detected

(BFI maxCount = 2)

Delay to indicate reports to higher layer

RACH

PDCCH

RACH for Recovery

CORESET activation latency

Set q0

RS

RS

RS

RS

Send UE triggered L1 report to switch beam

Proactively avoid the scenario where all beams fail

R-15 flow

R-16 Enhancement

Figure 7: L1-event triggered fast beam selection for enhancing BFR procedure

Figure 8: Index of the top 4 beams

Figure 9: SNR of the initial best four beams

Figure 10: Blocker centres’ evolution over time

# Enhancements to Beam Selection in CDRX

In this section we consider feature (B) of the approved WI looking at beam selection enhancements for lower latency and overhead reduction. In CDRX operation fast beam selection is crucial for power efficient operation in FR2. An example R-15 beam selection during the CDRX ON duration is shown in the figure below. In the example operation gNB configures two CORESETS in each slot to monitor PDCCH. Each CORESET maybe associated with a different TCI state for robustness.

When the gNB has some data to transmit in the ON duration, it may trigger AP CSI-RS via DCI. The UE may report its preferred beam, and further communication can continue based on the indicated report.  The latency for beam selection via this procedure for a few CSF combinations is provided below.

Table 3: Processing time in OFDM symbols (120Khz Numerology) for an example UE implementation

|  | RSRP (CRI/SSBIdx) | Wideband CRI-RI-CQI | Sub-band CRI-RI-CQI |
| --- | --- | --- | --- |
| TDCI-CSIRS:Time from DCI to AP-CSI-RS Tx | 28 | 28 | 28 |
| Num CSIRS Symbols | 2 | 2 | 2 |
| TCSIRS-UL: Time from end of AP-CSI-RS to First UL symbol CSI-Report | 28 | 36 | 140 |
| Total Latency of AP CSI-RS Based | 58 | 66 | 170 |

A component for the latency is the delay between the DCI trigger and AP-CSI-RS transmission. From a robustness aspect the DCI triggering the AP-CSI-RS might not be received due to potential blocking and may have to be retransmitted via a different beam. Hence schemes have to be studied and specified that have lower latency, at the same time good robustness for beam selection during CDRX operation.

Proposal 16: Mechanisms to reduce latency in beam selection for CDRX operation shall be studied and specified.

# Conclusion

In this contribution we discussed enhancements for Rel-16 NR MIMO beam management. Below are the observations and proposals

Observation 1: Considerable latency benefits when the dynamic RACH periodicity is smaller than regular RACH occasion periodicity.

Observation 2: Significant reduction in overhead and efficient utilization of resources can be obtained with the proposed PCell aided SCell recovery since the RACH resources are activated only after SCell BFD is indicated to the gNB via the PCell.

Observation 3: There is no need to report failed CC index, if all the SCells are cross-carrier QCLed, e.g. all SCells in same band on FR2 + PCell on FR1. Namely, in this case, the failed CC indices can be derived implicitly through a regular BFRQ without additional indication of the failed CC index.

Proposal 1: UE signalling of SCell beam failure can be based on dedicated SR in PCell or reusing an existing PCell SR mechanism to additionally report SCell beam failure.

Proposal 2: Support the following alternative

- For SCell BFR, BFRQ can be transmitted if UE declares beam failure

  - UE may report new beam information during BFR procedure

  - On the impact of new beam identification threshold

    - If a new beam is detected, it should always be reported together with its RSRP in or after BFRQ, regardless its RSRP is above or below identification threshold

    - Otherwise, UE should indicate no new beam is detected in or after BFRQ

Proposal 3: Support BFRQ transmission and candidate beam reporting from UE in the PCell for DL-only SCell beam reconfiguration

Proposal 4: Support PCell triggered dynamic RACH resources on SCell for SCell beam failure recovery.

Proposal 5: Support both explicit and implicit configuration for BFD RS in another CC.

Proposal 6: Support explicit report of failed CC index when not all SCells are cross-carrier QCLed.

Proposal 7: NR shall enable mechanism to associate each SRS resource set with a panel for both ‘codebook’ and ‘noncodebook’ based PUSCH transmission.

Proposal 8: SRI field in the DCI can be expanded to select multiple SRS resources belonging to multiple SRS resource sets, where each set is associated with a panel.  SRI to SRS resource table mapping shall be extended to include SRS resources across SRS resource sets.

Proposal 9: Study whether panel-level TA is needed for multi-panel UL transmissions.

Proposal 10: UL Beam Selection: SRI field in the DCI can be used to indicate multiple SRS resources associated with different SRS resource sets for UL transmission, and the UE may select for UL transmission a subset of the indicated SRS resources with corresponding UL beams that point away from human body.

Proposal 11: At least for the use cases other than simultaneous Tx of SRS resources for BM, the ID for indicating panel-specific UL transmission is clarified as below

  - Spatial Tx filters associated with same ID cannot be transmitted simultaneously.

  - Spatial Tx filters associated with different IDs may or may not be transmitted simultaneously, depending on UE capability.

    - UE may indicate max # or combinations of IDs whose spatial Tx filters can be transmitted simultaneously.

  - Maximum # of supported spatial layers associated with each ID depends on UE capability

Proposal 12: The use of ID for indicating panel-specific is optional.

Proposal 13: For enhanced reliability and robustness, support single DCI transmission over multiple TCI states

Proposal 14: Study and specify PUCCH repetition/ selection across multiple beams for enhanced reliability and robustness

Proposal 15: Study and specify L1 event trigger-based report for fast beam selection.

Proposal 16: Mechanisms to reduce latency in beam selection for CDRX operation shall be studied and specified.

# Appendix

# 11.1 Additional results for multi-TCI PDCCH

Table 3. Simulation Assumptions in Section 6.1

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Subcarrier Spacing for data | For 30 GHz: 120kHz |
| Data allocation | Agg level 4,8,16 |
| Channel Model | CDL-B model - delay spread =100ns , UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in subclause 7.7.5.1 in TR 38.901 accordingly. Companies to report phase noise modelling and PTRS considerations if used. |
| Criteria for beam selection | Best CPO beam pointing towards the average strongest cluster |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 2, 2; 1 , 1). ((dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS antenna element radiation pattern | For 30 GHz: According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 1; 1, 1); * Θmg,ng=90°; Ω0,1=Ω0,0+180°; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0, 360] degree |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| UE mobility feature | Blockage model added as Blockage A model in 3GPP 38.901 |
| Transmission scheme | Single TCI beam, single transmission; Single TCI beam, two transmissions (TDM, soft combine); 2 TCI beams, 2 transmissions (soft-combine) |
| MIMO mode | SU-MIMO |
| UE receiver type | MMSE-IRC as baseline |

# References

- RP-182067 Rel-16 MIMO Work Item

- Chairman’s Notes, 3GPP TSG-RAN WG1 Meeting #94b

- Chairman’s Notes, 3GPP TSG-RAN WG1 Meeting #95