3GPP TSG RAN WG1 Meeting #96 	R1-1903096

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Discussion of high resolution codebook design for rank 3 and 4

Document for:	Discussion and Decision

Introduction

Based on the WID of NR MIMO enhancements for Rel-16 in RAN meeting #80 [1], Rel-16 will specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead as follows:

- Extend specification support in the following areas

  - Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead.

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2.

It has been agreed in previous meetings [2-3] that:

Agreement:

The study and, if needed, work on Type II higher rank extension is performed as follows:

- Only for rank 3 and 4 by taking into account the outcome of Type II overhead reduction for rank 1-2.

- Simple extension of Rel.15 Type II without any additional optimization (which results in ~3-4x overhead over rank-1) is ruled out.

Agreement:

In RAN1 NR-AH 1901, companies are encouraged to evaluate the following alternatives for compression basis (Wf) subset selection scheme across different layers when RI=2. Select one of the following alternatives in RAN1#96:

- Alt1. Basis subset selection (Wf) for the 1st is the same as that for the 2nd layer

- Alt2. Basis subset selection (Wf) for the 1st can be different from 2nd layer

Assume Rel.15 3-bit amplitude and Rel.15 8PSK co-phasing for W2W2 quantization for evaluation purposes.

Only up to rank 2 is supported in type II codebook in Rel-15. However, up to rank 8 transmission per user is supported in Rel-15. That means the SU-MIMO performance will be limited by the maximal reporting rank of type II codebook. In this contribution, we will discuss whether to extend the Type II CSI feedback to rank 3/4 in Rel-16.

Probability of rank 3 and 4 for Type II CSI feedback

In NR Rel-15, up to 32 CSI-RS ports at gNB side has been supported and UEs with 4Rx are mandated to be supported in some NR bands. Fig. 1 shows the distribution of ranks of CSI reports, with the assumption of 32Tx at gNB and 4Rx at UE without any rank restriction. Simulation results show that the possibility of UE reporting rank 3 or 4 is 25%-30% for SU-MIMO in both UMa and UMi scenarios. On the other hand, for SU-MIMO case, since type II CSI feedback has higher resolution than type I, supporting rank 3 and 4 for type II can improve UE experience compared with type I CSI feedback. Therefore, the extension of type II codebook for rank 3 and 4 can be a good candidate technique of CSI feedback enhancement.

Observation 1: The probability of UE reporting rank 3 or 4 is 25%-30% in both UMa and UMi scenarios.

Figure 1. Distribution of feedback ranks in UMa and UMi scenarios.

One straightforward method to design high rank codebook is to separately quantize and feedback the combining coefficients in W2 for each layer. In other words, a type-II-like codebook for rank 3 and 4 can be obtained by simply extending type II codebook design for rank 1 and 2 with each layer quantized independently and same linear combination codebook for each layer.

However, the CSI reporting overhead with simply extension of type II codebook to high rank will be doubled. Therefore, compressing the feedback overhead of high rank codebook of type-II-like should be considered. Furthermore, frequency domain compression codebook is discussed in Rel-16 [4], the extension to rank 3/4 should consider the outcome of Type II overhead reduction for rank 1-2.

Performance evaluations of codebooks with rank 3 and 4

Preliminary system-level simulation results of rank adaptation for SU-MIMO are illustrated in Fig. 2. The performance gain of type II in Rel-15 and type-II-like codebook over type I codebook in Rel-15 is shown. For the type-II-like codebook, Rel-15 type II codebook is reused for rank 1 and 2 and the design principle of R15 type II codebook is simply extended to each layer of rank 3 or 4. In the simulation, the maximum rank of type-II-like codebook is 4, while the maximum rank of Rel-15 type II is 2 as supported in Rel-15. For the baseline using type I codebook, the maximum rank of type I codebook is 4. Detailed parameters are shown in Appendix I.

It can be seen that for SU-MIMO case type II codebook in Rel-15 up to rank 2 has about 10% performance gain over type I codebook. Moreover, the performance for type-II-like codebook in Rel-16 up to rank 4 has additional 6% gain over the Rel-15 type II with maximum rank 2. Therefore it is necessary to extend type II codebook to rank 3 and 4.

Figure 2. Performance comparison of type I, type II codebook in Rel-15 and the compression codebook in Rel-16.

Observation 2: Extending Rel-16 codebook with rank 3 and 4 can provide around additional 7-10% cell average gain compared with Rel-15 type II with rank 1 and 2 only.

Proposal 1: DFT-based compression codebook should support rank 3 and 4 PMI quantization in Rel-16.

Type-II codebook enhancement for rank 3 and 4

In the above sections, we have shown the benefits for type-II with extension to rank 3/4. However, considering the increment in feedback overhead, a basic principle of codebook design for rank 3/4 could be that CSI feedback overhead for rank 3/4 shall be comparable to the overhead of Rel-15 Type II codebook rank 1/2 in general.

Codebook extension on R15 type II codebook

For beam combination codebook, such as type II codebook, the performance of using that codebook can be considerably better than that of using beam selection codebook such as type-I codebook. Considering both of performance and overhead, two potential codebook structures can be considered.

One potential structure is a hybrid codebook design with both beam selection and beam combination. Beam combination codebook is utilized within dominant layer and beam selection is utilized within rest weak layer(s). For layers with beam combination, the beams for combination are selected from an orthogonal 2D-DFT beam group as Rel-15. As type II codebook in NR, wideband amplitude, subband amplitude and subband phase are feedback to represent the coefficients of linear combination of dominant layers. For rest weak layer(s) with beam selection, a beam can be selected from the same orthogonal 2D-DFT beam group with a co-phasing term between polarizations, but the set of beams for beam selection may be different from beams used for dominant layers with beam combination.

Another potential codebook structure is that the combination coefficients of layer 3/4 are constructed based on the coefficients of layer 1/2, guaranteeing the orthogonality among some layers. For example, the amplitude coefficients of layer 1 can be reused for layer 3, where the coefficients are reordered based on the power level of beams in layer 3. For the phases, the phases of the 1st polarization in layer 1 is reused for the 1st polarization in layer 3, the phases of the 2nd polarization in layer 1 are rotated for the 2nd polarization layer 3. The permutation of the amplitude and/or phase coefficients of beams in layer 1 and layer 3 can be designed to guarantee the orthogonality among layer 1 and layer 3. With above design, only coefficients of the first two layers are reported and the overhead of high rank type II codebook can be greatly reduced.

Codebook extension on frequency compression codebook

As discussed in the last two meetings, rank 3 and 4 codebook design should also take the outcome of Type II overhead reduction for rank 1-2 into account.

We have agreed the following codebook for a layer of type II codebook enhancement:

W=W1W2WfH

where W1=v0v1…vL-100v0v1…vL-1 is spatial domain (SD) compression matrix, Wf is frequency-domain (FD) compression matrix and W2 is two-dimension linear combination coefficients matrix.

As shown in figure 1, rank 2 PMI can contribute a great portion of rank distribution. Therefore, the performance of rank2 should be guaranteed, if rank adaption is considered. Therefore it is feasible to reuse the principle of Rel-16 rank 2 codebook in R15 for higher rank codebook design, for example:

- Each layer is quantized independently without restriction on the employed number of spatial beams or frequency beams or coefficients for the second layer.

- The codebook for each layer reuses the agreed codebook structure.

- Common or independent spatial/frequency beams for 2-nd layers can be further discussed.

Therefore, for rank 3/4, it can be straightforward to extend the codebook design of rank 1 to higher rank. One issue is the increment of feedback overhead. It is not desired to have the feedback overhead linearly increased for higher rank because of issues of UL resource allocation at the UE and the uncertainty of rank determination at the UE. Then considering the trade-off between overhead and performance, we have the following proposal:

Proposal 2: The feedback overhead of rank 3/4 should be comparable to that of rank 2 for type II codebook enhancement.

To reduce the feedback overhead, the configuration for rank 3/4 should be restricted. It is well known that the feedback overhead of type II enhancement codebook is mainly due to the number of coefficients to report. Therefore, the number of coefficients to report for rank 3/4 should be restricted, especially the number of coefficients for the weaker layers.

Two possible ways for rank ¾ codebook to reduce the feedback overhead are:

- Lower number of coefficients to report for each layer within rank 3/4 compared with those for each layer within rank 1/2.

- Lower number of coefficients to report for the weaker layers within rank 3/4 compared with those for stronger layers within rank 3/4.

  - Quantization method, such as uneven bit allocation or differential amplitude quantization, can be further combined and discussed to reduce the feedback overhead.

A bitmap is agreed to indicate the non-zero coefficients indices and the length of the bitmap depends on the value of M (the number of selected frequency beams). If the number of coefficients is restricted, the value of M should also be restricted to make the selection more efficient.

For rank 3/4 codebook design, different layers may experience different channel conditions and may have some difference in spatial propagation property. A larger value of L can provide fuller channel spatial information and guarantee the performance of rank 3/4.

Conclusions

The contribution discusses the Type II codebook enhancement for rank 3 and 4 in Rel-16, based on which the following observations and proposal are made.

Observation 1: The probability of UE reporting rank 3 or 4 is 25%-30% in both UMa and UMi scenarios.

Observation 2: Extending Rel-16 codebook with rank 3 and 4 can provide around additional 7-10% cell average gain compared with Rel-15 type II with rank 1 and 2 only.

Proposal 1: DFT-based compression codebook should support rank 3 and 4 PMI quantization in Rel-16.

Proposal 2: The feedback overhead of rank 3/4 should be comparable to that of rank 2 for type II codebook enhancement.

- References

RP-181453, Samsung, “WI Proposal on NR MIMO Enhancements”, 3GPP RAN#80, La Jolla, USA, 11-14 June, 2018.

“3GPP RAN1 #94bis Chairman’s Notes”, Chengdu, China, 8-12 October, 2018.

“3GPP RAN1 #95 Chairman’s Notes”, Spokane, USA, 12-16 November, 2018.

R1-1901566, Huawei, HiSilicon, “Discussion on CSI enhancement”, Athens, Greece, February 25- March 1, 2019.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz |
| System bandwidth | 10MHz (13 subbands, 4 PRB for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) for overhead reduction; (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2) for higher rank of Type II; the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; SU-MIMO for higher rank of Type II |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |