3GPP TSG-RAN WG1 Meeting #96	R1-1903120

Athens, Greece, Feb 25th – March 1st, 2019

Source:	Ericsson

Title:	IMT-2020 self-evaluation: mMTC non-full buffer connection density for LTE-MTC and NB-IoT

Agenda Item:	7.2.7.2

Document for:	Discussion & Decision

# Introduction

3GPP has as part of the New Study Item on Self Evaluation towards IMT-2020 Submission [1] agreed to “Provide self evaluation results against technical performance requirements for mMTC as per defined in Report ITU-R M.[IMT-2020. TECH PERF REQ]  [RAN1, RAN2], including Connection density”.

A submission time plan has also been agreed [2] where the 3GPP meetings for submission of description and compliance templates according to ITU-R report M.2411 [3] are set. The Compliance template for technical performance contains the minimum technical performance requirement item Connection density. To fulfil this requirement 3GPP must show that it’s candidate Radio Interface Technologies (RITs) submitted to ITU-R supports a connection density of 1 000 000 devices per km2. The evaluation is to be performed in accordance to test environment Urban Macro-mMTC as described in report ITU-R M.2412 [4].

In this contribution, we present results on Connection Density for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, and for NB-IoT. The LTE BL/CE mode operation is hereafter referred to as LTE-M operation. It is shown that LTE-M and NB-IoT meets the IMT-2020 requirement and should be part of the 3GPP submission to ITU-R. It is proposed to capture these results in the self-evaluation SI TR 37.919 [7].

The contribution also introduces the connection density requirement definition, the herein followed non-full buffer system simulation procedure and some of the most relevant simulation assumptions defining the test environment Urban Macro-mMTC.

This contribution is a revision of R1-1809780 including:

- A correction of the grade of service requirement

- An assessment of the resources used for NB-IoT NPSS, NSSS, NPBCH, SIB1-NB, and SIB2-NB transmissions

- An assessment of the resources used for LTE-M PSS, SSS, PBCH, SIB1-BR, SIB2, and PUCCH transmissions

# Introduction to the Connection density requirement

## Requirement definition

The connection density requirement requires a RIT to provide a certain Quality of Service (QoS) to 1 000 000 devices per km2 at a grade of service (GoS) of 1 percent. Service is considered provided when a message latency of less than 10 seconds is supported for a user attempting to send an uplink data packet of 32 bytes defined at layer 2. Besides the supported connection density, it is encouraged to report the connection efficiency which is defined as the connection density normalized by the required system bandwidth.

The connection density requirement requires 1% grade of service where acceptable quality of service is defined by a message latency of 10 seconds or less.

## System simulation procedure

Report IMT-2020.EVAL [3] outlines two system simulator procedures for evaluating connection density. The first is a non-full buffer system level simulation that requires a state of the art system simulator to perform the evaluations. In this paper, we have followed this setup. The second approach is for a full buffer system simulation that allows input based on a more rudimentary system simulator combined with post processing supported by link level simulations. Both approaches have their merits and we provide input for the full buffer methodology in [6].

Table 1 Non-full buffer system level simulation procedure [3].

## Test environment

Report IMT-2020.EVAL specifies the test environment to be used in the evaluations according to Table 2. The simulations presented in this contribution where to a large extent following this set of assumptions. Further detailed assumptions are outlined in section 3.

Table 2: Urban Macro-mMTC test environment definition [3].

| Parameters | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss Note: Applies only to Channel model B. | 20% high loss, 80% low loss Note: Applies only to Channel model B. |
| Inter-site distance | 500 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | Up to 64 Tx/Rx |
| Number of UE antenna elements | Up to 2 Tx Up to 2 Rx | Up to 2 Tx Up to 2 Rx |
| Device deployment | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Only 1 message/2 hours/device studied herein. |
| Simulation bandwidth | Up to 10 MHz | Up to 50 MHz |
| UE antenna height | 1.5 m | 1.5 m |

# Simulation configuration

## Simulation assumptions

Table 3 presents the herein investigated LTE-M and NB-IoT system level configurations, that overrides or comes in addition to those presented in Table 2.

Table 3: Simulation configuration.

| Parameter | LTE-M | NB-IoT |
| --- | --- | --- |
| Mode of operation | Inband | Inband |
| DL carrier configuration | NB not carrying SSS, PSS, PBCH or PDSCH containing SIB-BR transmission. | Non-anchor carrier not carrying NSSS, NPSS, NPBCH or NPDSCH containing SIB1-NB transmission. |
| UL carrier configuration | PRACH configured on 1 subframe in each radio frame corresponding to 10% overhead. | NPRACH configured on 7% of all UL resources. |
| (N)PRACH configuration | CE 0: 1 rep, 10 ms periodicity CE 1: 4 rep, 40 ms periodicity, -122 dBm CE threshold CE 2: 16 rep, 160 ms periodicity, -130 dBm CE threshold CE 3: 64 rep, 640 ms periodicity, -138 dBm CE threshold | CE 0: 2 rep, 160 ms periodicity CE 1: 8 rep, 640 ms periodicity, -126 dBm CE threshold CE 2: 32 rep, 2560 ms periodicity, -136 dBm CE threshold |
| Simulated system bandwidth | 1 NB (6 PRB) | 1 PRB |
| eNB total transmit power per TRxP | 29 dBm/PRB | 29 dBm/PRB Note: Power boosting not applied. |
| eNB number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] |
| eNB number of TXRU per TRxP | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] | 2 TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] |
| UE transmit power | 23 dBm | 23 dBm |
| UE number of TXRU | 1 TXRU [5] | 1 TXRU [5] |
| UE number of antenna elements | 1Tx/Rx 0° polarization [5] | 1Tx/Rx 0° polarization [5] |
| UE antenna element gain | 0 dBi [5] | 0 dBi [5] |
| UE antenna element pattern | Omni-directional [5] | Omni-directional [5] |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. | Conf A: 93° in GCS Conf B: 99° in GCS Note: Selected to optimize UL/DL SINR. |
| Handover margin (dB) | 2 dB (handover modelled explicitly) | 2 dB (handover modelled explicitly) |
| TRxP boresight | 30 / 150 / 270 degrees [5] | 30 / 150 / 270 degrees [5] |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] |
| Wrapping around method | Radio distance-based wrapping | Radio distance-based wrapping |
| Minimum distance of TRxP and UE | 10 m [5] | 10 m [5] |
| Polarized antenna model | Model-2 in TR36.873 [5] | Model-2 in TR36.873 [5] |
| System layout | 7 sites, 3 sectors per site | 7 sites, 3 sectors per site |
| TRxP number per site | 3 [5] | 3 [5] |
| Scheduling strategy | Round robin | Round robin |
| (N)PDCCH, (N)PDSCH, (N)PUSCH (F1) Link to system (L2S) model | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. | Based on Mutual information Effective SINR mapping. A doubling in the number of repetitions is modelled with a 3 dB Effective SINR gain. |
| PUCCH, NPUSCH F2 | PUCCH Format 1a is transmitted at the edges of the LTE system BW, i.e. outside of the NB. The PUCCH BLEP is therefore based on an average SNR to BLEP mapping derived from TDL-iii link level simulations. (See Annex.) | NPUSCH F2 is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. (See Annex.) |
| (N)PRACH L2S model | PRACH BLEP is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. (See Annex.) | NPRACH BLEP is based on an average SINR to BLEP mapping derived from TDL-iii link level simulations. Access is failed in case of intra-cell collision. (See Annex.) |
| (N)PBCH L2S model | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations. (See Annex.) | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) |
| (N)PSS/(N)SSS L2S model | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) | Average SINR to acquisition delay mapping derived from TDL-iii link level simulations (See Annex.) |
| Packet drop timer | 20 sec | 20 sec |
| RRC Connection setup procedure | CIoT UP Optimization (RRC Resume) | CIoT UP Optimization (RRC Resume) |
| RLC Mode | AM | AM |

The RCC Resume procedure modelled is aligned with the message flow presented in Figure 1.

Figure 1: Data and signalling flow used to model RRC Resume latency performance.

The message sizes used in the RRC Resume procedure are summarised in Table 4.

Table 4: RRC resume message sizes.

| Message | LTE-M | NB-IoT |
| --- | --- | --- |
| Msg2: RAR | 7 bytes | 7 bytes |
| Msg3: RRC Connection Resume Request | 7 bytes | 11 bytes |
| Msg4: RRC Connection Resume | 19 bytes | 19 bytes |
| Msg5: RRC Connection Resume Comp + RLC Ack for Msg4 | 17 bytes | 16 bytes |
| RRC Connection Release | 18 bytes | 17 bytes |

## Channel model

The coupling gain for the different configurations and channel models was recorded by the simulator and are shown in Figure 2 below. These include antenna gains, Urban Macro A and Urban Macro B SCM channel models and outdoor to indoor losses. The large cell size of Configuration B leads to a higher coupling loss than what is seen for Configuration A. Channel model UMA A contains aggressive assumptions for the indoor to outdoor loss which is also visible in the below figure.

Figure 2: Coupling gain distribution for test environment Urban Macro mMTC based on the settings of Table 1.

# Connection density evaluation

The achieved latencies for LTE-M and NB-IoT are presented in Figure 3. The results are reported per cell and per resource block (RB). To translate the x-axis to arrival intensity per km2 the Configuration A results should be scaled by a factor 1/0.072 and the Configuration B results by a factor 1/0.866. To get the LTE-M results per narrowband a scaling factor of 6 should be applied.

LTE-M offers a superior latency performance until the system reaches the point of congestion. This thanks to its support of higher order modulation and transmission bandwidths up to 6 PRBs. NB-IoT offers a higher connection density partly thanks to its robust performance under low SINR. These observations are supported by Figure 4 in the Annex: It is shown that LTE-M has a higher DL resource utilization than NB-IoT which is believed to be partly due to the use of higher bandwidth to facilitate low latency and high DL coverage. Figure 5 indicate that the higher resource utilization leads to higher DL interference levels, which eventually limits the performance of the systems. Figure 6 shows the UL interference statistics for the devices in worst radio conditions. For Configuration B a significant offset between LTE-M and NB-IoT UL interference levels are seen. This is explained by the minimal scheduling unit, which for NB-IoT is one subcarrier while for LTE-M it is 1 PRB. The LTE-M Release 15 feature for sub-PRB allocation is not supported in these simulations.

In absolute numbers the highest connection density is supported for Configuration A. This since the cell area is 12 times smaller for the Configuration A 500 m ISD, compared to the 1732 m ISD used in Configuration B. Configuration A supports a densification of the cell grid which can cope with higher access loads.

It should be noted that the presented latencies in Figure 3 includes the idle mode time to synchronize to the cell and read the master information block.

Figure 3: LTE-M and NB-IoT service latency at the 99th percentile.

In Table 5 the connection density is reported based on the results presented in Figure 3. Given the assumed cell size of 0.07 km2 in Configuration A and 0.86 km2 in Configuration B and the traffic model of 1 message/2 hours/device the maximum supported arrival intensities in Figure 3 can be translated into connection densities defined in terms of numbers of connections per km2.

Table 5 also presents the needed bandwidth to support the connection density target. In addition to the simulated LTE-M NBs, 2 PRBs are added to Table 5 to account for the PUCCH Format 1a transmissions configured at the LTE system bandwidth edges. Resources from LTE-M PSS, SSS, PBCH, SIB1-BR and SIB2 transmissions are not included as those are generally transmitted in 6-PRB regions in other parts of the LTE system bandwidth than the narrowbands used for user data transmission. The resource usage due to PSS, SSS, PBCH, SIB1-BR and SIB2 transmissions can be estimated to correspond to a total of 42% of the available downlink resources in a single narrowband according to the following example:

- PSS is transmitted in 2 OFDM symbols (OS) every radio frame: 1.4% of the resources

- SSS is transmitted in 2 OS every radio frame: 1.4% of the resources

- PBCH is transmitted with 5 repetitions using a total of 20 OS every radio frame: 14.2% of the resources

- SIB1-BR is mapped on 1 subframe, repeated 16 times, over 80 ms: 20% of the resources

- SIB2 is transmitted mapped on 1 subframe, every radio frame in a SI window of 160 ms, configured with a periodicity of 320 ms: 5% of the resources

For NB-IoT resources for NPSS, NSSS, NPBCH, SIB1-NB and SIB2-NB transmissions is not accounted for in Table 5, and can be estimated to correspond to a total of 32% of the available downlink resources in a single PRB according to the following example:

- NPSS is transmitted in 1 subframe every radio frame: 10% of the resources

- NSSS is transmitted in 1 subframe every second radio frame: 5% of the resources

- NPBCH is transmitted in 1 subframe every radio frame: 10% of the resources

- SIB1-NB is transmitted mapped on 8 subframes, repeated 16 times, and in total spread over 2.56 s: 5% of the resources

- SIB2-NB is transmitted mapped on 8 subframes, every 8th radio frame in a SI window of 1.6 m, configured with a periodicity of 10.24 s: 1.6% of the resources

It should be noted that scheduled transmissions of e.g. other system information messages and paging would in a real system add additional overhead, that is not taken into consideration here.

Table 5 LTE-M and NB-IoT performance metrics.

|  | Connection density @ 99 percent GoS [devices/NB or PRB] | Bandwidth to support 1 000 000 devices per km2 [NB or PRB] | Connection efficiency [devices/Hz] | Spectral efficiency [bits/s/Hz] |
| --- | --- | --- | --- | --- |
| LTE-M: UMA A, Conf A | 5.680.683 devices/NB | 1 NB + 2 PRBs | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA B, Conf A | 5.680.683 devices/NB | 1 NB + 2 PRBs | 5.26 devices/Hz | 0.19 bits/s/Hz |
| LTE-M: UMA A, Conf B | 342.218 devices /NB | 3 NBs + 2 PRBs | 0.32 devices/Hz | 0.01 bits/s/Hz |
| LTE-M: UMA B, Conf B | 444.983 devices /NB | 3 NBs + 2 PRBs | 0.41 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA A, Conf A | 1.233.109 devices /1 PRB | 1 PRBs | 6.85 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA B, Conf A | 1.225.128 devices/1 PRB | 1 PRBs | 6.81 devices/Hz | 0.24 bits/s/Hz |
| NB-IoT: UMA A, Conf B | 67.928 devices/1 PRB | 15 PRBs | 0.38 devices/Hz | 0.01 bits/s/Hz |
| NB-IoT: UMA B, Conf B | 94.035 devices/1 PRB | 11 PRBs | 0.52 devices/Hz | 0.02 bits/s/Hz |

# Text proposal to TR 37.910 v1.0.0

To capture the updates presented in this paper we below propose a set of updates to Chapter 7 in TR 37.910 v1.0.0 [7].

-----------------------------------------------------------Text start: TR 37.910 ---------------------------------------------------

# 7	Self evaluation of mMTC technical performance

## 7.1	Connection density

As specified in Report ITU-R M.2410, connection density is the system capacity metric defined as the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2) with 1% grade of service (GoS).

In Report ITU-R M.2412, the required QoS is that a 32-byte packet is successfully received within 10 s.

The connection density can be evaluated using one or both of two alternative methods: The Full-buffer system-level simulation followed by link level simulation, and the Non-full-buffer system level simulation. These are defined in section 7.1.3 of Report ITU-R M.2412. The detailed assumptions for these approaches, including system level configurations and traffic model, are defined in Table 5 – d) in Report ITU-R M.2412.

### 7.1.1	NR

The connection density of NR is evaluated by using the full buffer system level simulation followed by link level simulation (referred to as “full buffer system level simulation” below).

In a first step this evaluation method employs a full buffer system level simulation to derive the uplink SINR distribution for a candidate technology. In a second step link level simulation are performed to determine the uplink spectral efficiency and data rate as functions of SINR. When combined these three functions supports the calculation of the expected long-term time-frequency resources required for each SINR to support the specified traffic model.

Connection density is in a final step conceptually derived by the system bandwidth, declared for the candidate technology, divided by the average required frequency resource. The requirement is fulfilled if the recorded connection density exceeds the 1.000.000 devices/km2, while the time resource, i.e. the packet delay, at the 99th percentile SINR is less than 10 s.

This evaluation method is targeted to evaluate the connection density in terms of the capability of uplink data transmission. The capacity calculation is based on an  assumption of ideal resource allocation among the multiple packets and users (e.g., there is no collision on resource allocation). The packet delay calculation does not consider the delays introduced by the connection access procedure.

Under this evaluation method, NR FDD is evaluated. The Urban Macro – mMTC test environment is used for evaluation. Both evaluation configuration A (ISD=500 m) and evaluation configuration B (ISD=1732 m) are considered.

The evaluation results of NR FDD are shown in Table 7.1.1-1 expressed as the average performance presented by the contributing companies. It is observed that NR fulfills connection density requirement under full buffer system level simulation followed by link level simulation. Detailed simulation assumptions and results can be found in Annex B.4.5.

Table 7.1.1-1 Evaluation results of connection density for NR FDD
(Full buffer system level simulation followed by link level simulation 
packet arrival rate: 1 packet / 2 hour / device)

(a) Evaluation configuration A (ISD=500 m)

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| 1x2 SIMO OFDMA | 15 kHz | 1,000,000 | 3 | 35,569,150 | 180 | 3 | 35,082,937 | 180 |

(b) Evaluation configuration B (ISD=1 732 m)

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| 1x2 SIMO OFDMA | 15 kHz | 1,000,000 | 4 | 1,267,406 | 180 | 3 | 1,529,707 | 180 |

### 7.1.2	LTE

The connection density of NB-IoT and eMTC are evaluated using both the full buffer, and the non-full buffer system level simulation as defined in Report ITU-R M.2412.

As indicated in Section 7.1.1, the full buffer system level simulation method targets to evaluate the connection density in terms of the capability of uplink data transmission. It does also not model synchronization and system information acquisition, control channel and downlink data channel performance. It assumes ideal resource allocation among the multiple uplink packets and users (e.g., there is no collision on resource allocation), and the delays introduced by access procedure are not considered.

In the non-full buffer system level simulation, the idle mode synchronization and system information acquisition performance and delays are taken into account. The chosen system access procedure, the uplink data transmission and the connection release procedures are modelled. The link level performance of all relevant physical channels and signals are modelled, and the system level resource allocation and the delays associated with the modeled protocols are considered. The connection density is recorded at the user arrival rate at which 99% of all uplink packet is successfully received within 10 s.

Since the non-full buffer system level simulation provides a more detailed level of modelling the evaluation results are, as expected, different from the full buffer system level simulation.

For both evaluation methods, the Urban Macro – mMTC test environment is used. Both evaluation configuration A (ISD=500 m) and evaluation configuration B (ISD=1732 m) are considered.

The evaluation results of NB-IoT and eMTC are shown in Table 7.1.2-1 for full buffer system level simulation, and in Table 7.1.2-2 for non-full buffer system level simulation, respectively. For non-full buffer system level simulation, both the Early Data Transmission (EDT) and the RRC Resume procedures are evaluated. Since EDT procedure is shortened compared to RRC Resume procedure, the evaluation results are different.

In addition to the required bandwidths presented in Table 7.1.2-1 and Table 7.1.2-2, eMTC requires 2 PRBs to be configured at the edges of the configured LTE system bandwidth for supporting PUCCH transmissions. Overhead from (N)PSS, (N)SSS, (N)PBCH or (N)PDSCH system information broadcast is also not accounted for in the reported bandwidths. For NB-IoT a reasonable assumption is that the overhead from NPSS, NSSS, NPBCH and SIB1-NB transmissions constitutes ~30% of one downlink PRB. For LTE-M the corresponding overhead is approximately ~40% of one downlink narrowband.

It is observed that both NB-IoT and eMTC fulfill connection density requirement under either evaluation method, using either system access procedures.

Detailed simulation assumptions and results can be found in Annex B.4.5.

Table 7.1.2-1 Evaluation results of connection density for NB-IoT and eMTC
(Full buffer system level simulation followed by link level simulation
packet arrival rate: 1 packet / 2 hour / device)

(a) Evaluation configuration A (ISD=500 m)

| Technical feature | Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| NB-IoT | 1x2 SIMO | 15 kHz | 1,000,000 | 2 | 43,691,789 | 180 | 2 | 43,626,653 | 180 |
| eMTC | 1x2 SIMO | 15 kHz | 1,000,000 | 3 | 35,235,516 | 180 | 3 | 34,884,438 | 180 |

(b) Evaluation configuration B (ISD=1 732 m)

| Technical feature | Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| NB-IoT | 1x2 SIMO | 15 kHz | 1,000,000 | 3 | 2,335,319 | 180 | 2 | 2,376,936 | 180 |
| eMTC | 1x2 SIMO | 15 kHz | 1,000,000 | 3 | 1,212,909 | 180 | 3 | 1,511,989 | 180 |

Table 7.1.2-2 Evaluation results of connection density for NB-IoT and eMTC
(Non-full buffer system level simulation 
packet arrival rate: 1 packet / 2 hour / device)

(a) Evaluation configuration A (ISD=500 m)

| Technical feature | Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| NB-IoT (EDT) | 1x2 SIMO, Single-tone | 15 kHz | 1,000,000 | 1 | 8,047,087 | 180 | 1 | 8,077,017 | 180 |
| NB-IoT (EDT) | 1x2 SIMO, Multi-tone | 15 kHz | 1,000,000 | - | - | - | 1 | 16,000,000 | 180 |
| NB-IoT (RRC Resume) | 1x2 SIMO Multi-tone | 15 kHz | 1,000,000 | 1 | 1,233,000 | 180 | 1 | 1,225,000 | 180 |
| eMTC (RRC Resume) | 1x2 SIMO | 15 kHz | 1,000,000 | 1 | 1,893,000 | 360 | 1 | 1,893,000 | 360 |

(b) Evaluation configuration B (ISD=1 732m)

| Technical feature | Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement (device/km2) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) | Number of samples | Connection density (device/km2) | Required bandwidth (kHz) |
| NB-IoT (EDT) | 1x2 SIMO, Single-tone | 15 kHz | 1,000,000 | 1 | 1,198,000 | 360 | 1 | 1,203,880 | 360 |
| NB-IoT (EDT) | 1x2 SIMO Multi-tone | 15 kHz | 1,000,000 | - | - | - | 1 | 1,250,000 | 360 |
| NB-IoT (RRC Resume) | 1x2 SIMO Multi-tone | 15 kHz | 1,000,000 | 1 | 1,018,000 | 2700 | 1 | 1,034,000 | 1980 |
| eMTC (EDT) | 1x2 SIMO | 15 kHz | 1,000,000 | 1 | 1,107,000 | 540 | 1 | 1,140,000 | 540 |
| eMTC (RRC Resume) | 1x2 SIMO | 15 kHz | 1,000,000 | 1 | 1,026,000 | 3240 | 1 | 1,038,000 | 2520 |

-----------------------------------------------------------Text end: TR 37.910 ---------------------------------------------------

# Conclusions

In this contribution, we have discussed the IMT-2020 connection density requirement and provide input to the 3GPP Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that LTE-M and NB-IoT fulfils the IMT-2020 connection density requirement and should be part of the 3GPP IMT-2020 submission. A text proposal to TR 37.919 v1.0.0 is made to capture the updates made in this contribution compared to previous versions.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-172098, “3GPP submission towards IMT-2020”, ITU-R Ad-Hoc Contact Person

Report ITU-R M.2411-0 Requirements, evaluation criteria and submission templates for development of IMT-2020, ITU-R

Report ITU-R M.2412-0 Guidelines for evaluation of radio interface technologies for IMT-2020, ITU-R

RP-172728, LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”, RAN#78, Huawei

R1-1809262, “IMT-2020 self evaluation: mMTC full buffer connection density”, RAN1#93, Ericsson

TR 37.919 v1.0.0 Study on Self Evaluation towards IMT-2020 Submission (Rel-15)

# Annex: Simulation results

Figure 4 Average DL resource utilization.

Figure 5 1st percentile DL SINR.

Figure 6 1st percentile UL SINR.

Figure 7 PBCH and NPBCH acquisition delay link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective channel.

Figure 8 PSS/SSS and NPSS/NSSS acquisition delay link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.

Figure 9 PRACH and NPRACH misdetection link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.

Figure 10 PUCCH and NPUSCH F2 misdetection link to system model based on TDL-iii, 2 Hz. The SNR is defined at the bandwidth of the respective signal.