3GPP TSG-RAN WG1 Meeting # 96	R1-1903121

Athens, Greece, 25 Feb – 1 Mar 2019

Agenda Item:	7.2.7.2

Source:	Ericsson

Title:	IMT-2020 self-evaluation: CP latency

Document for:	Information

# 1	Introduction

The ITU target for control plane (CP) latency in IMT 2020 has been set to 20ms [1], and to 10ms in 3GPP [2]. In this paper we make an evaluation of the CP latency in NR with different configurations, and show that the target can be reached in both FDD and TDD. In this paper we focus more on evaluations towards ITU whereas [3] discusses what needs to be done before NR WI is completed.

This contribution was also submitted to RAN2 as R2-1901305.

# 2	CP latency

A text proposal on CP latency evaluation for NR was endorsed by RAN1 in [3] and then a TP has been proposed and agreed during the last RAN#81 plenary in [4]. However, the overall CP latency can be still enhanced when considering a value of UE and gNB RRC processing time for the NR RRC resume procedure equal to 3ms.

## 2.1	Definition

ITU has defined the CP latency and the requirement on it to be [1]:

“Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios.

The minimum requirement for control plane latency is 20 ms. Proponents are encouraged to consider lower control plane latency, e.g. 10 ms.”

A most battery efficient state should here be interpreted as Idle or Inactive state, which both have same energy consumption.

The start of continuous data transfer should here be interpreted as the point in time where the UE is ready to be scheduled, i.e. when it enters the Active state.

## Control plane signalling

According to the definition of CP latency we here study the transition from the RRC Inactive state to the RRC Active state, as shown in Figure 1.

Figure 1. Illustration of CP signaling during transition from Inactive to Active state.

## Processing delay

In this paper we assume the minimum timing capabilities are agreed for NR. With the UE capability 2 the minimum UL timing is set to be 3symbols for 15 and 30kHz SCS. For 120kHz we make the assumption of 9symbols timing.

With mini-slots the TTIs can have different length, and we have therefore counted the processing in terms of the shortest considered TTI, which is 4symbols in this paper. For simplicity the processing delay is therefore set to be 1TTI for 15 and 30kHz SCS and 3TTI at 120kHz SCS, in both gNB and UE.

The RRC processing delays are assumed to be of a fixed value of 3ms, as discussed in [5].

## Achievable latency in FDD

For the evaluation of latency, it has here been assumed that the UE works with n+2 timing and the eNB works with n+3 timing as the fastest option, i.e. that the processing budget is 1 and 2TTI, respectively. This is for 15 and 30kHz SCS. For 120kHz the processing delay is doubled in TTIs, giving n+3 timing for the UE and n+5 timing for gNB.

With the assumptions described above the resulting CP latency will be as outlined in Table 1. As can be seen the total worst-case delay sums up in the range 9-14TTI+6ms for FDD.

Table 1. CP latency in TTIs in NR Rel-15 FDD.

| Component | Description | Latency |  |
| --- | --- | --- | --- |
|  |  | 15/30kHz | 120kHz |
| 1 | Worst-case delay due to RACH scheduling period (1TTI period) | 1TTI | 1TTI |
| 2 | Transmission of RACH Preamble | 1TTI | 1TTI |
| 3 | Preamble detection and processing in gNB | 1TTI | 3TTI |
| 4 | Transmission of RA response | 1TTI | 1TTI |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Request) | 1TTI | 2TTI |
| 6 | Transmission of RRC Connection Resume Request | 1TTI | 1TTI |
| 7 | Processing delay in gNB (L2 and RRC) | 3ms | 3ms |
| 8 | Transmission of RRC Connection Resume (and UL grant) | 1TTI | 1TTI |
| 9 | Processing delay in the UE (L2 and RRC) | 3ms | 3ms |
| 10 | Transmission of RRC Connection Resume Complete (including NAS Service Request) | 1TTI | 1TTI |
| 11 | Processing delay in gNB (Uu –> S1-C) | 1TTI | 3TTI |
|  | Total delay | 9TTI+6ms | 14TTI+6ms |

- The worst-case CP latency in NR Rel-15 FDD is estimated to 9TTI+6ms at 15/30kHz SCS and 14TTI+6ms at 120kHz SCS.

### Achievable FDD latency in ms

With different TTI lengths, and SCS the absolute delay will differ, as shown in Table 2. Here we have assumed the values calculated above in Table 1.

Table 2. Achievable CP latency for NR Rel-15 in ms.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 15 | 10.5 | 7.8 |
| 7-symbol TTI | 10.5 | 8.3 | 6.9 |
| 4-symbol TTI | 8.6 | 7.3 | 6.5 |

As can be seen in the table, all considered configurations fulfil the 20ms 5G target on CP latency, and almost all configurations also reach the 10ms target.

- NR Rel-15 FDD can reach the 3GPP and ITU 5G targets on CP latency.

## Achievable latency in TDD

For TDD slot sequence, we here study two cases: an alternating UL-DL sequence, and a DL-heavy UL-DL-DL-DL sequence. Due to the slot sequence additional alignment delays are added.

With the assumptions described above the resulting CP latency will be as outlined in Table 3. As can be seen the total worst-case delay sums up in the range 12-26TTI+6ms for TDD.

Table 3. CP latency in TTIs in NR Rel-15 TDD.

| Component | Description | UL-DL Latency |  | UL-DL-DL-DL Latency |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15/30kHz | 120kHz | 15/30kHz | 120kHz |
| 1 | Worst-case delay due to RACH scheduling period (1TTI period) | 2TTI | 2TTI | 4TTI | 4TTI |
| 2 | Transmission of RACH Preamble | 1TTI | 1TTI | 1TTI | 1TTI |
| 3 | Preamble detection and processing in gNB | 1TTI | 3TTI | 1TTI | 3TTI |
| 4 | DL slot alignment | 1TTI | 1TTI | 0TTI | 1TTI |
| 5 | Transmission of RA response | 1TTI | 1TTI | 1TTI | 1TTI |
| 6 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Request) | 1TTI | 3TTI | 1TTI | 3TTI |
| 7 | UL slot alignment | 1TTI | 1TTI | 0TTI | 3TTI |
| 8 | Transmission of RRC Connection Resume Request | 1TTI | 1TTI | 1TTI | 1TTI |
| 9 | Processing delay in gNB (L2 and RRC) | 3ms | 3ms | 3ms | 3ms |
| 10 | DL slot alignment | 1TTI | 1TTI | 0TTI | 1TTI |
| 11 | Transmission of RRC Connection Resume (and UL grant) | 1TTI | 1TTI | 1TTI | 1TTI |
| 12 | Processing delay in the UE (L2 and RRC) | 3ms | 3ms | 3ms | 3ms |
| 13 | UL slot alignment | 1TTI | 1TTI | 0TTI | 3TTI |
| 14 | Transmission of RRC Connection Resume Complete (including NAS Service Request) | 1TTI | 1TTI | 1TTI | 1TTI |
| 15 | Processing delay in gNB (Uu –> S1-C) | 1TTI | 3TTI | 1TTI | 3TTI |
|  | Total delay | 14TTI+6ms | 20TTI+6ms | 12TTI+6ms | 26TTI+6ms |

- The worst-case CP latency in NR Rel-15 TDD with alternating UL-DL pattern is estimated to 14TTI+6ms for 15/30kHz SCS and 20TTI+6ms for 120kHz SCS.

### Achievable TDD latency in ms

With different TTI lengths and SCS the absolute delay will differ, as shown in Table 2. For the alternating UL-DL TDD pattern we get the latency as indicated in Table 4, and for the UL-DL-DL-DL TDD pattern the resulting latency is shown in Table 5.

Table 4. Achievable CP latency for NR Rel-15 in ms for TDD with alternating UL-DL pattern.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 20 | 13 | 8.5 |
| 7-symbol TTI | 13 | 9.5 | 7.3 |
| 4-symbol TTI | 10 | 8.0 | 6.7 |

Table 5. Achievable CP latency for NR Rel-15 in ms for TDD with UL-DL-DL-DL pattern.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 18 | 12 | 9.3 |
| 7-symbol TTI | 12 | 9.0 | 7.6 |
| 4-symbol TTI | 9.4 | 7.7 | 6.9 |

As can be seen in the table, all considered configurations fulfil the 20ms 5G target on CP latency for the alternating UL-DL TDD pattern, and several configurations can also fulfill the 10ms requirement.

- NR Rel-15 TDD can reach the ITU and 3GPP 5G targets on CP latency.

The observations above can be used as a starting point for discussion on CP latency in the self-evaluation study item.

# 3	Conclusion

In section 2 we made the following observations:

- The worst-case CP latency in NR Rel-15 FDD is estimated to 9TTI+6ms at 15/30kHz SCS and 14TTI+6ms at 120kHz SCS.

- NR Rel-15 FDD can reach the 3GPP and ITU 5G targets on CP latency.

- The worst-case CP latency in NR Rel-15 TDD with alternating UL-DL pattern is estimated to 14TTI+6ms for 15/30kHz SCS and 20TTI+6ms for 120kHz SCS.

- NR Rel-15 TDD can reach the ITU and 3GPP 5G targets on CP latency.

The observations above can be used as a starting point for discussion on CP latency in the self-evaluation study item.

# References

ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

R1-1810013, Text proposal to TR37.910 for NR control plane latency, Huawei, RAN1#94, August 2018.

RP-181807, Text proposal to TR37.910 for NR control plane latency, Huawei, Ericsson, Telecom Italia, RAN#81, September 2018.

3GPP TR 38.913, Study on scenarios and requirements for next generation access technologies, 3GPP, 14.3.0, August 2017.

R2-1901308, RRC UE processing time for RRCResume, Ericsson, RAN2 #105, February 2019.