3GPP TSG-RAN WG2 Meeting #96	 R1-1903149

Athens, Greece, 25th February - 1st March 2019

Agenda item:	7.2.2.1.1

Source:	Convida Wireless

Title:	On Additional RACH Opportunities for NR-U

Document for:	Discussion

# Introduction

During the NR-U SI, various options to alleviate the impacts of LBT failures on the random access procedure were discussed.  The following text related to supporting additional transmission opportunities for the RACH messages was included in TR 38.889 [1]:

“However, in order to alleviate the impact of LBT failures further, additional opportunities for the RACH messages may be introduced, e.g. in time or frequency domain, for both 4-step and 2-step RACH.”

In this contribution, we discuss our views related to the introduction of additional opportunities for the RACH messages and make some observations and proposals.

# Discussion

For Msg1 transmission, additional transmission opportunities in the time domain are beneficial.  However, the channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped closely together in time.  Increasing the time between the transmission opportunities helps to reduce the correlation, but also increases latency.

Observation 1:  The channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped closely together in time.

For wideband carriers, the channel may be partitioned into 20 MHz sub-bands, where LBT is performed on each sub-band independently [1].  Configuring additional RACH resources across multiple LBT sub-bands may provide additional robustness without increasing latency.  Therefore, for NR-U, it should be evaluated to configure additional RACH resources across LBT sub-bands at least at RRC CONNECTED mode.

Observation 2:  Configuring additional RACH resources across multiple LBT sub-bands may provide additional robustness without increasing latency.

Proposal 1:  For NR-U, it should be evaluated to configure additional RACH resources across LBT sub-bands at least for RRC CONNECTED mode.

In R1-1901040 [2], an observation was made that frequency domain enhancements would not have much benefit for initial access.  This is because for initial access, Msg1 is transmitted on the initial active UL BWP, which is likely to be ≤ 20 MHz.  Therefore, when discussing the configuration of additional RACH resources across LBT sub-bands, RAN1 should also discuss whether this will be applicable for UEs in RRC IDLE/RRC INACTIVE mode or only for UEs in RRC CONNECTED mode.

Proposal 2:  When discussing the configuration of additional RACH resources across LBT sub-bands, RAN1 should also discuss whether this will be applicable for UEs in RRC IDLE/RRC INACTIVE mode or only for UEs in RRC CONNECTED mode.

# Conclusion

In this contribution, we discuss our views related to the introduction of additional opportunities for the RACH messages and make the following observations and proposals:

Observation 1:  The channel access probabilities for transmission opportunities that are multiplexed in time may be highly correlated—especially if they are grouped closely together in time.

Observation 2:  Configuring additional RACH resources across multiple LBT sub-bands may provide additional robustness without increasing latency.

Proposal 1:  For NR-U, it should be evaluated to configure additional RACH resources across LBT sub-bands at least for RRC CONNECTED mode.

Proposal 2:  When discussing the configuration of additional RACH resources across LBT sub-bands, RAN1 should also discuss whether this will be applicable for UEs in RRC IDLE/RRC INACTIVE mode or only for UEs in RRC CONNECTED mode.

# References

- 3GPP TR 38.889, Study on NR-based access to unlicensed spectrum, V16.0.0

- R1-1901040 Discussion on time/freq-domain enhancements for RACH resources, Samsung