3GPP TSG-RAN WG1 Meeting #96			R1-1903155

Athens, Greece, 25th February – 1st March 2019

Agenda item:	7.2.4.1.3

Title:	Design Considerations for NR V2X Synchronization

Source:	Convida Wireless

Document for:	Discussion

# 1. Introduction

The following agreements were made on NR sidelink synchronization in the RAN1 AH 1901 meeting [1]:

- For NR SLSS, as the baseline:

  - The sequence type for S-PSS is the same type as the M-sequence used for NR-PSS

  - The sequence type for S-SSS is the same type as the Gold sequence for NR-SSS

- The frequency location for S-SSB is (pre-) configured

  - Note: it implies that there is no intended hypothesis detection in frequency location of S-SSB performed by the UE for a carrier in a given band

  - Note: the potential frequency locations for the (pre-)configured frequency location may be restricted, up to RAN4

- Combination 1:

  - Time domain: 2 symbols of length-127 S-PSS, 2 symbols of length-127 S-SSS

  - Frequency domain:11 or 12 RBs

  - BW containing S-SSB:

    - 2.5 MHz for 15 kHz SCS

    - 5 MHz for 30 kHz SCS

    - 10 MHz for 60 kHz SCS

    - 20 MHz for 120 kHz SCS

- Combination 2:

  - Time domain: 2 symbols of length-127 S-PSS, 2 symbol of length-127 S-SSS

  - Frequency domain:  20 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Combination 3:

  - Time domain: 1 symbol of length-127 S-PSS, 1 symbol of length-127 S-SSS

  - Frequency domain:  20 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Combination 4:

  - Time domain: 1 symbol of length-255 S-PSS, 1 symbol of length-255 S-SSS

  - Frequency domain:  24 RBs

  - BW containing S-SSB:

    - 5 MHz for 15 kHz SCS

    - 10 MHz for 30 kHz SCS

    - 20 MHz for 60 kHz SCS

    - 40 MHz for 120 kHz SCS

- Other combinations are not precluded.

- Note:  Company should specify the assumptions, such as total energy per SSB, when the performance results are compared between different combinations.

Based on these agreements, we discuss design considerations for NR sidelink synchronization.

# 2. Discussion

2.1 NR sidelink S-PSS/S-SSS sequence design consideration

Like NR Uu PSS/SSS, NR V2X SL S-PSS/S-SSS may carry SL-SSID for indication of synchronization sources [2]. In addition, NR SL S-PSS/S-SSS may need to consider minimizing the cross-correlation between SL S-PSS/S-SSSS and NR Uu PSS/SSS to avoid the timing estimation ambiguity when SL S-PSS/S-SSS and NR SSS/PSS are TDMed on the shared carrier on Uu interface. Therefore, the sequence design for S-PSS/S-SSS needs to be studied.

In this contribution, we addressed the design SL S-PSS/S-SSS sequences based on M-sequence which is similar to the Uu NR PSS/SSS sequence design specified in [3].

The base sequence dS-PSS(n) with length-127 is defined by:

dS-PSSn=1-2sn, n=0, …, 126,

where sn is an m-sequence with polynomial f(x) = x7 + x4 + 1 and the initial value specified in [3]. To evaluate the cross-correlation property between S-PSS and PSS, the Mean Square Aperiodic Cross-Correlation (MSACC) with the NR Uu PSS sequences may be used for assessment [5], [6].

The MSACC can be expressed as follow

RMSACCj=1M1N2i=1Mτ=1-NN-1ri,jτ2,

where M=3 denotes the total number of NR Uu PSS sequences, j denotes the j-th cyclic shift value for the dS-PSS sequence, j∈ {0, 1, 2, …, N-1} and ri,jτ denotes the aperiodic cross-correlation value [5], [6], between i-th NR Uu PSS sequence and j-th cyclic-shift of dS-PSS sequence.

To evaluate the auto-correlation property of SL S-PSS, the mean square auto-correlation (MSAAC) can be used for assessment [6].  The MSAAC can be expressed as follow

RMSAACj=1N2τ=1-NN-1rj,jτ2,

where rj,jτ denotes the aperiodic auto-correlation value for j-th cyclic-shift of dS-PSS sequence. Selecting m-sequence sets with good cross-correlation properties may result in the degradation of auto-correlation properties [6]. Therefore, it is desired to select cyclic-shift m-sequences which have lower MSACC and MSAAC values.

From Figure 1 (a) and (b), the simulation results show that the cyclic-shift values {20, 63, 106} have lower aperiodic cross-correlation with the NR Uu PSS and lower aperiodic auto-correlation values.

Figure 1 (a) Mean square aperiodic cross-correlation (MSACC) with the NR Uu PSS sequences (b) Mean square aperiodic auto-correlation (MAAAC) for each cyclic-shift

Proposal 1: M-sequence with cyclic-shift is used for NR S-PSS sequence design.

Proposal 2: NR SL S-PSS sequences carrying NSL-SSID(2) with low cross-correlation with NR Uu PSS sequences may be studied.

Similarly, SL S-SSS sequence generation may be like Uu SSS sequences specified in [3]. The sequence dS-SSSn is defined by:

dS-SSSn=1-2s0n+m0 ⨁ s1n+m1, n=0, …, 126,

where ⨁ denotes modulo-2 addition (or XOR) operation, s0n is an m-sequence generating from primitive polynomial f0(x) = x7 + x6 +1, and  s1n is an m-sequence generating from polynomial f1(x) = x7 + x3 +1. Any non-zero vector can be used for the initial value for s0n and s1(n). Similar to m0 and m1 defined in NR Uu SSS specified in [3], where m0 =zqNSL-SSID(1)112+ NSL-SSID(2) , m1=NSL-SSID(1)mod 112, NSL-SSID(1) ∊ {0, 1, …, M},  q ≥3, z is an positive integer and M denotes number of  NSL-SSID(1). The number of SL-SSID can be written as follow:

NSL-SSID= 3NSL-SSID(1)+ NSL-SSID(2)

Proposal 3: NR SL S-SSS sequences carrying NSL-SSID(1) and NSL-SSID(2) with the sequences similar to NR Uu SSS sequences may be studied.

2.2 Resource Mapping for S-PSS/S-SSS

For S-PSS length 127 and SL SSB bandwidth 20 RB, the S-PSS may be mapped to the first symbol or first two symbols of an SL SSB in time, as shown in Figure 1. Similar to LTE V2X, if two symbols of S-PSS in an SL SSB are supported then the same sequence dS-PSS(n) may be repeated in each of the S-PSS symbols.

Similarly, the S-SSS may also be mapped to the last or last two symbols within an SL SSB, as shown in Figure 1. If mapped to two symbols, the same dS-SSS(n) may be repeated in each of the S-SSS symbols.

Figure 1  (a) One-symbol S-PSS/S-SSS (b) Two-symbol S-PSS/S-SSS with SL SSB bandwidth of 20RBs

For S-PSS and S-SSS with sequence length of 127, the corresponding sequence dS-PSS(n) and dS-SSS(n) mapped to 12 RBs in frequency are depicted in Figure 2.

Figure 2 S-PSS and S-SSS sequence mapping in frequency with SSB Bandwidth of 20 RBs

Proposal 4: NR S-PSS and S-SSS sequences mapped to 12 RBs in frequency with one or two symbols in an SL SSB with bandwidth of 20 RB may be studied.

2.3 Resources for SL SS/PBCH

For shared carrier scenario, SL SS/PBCH resources may be allocated in a sub-band which may be fully, partially, or non-overlapped with Uu SS/PBCH in time and frequency.

As shown in Figure 4, for paired spectrum, SL SS/PBCH can be allocated to a UL frequency band thus UEs under network coverage may not receive SL SS/PBCH. However, this allocation method may sacrifice the utilization of UL resources allocation.  In addition, for unpaired spectrum, DL and UL are at a same frequency band, and the allocation of SL SS/PBCH resources may be overlapped with Uu SS/PBCH in frequency domain.  To avoid the possible overlapping, SL SS/PBCH and Uu SS/PBCH may be TDMed [4] as shown in Figure 5. However, in this case, SL S-PSS/S-SSS design has to consider how to reduce the correlation between Uu SS/PBCH and SL SS/PBCH to avoid the timing estimation ambiguity.

Proposal 5: NR should study SL SS/PBCH resource allocation methods especially when sharing the carrier with the Uu interface.

Figure 4 SL SS/PBCH and Uu SS/PBCH are FDMed

Figure 5 SL SS/PBCH and Uu SS/PBCH are TDMed

# 3. Conclusion

In this contribution, we discussed the NR V2X sidelink synchronization design considerations, and we propose the follows.

Proposal 1: M-sequence with cyclic-shift is used for NR S-PSS sequence design.

Proposal 2: NR SL S-PSS sequences carrying NSL-SSID(2) with low cross-correlation with NR Uu PSS sequences may be studied.

Proposal 3: NR SL S-SSS sequences carrying NSL-SSID(1) and NSL-SSID(2) with the sequences similar to NR Uu SSS sequences may be studied .

Proposal 4: NR S-PSS and S-SSS sequences mapped to 12 RBs in frequency with one or two symbols in an SL SSB with bandwidth of 20 RB may be studied.

Proposal 5: NR should study SL SS/PBCH resource allocation methods especially when sharing the carrier with the Uu interface.

# 4. References

- RAN1 Chairman Notes, TSG RAN WG1 RAN1 AdHoc 1901, Taipei, Taiwan, January 2019.

- R1-1808695, “Sidelink synchronization for NR V2X communication,” Intel Corporation, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

- TS 38.211, V15.2.0, NR; Physical channels and modulation, 2018-August-06.

- R1-1808814, “Discussion on NR V2X sidelink synchronization,” ETRI, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden, Aug 20th – 24th, 2018.

- M.B. Pursley and D.V. Sarwate, “Bounds on aperiodic cross-correlation for binary sequences,” IEEE Electronics Letters vol. 12, Jun.1976.

- I. Oppermann and B. S. Vucetic, “Complex spreading sequences with a wide range of correlation properties,” IEEE Trans. Commun., vol. COM-45, pp. 365-375, March 1997.