3GPP TSG RAN WG1 Meeting #96	R1-1903191

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	7.2.7.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for mMTC connection density

Document for:	Discussion and Decision

Introduction

In this contribution, we provide the evaluation results of connection density for both non-full buffer system level simulation and full buffer system-level simulation and link-level simulation, using both evaluation configurations (ISD=1732m and ISD=500m) for Urban Macro - mMTC test environment.

Connection density definition

According to Report ITU-R M.2412, connection density is said to be C (# of devices per km2), if, under the number of devices, N=C×A (A is the simulation area in terms of km2), that the packet outage rate is less than or equal to 1%, where the packet outage rate is defined as the ratio of

- The number of packets that failed to be delivered to the destination receiver within a transmission delay of less than or equal to 10s

to

- The total number of packets generated by the (N=C×A) devices within the time T.

The transmission delay of a packet is understood to be the delay from the time when uplink packet arrives at the device to the time when the packet is correctly received at the destination (BS) receiver.

In addition, it is encouraged that the self evaluation reports the connection efficiency which is given by

(# of device/Hz/TRxP)							(1)

where C is the connection density (# of devices per km2), A is the simulation area in terms of km2, M is the number of TRxP in the simulation area A, and W is the UL bandwidth (for FDD).

In Report ITU-R M.2412, There are two possible evaluation methods to evaluate connection density requirement defined in ITU-R M.2410-0:

-non-full buffer system-level simulation;

-full-buffer system-level simulation followed by link-level simulation.

In this contribution, the evaluation results of the two evaluation methods are provided.

Evaluation results

Evaluation results of non-full buffer system level simulation

NB-IoT and eMTC using the early data transmission procedure are evaluated using non-full buffer system level simulation as defined in Report ITU-R M.2412, following the model as descripted in Annex A and Annex B. It is assumed that NB-IoT is working in standalone mode, eMTC is working on one narrow band which is 1.08MHz. The detailed simulation assumption is shown in Annex C. The evaluation results are given in Table 1 and Table 2, respectively.

Table 1 Evaluation results for NB-IoT and eMTC for ISD=1732m 
using non-full buffer system level simulation

|  |  | Config B (ISD = 1732m), channel mode A | Config B (ISD = 1732m), channel mode B |
| --- | --- | --- | --- |
| NB-IoT | Devices supported per km2 per 180kHz | 599,000 | 601,940 |
|  | Required bandwidth to support 1,000,000 devices | 360kHz | 360kHz |
|  | Connection efficiency (#of devices/Hz/TRxP) | 2.88 | 2.896 |
| eMTC | Devices supported per km2 per 180kHz | 369,000 | 380,000 |
|  | Required bandwidth to support 1,000,000 devices | 540kHz | 540kHz |
|  | Connection efficiency (#of devices/Hz/TRxP) | 1.777 | 1.828 |

Table 2 Evaluation results for NB-IoT for ISD=500m
using non-full buffer system level simulation

|  |  | Config A (ISD = 500m), channel mode A | Config A (ISD = 500m), channel mode B |
| --- | --- | --- | --- |
| NB-IoT | Devices supported per km2 per 180kHz | 8,047,087 | 8,077,017 |
|  | Required bandwidth to support 1,000,000 devices | 180kHz | 180kHz |
|  | Connection efficiency (#of devices/Hz/TRxP) | 3.226 | 3.238 |
| eMTC | Devices supported per km2 per 180kHz | 5,349,310 | 5,348,472 |
|  | Required bandwidth to support 1,000,000 devices | 1.08MHz | 1.08MHz |
|  | Connection efficiency (#of devices/Hz/TRxP) | 2.145 | 2.144 |

Observation1: NB-IoT and eMTC can fulfil IMT-2020 requirement under non-full buffer system level simulation, and NB-IoT demonstrates higher connection efficiency.

Evaluation results of full buffer system-level simulation followed by link-level simulation

The connection density of NB-IoT, eMTC and NR are evaluated using full buffer system level simulation with link level simulation as defined in Report ITU-R M.2412. The evaluation results are provided in Table 3. The UL SINR distribution of full buffer simulation is shown in Annex E and the link level spectrum efficiency of NB-IoT, eMTC and NR is shown in Annex F. The 99% latency derived by SINR could fulfill the 10s latency requirement.

Table 3 Evaluation result of full buffer system-level followed by link-level simulation

|  |  | Config A (ISD=500m) Channel mode A | Config B (ISD=500m) Channel mode B | Config B (ISD=1732m) Channel mode A | Config B (ISD=1732m) Channel mode B |
| --- | --- | --- | --- | --- | --- |
| NB-IoT | Devices supported per km2 per 180kHz | 43,271,000 | 43,846,000 | 2,567,000 | 2,702,000 |
|  | Connection efficiency (#of devices/Hz/TRxP) | 17.348 | 17.579 | 12.35 | 13.0 |
| eMTC | Devices supported per km2 per 180kHz | 39,995,000 | 38,502,000 | 1,344,000 | 1,699,000 |
|  | Connection efficiency (#of devices/Hz/TRxP) | 16.035 | 15.436 | 6.466 | 8.174 |
| NR | Devices supported per km2 per 180kHz | 36,574,000 | 35,021,000 | 1,138,000 | 1,465,000 |
|  | Connection efficiency (#of devices/Hz/TRxP) | 14.663 | 14.041 | 5.475 | 7.048 |

The evaluation result of full buffer system-level simulation followed by link-level simulation is quite higher than non-full buffer system-level simulation since it has an ideal assumption of resource scheduling and the delays due to access procedure is not taken into account. In addition, the DL resource allocation is not considered in this evaluation method, while in practice DL resource allocation may be the bottleneck of the access procedure, which will introduce large delay, and result in packet drop. In this sense, the evaluation method of full buffer system-level simulation with link level simulation demonstrates a best case result for the candidate technology.

It is observed that NB-IoT has the advantage of higher UL SINR due to higher power spectral density, which is the result of its finer frequency granularity on data allocation. It in turns results in higher spectrum efficiency from system view. In summary, the evaluation results show that NB-IoT, eMTC and NR could fulfill the IMT-2020 requirement.

Observation2: NB-IoT, eMTC and NR can fulfil IMT-2020 requirement under full buffer system-level simulation followed by link-level simulation, and NB-IoT demonstrates higher connection efficiency.

Conclusions

In this contribution, the evaluation result on self evaluation for mMTC is provided. The evaluation demonstrates that NB-IoT and eMTC could fulfill the mMTC connection density requirements for Urban Macro – mMTC test environment under both evaluation methods. NR could fulfill the requirement under evaluation method of full buffer system level simulation with link level simulation.

Observation1: NB-IoT and eMTC can fulfil IMT-2020 requirement under non-full buffer system level simulation, and NB-IoT demonstrates higher connection efficiency.

Observation2: NB-IoT, eMTC and NR can fulfil IMT-2020 requirement under full buffer system-level simulation followed by link-level simulation, and NB-IoT demonstrates higher connection efficiency.

- References

- R1- 1719421, “Consideration on self evaluation of mMTC for IMT-2020”, Huawei, HiSilicon, November 2017.

- Appendix A. 	Evaluation model for non-full buffer system level simulation for NB-IoT

- A.1	Procedure and delay modeling

To evaluate NB-IoT and eMTC, the procedure needs to be assumed for a packet transmission. Considering the packet arrival rate of a device is very sparse (1 message/day/device to 1 message/2 hours/device), it is appropriate to assume that the devices (both for NB-IoT and eMTC) are within idle mode when an uplink message packet arrives.

In [1], it is shown that legacy procedure and small data transmission procedure are available for both NB-IoT and eMTC. It is noted that ITU-R assumes a L2 packet size of 32 byte (i.e., 256 bit) for mMTC evaluation, which is typically within the range of small data packet. Therefore the small data transmission procedure is considered in this contribution for delay modeling. Besides, it is assumed that the devices are in non-initial state, that is, the SIB information is assumed to have been received by the devices. In this case, the SIB reception is ignored. This procedure is shown in Table 1.

Based on the understanding of transmission delay, Step 1 to Step 4 is considered to be contributing to the total transmission delay. Besides, the following Step 5 and Step 6 are considered to be contributing to the DL or UL resource occupation, but do not contribute to the delay since the timer will stop when Step 4 finished.

Table 4 Early data transmission procedure of NB-IoT and eMTC under consideration

| NB-IoT |  |  |  | eMTC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Device |  | BS |  | Device |  | BS |
|  | Step1: Sync + MIB |  |  |  | Step1: Sync + MIB |  |
|  | Step 2: PRACH Msg1 |  |  |  | Step2: PRACH Msg1 |  |
|  | Step 3: NPDCCH + RAR (including UL grant) |  |  |  | Step3: MPDCCH + RAR (including UL grant) |  |
|  | Step 4: UL data transmission |  |  |  | Step4: UL data transmission |  |
|  | Step 5: RRCEarlyDataComplete |  |  |  | Step5: RRCEarlyDataComplete |  |
|  | Step 6: HARQ Ack |  |  |  | Step 6: HARQ Ack |  |

- A.2	Evaluation method of full system level simulation

Generally, the system level simulation should evaluate each packet’s total delay tpacket. If tpacket > 10s, this packet is regarded as failed to be delivered to the destination receiver.

The total delay consists of the delays from Step 1 to 4,

where tUL_data is the UL data transmission time duration (for step 4), and t1, t2, and t3 are the time delay for step 1~3, respectively.

Conventionally, the value of tUL_data can be well derived in system level simulation.

For derivation of t1, t2, and t3, a full implementation of these steps may result in high complexity simulation. Therefore a simplified model is needed for Step 1 to 3. In the following sub-sections, the considered delay models are presented.

- A.3	Delay Modeling of Step 1: Sync + MIB

The step 1 delay is given by

where tSS is the delay for synchronization, and tMIB is the delay of MIB reception.

- A.1.1	Synchronization delay

For NB-IoT, NPSS and NSSS are transmitted for the device to conduct synchronization. NPSS is transmitted in sub-frame 5 with 132 REs of each frame. NSSS is transmitted in sub-frame 9 with 132 REs of every other frameas illustrated in Figure 1.

Figure 1 NPSS and NSSS for NB-IoT

Denote LNPSS and LNSSS as the repetition times needed to successfully accomplish primary synchronization and secondary synchronization, respectively. In this case, the synchronization delay is given by

tss=tPSS+tSSS

tPSS=tNPSS_0+LNPSS-1× TPSS

tSSS=tNSSS_0+LNSSS-1× TSSS

where tNPSS_0 = tNPSS-t0 is the time interval between the nearest NPSS transmission at tNPSS, and the packet arrival time, t0, TPSS =10ms is the transmission period of NPSS; tNSSS_0 = tNSSS-t0 is the time interval between the nearest NSSS transmission at tNSSS, and the packet arrival time, t0,  and TSSS=20ms is the transmission period for NSSS.

The value of LNPSS and LNSSS can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding value of LNPSS/ LNSSS for a given SINR can be determined according to the 90th percentile point successful detection of NPSS/NSSS (under this SINR value) as shown in the following mapping table.

- A.1.2	PBCH receiving delay

NPBCH (for MIB) is transmitted in sub-frame 0 in every radio frame with 100 REs on anchor-PRB with at most 64 sub-frame combination as illustrated in Figure 2

Figure 2 NPBCH for NB-IoT

Denote LNPBCH as the repetition times for correctly receiving NPBCH. Hence, the receiving PBCH delay is given by

tnpbch=tNPBCH_0+LNPBCH-1× TNPBCH

where tNPBCH_0 = tNPBCH-t0 is the time interval between the nearest NPBCH transmission at tNPBCH, and the synchronization end time, t0, TNPBCH =10ms is the transmission period of NPBCH.

The value of LNPBCH can be determined by DL SINR, based on link level simulation using TDL-iii channel model (with QPSK, coding rate of 0.25, and code block size of 50bit). The corresponding SNR threshold for a given LNPBCH would guarantee larger than 90% successful reception ratio with LNPBCH times repetition of NPBCH reception.

- A.4	Delay Modeling of Step 2: PRACH Msg1

For PRACH delay model, it is dependent on two aspects. One is the number of collisions encountered by the device, ncollision. The other aspect is the time duration, tPRACH, for correctly receiving PRACH without collision. The latter is depending on UL SINR. Therefore the PRACH delay is given by

t2=f2(ncollision, tPRACH)

If collision happens during the PRACH transmission, all of the collided PRACH transmissions are assumed to be failed, and another round of PRACH transmission for the collided UEs is needed.

For NB-IoT, the UEs will transmit PRACH according to its CE level. The CE level is determined by its RSRP (see TS36.331). For UEs in a specific CE level, the time domain resource for PRACH could be configured according to TS36.331 by the transmission periodicity, transmission duration (or repetition times), and the transmission start time (within the period). If the three CE levels share the same PRACH frequency resource, the UEs with lower CE level could not use the PRACH resource that is overlapped with higher CE level. One illustration is shown in Figure 3.

Device starts the PRACH transmission at available transmission time according to its CE level as shown in colored box in Figure 3, and randomly selects one PRACH channel among the available number of channels (assumed to be 24 in this simulation).

Figure 3 Illustration of PRACH configuration for three CE levels 
(PRACH shares the same frequency) resource)

The NPRACH of each CE level has its own parameter nprach-Periodicity-r14 and nprach-StartTime-r14 to decide the NPRACH candidate opportunity (see Section 6.7.3 in TS36.331). And the corresponding SNR threshold is given by link level simulation using TDL-iii channel, with the guarantee of 90% successful reception ratio of NPRACH. The example configuration of NPRACH resource is shown as in Table 5.

Table 5 Example NPRACH configuration for each CE level

| Coverage Enhancement level | Repetition times of NPRACH transmission | Transmission Duration tPRACH_duration | Periodicity | NPRACH start time |
| --- | --- | --- | --- | --- |
| CE Level 0 | 2 | 11.2ms | 40ms | 8ms |
| CE Level 1 | 8 | 44.8ms | 80ms | 32ms |
| CE Level 2 | 32 | 179.2ms | 1280ms | 8ms |

For a given UE, it is assumed that its CE level is determined by DL RSRP in the simulation. Meanwhile, a full buffer UL SINR is calculated for this UE to determine whether its PRACH could be detected by BS. If its UL SINR is lower than the corresponding CE level SNR threshold, it is assumed that BS could not detect its PRACH signal, and the UE could not receive any response from BS, then this UE would continue to send PRACH signal until maxNumPreambleAttemptCE times (See TS 36.331), after that UE should switch its CE level downward, and then starts another round of PRACH transmission.

If only one device occupies the channel at that time, it is assumed that BS could receive the PRACH correctly. If multiple UEs within the same CE level start the PRACH transmission at the same time (say t0), and all of them occupies the same sub-carrier (channel) at t0, these UEs are collided, and their PRACH reception at BS side would be failed.

When collision occurs, a backoff mechanism is used to avoid further collision to next transmission (see Section 5.1.5 in TS36.321). The backoff length (until the next PRACH transmission) consists of two parts: one is the backoff window with length of tbackoff, which is a random value between {0, Twindow}ms; the other part is the RAR window with the minimum length of 2×TPDCCH, where TPDCCH is the transmission period of NPDCCH in Step 3. In this case, the total latency for PRACH is given by

tPRACH=i=1ncollision+1tPRACH_0_i+tPRACH_duration_i+tbackoff_i+2×TPDCCH

where tPRACH_0_i is from the time when the device is ready to send PRACH to the time of the nearest PRACH transmission opportunity for this device, tPRACH_i is the transmission duration of the i-th PRACH transmission of the device (depending on UL SINR; tPRACH_duration_i = LPRACH×TPRACH, where LPRACH is the NPRACH repetition time required by UL SINR, and TPRACH is the interval of preamble format 0), tbackoff_i is the window length of the i-th back off which is randomly selected in (0, Twindow)ms, while it is 512ms in the simulation, where the value of Twindow could be configured as in Table 7.2-2 in TS 36.321, and TPDCCH is the period of common search space (CSS), it is configured as 24ms, 48ms and 96ms for each CE level respectively, and ncollision≥0 is number of collisions encountered by the device that is provided by the system level simulation.

Table 6 Backoff Parameter values for NB-IoT

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 256 |
| 2 | 512 |
| 3 | 1024 |
| 4 | 2048 |
| 5 | 4096 |
| 6 | 8192 |
| 7 | 16384 |
| 8 | 32768 |
| 9 | 65536 |
| 10 | 131072 |
| 11 | 262144 |
| 12 | 524288 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

- A.5	Delay Modeling of Step 3: NPDCCH + RAR (including UL grant)

For NB-IoT, the Step 3 transmission (downlink) consists of NPDCCH transmission and RAR transmission through PDSCH,

t3=tNPDCCH+tRAR

where tNPDCCH is the delay for correctly receiving NPDCCH, and tRAR is the delay for correctly receiving RAR.

- A.5.1	Scheduling scheme of NPDCCH and RAR

The scheduling of NPDCCH and RAR (transmitted on NPDSCH) are based on system level simulation. For a given time instance, NPDCCH is assumed to have higher scheduling priority over RAR transmission. However, once RAR transmission starts, it is assumed that this RAR transmission has the highest scheduling priority at that time instance.

- A.5.2	NPDCCH delay

The NPDCCH transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tNPDCCH=tNPDCCH_0+LNPDCCH-1× TTI

where tNPDCCH_0 = tNPDCCH_sche - t0 is the time interval between the time when the available NPDCCH resource exist for the specific device at tNPDCCH_sche, and the PRACH end time, t0, the available PDCCH means a candidate PDCCH resource which is not scheduled for other UEs, LNPDCCH is the repetition times for correctly receiving NPDCCH, and TTI=1ms. The value of tNPDCCH_sche is related to the scheduling scheme of NPDCCH and RAR.

If multiple devices which belong to the same CE level request PDCCH transmission at a specific time instance, they may share the NPDCCH resource. In this case, the value of LNPDCCH for device k will be aligned with the device that requests the largest value of LNPDCCH.For a given CE level, RMAX and G are configured to determine the period of common searching space of this CE level, where TPDCCH = G* RMAX as illustrated in Figure 4. UE should monitor each PDCCH candidate within a set of repetition { RMAX /8 , RMAX /4  , RMAX /2  , RMAX }. So the LNDCCH should be selected in this repetition set.

Figure 4 Example of Common Searching Space

The exact value of LNDCCH can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding SNR threshold for a given LNDCCH would guarantee larger than 90% successful reception ratio with LNDCCH times repetition of NPDCCH reception. It is noted that NPDCCH is transmitted using QPSK, coding rate of 0.128, and code block size of 39bit for the case of using 12 sub-carriers.

- A.5.3	RAR delay

The RAR transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tRAR=tNPDSCH_0+LNPDSCH-1×TRAR

where tNPDSCH_0 = tNPDSCH_sche - t0 is the time interval between the available NPDSCH transmission for the specific device at tNPDCCH_sche, and the NPDCCH end time, t0, the available NPDSCH means an unused DL resource after time t0, LNPDSCH is the repetition times for correctly receiving NPDSCH, and TRAR is the transmission duration for one RAR packet. The value of tNPDSCH_sche is related to the scheduling scheme of NPDCCH and RAR.

For the case of BS scheduling RAR for multiple UEs simultaneously, the RAR packet size would be nS where S=56bits is the size of a single RAR, and n denotes the number of scheduled devices . The value of LNPDSCH and the MCS for device k will be aligned with the device which requests the largest value of LNPDSCH, and it could be derived from simulation according to RAR packet size and DL SINR, while DL SINR could be derived from DL wideband SINR.

For NB-IoT, one RAR transmission duration is derived by

tRAR=nSSE(MCS,OH)×180kHz×1000

where n is the number of scheduled devices for RAR transmission, S=56bit is the size of RAR for one device, SE is the expected spectral efficiency (bps/Hz) that is related to MCS and overhead OH. MCS is selected based on DL PDSCH SINR, which can be derived from DL wideband SINR. When multiple devices are scheduled (n>1), the MCS is selected based on the device that experiences the worst SINR.

- A.6	Delay Modeling of Step 4: UL data

The UL data transmission is fully modeled in the system level simulation as in conventional system level simulation. UL data contains two parts, RRC connection request message (88bits) and UL traffic packet (256bits), so the total packet size is 344bits.To facilitate the system level simulation, only single-tone is used for scheduling, while UL data transmission is based on MAC scheduling according to UL resource utilization condition and UL SINR of devices. The transmission delay of step 4 could be depicted as the following.

tUL_data=i=1N+1tSCHED_i+tPUSCH_duration_i

where tSCHED_i is the scheduling delay for the i-th transmission, tPUSCH_duration_i is the transmission duration for i-th PUSCH transmission for the specific device. N is the retransmission times for UL data transmission. These values are derived by system level simulation.

- A.7	DL and UL resource occupation model for Step 5 and Step 6

Since Step 5 RRC early data complete message and Step 6 HARQ Ack are after the UL data reception, the transmission delay should not be taken into consideration, but both steps would occupy a few DL and UL resource which may impact the simulation result. So the resource occupation of Step 5 and Step 6 are modeled in the system level simulation.

- A.7.1	DL resource occupation for Step 5: RRC Early Data Complete

This model is similar to the MSG2 transmission model (Step 3). It contains two parts, PDCCH occupation and PDSCH occupation. BS should assign PDCCH and PDSCH resource to this step according to the DL SINR to the device. Since the PDCCH of this message is masked with temporary C-RNTI, so the PDCCH would be transmitted in common searching space as depicted in Section 4.5, and the LNPDCCH should be derived based on DL SINR same as MSG2. Besides, the LNPDSCH should be derived based on DL SINR and the message size. Then the DL resource occupied by transmission of RRCEarlyDataCompele can be determined, and it will reduce the DL resource for transmission of Step 3.

- A.7.2	UL resource occupation for Step 6: HARQ Ack

NB-IoT uses PUSCH format 2 to transmit HARQ Ack, and each resource unit is 2ms length for 15kHz sub-carrier and it has only 1 bit data. It is assumed that BS determines the UL resource occupation for a specific device according to its UL SINR, and the HARQ Ack is assumed to be error free. The UL resource occupied by this step will reduce the UL resource for Step 4.

- Appendix B. 	Evaluation model for non-full buffer system level simulation for eMTC

- B.1	Procedure and delay modeling

The procedure is as descripted in Section A.1

- B.2	Evaluation method of full system level simulation

The overall method is as descripted in Section A.2

- B.3	Delay Modeling of Step 1: Sync + MIB

The step 1 delay is given by

where tSS is the delay for synchronization, and tMIB is the delay of MIB reception.

- B.3.1	Synchronization delay

For eMTC, PSS and SSS are transmitted for the device to conduct synchronization, both are using 62REs  in sub-frame 0 and  sub-frame 5 as illustrated in Figure 5.

Figure 5 PSS and SSS for eMTC

Denote LPSS and LSSS as the repetition times needed to successfully accomplish primary synchronization and secondary synchronization, respectively. In this case, the synchronization delay is given by

tss=tPSS+tSSS

tPSS=tPSS_0+LPSS-1× TPSS

tSSS=tSSS_0+LSSS-1× TSSS

where tPSS_0 = tPSS-t0 is the time interval between the nearest PSS transmission at tPSS, and the packet arrival time, t0, TPSS =5ms is the transmission period of NPSS; tSSS_0 = tSSS-t0 is the time interval between the nearest SSS transmission at tSSS, and the packet arrival time, t0,  and TSSS=5ms is the transmission period for SSS.

The value of LPSS and LSSS can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding value of LPSS/ LSSS for a given SINR can be determined according to the 90th percentile point successful detection of PSS/SSS (under this SINR value). The DL SINR for PSS and SSS of a device could be derived from DL wideband SINR (geometry) of that device.

- B.3.2	PBCH receiving delay

PBCH (for MIB) is transmitted in sub-frame 0 and sub-frame 9 in every radio frame with at most 4 frame combination as illustrated in Figure 6. Meanwhile, PBCH for eMTC has 5 repetition transmission (R0~R4) within sub-frame 9 and sub-frame 0 totally, as illustrated in Figure 7.

Figure 6 PBCH for eMTC

Figure 7 PBCH repetition mapping for FDD

Denote LPBCH as the repetition times for correctly receiving PBCH, each repetition time contains sub-frame 9 and sub-frame 0. Hence, the receiving PBCH delay is given by

tpbch=tPBCH_0+LPBCH-1× TPBCH

where tPBCH_0 = tPBCH-t0 is the time interval between the nearest PBCH transmission at tPBCH, and the synchronization end time, t0, TPBCH =10ms is the transmission period of PBCH.

The value of LPBCH can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding SNR threshold for a given LPBCH would guarantee larger than 90% successful reception ratio with LPBCH times repetition of PBCH reception. The DL SINR could be derived from DL geometry of that device.

- B.4	Delay Modeling of Step 2: PRACH Msg1

For PRACH delay model, it is dependent on two aspects. One is the number of collisions encountered by the device, ncollision. The other aspect is the time duration, tPRACH, for correctly receiving PRACH without collision. The latter is depending on UL SINR. Therefore the PRACH delay is given by

t2=f2(ncollision, tPRACH)

If collision happens during the PRACH transmission, all of the collided PRACH transmissions are assumed to be failed, and another round of PRACH transmission for the collided UEs is needed.

ForeMTC, the UEs will transmit PRACH according to its CE level. The CE level is determined by its RSRP (see TS36.331). For UEs in a specific CE level, the time domain resource for PRACH could be configured according to TS36.211 1 by PRACH configuration index, and the number of repetitions required for preamble transmission.  If the four CE levels share the same PRACH resource, the CE levels are differentiated by different preamble indices.

For eMTC, the frequency resource for PRACH could be configured as 6PRBs on 1.08MHz bandwidth.

Device starts the PRACH transmission at available sub-frame according to PRACH configuration index, and randomly selects one PRACH preamble index according to its CE level..

The PRACH of each CE level has its own parameter numRepetitionPerPreambleAttempt. And the corresponding SNR threshold is given by link level simulation using TDL-iii channel, with the guarantee of 90% successful reception ratio of PRACH. The example configuration of PRACH resource is shown as in Table 5.

Table 7 Example PRACH configuration for each CE level

| Coverage Enhancement level | Repetition times of PRACH transmission | Sub-frames for Transmission |
| --- | --- | --- |
| CE Level 0 | 1 | 1 |
| CE Level 1 | 4 | 4 |
| CE Level 2 | 8 | 8 |
| CE Level 3 | 32 | 32 |

For a given UE, it is assumed that its CE level is determined by DL RSRP in the simulation. Meanwhile, a full buffer UL SINR is calculated for this UE to determine whether its PRACH could be detected by BS. If its UL SINR is lower than the corresponding CE level SNR threshold, it is assumed that BS could not detect its PRACH signal, and the UE could not receive any response from BS, then this UE would continue to send PRACH signal until maxNumPreambleAttemptCE times (See TS 36.331), after that UE should switch its CE level downward, and then starts another round of PRACH again.

If only one device occupies the channel with a given preamble index at that time, it is assumed that BS could receive the PRACH correctly. If multiple UEs within the same CE level start the PRACH transmission at the same time (say t0), and all of them using the same preamble index at t0, these UEs are collided, and their PRACH reception at BS side would be failed.

When collision occurs, a backoff mechanism is used to avoid further collision to next transmission (see Section 5.1.5 in TS36.321). The backoff length (until the next PRACH transmission) consists of two parts: one is the backoff window with length of tbackoff, which is a random value between {0, Twindow}ms; the other part is the RA response window with the length of TRAwindow. In this case, the total latency for PRACH is given by

tPRACH=i=1ncollision+1tPRACH_0_i+tPRACH_duration_i+tbackoff_i+TRAwindow

where tPRACH_0_i is from the time when the device is ready to send PRACH to the time of the nearest PRACH transmission opportunity for this device, tPRACH_duration_i is the transmission duration of the i-th PRACH transmission of the device (depending on UL SINR, PRACH repetition and available PRACH sub-frame configuration), tbackoff_i is the window length of the i-th back off which is randomly selected in (0, Twindow)ms, where the value of Twindow could be configured as Table 8 Backoff Parameter values for eMTCTable 8(See Table 7.2-1 in TS 36.321), and TRAwindow could be configured as 400 sub-frames(See Section 6.3.2 in TS 36.331), and ncollision≥0 is number of collisions encountered by the device that is provided by the system level simulation.

Table 8 Backoff Parameter values for eMTC

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 20 |
| 3 | 30 |
| 4 | 40 |
| 5 | 60 |
| 6 | 80 |
| 7 | 120 |
| 8 | 160 |
| 9 | 240 |
| 10 | 320 |
| 11 | 480 |
| 12 | 960 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

- B.5	Delay Modeling of Step 3: MPDCCH + RAR (including UL grant)

For eMTC, the Step 3 transmission (downlink) consists of NMPDCCH transmission and RAR transmission,

t3=tMPDCCH+tRAR where tMPDCCH is the delay for correctly receiving MPDCCH, and tRAR is the delay for correctly receiving RAR. The modeling of MPDCCH and MPDSCH are based on system level scheduling, while MPDCCH has a higher scheduling priority for a given scheduling time. Currently, the simulation based way is used in the evaluation.

- B.5.1	MPDCCH Delay

For eMTC, device needs to monitor every MPDCCH candidate with maximum 24 aggregation level within configured set of (at most 4) repetition numbers from the start of the search space to the end of the search space. And the common search space is illustrated in Figure 8. For a given CE level, RMAX and G are configured to determine the period of common searching space of this CE level, Where TPDCCH = G* RMAX. UE should monitor each PDCCH candidate within a set of repetition { RMAX /8 , RMAX /4  , RMAX /2  , RMAX }. So the LNDCCH should be selected in this repetition set.

Figure 8 Example of Common Searching Space

The MPDCCH transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tMPDCCH=tMPDCCH_0+LMPDCCH-1× TTI

where tMPDCCH_0 = tMPDCCH_sche - t0 is the time interval between the available MPDCCH transmission for the specific device at tMPDCCH_sche, and the PRACH end time, t0, LMPDCCH is the repetition times for correctly receiving MPDCCH, and TTI=1ms.

If multiple devices which belong to the same CE level request MPDCCH transmission at a specific time instance, they may share the MPDCCH resource. In this case, the value of LMPDCCH for device k will be aligned with the device that requests the largest value of LMPDCCH.

The value of tMPDCCH_0 can be derived from simulation or an appropriate model. When multiple devices request MPDCCH transmission, there might be waiting time for a specific device, which is accounted in tNPDCCH_0.

The value of LMDCCH can be determined by DLSINR, based on link level simulation using TDL-iii channel model. The corresponding SNR threshold for a given LMDCCH would guarantee larger than 90% successful reception ratio with LMDCCH times repetition of NPDCCH reception. It is noted that MPDCCH is transmitted using QPSK, with 31 bits for ModeB and 45 bits for ModeA, including 16 bits CRC.

- B.5.2	RAR delay

The RAR transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tMPDSCH=tMPDSCH_0+LMPDSCH-1× TTI

where tMPDSCH_0 = tMPDSCH_sche - t0 is the time interval between the available MPDSCH transmission for the specific device at tMPDCCH_sche, and the MPDCCH end time, t0, the available MPDSCH means an unused DL resource after time t0, note that the MPDSCH must have at least 1 TTI interval with the end of MPDCCH, LMPDSCH is the repetition times for correctly receiving MPDSCH, and TTI=1ms.

For the case of BS scheduling RAR for multiple UEs simultaneously, the RAR packet size would be nS where S=56bits is the size of a single RAR, and n denotes the number of scheduled devices. The scheduled devices share the same MCS which is selected based on DL SINR. The value of LMPDSCH for device k will be aligned with the device which requests the largest value of LMPDSCH and it could be derived from simulation according to RAR packet size and DL SINR, while DL SINR could be derived from DL wideband SINR.

- B.6	Delay Modeling of Step 4: UL data

The UL data transmission is fully modeled in the system level simulation as in conventional system level simulation. UL data contains two parts, RRC connection request message (88bits) and UL traffic packet (256bits), so the total packet size is 344bits.To facilitate the system level simulation, the UL MCS is selected based on UL resource utilization and the repetition times is determined by MCS and UL SINR, which means BS would select MCS  for a given UE according to the available consecutive RB number and the packet size(344bits) to ensure that UE could send the total packet within one transmission to follow the early data transmission procedure. After that BS could determine the transmission repletion times based on the selected MCS and device’s UL SINR.. The transmission delay of step 4 could be depicted as the following.

tUL_data=i=1N+1tSCHED_i+tPUSCH_duration_i

where tSCHED_i is the scheduling delay for the i-th transmission, tPUSCH_duration_i is the transmission duration for i-th PUSCH transmission for the specific device. N is the retransmission times for UL data transmission. These values are derived by system level simulation.

- B.7	DL and UL resource occupation model for Step 5 and Step 6

Since Step 5 RRC early data complete message and Step 6 HARQ Ack are after the UL data reception, the transmission delay should not be taken into consideration, but both steps would occupy a few DL and UL resource which may impact the simulation result, so the resource occupation of Step 5 and Step 6 are modeled in the system level simulation.

- B.7.1	DL resource occupation for Step 5: RRC Early Data Complete

This model is similar to the MSG2 transmission model (Step 3), it contains two parts, PDCCH occupation and PDSCH occupation. BS should assign PDCCH and PDSCH resource according to its DL SINR of the device. Since the PDCCH of this message is masked with temporary C-RNTI, so the PDCCH would be transmitted in common searching space as depicted in Section 4.124.5, the LMPDCCH should be derived based on DL SINR and the message size. Then the DL resource occupied by transmission of RRCEarlyDataCompele can be determined, and it will reduce the DL resource for transmission of Step 3.

- B.7.2	UL resource occupation for Step 6: HARQ Ack

eMTC uses PUCCH to transmit HARQ Ack, it is assumed that PUCCH resource is not within 1.08MHz of eMTC, and the HARQ Ack is assumed to be error free. So the UL resource occupation of this step is not considered in the simulation.

- Appendix C. System level simulation assumption

- C.1	Simulation assumption for mMTC

Table 9 Simulation assumption

| Urban Macro - mMTC | Parameter |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| ISD | Config A: 500m Config B:1732m |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 43 dBm on 180kHz |
| Device power class | 23 dBm |
| Inter-site distance | 1732 m |
| Number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of device antenna elements | 1Tx/Rx 0° polarization |
| Number of TXRU per device | 1TXRU |
| Device deployment | 80% indoor, 20% outdoor Randomly and uniformly distributed over the area |
| Device mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| Device speeds of interest | 3 km/h for indoor and outdoor |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| Device noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| Device antenna element gain | 0 dBi |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | With layer 2 PDU (Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device Packet arrival follows Poisson arrival process |
| Device antenna height | 1.5 m |
| Channel model | Channel model A Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | Config A: 99° in LCS Config A: 93° in LCS |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and device | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 |

- C.2	Simulation assumption for NB-IoT

Table 10 Simulation assumption for NB-IoT

| Urban Macro – NB-IoT | Parameter |
| --- | --- |
| Simulation bandwidth | 180 kHz |
| Sub-carrier spacing for PDCCH, PDSCH | 15 kHz |
| Sub-carrier spacing for PUSCH | 15 kHz |
| PRACH | 90kHz with 24 sub-carriers (channels) in 180 kHz BW, 3.75kHz sub-carrier spacing for PRACH Back off window Twindow=512ms |
| UL DMRS | 2 symbols per 14 OFDM symbols |
| PUSCH scheduling unit | Single tone (15kHz) |
| NPDCCH period TPDCCH | CE level 0: 24ms, CE Level 1: 48ms CE level 2: 96ms |
| Power control | ISD 1732m: Alpha = 1, P0 = -115.8 dBm on 15kHz ISD 500m: Alpha = 1, P0 = -100 dBm on 15kHz |

- C.3	Simulation assumption for eMTC

Table 11 Simulation assumption for eMTC

| Urban Macro – eMTC | Parameter |
| --- | --- |
| Simulation bandwidth | 1.08MHz |
| Sub-carrier spacing for PDCCH, PDSCH | 15 kHz |
| Sub-carrier spacing for PUSCH | 15 kHz |
| PRACH | 2 sub-frame per frame |
| UL DMRS | 2 symbols per 14 OFDM symbols |
| PUSCH scheduling unit | 180KHz |
| Simulation bandwidth | 1.08MHz |
| Power control | ISD 1732m: Alpha = 1, P0 = -113 dBm on 180kHz ISD 500m: Alpha = 1, P0 = -103 dBm on 180kHz |

- C.4	Simulation assumption for NR

Table 12 Simulation assumption for NR

| Urban Macro – eMTC | Parameter |
| --- | --- |
| Simulation bandwidth | 5MHz |
| Sub-carrier spacing for PDCCH, PDSCH | 15 kHz |
| Sub-carrier spacing for PUSCH | 15 kHz |
| UL DMRS | 12 symbols per RB |
| PUSCH scheduling unit | 180kHz |
| Simulation bandwidth | 5 MHz |
| Power control | ISD 1732m: Alpha = 1, P0 = -113 dBm on 180kHz ISD 500m: Alpha = 1, P0 = -103 dBm on 180kHz |

- Appendix D. Link level simulation assumption

Table 13 Link-level simulation assumption

|  | NB-IoT Parameter | eMTC Parameter | NR Parameter |
| --- | --- | --- | --- |
| Simulation bandwidth | 15kHz | 180kHz | 180kHz |
| Sub-carrier spacing | 15 kHz | 15 kHz | 15 kHz |
| Modulation order | π/2 BPSK /π/4 QPSK | QPSK/16QAM | QPSK/16QAM |
| Number of Resource unit | 2,3,4,5,6,8,10 | - | - |
| Number of TTI | - | 1,4 | 1 |
| Number of repetition | 1,2,4,8,16 | 1,2,4,8,16,32 | - |
| Channel model | TDL-iii |  |  |
| Delay spread | 363ns |  |  |
| TBS | 256 | 16,24,32,40,56,72,104,120,136,144,176,208,224,256 | 40,48,64,80,96,144,152,168,184,208,240,256 |
| Channel estimation | Realistic |  |  |

- Appendix E. SINR distribution of full buffer system level simulation

Figure 9 SINR distribution of NB-IoT, eMTC and NR for config A/B with Channel model A/B

- Appendix F.  Spectrum efficiency from link-level simulation

Figure 10 Spectrum efficiency of NB-IoT, eMTC and NR