3GPP TSG RAN WG1 Meeting #96                                                                        	R1-1903202

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.14

Source:	Huawei, HiSilicon

Title:	Discussion on cell measurement for mobility management in NTN

Document for:	Discussion and decision

Introduction

In RAN plenary #80, a new SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. The objectives of the SI for Layer 2 and above, and RAN architecture are reported as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

To facilitate mobility management (e.g., handover), physical layer cell measurements are needed. In this contribution, we discuss necessary cell measurement for efficient mobility management in NTN.

Discussion

Cell measurement for mobility management in NR

One of the fundamental requirements for mobility management is to identify the cell boundary, such that necessary mobility handling operations can be implemented when UE is about to cross cell boundaries. For this purpose, NR utilizes signal strength based cell measurement, e.g., x-RSRP, x-RSRQ, x-RSSI and x-SINR, where x represents Synchronization Signal (SS) or CSI [2].

- x reference signal received power (x-RSRP) is defined as the linear average over the power

contributions (in [W]) of the resource elements that carry x.

- x reference signal received quality (x-RSRQ) is defined as the ratio of N×x-RSRP / NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks.

- x Received Signal Strength Indicator (x-RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over N number of resource blocks from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc.

- x signal-to-noise and interference ratio (x-SINR), is defined as the linear average over the

power contribution (in [W]) of the resource elements carrying x signals divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying x within the same frequency bandwidth.

Specifically, the signal strength based measurement event A3 triggering handover in NR is listed as [3]:

- Event A3 (Neighbour becomes offset better than SpCell)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

where Mn and Mp are the measurement results of the neighbouring cell and SpCell, respectively, not taking into account any offsets; Ofn and Ofp are the measurement object specific offsets of the reference signal of the neighbour cell and SpCell, respectively (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell or SpCell); Ocn and Ofp are the cell specific offsets of the neighbour cell and SpCell, respectively (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell or SpCell), and set to zero if not configured for the neighbour cell or SpCell; Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event); Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event); Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Observation 1: The cell measurement for NR mobility management (e.g., handover) is executed based on signal strength related parameters.

Cell measurement for mobility management in NTN

The signal strength based measurement is reasonable and efficient for NR since a clear near-far effect is expected in terrestrial networks. As depicted in Figure 1(a), a UE can approximately determine whether it is located in the center or edge of the serving cell by measuring the value of RSRP. However, regarding the NTN context shown in Figure 1(b), the near-far effect is not clearly exhibited within a cell.

Figure 1. A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

The unclear near-far effect in NTN is further verified by the path loss value shown in Table 1. For instance, in an NTN with a height of 600km and a cell range of 200km, the difference of free space loss between the cell center and cell edge is only around 0.12dB. Considering a 3dB EIRP difference within a cell due to the antenna pattern, the overall differential signal power within a cell is around 3.12dB. Besides, the typical measured intra-frequency/inter-frequency RSRP accuracy requirements for FR1 under normal condition is ±4.5dB [4]. Therefore, it is very hard to distinguish where a UE is located in an NTN cell by pure received signal power based measurement. As such, cell boundary determination can be restricted to some extent or even ineffective sometimes.

Table 1: Path loss in different network scenarios

| Scenarios |  | LEO based NTN |  | GEO based NTN |
| --- | --- | --- | --- | --- |
| Altitude |  | 600 km | 1200 km | 35786 km |
| Beam footprint size |  | 200 km | 200 km | 500 km |
| Path loss | Cell center | 154.05 dB | 160.05 dB | 206.74 dB |
|  | Cell edge | 154.17 dB | 160.09 dB | 206.75 dB |

Observation 2: The near-far effect is not clearly exhibited in NTN compared with terrestrial network.

Furthermore, even though a UE with high sensitivity can determine its relative position with respect to the center of a serving cell by measuring the received signal power, it is still not sufficient for efficient mobility management in NTN. To be specific, as plotted in Figure 2, a UE residing on the dotted circle with a radius of 50km has a same signal strength. For example, the received RSPP of the three points A, B and C is equal.

Figure 2. An example of different points on a circle inside an NTN cell

However, due to the high mobility of LEO based NTN cell, the residual time required for points A, B and C to cross the cell boundary can be quite different. As seen in Figure 2, the NTN cell diameter is 200km and its movement is indicated by the dashed arrow from the left to the right. Assuming that the speed of the LEO satellite is 7.58km/s. Thus, as shown in Table 2, the residual time for a UE residing in A, B and C to approach the cell boundary is different (i.e., 19.74s, 6.58s and 13.16s, respectively), and so does its mobility management requirement (e.g., preparing time before handover).

Table 2: Residual time for different points on a circle to cross cell boundary

| Reference points | Distance to cross cell boundary | Residual time to cross cell boundary |
| --- | --- | --- |
| A | 150 km | 19.74 s |
| B | 50 km | 6.58 s |
| C | 100km | 13.16 s |

Observation 3: In an LEO based NTN cell, UEs with similar received signal strength can have distinct mobility management requirement.

Combining Observations 2 and 3, RAN1 is kindly advised to discuss related physical layer measurements besides traditional signal strength to enhance NTN mobility management in high layers.

Observation 4: Besides signal strength related physical layer measurements, other measurements can be exploited to enhance network mobility management.

Furthermore, as shown in Table 3, six reference scenarios are identified in [5]. Different scenarios have distinct characteristics and thus may call for different measurement parameters. In particular, for scenarios A and B, measuring signal strength may not work at all due to the very small difference in the received power. On the other hand, for scenarios C1, C2, D1 and D2, the UE and satellite position measurement can be crucial, and signal strength based measurement may be taken as a second criteria.

Table 3: Reference scenarios (adopted from [5])

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

Observation 5: For different network scenarios (i.e., scenario A, B, C1, C2, D1 and D2), suitable combination of parameters (e.g., position and signal strength) should be chosen to facilitate efficient cell measurements.

Conclusions

Based on the discussion in the previous sections, we made the following observations:

Observation 1: The cell measurement for NR mobility management (e.g., handover) is executed based on signal strength related parameters.

Observation 2: The near-far effect is not clearly exhibited in NTN compared with terrestrial network.

Observation 3: In an LEO based NTN cell, UEs with similar received signal strength can have distinct mobility management requirement.

Observation 4: Besides signal strength related physical layer measurements, other measurements can be exploited to enhance network mobility management.

Observation 5: For different network scenarios (i.e., scenario A, B, C1, C2, D1 and D2), suitable combination of parameters (e.g., position and signal strength) should be chosen to facilitate efficient cell measurements.

- References

TR 38.811 V15.0.0, “Study on New Radio (NR) to support non terrestrial networks”.

TS 38.215 V15.2.0, “Physical layer measurements”.

TS 38.331 V15.3.0, “Radio Resource Control (RRC) Protocol specification”.

TS 38.133 V15.4.0, “Requirements for support of radio resource management”.

TR 38.821 V0.3.0, “Solutions for NR to support non-terrestrial networks (NTN)”.