3GPP TSG RAN WG1 Meeting #96	R1-1903205

# Athens, Greece, Febuary 25  – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCCH selection among different QCL-TypeD property |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-3-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If a UE monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have different QCL-TypeD properties on active DL BWP(s) of one or more cells, it was agreed in RAN1 #94bis that the UE will first select the cell with the lowest serving cell index containing a CSS, then select the CSS with the lowest CSS ID in the cell. However, the current specification may be interpretated in two ways. The first one is in line with the agreement and second one is that the UE will first select the CSS with lowest CSS ID among all serving cells, then select the cell with lowest serving cell index containing the CSS with the lowest CSS ID. Consider the following configuraions as an example - Cell#1 with CSS#2 - Cell#2 with CSS#1 The UE will select the CORESET correspoding to CSS#2 in Cell 1 based on the first interpretation while the UE will select the CORESET correspoding to CSS#1 in Cell#2 based on the second interpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the ambiguity of PDCCH monitoring in multiple CORESETs that have different QCL-TypeD properties to align with the agreement in RAN1#94bis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCCH monitoring in multiple CORESETs with different QCL-TypeD properties would be ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR impacts the PDCCH monitoring in multiple CORESETs with different QCL-TypeD properties. - If the UE is implemented in accordance to the CR and the gNB is not, the UE may fail to detect the PDCCH. - If the gNB is implemented in accordance to the CR and the UE is not, the UE may fail to detect the PDCCH. However, it is expected that UEs have been implemented in accordance to this CR, given the RAN1 agreement. |  |  |  |  |  |  |  |  |

------------------------------------------ Start of Text Proposal ----------------------------------------------

10.1 UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have different QCL-TypeD properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell with the lowest index from the one or more cells that corresponds to a CSS set, and the CORESET corresponds to the CSS set with the lowest index in the cell, if any; otherwise, to the USS set with the lowest index in the cell

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

-	for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL-TypeD properties than a CSI-RS

-	for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL-TypeD properties

-	the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells

-	the number of active TCI states is determined from the multiple CORESETs

< Unchanged parts are omitted >

------------------------------------------- End of Text Proposal ----------------------------------------------