3GPP TSG RAN WG3 Meeting #103 	R3-1903216

Athens/Greece, 25 February-1 March 2019

Title:	[NR-NTN] Link and system level performance evaluation for NR-NTN

Source: 	Thales

Type:	discussion

Document for:	Information

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

A first Study Item “Study on NR to support non-terrestrial networks” was approved during the #80 meeting in La Jolla, with the following achievements

- Definition of deployment scenarios and related system parameters

- Adaptation of the 3GPP channel models for non-terrestrial networks

- Identification of any key impact areas on the New Radio

A second Study Item “Study on solutions for NR to support non-terrestrial networks” was approved during the same meeting, where solutions will be investigated on the key issues identified in [1].

This contribution proposes performance evaluation parameters for this new Study Item.

# References

[1] 3GPP TR 38.811 “Study on NR to support non-terrestrial networks”

[2] ETSI 302 307-1 v1.4.1 “Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 1 (DVB-S2)”, July 2014

[3] DVB Document A171-2, “Digital Video Broadcasting (DVB) Implementation guidelines for the second generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X)”, March 2015

[4] TR 38.803 “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects” v14.2.0

[5] 3GPP TR 38.802 “Study on New Radio (NR) Access Technology; Physical Layer Aspects“

[6] 3GPP R1-1802551 “UE antenna assumption and beam modeling for NTN”

# Discussion

Section 7 of [1] highlights the design constraints specific to non-terrestrial networks compared to cellular systems and the potential features of NR that may require modifications. From a RAN1 perspective, the table below summarizes these constraints and features, and identifies the required simulation type (link- or system level).

Table 1: Non-terrestrial design constraints and potential NR feature modifications from a RAN1 perspective

| Non-terrestrial design constraint | NR features that may require modifications | Simulation type needed |
| --- | --- | --- |
| Specific propagation channel (high Doppler, satellite impairments in case of a transparent satellite) | Reference signals Preamble sequence SCS | Link-level - study of new pilot arrangement/SCS if needed - related to initial access |
| Power limited link budget | Very low SNR MCS may be required | Link-level - study of new MCS if needed |
| Large differential delay due to large cell size | Initial access procedure | Link-level - related to initial access System-level |
| Large propagation delay | Initial access procedure HARQ procedure Power control command ACM | Link-level - related to initial access - channel coding (to target low BLER) System-level |
| Specific spectrum | Carrier numbering/pairing Carrier bandwidth Carrier aggregation method | System-level |

While RAN3 has provided reference scenarios, performance evaluation parameters need to be agreed at RAN1 level for both link-level and system-level simulations.

Two reference scenarios are considered

- A scenario with transparent GEO satellite

- A scenario with a LEO constellation at 600km altitude

# UE characteristics

For performance simulations, it is proposed to adopt the RF characteristics of UE described in Table 2. These characteristics are derived from [1] and [6].

Table 2: RF characteristics of UE

|  | Fixed or mounted on moving platforms | Handheld for < 6GHz | Handheld for > 6GHz |
| --- | --- | --- | --- |
| Transmit Power | 2 W (33 dBm) | 200 mW (23 dBm) | 200 mW (23 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) | Co-phased array – Dual linear polarization as described in Table 1 of [6] | Co-phased array – Dual linear polarization as described in Table 1 of [6] |
| Polarisation | Circular | Linear | Linear |
| Number of antenna elements | 1 | 4 | 32 |
| Number of antenna pannels | N/A | 1 | 2 |
| Antenna panel orientation (bearing angle, downtilt) | N/A | (0°, 0°) | Panel 1: (0°, 0°) Panel 2: (180°, 0°) |
| Antenna panel configuration | N/A | (M, N, P) = (1, 2, 2) dH=0.5λ Polarization: 0°, 90° | M, N, P) = (2, 4, 2) dH, dV=0.5, 0.5λ Polarization: 0°, 90° |
| TXRU to element mapping | N/A | One TXRU per element | One TXRU per panel per polarization |
| Antenna element gain pattern | Very directional | Omni-directional | See Table 2 of [6] |
| Antenna element gain | Tx: 43.2 dBi Rx: 39.7 dB | Tx and Rx: 0 dBi | Tx and Rx: 5 dBi |
| Noise figure | 1.2 dB | 7 dB | 10 dB |

# Satellite characteristics

## Geo satellite

| Assumptions | Value |
| --- | --- |
| Altitude from ground | 35789km |
| Antenna pattern | As describe in clause 6.4.1 of TR38.811 with ka = TBD |
| Min EIRP density at beam center | TBD |
| Phase noise | See Annex A.1 |

## LEO satellite

| Assumptions | Value |
| --- | --- |
| Altitude from ground | 600km |
| Antenna pattern | As describe in clause 6.4.1 of TR38.811 with ka = TBD |
| Min EIRP density at beam center | TBD |
| Phase noise | See Annex A.1 |

# Link-level simulation assumptions

The metrics for link-level simulations given in Section 7.1.1 of [5] are considered. Other metrics are not precluded (e.g. required satellite power per useful transmitted bit).

## Simulation assumption for evaluation of new pilot arrangement/SCS/MCS (if needed)

Table 3: Simulation assumptions for evaluation of new pilot arrangement/SCS/MCS

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 2/20 GHz |
| Duplex | FDD/TDD |
| Waveform | OFDM (Downlink) OFDM /SC-FDMA (Uplink) |
| System Bandwidth | 30 MHz for < 6GHz 400 MHz for > 6GHz |
| TTI length | 1 ms as baseline |
| Subcarrier spacing | 15kHz, 30kHz, 60kHz for f0 < 6GHz 60kHz, 120kHz f0 > 6GHz Other SCS may also be considered if needed |
| Guard time interval | Interval of normal CP as baseline |
| FFT size | e.g. 1024 for 15kHz subcarrier spacing |
| Data transmission bandwidth | FFS |
| Antenna configuration | 1T1R as baseline |
| MIMO mode | N/A |
| Rank per UE | N/A |
| MCS | QPSK 1/3 1/2 2/3 16-APSK 1/2 2/3 Other MCS for very low SNR may also be considered if needed |
| Control Overhead | Zero |
| Channel estimation * | Realistic (Pilot pattern to be given) |
| Channel Model ** | TDL from [1] |
| Phase noise model (only for >6GHz) | As proposed in [2][3][4] depending on the considered UE type, see Annex A.1 for typical values |
| Satellite power amplifier model (transparent scenarios only) | As proposed in [2] for GEO, see Annex A.2 FFS for LEO |

## Simulation assumption for evaluation related to initial access

Target requirements defined in Section 7.1.5 of [5] are considered as baseline, with an additional frequency offset coming from the satellite motion.

Table 4: Simulation assumptions for synchronization signals/channels

| Assumptions | Value |
| --- | --- |
| Carrier Frequency | 2GHz , 20GHz |
| Channel Model | TDL from [1] |
| Subcarrier Spacing(s) | As in Table 2 |
| SNR | [-12dB -6dB 0dB] |
| UE speed | 3 km/h 500 km/h (mandatory) 1000km/h (optional in LEO, mandatory in GEO) |
| Search window | As in [5], Table A.1.5-1 |
| Frequency Offset | As in [5], Table A.1.5-1 |
| Doppler from satellite | Worst case at 600km considering 10° as UE minimum elevation angle with beam center precompensation |
| Number of interfering TRPs | 0 TRP (TBC) |

## Simulation assumption for evaluation of channel coding

Simulation assumptions for eMBB from [5], Table A.1.3-1, are considered (or a subset to be defined).

# System-level simulation assumptions

In the following, the coordinate system is the one described in Section 6.3 of [1] (“Earth Centred Earth fixed”).

## LEO constellation at 600km

For some performance evaluations (e.g. HARQ performance evaluation), it is expected that the simulation of a single satellite may be sufficient.

Table 5: Simulation assumptions for LEO constellation at 600km

| Parameter | Values |
| --- | --- |
| Carrier frequency | 2GHz and 20GHz |
| System Bandwidth | 30 MHz for f0 < 6GHz 400 MHz for f0 > 6GHz |
| Number of satellites | 1 or to be provided by each proponent |
| Reference satellite initial position | (6971,0,0) Other values are not precluded |
| Beam layout | Regular, as illustrated in Figure 1 |
| Number of beams per satellite | To be provided by each proponent |
| Beam size (diameter) | 200 and 500 km |
| Channel Bandwidth | 10/30MHz for f0< 6GHz 400MHz for f0>6GHz |
| UE orientation | FFS |
| Handover margin (for calibration) | 0dB |
| UE distribution | 100% outdoor Uniform dropping, number of users per beam FFS |
| UE attachment | Geometric |
| Traffic model | FFS |
| Traffic load | FFS |
| Fast fading channel | Fast fading channel is not modelled |

Figure 1: Example of regular beam layout (The centre of Beam 1 lies on the intersection between the earth surface and the axis satellite - earth centre)

As illustration, Table 4 gives the beam centre positions calculated from a given satellite position (6971,0,0) and the beam layout from Figure 1, assuming that the straight line from centre of beam 7 to the centre of beam 4 is parallel to the satellite direction. The distance between the centre of two neighboring beams is equal to 200km.

Table 6: Example of satellite and beam positions

| Satellite | (6971,0, 0) |
| --- | --- |
| Beam 1 | (6371,0,0) |
| Beam 2 | (6351,-433,-250) |
| Beam 3 | (6351,-433,250) |
| Beam 4 | (6351,0,500) |
| Beam 5 | (6351,433,250) |
| Beam 6 | (6351,433,-250) |
| Beam 7 | (6351,0,-500) |

## Transparent GEO satellite

Table 7: Simulation assumptions for transparent GEO satellite

| Parameter | Values |
| --- | --- |
| Carrier frequency | 2GHz and 20GHz |
| System Bandwidth | 30 MHz for f0 < 6GHz 400 MHz for f0 > 6GHz |
| Satellite orbit | Geostationary orbit |
| Number of satellites | 1 |
| Satellite position | (42157,0,0) |
| Beam layout | Regular, as illustrated in Figure 1 |
| Number of beams per satellite | To be provided by each proponent |
| Beam size (diameter) | 500km |
| Channel Bandwidth | 10/30MHz for f0< 6GHz 400MHz for f0>6GHz |
| UE orientation | FFS |
| Handover margin (for calibration) | 0dB |
| UE distribution | 100% outdoor Uniform dropping, number of users per beam FFS |
| UE attachment | Geometric |
| Traffic model | FFS |
| Traffic load | FFS |
| Fast fading channel | Fast fading channel is not modelled |

# Annex A.1: Phase noise models in Ka band from [2][3]

Table 8: Example of typical phase noise mask in Ka-band (from 3])

| Offset (Hz) | SSB (dBc/Hz) |  |  |
| --- | --- | --- | --- |
|  | Uplink | Satellite | Downlink |
| 10 | -42 | -33 | -42 |
| 100 | -72 | -62 | -72 |
| 1K | -82 | -80 | -82 |
| 10K | -92 | -90 | -92 |
| 100K | -102 | -95 | -102 |
| 1M | -112 | -106 | -112 |
| 10M | -122 | -116 | -122 |
| 50M | -124 | -118 | -124 |

# Annex A.2: Power amplifier model from [2]

Figure 2: Power amplifier model from [2]