3GPP TSG RAN WG1 #96												R1-1903407

Athens, Greece, 25th February – 1st March, 2019

Agenda Item:	 7.2.5.3

Source:	CMCC

Title: 	Proposal summary for NR RIM

Document for:	Discussion and Decision

Overall RIM-RS configuration parameters

The OAM configures the following parameters for RIM RS.

Table 1 Parameters common to all gNB sets

|  | Parameter | Detail | Value range |
| --- | --- | --- | --- |
| Frequency domain | RIM RS subcarrier spacing and bandwidth | A combination of RIM RS SCS and number of PRBs The length of RIM RS sequence length = number of PRBs *12 | For carrier bandwidth larger than 20MHz - {15kHz, 96 RB} - {30kHz, 48 RB} - {30kHz, 96 RB} For carrier bandwidth smaller than 20MHz, - For 15KHz, min{96 RB, carrier bandwidth} - For 30KHz, min{48 RB, carrier bandwith} |
|  | Frequency resource set | - Number of candidate frequency resources in the whole network: NF - A common reference point (in NR-ARFCN) - A list of starting frequency offset for each candidate resource from common reference point | NF∈{1,2,4} |
| Sequence domain | Sequence set | - List of S1 scramble IDs for RIM RS-1 - List of S2 scramble IDs for RIM RS-2 (if RS-2 is configured) Note: each scramble ID corresponds to one sequence | S1 ≤8, S1 should be even when “Enough”/”Not enough” indication for RS-1 is ON S2 ≤8 |
|  | “Enough”/ “Not enough” indication for RS-1 | - ON/OFF |  |
|  | Other parameters needed for RIM RS initialization seed | Pending on discussion in Section 4 |  |
| Time domain | TDD DL/UL switching period | This field configures the TDD DL/UL switching period for RIM RS transmission in the network, where one RIM RS is configured in one TDD DL/UL switching period - Period 1 (P1) and Reference point - Period 2 (P2) and Reference point (optional) | P1 and P2 are in units of ms P1,P2∈0.5,0.625,1,1.25, 2, 2.5, 3,4, 5,10,20ms |
|  | Total number of set IDs | - Total number of set IDs for RS-1: NSetIdRS1 - Total number of set IDs for RS-2(if configured): NSetIdRS2 | Maximum number of set ID is 22 bit |
|  | Number of consecutive TDD DL/UL switching periods for repetition/near-far | - For RS-1: R1 - For RS-2 (if configured): R2 | R1∈{1,2,4,8} R2∈{1,2,4,8} |
|  | OFDM symbol position | This field configures the transmission position of RIM RS within a TDD DL-UL switching period. - List of R1 OFDM symbol positions (symbol offset with respect to the reference point) for RS-1 - List of R2 OFDM symbol positions (symbol offset with respect to the reference point) for RS-2 (if configured) |  |
|  | Near/Far indication Flag | Indication of whether near-far functionality is enabled (optional) - ON/OFF for RS-1 - ON/OFF for RS-2 (if configured) |  |

Table 2 Set-specific parameters configured by OAM

| Parameter | Explanation | Value range |
| --- | --- | --- |
| Value of set ID | - Set ID for RS-1 - Set ID for RS-2 (if configured) | At most two set IDs can be configured for a gNB. The bit length of the set ID is maximum 22bit. |

Offline Agreement 1:

Above two tables are agreed

Offline Agreement 2:

Following 3 cases should be supported by configuration.

Case 1: When only one TDD-UL-DL-Pattern is configured, only Period 1 is configured. For this case, the reference point configured for Period 1 is the DL transmission boundary (i.e., the 1st reference point as captured in TR 38.866) of this TDD-UL-DL-Pattern.

Case 2: When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, only period 1 is configured. Here Period 1 equals to the concatenated periodicity of the two TDD-UL-DL-Patterns. For this case, the reference point configured for Period 1 is the DL transmission boundary of the TDD-UL-DL-Pattern where the RIM-RS resource is configured.

Case 3: When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both period 1 and period 2 are configured. Period 1 equals to the transmission periodicity of pattern 1, while period 2 equals to the transmission periodicity of pattern 2. For this case, the reference points configured for Period 1 and Period 2 are the DL transmission boundary of the corresponding TDD-UL-DL-Pattern, respectively.

The above description does not need to be captured and are mainly for understanding.

Offline Agreement 3: global RIM RS transmission periodicity (Pt) is then determined as follows,

Pt= Pt+n

where

- Pt=NSetIdRS1NF∙S1∙R1+ NSetIdRS2NF∙S2∙R2, if the indication for “enough” or “not enough” is OFF, or

Pt=2∙NSetIdRS1NF∙S1∙R1+ NSetIdRS2NF∙S2∙R2, if the indication for “enough” or “not enough” is ON

- n is the smallest non-negative integer such that

modPt∙P1, 10240=0, in case 1 and case 2 or

modPt∙(P1+P2)/2, 10240=0, in case 3

More specially, Pt can be determined as follows,

- if one TDD-UL-DL-Pattern is configured (case 1), or only one of the TDD-UL-DL-Patterns is configured for RIM-RS transmission when two concatenated TDD-UL-DL-Patterns are configured (case 2), i.e., only P1 is configured,

Pt=Pt⋅P1∙2μRIM1024∙Nslotframe,μRIM∙1024∙Nslotframe,μRIMP1∙2μRIM

- if both TDD-UL-DL-Patterns are configured for RIM-RS transmission when two concatenated TDD-UL-DL-Patterns are configured (case 3),  , i.e., both P1 and P2 are configured,

Pt=Pt⋅P1+P22∙2μRIM1024∙Nslotframe,μRIM∙1024∙Nslotframe,μRIMP1+P22∙2μRIM

Where μRIM is subcarrier spacing configuration for RIM, and only two values, i.e., 0 for 15KHz subcarrier spacing and 1 for 30KHz subcarrier spacing are applicable, correspondingly, Nslotframe,μRIM is same as the definition in 38.211.

It is expected that P1，or (P1+P2) divides 20 msec

Offline agreement 1

P1,P2∈0.5,0.625,1,1.25, 2, 2.5, 3,4, 5,10,20ms

It is not expected that (P1+P2) larger than 20 msec

Unique RIM-RS resource indexing

The RIM-RS resources can be uniquely indexed via global resource tuple it,if,is, where, it is time-domain resource index, if is frequency-domain resource index, is is sequence index.

- it∈{0,1, …, Pt-1}, where Pt is the global RIM-RS periodicity in unit of TDD DL/UL switching period;

- if∈0,1,⋯,NF-1, where NF is the number of candidate frequency resources in the whole network;

- is∈0,1,⋯,NS-1, where NS is the number of RIM-RS sequences in one TDD DL/UL switching period, NS=S1 for RS-1 and NS=S2 for RS-2

Mapping between set ID and RIM-RS resource indies

For transmission, a gNB is configured with one or multiple set ID(s) nsetID. Then the RIM RS resource indices  it,if,is for transmission are determined as follows:

is=Sstart+nSetId mod ns

it=Tstart+itR+r, with r=0,1,⋯,R-1, and it=modnSetIdns, NT

if=modnSetId NTns, NF

where

- nsetID is the set ID configured for RS1 or RS-2;

- Sstart is start sequence index;

  - For RS-1 with “Enough”/ “Not enough” indication as OFF, Sstart=0

  - For RS-1 with “Enough”/ “Not enough” indication as ON,

    - Sstart=0, for indicating “Not enough”

    - Sstart= S1/2, for indicating “Enough”

  - For RS-2, Sstart=0

- ns is the number of sequence candidates for the current functionality

  - For RS-1 with “Enough”/ “Not enough” indication as OFF, ns=S1

  - For RS-1 with “Enough”/ “Not enough” indication as ON, ns=S1/2

  - For RS-2, ns=S2

- Tstart is starting time-domain offset;

  - For RS-1, Tstart=0

  - For RS-2, Tstart is determined as

Tstart=NSetIdRS1NF∙S1∙R1，if the indication for “enough” or “not enough” is OFF or

Tstart=2∙NSetIdRS1NF∙S1∙R1, if the indication for “enough” or “not enough” is ON

- NT is the number of set IDs to be differentiated via time-domain;

    - For RS-1, NT =NSetIdRS1NF∙S1，if the indication for “enough” or “not enough” is OFF or

    - For RS-1, NT =2∙NSetIdRS1NF∙S1, if the indication for “enough” or “not enough” is ON

    - For RS-2, NT=NSetIdRS2NF∙S2

- R is the number of consecutive TDD DL/UL switching periods;

    - For RS-1, R=R1

    - For RS-2, R=R2

Conversely, for reception, gNB obtains the set ID from the RIM RS resource indices it,if,is of detected RIM RS based on the mapping rule, as follows,

nsetID=is-Sstart+nsit-TstartR+NTnsif

Initialization seed

Offline Agreement 2:

Regarding generating time-varying sequence for RIM-RS, it is suggested that

cinit=2M∙fnt+nSCID mod 231

where,

- nSCID is scramble ID with value space of 0..2M-1. nSCID is selected from a scramble ID table via global sequence index is, where the scramble ID table is configured by OAM;

- nt is a counter of RIM-RS transmission periodicity (Pt) since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900);

- fnt is equal to Z∙nt+δ mod 231-M, where

- Z and δ are configured by OAM with value space of 0..231-1,  and they are both periodically updated by OAM with scrambel ID group (up to 8).

Alternative Possible agreement for initialization seed

Regarding generating time-varying sequence for RIM-RS, it is suggested that

cinit=2M∙fnt+nSCID mod 231

where,

- nSCID is scramble ID with value space of 0..2M-1. nSCID is selected from a scramble ID table via global sequence index is, where the scramble ID table is configured by OAM;

- nt is a counter of RIM-RS transmission periodicity (Pt) since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900);

- fnt=i=031-M-1c(i)⋅2i , where the pseudo-random sequence c(i) is defined in TS 38.211 clause 5.2.1, with initializing seed cinit=Z∙nt+δ mod 231, where,

- Z and δ are configured by OAM with value space of 0..231-1,  and they are both periodically updated by OAM with scrambel ID group (up to 8).

- 5.2.1 Pseudo-random sequence generation Generic pseudo-random sequences are defined by a length-31 Gold sequence. The output sequence of length, where, is defined by where NC=1600 and the first m-sequence shall be initialized with. The initialization of the second m-sequence, , is denoted by with the value depending on the application of the sequence.

OAM functionality

Offline Agreement 3: The OAM configures gNBs to report the all necessary information derived from the detected RIM-RS, including

- The detected set ID;

- Propagation delay in number of OFDM symbols

- Functionality of the RS (RS-1 or RS-2, Enough or Not enough mitigation for RS-1)

Offline Agreement 4: For RIM-RS monitoring, the OAM configures a monitoring window, in which the RIM-RS is periodically monitored. To support the periodic RIM-RS monitoring, the OAM configures the following parameters:

- Monitoring window: indicating a duration in which gNB monitors the RIM-RS, multiple of Pt

- Monitoring periodicity within the window: M∙P1∙R1 or M∙(P1+P2)/2∙R1 , where M is mutually prime to  NSetIdRS1NF∙S1;

  - P1∙R1 or (P1+P2)/2∙R1 is the consecutive detection duration per Monitoring periodicity

- Only the earliestNSetIdRS1NF∙S1∙P1∙R1 or  NSetIdRS1NF∙S1∙(P1+P2)/2∙R1 of each Pt in the Monitoring window are taken as valid time for monitoring potential interference, and they are consecutively monitored in the Monitoring window, i.e., residual part of each Pt is not used for discovering potential interference

- Starting offset of the first monitoring occasion within the window: SM

- Periodicity of Monitoring window: PM

Offline Proposal 5: The gNB ID of a detected RIM-RS is conveyed by OAM to the source of the RIM-RS.

Annex.

Example of offline proposal 4