3GPP TSG-RAN WG1 Meeting #96	Tdoc R1-1903447

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Factory Automation Scenario at 4 GHz

Document for:	Discussion, Decision

# Introduction

Factory automation is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In order to check a performance the current Rel-15 NR system can guarantee for this use case, we provide system level simulation results, which are based on the assumption in [2] with additional adjustment to appropriately capture relevant deployment. The results show that it is possible to achieve reliability target with existing NR Rel. 15 design with certain number of users per cell.

# Discussion

According to [2], requirements for factory automation use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for factory automation use case

| Factory automation requirement | 99.9999% PHY reliability | 1 ms PHY one-way latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms |
| --- | --- | --- | --- |

We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate reliability performance of URLLC for factory automation using the system level simulations. For calibration purposes we obtain SINR distribution from a system level simulation. In our simulations we follow methodology defined in email discussion [2].

## Additional assumptions and link level results

For system level simulations in addition to Table A-1 in appendix and agreements in email discussion [2] we use the following assumptions:

- PDCCH is allocated in OFDM-symbols 0,5 and 10 in every slot;

- PUCCH periodicity for SR is 2os;

- PUCCH for HARQ-ACK has only one occasion per slot;

- For traffic:

  - Starting point of data arrival time is randomized within a slot between different seeds;

  - Data arrives in all UEs simultaneously assuming perfect synchronization between UEs;

- PDCCH and PUCCH channels are assumed ideal.

- Scheduler targets to one-shot transmissions with target BLER 10-6, while 1 more retransmission is allowed.

- PUSCH and PDSCH use 4os mini-slots.

- Resource utilization calculation considers only data and only used resources in case of CG.

- For TDD we use the following pattern: [D D D D D D G U U U U U U U];

In our simulations we follow methodology to report performance according to Option 1 (Section 5.1 in [2]).

In addition, based on link level simulation assumption in Table A-2 in the appendix, we present BLER performances of PDCCH and PDSCH.

For PDCCH, we consider DCI size =40 bits excluding CRC, AL 4,8,16, and 1os CORESET duration. PDCCH BLER for different AL are given in Fig. A-1.

For data channels, we consider TBS = 256 bits (=32 bytes), transmission duration of 4 OFDM symbols with 1 DMRS symbol overhead. PDSCH/PUSCH BLER for different MCSs supported within 40 MHz BW (e.g., MCS1 to MCS6 from Table 5.1.3.1-3 [3]) are given in Fig. A-2.

## Simulation results

We simulated factory automation scenario with different number of users per cell and with randomization of data arrival time. It can be noted, that depending on when data arrives within the slot, the packets can be transmitted in different mini-slots

Table 2: Percentage of UEs fulfilling 1ms latency and 99.9999% reliability requirements for FDD

| Num. of UEs per cell | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 35 |  | 40 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% |
| DL | 100 | 6.09 | 99.97 | 9.17 | 99.58 | 12.24 | 98.05 | 15.28 | 94.73 | 18.34 | 89.46 | 21.37 | 82.22 | 24.38 |
| UL SR-based | 74.95 | 6.93 | 57.71 | 10.42 | 43.92 | 13.91 | 35.25 | 17.37 | 29.42 | 20.86 | 25.27 | 24.31 | 22.13 | 27.74 |
| UL CG | 100 | 6.93 | 100 | 10.42 | 99.96 | 13.19 | 99.61 | 17.37 | 97.65 | 20.86 | 93.16 | 24.31 | 86.72 | 27.75 |

Table 3: Percentage of UEs fulfilling 1ms latency and 99.9999% reliability requirements for FDD

| Num. of UEs per cell | 5 |  | 10 |  |
| --- | --- | --- | --- | --- |
|  | UE% | RU% | UE% | RU% |
| DL | 99.68 | 6.56 | 96.21 | 13.02 |
| UL CG | 99.79 |  | 95.68 |  |

It is possible to achieve less than 1% of UEs in outage in DL and UL with 4os duration in FDD configuration with 30 kHz SCS within 1 ms one-way latency having up to 20UEs/cell.

It is possible to achieve less than 1% of UEs in outage in DL and UL with 4os duration in a TDD configuration with 30 kHz SCS within 1 ms one-way latency having up to 5UEs/cell if traffic arrival does not correspond to the worst latency case.

Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation can be achieved with several transmission configurations.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is possible to achieve less than 1% of UEs in outage in DL and UL with 4os duration in FDD configuration with 30 kHz SCS within 1 ms one-way latency having up to 20UEs/cell.

Observation 2	It is possible to achieve less than 1% of UEs in outage in DL and UL with 4os duration in a TDD configuration with 30 kHz SCS within 1 ms one-way latency having up to 5UEs/cell if traffic arrival does not correspond to the worst latency case.

Observation 3	Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation can be achieved with several transmission configurations.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- [AH1901-NR-01] [eURLLC 7.2.6.1.4] Email discussion/approval on converging the proposals for eURLLC processing timeline

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019.

- R1-1900168 Channel Model for Factory Automation Scenario, Ericsson, January 2019.

# Appendix A

Figure A-1: PDCCH BLER for different AL

Figure A-2: PDSCH/PUSCH BLER (single transmission) for different MCSs

# Appendix B

Table A-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Inter-BS distance | 20m |
| Carrier frequency | 4 GHz |
| Channel model | Modified Indoor Hotspot LOS model with blockers 3GPP TR 38.901 where extra blockage loss is added on top of the InH LoS loss (see e.g., Fig. A-1 and [5]) |
| UE Tx power | 23dBm |
| BS antenna configurations | 16Tx/16Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8), dH = dV = 0.5 λ |
| BS antenna height | 10m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| BS Tx power | 24 dBm per 20 MHz |
| Simulation bandwidth | 40 MHz |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are indoor |
| Traffic | 1. Periodic deterministic traffic model with data arrival interval 2 ms, 32B TBS on Physical layer. |

Table A-2: Link level simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-D 30ns |
| Deployment | Modified indoor hot-spot with blockers |
| UE speed | 3 km/h |
| BS TX antenna configuration | 2 Tx ports |
| UE RX antenna configuration | 4 RX ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Figure A-3 Modified indoor layout of factory hall with blockers