3GPP TSG RAN WG1 Meeting 96	Tdoc R1-1903448

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of URLLC Factory Automation Scenario at 30 GHz

Document for:	Discussion, Decision

# Introduction

Factory automation is one of the main use cases considered for Rel. 16 eURLLC SI [1]. In this contribution we provide the system level simulation results with assumptions from [2-3] to demonstrate baseline performance of Rel-15 NR for FR2. In addition, we show a performance gain with potential Rel-16 UE capability enhancements, DL and UL geometry, link level simulation results.

The results show that it is possible to achieve latency and reliability target with existing NR Rel. 15 design for certain configurations.

# Discussion

According to [3], requirements for factory automation use cases are given in Table 1 below.

Table 1: Assumption of reliability requirement for factory automation use case

| Factory automation requirement | 99.9999% PHY reliability | 1 ms PHY one-way latency | DL & UL: 32 bytes Periodic deterministic traffic model with data arrival interval 2 ms |
| --- | --- | --- | --- |

We note that reliability requirement of 99.9999% at PHY layer assumes that higher layer mechanisms like PDCP duplication are not available. On the other hand, for deployments that have higher layer mechanisms like PDCP duplication, physical layer reliability requirement can be relaxed.

With this requirement in mind, we evaluate reliability of URLLC for factory automation using the system level simulations. For calibration purposes we obtain SINR distribution from a system level simulation shown in Figure A-1. In addition, based on link level simulation assumption in Table B-2 in the appendix, we present BLER performances of PDCCH and PDSCH on figures A-2 and A-3.

## Additional assumptions

For system level simulations in addition to Table A-1 in appendix we follow methodology defined in email discussion [2]. Moreover, we use the following assumptions:

- PUCCH for HARQ-ACK has only one occasion per slot;

- For traffic:

  - Starting point of data arrival time is randomized within a slot between different seeds;

  - Data arrives in all UEs simultaneously assuming perfect synchronization between UEs;

  - Data arrival time in UL and DL are not the same, a constant time shift is used.

- PDCCH and PUCCH channels are assumed ideal.

- Scheduler targets to one-shot transmissions with target BLER 10-6, while 1 more retransmission is allowed.

- PUSCH and PDSCH use 7os mini-slots.

- Resource utilization calculation considers only data and only used resources in case of CG.

- We use the following TDD pattern [D D D D D D F F U U U U U U].

In our simulations we follow methodology to report performance according to Option 1 (Section 5.1 of TR38.824 v.1.0.0).

## Simulation results

We simulated factory automation scenario with different number of users per cell and with randomization of data arrival time. It can be noted, that depending on when data arrives within the slot, the packets can be delayed on various amount of time.

Table 2: Percentage of UEs fulfilling 1ms latency and 99.9999% reliability requirements for TDD

| Num. of UEs per cell | 5 |  | 10 |  | 15 |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE% | RU% | UE% | RU% | UE% | RU% | UE% | RU% |
| DL | 100 | 9.02 | 99.87 | 17.14 | 95.68 | 25 | 81.77 | 33.05 |
| UL CG | 100 |  | 99.95 |  | 98.28 |  | 90.47 |  |

Table 3: Percentage of UEs fulfilling 1ms latency and 99.9999% reliability requirements for TDD with capability 3

| Num. of UEs per cell | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| DL | 100 | 99.995 | 99.85 | 97.5 |
| UL CG | 100 | 100 | 99.99 | 99.63 |

That is, we show that for factory automation scenario at 30 GHz carrier, the requirement of 99.9999% PHY reliability within 1 ms one-way latency for DL and UL transmissions can be achieved.

It is possible to have less than 1% of UEs in outage in DL and UL with 7os duration in TDD configuration with 120 kHz SCS within 1 ms one-way latency having up to 10UEs/cell.

With enhanced UE processing capability for 120 kHz SCS, it is possible to have less than 1% of UEs in outage in DL and UL with 7os duration in TDD configuration with 120 kHz SCS within 1 ms one-way latency having up to 15UEs/cell.

Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation at 30 GHz carrier can be achieved.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is possible to have less than 1% of UEs in outage in DL and UL with 7os duration in TDD configuration with 120 kHz SCS within 1 ms one-way latency having up to 10UEs/cell.

Observation 2	With enhanced UE processing capability for 120 kHz SCS, it is possible to have less than 1% of UEs in outage in DL and UL with 7os duration in TDD configuration with 120 kHz SCS within 1 ms one-way latency having up to 15UEs/cell.

Observation 3	Reliability requirement of 99.9999% within 1 ms one-way latency for factory automation at 30 GHz carrier can be achieved.

# References

- RP-181477 New SID on Physical Layer Enhancements for NR URLLC, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- [AH1901-NR-01] [eURLLC 7.2.6.1.4] Email discussion/approval on converging the proposals for eURLLC processing timeline

- 3GPP RAN1 #94bis Chairman’s note

- 3GPP TS 38.214 V15.3.0

- R1-1900161 Scheduling/HARQ/CSI Processing Timeline Enhancements for NR URLLC, Ericsson, January 2019.

- R1-1900168 Channel Model for Factory Automation Scenario, Ericsson, January 2019.

# Appendix A

Figure A-1: DL and UL SINR

From Fig. A-1, the 1%-tile DL SINR and UL SINR are -1.57 dB and -1 dB, respectively.

Based on link level simulation assumption in Table A-2 in the appendix, we present BLER performances of DL control channel and data channel. For PDCCH, we consider DCI size =40 bits excluding CRC, AL8,16, and 1os CORESET. PDCCH BLER for different AL are given in Fig. A-2.

Figure A-2: PDCCH BLER for different AL

For data channel, we consider TBS = 256 bits (=32 bytes), transmission duration of 7 OFDM symbols with 1 DMRS symbol overhead. BLER for different MCSs supported within 160 MHz BW (e.g., MCS0 to MCS2 from Table 5.1.3.1-3 [4]) are given in Fig. A-3 for both single transmission and one retransmission with soft combining.

Figure A-3: BLER of single transmission and one retransmissions for different MCSs

# Appendix B

Table B-1: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer as defined in 38.802 Indoor floor: 12 BSs per 120 m x 50 m |
| Inter-BS distance | 20m |
| Carrier frequency | 30 GHz |
| Channel model | Modified Indoor Hotspot LOS model with blockers 3GPP TR 38.901 where extra blockage loss is added on top of the InH LoS loss (see e.g., Fig. A-1 and [7]) |
| UE Tx power | 23dBm |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 2, 4), dH = dV = 0.5 λ |
| BS antenna height | 10m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 7 dB |
| UE antenna configuration | 2 Tx/Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 5 dBi |
| UE receiver noise figure | 10 dB |
| BS Tx power | 26 dBm |
| Simulation bandwidth | 160 MHz |
| Duplex mode | TDD (50% DL) |
| SCS | 120 kHz |
| UE distribution | 100% of users are indoor |
| Traffic | - Full buffer (for geometry results) - Periodic deterministic, 2 ms periodicity, 32B TBS |

Table B-2: Link level simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 30GHz |
| Channel model | CDL-A 20ns |
| Deployment | Modified indoor hot-spot with blockers |
| UE speed | 3 km/h |
| BS TX antenna configuration | 2 Tx ports |
| UE RX antenna configuration | 2 RX ports |
| System bandwidth | 160 MHz |
| Sub-carrier spacing | 120 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Figure B-1 Modified indoor layout of factory hall with blockers