3GPP TSG RAN WG1 Meeting #96	R1-1903516

# Athens, Greece, Febuary 25-March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | CRNum | rev | - | Current version: | 14.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on MPDCCH USS/CSS differentiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2019-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The resource of USS and Type0-CSS are physically overlapped based on current configuration. DCI scrambled by C-RNTI and SPS C-RNTI can be transmitted in both USS and Type0-CSS, but current specification defines no UE behaviour to differentiate DCI from USS or Type0-CSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE should prioritize USS over CSS if UE cannot decide whether detected DCI is from CSS or USS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-14 MTC feature of larger maximum TBS is adopted to USS only. If UE cannot differentiate DCI from USS or CSS, it cannot decide to interpret MCS as Rel-14 values or legacy values. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.1.5	MPDCCH assignment procedure

### < Unchanged parts are omitted >

A BL/CE UE is not expected to be configured with values of and that result in non-integer values of .

For Type1-MPDCCH common search space, = and is determined from locations of paging opportunity subframes,

If SystemInformationBlockType1-BR or SI message is transmitted in one narrowband in subframe , a BL/CE UE shall assume MPDCCH in the same narrowband in the subframe  is dropped.

The BL/CE UE is not required to monitor an MPDCCH search space if any ECCEs corresponding to any of its MPDCCH candidates occur within a frame before and also occur within frame.

The BL/CE UE is not expected to be configured with overlapping MPDCCH search spaces.

A BL/CE UE configured to monitor MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI with the same payload size and with the same aggregation level in the Type0-MPDCCH common search space and the MPDCCH UE-specific search space shall assume that for the MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI, only the MPDCCH in the UE specific search space is transmitted.

For MPDCCH UE-specific search space or for Type0-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition is set to 1; or for Type2-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition-RA is set to 1; or for Type2A-MPDCCH common search space if the higher layer parameter mpdcch-NumRepetitions-SC-MTCH is set to 1;

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0 and 5 shown in Table 4.2-1 of [3]

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4 and 7 shown in Table 4.2-1 of [3];

otherwise

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD, in special subframes, if the BL/CE UE is configured with CEModeB

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0, 1, 2, 5, 6, 7, 9 and 10 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4, 7, 8, 9 and 10 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA.

-	For TDD, in special subframes, for MPDCCH in Type1/1A-MPDCCH common search space.

### < Unchanged parts are omitted >