3GPP TSG RAN WG1 Meeting #96	R1-1903763

# Athens, Greece, February 25th- March 1st, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1243 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UL HARQ for EN-DC with SUO Case 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed in RAN1#92 that for a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the PUSCH HARQ RTT is 10 ms in which UL grant received in subframe n schedules PUSCH in subframe n+4, and an UL grant for the same UL HARQ process occurs in subframe n+10. Such agreement was captured in RAN2 specification TS 36.321 but removed, according to the RAN2 in R2-1818775. The RAN2 LS also requested RAN1 to capture 10 ms HARQ RTT for SUO Case 1 in RAN1 specification. The RAN2 LS also suggests to capture that ACK is delivered to higher layers for a transport block. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the agreement that for a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the PUSCH HARQ RTT is 10 ms into TS 36.213. According to the RAN2 LS, it is also captured that ACK is delivered to higher layers for a transport block. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification on SUO Case 1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: The CR captures an agreed UE behaviour which was caputred in TS 36.321 but is requested by RAN2 to be moved to RAN1 specification. Therefore, no UE behaviour is changed. |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

## 8.3	UE HARQ-ACK procedure

<Unchanged parts are omitted>

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to receive PHICH corresponding to a transport block on the serving cell and ACK for that transport block shall be delivered to the higher layers.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, UL grant in subframe n schedules the same UL HARQ process as that in subframe n-6.

Table 8.3-1: k for TDD configurations 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 7 | 4 |  |  |  | 7 | 4 |  |  |  |
| 1 |  | 4 |  |  | 6 |  | 4 |  |  | 6 |
| 2 |  |  |  | 6 |  |  |  |  | 6 |  |
| 3 | 6 |  |  |  |  |  |  |  | 6 | 6 |
| 4 |  |  |  |  |  |  |  |  | 6 | 6 |
| 5 |  |  |  |  |  |  |  |  | 6 |  |
| 6 | 6 | 4 |  |  |  | 7 | 4 |  |  | 6 |

Table 8.3-2: k for TDD configurations 0-6 and UE configured with symPUSCH-UpPts-r14

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 7 | 5,4 |  |  |  | 7 | 5,4 |  |  |  |
| 1 |  | 5,4 |  |  | 6 |  | 5,4 |  |  | 6 |
| 2 |  |  |  | 7,6 |  |  |  |  | 7,6 |  |
| 3 | 6 |  |  |  |  |  |  |  | 7,6 | 6 |
| 4 |  |  |  |  |  |  |  |  | 7,6 | 6 |
| 5 |  |  |  |  |  |  |  |  | 7,6 |  |
| 6 | 6,4 | 4 |  |  |  | 7,4 | 4 |  |  | 6 |

For a non-BL/CE UE, the physical layer in the UE shall deliver indications to the higher layers as follows:

For FDD with a UE not configured with subframeAssignment-r15, and for TDD with a UE configured with one serving cell, and for TDD with a UE configured with more than one serving cell and with TDD UL/DL configuration of all configured serving cells the same, and UE is not configured with EIMTA-MainConfigServCell-r12 for any serving cell, for downlink or special subframe i, if a transport block was transmitted in the associated PUSCH subframe then:

if ACK is decoded on the PHICH corresponding to that transport block in subframe i, or if that transport block is disabled by PDCCH/EPDCCH received in downlink or special subframe i, ACK for that transport block shall be delivered to the higher layers; else NACK for that transport block shall be delivered to the higher layers.

For TDD, if the UE is configured with more than one serving cell, and if at least two serving cells have different UL/DL configurations, or the UE is configured with EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD, for downlink or special subframe i, if a transport block was transmitted in the associated PUSCH subframe then:

if ACK is decoded on the PHICH corresponding to that transport block in subframe i, or if that transport block is disabled by PDCCH/EPDCCH received in downlink or special subframe i, ACK for that transport block shall be delivered to the higher layers; or

if a PHICH resource corresponding to that transport block is not present in subframe i or if UE is not expected to receive PHICH corresponding to that transport block in subframe i, ACK for that transport block shall be delivered to the higher layers.

else NACK for that transport block shall be delivered to the higher layers.

<Unchanged parts are omitted>