3GPP TSG RAN WG1 Meeting #96	R1-1901526

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.2.2

Source:	Huawei, HiSilicon

Title:	Initial access in NR unlicensed

Document for:	Discussion and decision

Introduction

In the previous RAN plenary meeting #82bis, the WI for NR unlicensed (NR-U) was approved [1]. Based on the new WID for NR-U [2], the following physical layer aspects for initial access should be specified:

-	Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SSB numerology assumed per band for Pcells in unlicensed spectrum.

- 	Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure; and, time-permitting, specification of 2-step RACH procedure.

-	Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

-	System information: specify required NR modifications to enhance acquisition of system information in NR-U including increased transmission opportunities in time domain for SI transmission.

- 	RLM/RRM (including multi-beam operation) extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting.

In this contribution, we mainly discuss enhancements on initial access including DRS, PRACH. Considerations on paging and RRM/RLM are also discussed.

NR-U DRS transmission scheme

Enhancement on the DRS transmission

In our companion contribution [3], a DRS consists of one or multiple “DRS blocks”. Each DRS block contains at least a SSB and optionally RMSI/OSI/paging with the same QCL assumption. The duration of a DRS block depends on whether RMSI/OSI/paging are included and which MCS(s) are used. The composition of DRS block(s) within a DRS should be same. The SSB in a DRS block should be transmitted at a pre-defined SSB position within DRS transmission window. Given one to one mapping between possible DRS block transmission position and SSB transmission position when composition of DRS block is configured, as illustrated in Fig 1, we will discuss how to transmit equivalent SSB for simplicity.

Fig 1. DRS block and SSB position in DRS transmission window

Channel occupation at a predefined position for NR-U SS/PBCH block transmission cannot be guaranteed in unlicensed band due to LBT failure. As per RAN1#94 meeting agreement, the following 3 alternatives are identified for NR-U SS/PBCH block transmission.

- Alt-1: Shift SSB(s) in time to the next transmission instance

- Alt-2: Cyclically wrap the SSBs dropped due to LBT failure around to the end of the burst set  transmission

- Alt-3: Network to flexibly position SSB index and indicate the timing information

If Alt-1 is used for SS/PBCH block transmission, as illustrated in Fig 2, after the gNB taking the ownership of the channel, it will commence SS/PBCH block transmission at the next available transmission instance and the order of transmitted SS/PBCH block will not change. In this case, the UE can acquire time/frame synchronization through PBCH DMRS of the detected SS/PBCH block and additional signalling in PBCH payload that indicates time shifting between the original SSB position and the actual transmitted SSB position. Time offset remains the same for each transmitted SS/PBCH block within a DRS transmission window. If Alt-2 is adopted for SS/PBCH block transmission, as shown in Fig 3, after a gNB taking the ownership of the channel, it will commence SS/PBCH block transmission at the next available transmission instance. In this case, part of SS/PBCH block burst set is transmitted at the original configured SSB positions while the others are transmitted in the following transmission instances. More precisely, PBCH DMRS of the first transmitted SS/PBCH block in a SSB burst set is not fixed, which depends on dynamic LBT results. The UE can obtain time/frame synchronization through PBCH DMRS of the detected SS/PBCH block and additional signalling that indicates whether cyclic shifting occurs or not. Assuming the maximum supported delay is 4 transmission occasions, as example shown in Fig 2 using Alt-1 requires 2 additional bits to indicate time offset while example shown in Fig 3 using Alt-2 requires only 1 additional bits to indicate time offset incurred by LBT, and therefore less specification change is needed for supporting enhancement on SS/PBCH block transmission using Alt-2. Besides, both Alt-1 and Alt-2 needs DRS block length information to acquire the accurate time synchronization in case DRS block length could be dynamically configured (e.g., half slot or one slot). In addition, Alt-2 requires the number of transmitted SSB (DRS) for purpose of time synchronization. However, this information is also useful for rate matching when Alt-1 is adopted. To summarize, DRS block length and the number of transmitted SSB (DRS) are needed for both Alt-1 and Alt-2, and the corresponding signalling overhead is identical for both alternatives.

The main purpose of Alt-3 is to ensure that all SS/PBCH blocks could be transmitted with equal probability. While for multiple beam case, SS/PBCH blocks could be transmitted with equal probability if limited transmission starting positions is supported using Alt-1/Alt-2, i.e. if the number of configured SSB positions for SS/PBCH block transmission is 8 and the number of SS/PBCH block within each SS/PBCH block burst set is 4, as shown in Fig 2/Fig 3, if the first SS/PBCH block is required to be transmitted at or before SSB position 5 then the whole SS/PBCH block set is guaranteed to be transmitted within the current DRS window. However, more signalling bits are required using Alt-3 than that using Alt-1 and Alt-2 since all SS/PBCH blocks are dynamically configured and more specification revisions/updates are needed.

Based on the above discussion, Alt-2 is preferred to be used for SS/PBCH block transmission in NR-U regarding the required signalling bits and specification impact.

Fig 2. NR-U SS/PBCH block transmission scheme using Alt-1

Fig 3. NR-U SS/PBCH block transmission scheme using Alt-2

In NR R15 FR1, a UE acquires QCL assumption only based on PBCH DMRS. It is beneficial for neighbor cell RRM measurement since a neighbor cell UE is only required to detect PBCH DMRS to acquire QCL assumption of SSB without further decoding PBCH payload. In Rel-15 NR, 3 bits could be carried by PBCH DMRS, and it is sufficient to carry QCL assumption indication for NR-U operation considering the number of transmitted SSB will not exceed 8 in NR-U.

Shift granularity between candidate SSBs positions/candidate groups of SSBs depends on the deployment scenarios and composition of DRS. In CA mode, only SSB is transmitted and the shift granularity of SSB could be as small as one SSB location. While in DC/SA mode, SSB and other signal(s)/channel(s)  may be jointly transmitted in a DRS, and the shift granularity of SSB could be one DRS block, which is the basic unit of DRS. It consists of SSB, at least one of the corresponding RMSI CORESET/PDSCH, OSI (optional) and paging (optional) with same QCL assumption. For example, if SSB and the corresponding RMSI require one slot to transmit, the shift granularity of SSB should be one slot. As illustrated in Fig 4 below, if 2 DRS blocks are configured within one slot, then DRS shift granularity is 7 symbols, and the gNB could commence DRS transmission at either symbol 0 or symbol 7 depending on LBT results. In this case, two alternatives as shown in left side and right side of Fig 4 could be considered for NR-U DRS transmission. Both alternatives are symmetric within a slot, which facilitates DRS transmission in case LBT fails at the slot boundary. The only difference between two alternatives is that Alternative 2 supports 3 symbol CORESET configuration while Alternative 1 only supports 1or 2 symbol CORESET configuration. In NR R15, 3 symbol CORESET configuration is supported. Since RMSI CORESET is essential for RMSI acquisition, CORSET configuration with more symbols (e.g., more available CCEs) provides better detection performance when strong inter-cell interference is expected. To achieve same configuration flexibility as R15, Alternative 2 is preferable compared with Alternative 1.

Fig 4. Illustration of NR-U DRS in DC/SA mode

Proposal 1: Considering the standard impact, Alt-2 should be used for SS/PBCH block transmission in NR-U.

Proposal 2: Shift granularity of SS/PBCH blocks positions/candidate groups of SS/PBCH blocks depends on the deployment scenarios. In CA mode, the shift granularity could be at least one SS/PBCH block location, while in DC/SA mode the shift granularity could be one DRS block.

Proposal 3: QCL assumption indication of SS/PBCH block should be indicated only by PBCH DMRS. The number of PBCH DMRS sequence in NR R15 is sufficient for NR-U.

Proposal 4: A UE acquires serving cell time synchronization based on both PBCH DMRS and at least wrap around indication carried in PBCH payload.

FFS: other parameters such as DRS block length, the number of transmitted DRS block.

DRS duration and SSB candidate positions

According to ETSI regulation [4], short control signaling could be transmitted without CAT 4 LBT if the number of transmission is less than 50 times and the total transmission duration is less than 2500 µs within an observation period of 50 ms.

As discussed in TR 38.889 [5], CAT 2 LBT could be adopted for DRS transmission only when the DRS duty cycle ≤ 1/20, and the total duration is up to 1 ms. Thereby, at maximum 6 SSBs or 4 SSB multiplexed with RMSI could be transmitted within 2 slots (1 ms).

The DMTC window should be shorter than the DRS periodicity in order to avoid ambiguity of SSB transmission in two consecutive DRS periods. In LTE LAA, 6ms DMTC window was adopted. In Multefire, the DMTC may be configured up to 10 ms in length during which the gNB attempts to transmit DRS. It can efficiently overcome LBT’s impact on DRS transmission. It is also beneficial that DMTC window of nearby gNBs don’t collide with each other within a DRS period for better cell search and measurement performance. Typically, gNB can hear transmission from 2 or 3 other gNBs in a NR-U indoor deployment. Considering all above, a DMTC window with 5ms to 10ms should be enough for NR-U. The maximum number of SSB candidate position(s) Y depends on SSB numerology as well as the duration of DMTC window. For example, a DMTC window of 8ms (Y=32) can be considered, given SSB with 30 kHz SCS and 2 SSB positions per slot for DC and SA mode.

Proposal 5: If CAT 2 LBT is adopted for DRS transmission, the maximum allowed transmission duration of DRS is 1ms.

Proposal 6: The maximum number of candidate SSB position (Y) should be decided according to the minimum periodicity of DRS, maximum length of DMTC window and SSB pattern within a slot.

At SI stage, the transmission of OSI and paging within the NR-U DRS was agreed. It is beneficial to reduce the required number of channel access attempts and better satisfy the OCB requirement in NR-U. From another perspective, it is also not desirable to always consist of all these channels/signals in the NR-U DRS due to limited resources available for the DRS transmission.

In NR, PBCH within SS/PBCH block can indicate the multiplexing pattern between SS/PBCH block and RMSI CORESET. When additional signal (i.e. OSI, paging) may also be included in DRS, it is natural to extend the configuration information in PBCH to carry the indication of inclusion for additional signal and the multiplexing pattern between SS/PBCH block and those included signals in DRS.

Proposal 7: PBCH within SS/PBCH block can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

NR-U Physical RACH (PRACH)

Resource allocation of PRACH

In Rel-15 NR, gNB can semi-statically configure one or multiple PRACH occasions by RMSI. The configuration parameters include periodicity, preamble format, available SFN number, slot number and PRACH occasion number within a PRACH slot etc. In NR-U, the transmission of PRACH may be blocked by uncertain LBT failure, semi-statically configured SFN number, slot number may become un-available. Thereby, a dynamic PRACH resource allocation scheme should be complementally considered. In TR38.889 [4], several dynamic resource allocation alternatives were given as following:

-	Alt.1: For connected mode UE, scheduling of PRACH resources via DCI.

-	Alt.2: For idle mode UE, scheduling of PRACH resources via paging

-	Alt.3: New RACH resources are used immediately following detection of DRS transmission

In Alt.1, scheduling of PRACH is triggered by DCI, e.g. a group common PDCCH. The scheduling information at least includes time and frequency resource allocation for PRACH. This trigger method is mainly applicable for connected mode UE. For idle mode UE, Alt.2 and Alt.3 can both be considered. While for Alt.2, a single paging resource may belong to a tracking area consisting of multiple cells. Thereby, potential collision among PRACH resources of UEs that belong to different cells will occur. If this issue can’t be addressed, Alt.2 will not be an attractive scheme for idle mode UE. In Alt.3, additional RACH resource can be immediately followed within DRS duration after of detection of DRS transmission. It can avoid new LBT procedure for additional PRACH resource, thereby more efficient PRACH   transmission opportunities can be clearly foreseen.

Based on the above analysis, we propose that Alt.1 should be supported at least for connected mode UE. And Alt.3 can be considered as a supplementary scheme for idle mode UE.  Furthermore, for Alt.1, to reduce signaling overhead, only parameters subject to LBT results need to be configured in DCI, e.g. starting OFDM symbol and frequency resources etc.  And the other parameters including PRACH periodicity, format and RO number per PRACH slot can still be acquired by RMSI.

Proposal 8: Semi-static resource allocation of PRACH supported in NR could be the baseline of resource allocation design of PRACH in NR-U.

Proposal 9: Dynamic scheduling of PRACH occasion via DCI (i.e. Alt.1) and via DRS (i.e. Alt.3) shall be supported in NR.

In Rel-15 NR, an association between RACH resource(s) and SS/PBCH block(s) is established. In NR-U, actual transmission of one SSB is associated with multiple SSB candidate positions to overcome uncertain LBT results. For Alt.2 (i.e. cyclical wrapping’s method), it is natural to map multiple candidate SSB positions in different cyclically wrapped periodicity to same RO. For example, SSB(s) which satisfy SSB candidate position(s) % X = k will be all mapped to the k-th RO in RO pool.

In R15, open loop power parameters for PRACH are semi-statically configured by RMSI. Different power parameters are needed for ROs associated with different SS/PBCH blocks (i.e. different large scale fading).While in NR-U, an extra RO can be dynamically scheduled via DCI. Thereby, an extra power parameter should be accordingly updated in the new scheme. To achieve this, a more flexible power control scheme for RACH except for semi-static manner should be studied in NR-U.

Proposal 10: Dynamic power allocation for PRACH should be supported in NR-U.

In R15 NR, multiple Msg.1 transmission before RAR reception had been extensively discussed and agreed for CFRA. While due to limited time budget for detailed design, it was not be captured in specification.

In TR38.889 [4], multiple Msg1 transmissions before RAR reception and group wise SSB-to-RO mapping in time domain had been captured as alternative of PRACH enhancement schemes. In NR-U, when RACH retransmission occurs, the access delay of the RACH procedure may be seriously deteriorated. As shown in Fig 5, less LBT and reduced access delay can be achieved by multiple Msg.1 transmissions, wherein duplicated Msg.1 transmissions plus corresponding response window can be constrained into a COT. Thereby, it is potential beneficial to support multiple Msg.1 transmission in NR-U.

In addition, multiple beam sweeping is not necessary and lower transmission power (i.e.18dBm) is assumed in unlicensed band, thereby, the interference issue due to multiple RACH transmissions in licensed band will be significantly alleviated.

Proposal 11: Multiple Msg1 transmissions for both CBRA and CFRA should be supported in NR-U.

Fig 5. Illustration of multiple Msg1 transmission

In licensed band, a UE attempt to detect a DCI with CRC scrambled by RA-RNTI within a RAR window corresponding to the RO where a PRACH is transmitted. Considering that multiple ROs within a group are actually associated with the same UE/preamble index. When multiple Msg1 transmissions before RAR reception were supported, a common RNTI associated with these Msg1 is beneficial to reduce UE’s blind detection complexity and the probability of false alarm. Alternatively, this common RNTI can be determined by a reference RO in this group and the group information of ROs can be configured by the RMSI. For example, this reference RO can be the first RO or the last RO in the group.

Proposal 12: When multiple Msg1 transmissions before Msg2 reception is configured, a common RA-RNTI is used for all the ROs where msg1 are duplicated transmitted.

In licensed band, power ramping is performed according to the configured value (herein termed as delta) in the time occasion that is out of the RAR window, while in unlicensed band, as discussed above, it is potential that UE could also send Msg1 for multiple times within the RAR window. Considering that the access chance is precious due to the requirement of being LBT successful, while the gNB might suffer from the unsuccessful detection of Msg1, it should be careful to set a suitable power for the transmission of Msg1 within the RAR window to try to ensure the successful detection. Therefore, it is suggested that within the RAR window, power ramping could be performed for Msg1. Since this is additional chance for Msg1 transmission, it is proposed to use delta1 as power ramping value within the RAR window and use delta2 as power ramping value while outside the RAR window, wherein delta1 and delta2 could be separately configured by the gNB.

Proposal 13: Power ramping could be performed within the RAR window, and power ramping could be set separately for Msg1 transmission within and outside the RAR window.

RACH related procedure

Random access response

In NR, the UE expects to receive random access response (RAR) within a RAR time window after successful PRACH preamble transmission, which is determined by higher layer parameter. A RAR carries scheduling information of a PUSCH transmission from the UE (Msg3 PUSCH). If a UE does not detect the PDCCH with a corresponding RA-RNTI and a corresponding DL-SCH transport block within the time window, it retransmits the PRACH preamble. In unlicensed operation, the RAR transmission could be blocked due to LBT failure. In last meeting, it was identified beneficial to extend the maximum RAR time window size which provide chances for the gNB to transmit RAR when LBT failure occurs.

However, in NR unlicensed system, always setting a large value of RAR window may bring the accessing latency in case that the retransmission of Msg1 is due to detection failure of Msg.1 at the gNB. Consequently, suitable setting of RAR window is preferred by taking into account two factors, i.e., successful detection of Msg1 and potential LBT failure to send RAR by the gNB. Therefore, it would be beneficial to dynamically configure the RAR window size. The RAR window size should be increased with the number of Msg1 retransmission. The more times Msg1 is retransmitted, the higher probability RAR reception failure is due to LBT failure at the gNB side.

Proposal 14: It is beneficial to reduce RACH latency that RAR window could be configured with multiple values and the RAR window size could be increased with the number of Msg1 retransmission.

Msg3 PUSCH

As discussed above, a UE will transmit Msg3 PUSCH according to the scheduling information indicated by the RAR. There is a minimum time requirement between the last symbol of a PDSCH reception conveying a RAR and the first symbol of a corresponding Msg3 PUSCH transmission for a UE. Due to the time duration limitation of COT and required time gap between RAR and Msg3, they may not be able to be transmitted within the same COT. At the end of time duration of COT, gNB will stop data transmission and do LBT procedure to obtain new COT. Due to the uncertainty of LBT result, the delay of Msg3 transmission may become large.  To reduce access latency and increase Msg3’s transmission opportunity, it is beneficial to allow multiple Msg3’s transmission occasions out of COT.   UE can do LBT before each transmission occasion in non-COT duration after RAR grant is received, and Msg3 is allowed to transmit if LBT is successful. For example, gNB can configure multiple Msg3 transmission occasions (e.g. number) to UE when the transmission of Msg.3 is even out of COT.  An illustrative procedure can be found in following Fig 6. When Msg2 is transmitted at the end of COT N and the remaining symbols are all downlink symbols in this COT, it is foreseeable that the transmission of Msg3 will be out of COT N. If the transmission of Msg3 can be located in next COT N+1, UE sends Msg3 with a single transmission occasion within COT. Otherwise, UE can do LBT until the try is up to configured number of Msg3’s transmission occasions out of COT.

Fig 6. More transmission occasions for Msg3 out of COT

Furthermore, to support this cross-COT transmission scheme, concept of 2-step triggered scheduling mechanism in LTE eLAA could be considered, i.e., Msg3 is triggered to transmit in the subsequent COT and the scheduling information of trigger DCI is indicated by RAR in the previous COT.

Proposal 15: NR-U should support following mechanisms to reduce transmission latency of Msg3 in 4-step RACH:

- Allocating multiple Msg3’s transmission occasions

- 2-step triggered mechanism

PDSCH with UE contention resolution identity

In response to an Msg3 PUSCH transmission, the UE attempts to detect a PDCCH with a corresponding TC-RNTI or C-RNTI scheduling a PDSCH that includes a UE contention resolution identity, and it transmits HARQ-ACK information in a PUCCH. Similar to RAR transmission discussed above, Msg4 could also be enhanced in the same manner in NR-U.

Paging occasion in frequency domain

Paging is used to inform one or a group of UE(s) the arrival of incoming data or SI changing. In Rel-15 NR, the UE need to monitor the Type0-PDCCH to search the paging message using the DRX parameters provided in the system information, which is similar to LTE paging mechanism, during the cell search. Bandwidth part (BWP) is introduced in NR to support flexible channel bandwidth within a single wideband component carrier (CC), multiple SS/PBCH blocks could be transmitted in the frequency domain within a wideband CC. NR-U will support the same wideband operation. Considering the receiving bandwidth UE supports, a paging message corresponding to a sub-group of UEs, as illustrated in Fig 7, will only be associated to a single default BWP within the wideband CC in order to reduce the paging overhead. A gNB should configure different default BWPs for a UE to reduce the paging overhead.

Fig 7. NR-U POW mapping in multiple BWPs

From another perspective, paging transmission opportunities may be reduced due to LBT failure, which may lead to fatal system messages loss at UE side, e.g. updated system information, ETWS, CMAS, and etc. To increase UE’s paging transmission opportunity, paging/paging monitoring occasion enhancement in time domain had been captured in TR. Furthermore, to reduce UE’s power consumption and improve the battery life, the paging monitoring time span for RRC_IDLE UE should be minimized. A modified paging procedure can be considered to wake up UE with a limited time interval.

In this modified paging procedure, a shifting window in time domain are introduced together to increase paging occasions. When UE’s paging transmission (i.e. PO) in time domain is blocked due to LBT failure, a UE can shift this PO to next available position within a shifted paging window.

Proposal 16: The paging occasion of a UE can be associated to a single BWP within the wideband CC in order to reduce the paging overhead.

Proposal 17: The paging monitoring occasion of a UE can be shifted in a time window within a DRX cycle.

Enhancement on RRM/RLM

In Rel-15 NR, the downlink radio link quality of the serving cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE could be configured with a set of periodic CSI-RS resources within the active DL BWP. The maximum number of configured CSI-RS resources depends on the maximum number of SS/PBCH blocks within a half radio frame. The physical layer in the UE indicates, in the frame where the radio link quality is assessed, out-of-sync to higher layer when the radio link quality is worse that the configured threshold for all resources in the set of resource for radio link monitoring (RLM). When the radio link quality is better than the configured threshold for any resource in the configured resource set, the physical layer in the UE indicates in-sync to higher layers.

In NR-U, periodic CSI-RS resources configured for RLM, which are transmitted at the scheduled occasions, cannot always be guaranteed due to uncertainty of LBT. In case of CSI-RS transmission fails in the frames that the UE is configured for radio link quality assessment, it may declare the radio link failure even if the radio link quality is still good. The UE may also perform re-connection by a PRACH signal transmission, which could increase the UE’s power consumption and decrease the spectral utilization efficiency.

Based on the above discussion, enhancement on CSI-RS based measurements shall be investigated, i.e., robust CSI-RS transmission scheme by introducing more CSI-RS transmission opportunities and/or CSI-RS transmission status indication. As illustrated in Fig 8, assuming each radio link quality assessment window contains three CSI-RS transmission occasions (TO). In case A, the UE will indicate out-of-sync to high layers since all CSI-RS transmission fails due to LBT. In case B, the gNB may transmit the CSI-RS replacement that triggered by PDCCH until the beginning of the next pre-configured CSI-RS TO when the transmission fails at the current TO. Once the UE receives the CSI-RS replacement, it knows that the previous CSI-RS is missing due to LBT. In addition, the gNB could only indicate the previous CSI-RS transmission status, as shown in case C, to the UE via PDCCH. The UE may then update the measurement results according to the indication carried in DCI.

Fig 8. NR-U CSI-RS transmission schemes

Proposal 18: The gNB shall indicate the transmission status of CSI-RS that is configured for RLM/RRM to UE.

As shown in following Fig 9, if the UE detects n310 consecutive out-of-sync indications, it starts timer t310. If the timer expires, UE claims link failure. Otherwise, if UE detects consecutive n311 in-sync indications prior to timer t310 expiration, the timer is reset and the link is claimed to be recovered.

Fig 9. NR-U in-sync and out-of-sync indications

In TR 38.889[5], a new metric for identification of unsuccessful RLM-RS detection was proposed. Different from R15 NR, the unsuccessful reception may be induced by LBT failure or serious channel degradation.

Once receiving UE’s request, the gNB could send actual transmission status of one or multiple RLM RS(s) to the UE. Based on this information, UE could update its timer(s) accordingly. This is beneficial since more accurate radio link monitoring result is obtained and undesired link reconnection could be avoided by the UE. As illustrated in Fig 10, a UE sends report to the gNB as timer T310 starts. However, the UE will reset timer T310 and update counter N310 after receiving feedback from the gNB. To this end, LBT impact on NR-U RLM is mitigated.

Fig 10. Handshaking between gNB and UE for RLM

Proposal 19:  UE could update its timer based on the gNB’s indication by reporting non-detected RLM RS, which can also mitigate LBT impact on NR-U RLM.

For RRM measurement in NR-U, to detect hidden node, UE reporting of RSSI measurements to gNB is considered useful. The RSSI measurement and report can also reflect the load condition around UE. Except for RSSI report, channel occupancy percentage which can help gNB to know the overview of channel occupancy and medium contention should also be considered. This channel occupancy percentage is defined as percentage of measurement samples that RSSI value is above a threshold.

In LAA, report of RSSI and channel percentage had been supported. If there is no new issue to be addressed, same design can be reused and adopted for NR-U, e.g. RMTC per a frequency to perform RSSI measurement and to report RSSI and channel occupancy.

Proposal 20: The report mechanism of RSSI and channel occupancy percentage in LAA could be the baseline of RSSI and channel percentage report in NR-U.

Conclusions

In this contribution, we mainly discuss enhancements on SS/PBCH block transmission as part of DRS and PRACH. Considerations on paging and CSI-RS based measurements are also discussed. Based on the discussion, we have made the following proposals:

Proposal 1: Considering the standard impact, Alt-2 should be used for SS/PBCH block transmission in NR-U.

Proposal 2: Shift granularity of SS/PBCH blocks positions/candidate groups of SS/PBCH blocks depends on the deployment scenarios. In CA mode, the shift granularity could be at least one SS/PBCH block location, while in DC/SA mode the shift granularity could be one DRS block.

Proposal 3: QCL assumption indication of SS/PBCH block should be indicated only by PBCH DMRS. The number of PBCH DMRS sequence in NR R15 is sufficient for NR-U.

Proposal 4: A UE acquires serving cell time synchronization based on both PBCH DMRS and at least wrap around indication in PBCH payload.

FFS: other parameters such as DRS block length, the number of transmitted DRS block.

Proposal 5: If CAT 2 LBT is adopted for DRS transmission, the maximum allowed transmission duration of DRS is 1ms.

Proposal 6: The maximum number of candidate SSB position (Y) should be decided according to the minimum periodicity of DRS, maximum length of DMTC window and SSB pattern within a slot.

Proposal 7: PBCH within SS/PBCH block can be used to indicate the inclusion of RMSI, OSI and paging in DRS.

Proposal 8: Semi-static resource allocation of PRACH supported in NR could be the baseline of resource allocation design of PRACH in NR-U.

Proposal 9: Dynamic scheduling of PRACH occasion via DCI (i.e. Alt.1) and via DRS (i.e. Alt.3) shall be supported in NR.

Proposal 10: Dynamic power allocation for PRACH should be supported in NR-U.

Proposal 11: Multiple Msg1 transmissions for both CBRA and CFRA should be supported in NR-U.

Proposal 12: When multiple Msg1 transmissions before Msg2 reception is configured, a common RA-RNTI is used for all the ROs where msg1 are duplicated transmitted.

Proposal 13: Power ramping could be performed within the RAR window, and power ramping could be set separately for Msg1 transmission within and outside the RAR window.

Proposal 14: It is beneficial to reduce RACH latency that RAR window could be configured with multiple values and the RAR window size could be increased with the number of Msg1 retransmission.

Proposal 15: NR-U should support following mechanisms to reduce transmission latency of Msg3 in 4-step RACH:

- Allocating multiple Msg3’s transmission occasions

- 2-step triggered mechanism

Proposal 16: The paging occasion of a UE can be associated to a single BWP within the wideband CC in order to reduce the paging overhead.

Proposal 17: The paging monitoring occasion of a UE can be shifted in a time window within a DRX cycle.

Proposal 18: The gNB shall indicate the transmission status of CSI-RS that is configured for RLM/RRM to UE.

Proposal 19:  UE could update its timer based on the gNB’s indication by reporting non-detected RLM RS, which can also mitigate LBT impact on NR-U RLM.

Proposal 20: The report mechanism of RSSI and channel occupancy percentage in LAA could be the baseline of RSSI and channel percentage report in NR-U.

References

RAN Chairman’s Notes, 3GPP TSG RAN Meeting #82, Sorrento, Italy, Dec. 10-13, 2018

Qualcomm Inc., “New WID on NR-based Access to Unlicensed Spectrum” , RP-182878, Sorrento, Italy, Dec. 10-13, 2018

R1-1901522,  “Initial access signal and channels in NR unlicensed band”, Huawei, HiSilicon

ETSI EN 301 893 (2.1.1),“Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”

3GPP TR 38.889 (v16.0.0): “Study on NR-based access to unlicensed spectrum ".