3GPP TSG RAN WG1 Meeting #96 			                                                                 	R1-1901874

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	7.2.2.1.2

Source:	Charter Communications

Title:	On Initial Signal Design

Document for:	Discussion/Approval

# Introduction

Following the recently concluded study item on the feasibility of using NR in unlicensed spectrum, the NR-U WI is in progress in RAN1 [1]. In the last meeting, the following agreement was reached regarding DL signals and channels [2]:

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

- Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

- FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

In this contribution, we further discuss the role of non-NR-based signals/channels as an initial signal.

# NR-U Initial Signal

The NR-U SI phase extensively discussed the role and structure of an initial signal in NR-U. In RAN1#93, the following agreement was reached regarding the purpose(s) of an initial signal:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

Unfortunately, the subsequent discussions tried to address each scenario individually, instead of studying a holistic design that can address all of the above use cases. This trend continues in the WI, where the agreement in Section 1 only focuses on DL transmission burst detection. A more comprehensive assessment of the options is shown in Table 1, setting aside implementation/complexity concerns for now.

Table 1: Comparison of initial signal options

| Use case | DM-RS | PDCCH | 802.11a/802.11ax preamble |
| --- | --- | --- | --- |
| Power saving due to avoidance of blind PDCCH decodes | Yes | No | Yes |
| Power saving due to indication of COT structure | No | Yes | Yes* |
| Improved coexistence | No | No | Yes† |
| Spatial reuse within same operator network | No | No | Yes‡ |
| Serving cell burst identification | Yes | Yes | Yes* |
| *: Requires modification in 802.11ax preamble PLCP and/or MAC header †: Preamble-based coexistence has been shown to satisfy the fair coexistence criterion in TR 38.889 ‡: 802.11ax BSS color/spatial reuse group feature |  |  |  |

The pros and cons of NR-based versus non-NR-based signals for NR-U have been discussed repeatedly and no further reconciliation is deemed possible for the two perspectives. In order to make progress, three high-level observations can then be made as follows:

Observation 1: It is unreasonable to not utilize NR signals, such as DM-RS that will anyway be present, for any applicable initial signal use cases, such as serving cell transmission identification.

Observation 2: Customizing the 802.11ax preamble to carry NR-U PCI/RAT identifier/COT structure would potentially make it non-compatible with legacy Wi-Fi devices in the 5 GHz band. Therefore, customized preambles are more suitable for ‘greenfield’ bands such as 6 GHz.

Observation 3: Certain network operators with existing 802.11ax deployments may want NR-U devices to also transmit/receive preambles in the case of overlapping, co-channel deployments.

Therefore, for the case of 5 GHz, the use of preambles can be left to implementation. In terms of specification impact, two options should then be allowed for channel access: ED-only LBT and preamble+ED detection. This would be similar to the two options allowed in ETSI EN BRAN 301893. For the case of 6 GHz, a customized preamble that meets all use cases in Table 1 and can be decoded by both NR-U and 802.11ax is suggested to be adopted and pursued further in consultation with other stakeholders in unlicensed spectrum.

Proposal 1:  For the case of 5 GHz, the use of preambles can be left to implementation. In terms of specification impact, two options are allowed for channel access: ED-only LBT and preamble+ED detection.

Proposal 2: For the case of 6 GHz, a customized preamble that also carries NR-U RAT identifier/PCI/COT structure and can be decoded by both NR-U and 802.11ax is suggested to be adopted and pursued further in consultation with other stakeholders in unlicensed spectrum.

Finally, regarding implementation complexity, some sources have claimed that decoding 802.11 signals entails the support of a new sampling frequency by NR hardware, while another potential issue is the long delay (up to several hundred ms) needed to change BWPs of a different subcarrier spacing. While this is up to implementation at least for 5 GHz, it is noted that the NR modem need not directly decode the 802.11 preamble, since a secondary WiFi modem can be used for the purposes of preamble detection/decoding and passing the results to the NR scheduler, as is the case in earlier variants of 3GPP technologies in unlicensed spectrum [3].

# Summary

In this contribution we examined the use of non-NR signals/channels as an initial signal for NR-U. The following observations and proposals ensued.

Observation 1: It is unreasonable to not utilize NR signals, such as DM-RS that will anyway be present, for any applicable initial signal use cases, such as serving cell transmission identification.

Observation 2: Customizing the 802.11ax preamble to carry NR-U PCI/RAT identifier/COT structure would potentially make it non-compatible with legacy Wi-Fi devices in the 5 GHz band. Therefore, customized preambles are more suitable for ‘greenfield’ bands such as 6 GHz.

Observation 3: Certain network operators with existing 802.11ax deployments may want NR-U devices to also transmit/receive preambles in the case of overlapping, co-channel deployments.

Proposal 1:  For the case of 5 GHz, the use of preambles can be left to implementation. In terms of specification impact, two options are allowed for channel access: ED-only LBT and preamble+ED detection.

Proposal 2: For the case of 6 GHz, a customized preamble that also carries NR-U RAT identifier/PCI/COT structure and can be decoded by both NR-U and 802.11ax is suggested to be adopted and pursued further in consultation with other stakeholders in unlicensed spectrum.

# Reference

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, RAN#82

- Chairman’s Notes, RAN1#1901 Ad hoc, January 2019.

- https://mentor.ieee.org/802.19/dcn/15/19-15-0057-00-0000-lte-u-forum-and-coexistence-overview.pdf