3GPP TSG RAN WG1 #96                                        R1-1901925

Athens, Greece, 25th February - 1st March, 2019

Source:	OPPO

Title:	On configured grant for NR-U

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

In RAN1 Ad-Hoc Meeting 1901, the following agreements were achieved for configured grant (CG) uplink transmission:

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

In this contribution, some considerations on configured grant uplink transmission for NR-U are discussed. This contribution is a revision of [1].

Resource configuration

In NR CG transmission, time domain resource is determined by periodicity, time domain offset, starting symbol and transmission duration based on the configured subcarrier spacing through RRC configuration. In FeLAA, the resource configuration periodicity is fixed to 40 subframes, a set of subframes used for autonomous uplink (AUL) transmission are indicated through 40 bits bitmap by RRC parameters and activated/deactivated by dynamic signalling. It is slightly preferred to follow FeLAA rules to enhance the resource configuration of CG-PUSCH in time domain for NR-U.

Following FeLAA rules, the resource allocation unit for each bit could be changed from “subframe” to “transmission duration”, and the transmission duration could be pre-configured in RRC parameters. Figure 1 shows one example of time domain resource configuration with each bit indicating 4 slots.

Figure 1: Example of bitmap based resource configuration in time domain

For a UE configured with AUL transmission in FeLAA, firstly the UE will be RRC configured with a set of subframes which may be used for AUL transmission. Those AUL subframes are activated and de-activated by a PDCCH command. The activation command also indicates PRB allocation, MCS, DMRS cyclic shift and orthogonal cover code to the UE. The UE may initiate an AUL transmission after receiving the activation command, or the UE may skip an AUL resource if there is no data in UL buffers.

Similarly, for resource allocation in frequency domain for NR-U, dynamically indicated signaling along with activation command can be considered to achieve better multiplexing between scheduled PUSCH and CG-PUSCH.

Proposal 1: RRC configured resource with bitmap indication in time domain and dynamically allocated resource in frequency domain can be considered for CG-PUSCH.

UCI enhancement

According to the agreement, UCI which carries necessary demodulation information, i.e., HARQ process ID, NDI, RVID, and so on, should be piggybacked on PUSCH in NR-U CG transmission. In FeLAA, DMRS is transmitted at the fourth symbol of each slot and UCI is mapped from the second symbol to the thirteenth symbol in one subframe in case the first symbol or the last symbol in the subframe is punctured for LBT purpose. Following the same philosophy, the first symbol and the last symbol within one CG-PUSCH duration should not be used to transmit DMRS or UCI. On the other hand, DMRS and UCI should be placed at the beginning of the transmission duration as much as possible for fast demodulation. Assume the transmission duration for CG-PUSCH is one slot, Figure 2 gives an example for DMRS and UCI mapping pattern for NR-U CG transmission.

Figure 2: Illustration of DMRS and UCI mapping pattern for CG transmission

Proposal 2: The first symbol and the last symbol within one CG-PUSCH duration should not be used to transmit DMRS or UCI for LBT purpose.

Support link adaptation for CG-PUSCH transmission is beneficial, since it is hard for gNB to determine an accurate MCS/TBS for an UE-initiated PUSCH, while from UE perspective, it could make use of channel reciprocity to determine uplink channel quality by measure downlink reference signals in unlicensed carrier. In addition, the demodulation of CG-PUSCH and UCI piggybacked on CG-PUSCH are independent and gNB relies on the UCI result for CG-PUSCH demodulation, which makes it possible for UE to determine its own MCS/TBS parameters for CG-PUSCH and report the corresponding determined parameters in the UCI at least for the first transmission.

Proposal 3: Support UE determined MCS/TBS for CG-PUSCH.

HARQ and DFI

According to SI agreement, NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz, so it is possible that resource allocation in frequency domain span larger than 20MHz also for CG-PUSCH. To improve transmission efficiency for CG-PUSCH with potential large TBS, CBG based HARQ-ACK feedback and retransmission should be supported so that retransmission can be performed only for those erroneously decoded CBGs instead of the whole large TB.

Proposal 4: Support CBG based HARQ-ACK feedback for CG-PUSCH.

Downlink Feedback Information (DFI) including HARQ-ACK feedback should be transmitted to UE after demodulation of CG-PUSCH. If the HARQ-ACK feedback of one HARQ process is NACK, gNB may schedule a grant-based retransmission for the same HARQ process, or gNB may indicate the NACK information to UE so that UE could start the retransmission for the same HARQ process via the same configured grant resource for initial transmission.

In FeLAA, DFI information for all the HARQ process are transmitted via PDCCH. However, unlike TB-based HARQ-ACK feedback in LTE, CBG-based HARQ-ACK feedback is introduced in NR and should be supported for NR-U CG-PUSCH, which would lead to a large feedback payload and would impact on the performance of PDCCH.

To reduce the potential large HARQ-ACK payload for PDCCH, one option is to determine HARQ-ACK codebook semi-statically corresponding to CG-PUSCH resources within a pre-configured time window, as shown in Figure 3-a. Another option is to use dynamic HARQ-ACK codebook determination, in which case the UE should transmit uplink assignment index (UAI) information to gNB through UCI piggyback on CG-PUSCH to help gNB determine the corresponding HARQ-ACK codebook, as shown in Figure 3-b.

On the other hand, instead of reducing the large HARQ-ACK payload, full HARQ-ACK feedback transmission can be supported via PDSCH.

Proposal 5: Study how to transmit DFI in PDCCH for CBG-based HARQ-ACK feedback for CG-PUSCH.

Figure 3: DFI transmission for CBG-based feedback for CG-PUSCH

Conclusions

In this contribution, some considerations on CG transmission for NR-U are discussed. The following proposals are made.

Proposal 1: RRC configured resource with bitmap indication in time domain and dynamically allocated resource in frequency domain can be considered for CG-PUSCH.

Proposal 2: The first symbol and the last symbol within one CG-PUSCH duration should not be used to transmit DMRS or UCI for LBT purpose.

Proposal 3: Support UE determined MCS/TBS for CG-PUSCH.

Proposal 4: Support CBG based HARQ-ACK feedback for CG-PUSCH.

Proposal 5: Study how to transmit DFI in PDCCH for CBG-based HARQ-ACK feedback for CG-PUSCH.

References

- R1-1900275, “On configured grant for NR-U”, OPPO.