3GPP TSG RAN WG1 #96	R1-1901970

Athens, Greece, 25th February - 1st March, 2019

Title:                     Discussion on remaining issues of network coordination mechanisms

Source: 	ZTE

Agenda Item:	7.2.5.2

Document for:	Discussion and Decision

Introduction

The revised WID on NR CLI (Cross Link Interference) and RIM (Remote Interference Management) [1] was approved in RAN#82 plenary meeting. In CLI part, RAN1 aims to specify cross-link interference mitigation techniques to support flexible resource adaptation for unpaired NR cells. In RAN1 AH#1901, some further agreements were reached for network coordination mechanism(s) [2]. The detailed objectives for CLI in WID and the agreements reached in last RAN1 meeting are as follows:

- Specify cross-link interference measurements and reporting at a UE (e.g., CLI-RSSI and/or CLI-RSRP) [RAN1, RAN2, RAN4]

- Specify network coordination mechanism(s) including at least exchange of intended DL/UL configuration [RAN1, RAN3]

- Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels [RAN4]

  - Target no or very minimal impact on RF requirement

Note: Measurement and coordination mechanisms should be applicable to IAB nodes.

Agreement

For inter-gNB exchange of intended UL/DL configuration, time-domain resources indication is exchanged.

- The direction of time resources is designated as an intended DL slot(s)/symbol(s) or intended UL slot(s)/symbol(s).

- FFS: whether the remaining region which is not indicated as DL or UL is interpreted as unused or flexible

- FFS: detail message format

- Note: Need to further check this information (e.g. TDD DL/UL configuration, Actually Transmitted SSB, RACH configuration)

The indicated configuration is assumed to be valid until a new configuration is received

The above information exchange is not to mandate specific behavior at the receiving gNB

The above information exchange is not to mandate specific behavior at the transmitting gNB

In this contribution, we will show our views for remaining issues of network coordination mechanisms for CLI mitigation.

Discussion on network coordination mechanism(s)

Exchange of measurement resource configuration

Two metrics for CLI measurement identified in RAN1 AH#1901 [2] are copied below, i.e. SRS-RSRP and RSSI based UE-UE CLI measurement. Besides, in our companion contribution [4] we suggest that both NZP-SRS and ZP-SRS can be used for CLI measurement.

- ---------------------------------------------------------------------------------------------------------------------------------------------

The following CLI measurements are supported:

- SRS-RSRP:

  - Linear average of the power contributions of the SRS to be measured over the configured resource elements within the considered measurement frequency bandwidth in the time resources in the configured measurement occasions

- RSSI:

  - The linear average of the total received power observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over the configured resource elements for measurement by the UE

- ---------------------------------------------------------------------------------------------------------------------------------------------

For SRS and ZP-SRS based CLI measurement, the SRS senders and the SRS receivers are located in different cells. Therefore, adjacent gNBs should exchange/coordinate SRS configurations or measurement configurations and then configure them to their UEs before the UEs performing CLI measurement.

SRS configurations or measurement configurations exchanging between gNBs may include: SRS ports / symbols / symbol offsets / comb, sequence identifications, cyclic shifts, group or sequence hopping, transmission or measurement periodicity, slot offset, power setting and measurement triggering, etc. In order to reduce the exchanged signaling overhead, some parameters can be fixed  to a constant value via standardization, e.g. port, SRS symbols and its position in a slot.

Proposal 1: Network coordination mechanism should support exchange of SRS configurations or measurement configurations for CLI measurement.

Exchange of TDD DL/UL configuration

In NR Rel-15, NR slot format is more flexible and can be (re-)configured by high layer parameter TDD-UL-DL-ConfigurationCommon (including pattern1 and pattern2 if it is configured), TDD-UL-DL-ConfigDedicated, PDCCH for DCI format 2-0 or via DCI scheduling, etc. Take high layer parameter  TDD-UL-DL-ConfigurationCommon as a example, the slot format in a configuration period can be include three parts: DL slot(s)/symbol(s), flexible slot(s)/symbol(s), UL symbol(s)/slot(s). Only flexible slot(s)/symbol(s) in a configuration period can be reconfigured to DL or UL slots(s)/symbol(s) by tdd-UL-DL-ConfigDedicated, PDCCH for DCI format 2-0, or DCI scheduling. Furthermore, PDCCH for DCI format 2-0 or DCI scheduling can only change the directions of flexible part configured by semi-static UL/DL configuration.

According to above analysis on the existing specification in Rel-15, we can see that the characteristic of NR slot format, i.e. flexible resource adaptation named in WID (or called duplexing flexibility / dynamic TDD in the earlier days), is mainly reflected in the flexible part which can be dynamically re-configured.

One option (Option 1) for an intended UL/DL configuration is reusing a semi-static UL/DL configuration given by  TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated. In Option 1, only such a semi-static UL/DL configuration is needed to be exchanged between gNBs. It can only help to solve the cross-link interference caused by the direction misalignment in the DL/UL part configured by semi-static configuration. However, it is hardly helpful in resolving the cross-link interference generated in flexible part at both two gNBs sides. Therefore, this option is almost useless for cross-link interference mitigation to support flexible resource adaptation in flexible part.

Observation 1: Exchange of a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated can at least help to solve the cross-link interference caused by the direction misalignment in the DL/UL part configured by above semi-static configuration.

Observation 2: If only a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated, as an intended UL/DL configuration, is exchanged between the gNBs, it is hardly helpful in resolving the cross-link interference to support flexible resource adaptation in the flexible slots/symbols configured by above high layer parameters.

Option 2 is that a whole intended UL/DL configuration consists of two parts: Part I with a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated, and Part II with an intended UL/DL configuration on flexible part configured by above high layer parameters. Part II is not equivalent to dynamic UL/DL configuration configured by PDCCH for DCI format 2-0 or DCI scheduling. Obviously, dynamic UL/DL configuration exchanged between gNBs is unrealistic due to time delay of backhaul transmission. Part II can also configured as a semi-static manner and only gives the “intended” direction information of flexible part. The “intended” direction can reflect the the information of direction priorities, but it is not necessarily the actual direction of subsequent  transmission.

There are two sub-options to exchange the information of a whole intended UL/DL configuration (Part I and Part II) in Option 2 between inter-gNBs.

Option 2-1: Part I and Part II can be informed to the neighboring gNBs through a unified set of parameters. This option does not need to differentiate Part I and Part II in the parameters, which are transparent to the receiving gNBs. Therefore, the receiving gNB can not distinguish which part of the transmission gNB is semi-statically configured and which part can be dynamically configured.

Option 2-2: Part I and Part II can be exchanged independently between inter-gNBs, i.e. by two sets of parameters. Based on two different sets of parameters, the receiving gNB can know which part of the transmission gNB is semi-statically configured and which part can be dynamically configured.

For Option 2-2, The receiving gNB  may take such information into account when setting its configuration and scheduling policy, e.g. align their semi-static configuration and apply sensing before the transmission in a lower priority direction in flexible part. For example, the message “DL DL DL UL UL ” for flexible slots/symbols transmission is transferred by the gNB1 to the gNB2. It means gNB1 sets DL direction in the first three slots/symbols and UL direction in the last two slots/symbols to a high priority. Which also means UL direction in the first three slots/symbols and DL direction in the last two slots/symbols has a lower priority. If the device sends the DL data in the first three slots/symbols, it does not need to perform CLI mitigation scheme (e.g. sensing) since the direction DL has a high priority in these slots/symbols. If the device sends the UL data in the first three slots/symbols, it needs to perform sensing since the priority of direction UL in these slots/symbols is lower.  Based on sensing, if strong cross-link interference is detected, the device can stop or delay the data transmission, or even reconfigure/schedule these slots/symbols for another direction transmission.

- The sets of parameters for exchanging intended UL/DL configuration can include numerology, configuration period, direction information, etc.  The remaining slot(s)/symbol(s) in a configuration period which is not indicated as DL or UL by above two sets of parameters are flexible slot(s)/symbol(s).

Observation 3: If a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated and an intended UL/DL configuration on the flexible slots/symbols, composing a whole intended UL/DL configuration, is exchanged between the gNBs by a unified set of parameters, the receiving gNB can not distinguish which part of the transmission gNB is semi-statically configured and which part can be dynamically configured.

Proposal 2: A whole intended UL/DL configuration should include two parts:

- Part I: a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated

- Part II: an intended UL/DL configuration on the flexible slots/symbols

Proposal 3: The parameters exchanged between the gNBs to indicate the whole intended UL/DL configuration should include two different set of parameters:

- One set for the semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated

- The other set for  the intended UL/DL configuration on the flexible slots/symbols

Proposal 4: The remaining slot(s)/symbol(s) in a configuration period which is not indicated as DL or UL by exchanged parameters are flexible slot(s)/symbol(s).

Besides, the impacts brought by CLI on some important reference signals/channels such as SS/PBCH block, CORESET0, and PRACH should be considered. Note that exchange of semi-statically used resources (to avoid collisions with, e.g., PSS/SSS, PRACH, PBCH) exchange between LTE eNB and NR gNB via X2/Xn interface had already been supported in LTE-NR coexistence scenario. Thus, RAN1/3 can do some enhancements referring to this. Another option is to set the directions of flexible slots/symbols on which SS/PBCH block, CORESET0, and PRACH will be transmitted as intended direction “DL” or “UL”.

Proposal 5: Two alternatives can be considered to reduce the impacts of CLI on SS/PBCH, CORESET0, and PRACH.

- Alt.1: Exchange of time/frequency resource configuration of SS/PBCH, CORESET0, and PRACH

- Alt.2: Set the directions of flexible slots/symbols on which SS/PBCH block, CORESET0, and PRACH will be transmitted as intended direction “DL” or “UL”

Evaluation on network coordination mechanisms

Based on sensing based CLI mitigation scheme and network coordination mechanisms (via exchange of intended UL/DL configuration), we evaluate the performance of dynamic TDD with sensing/coordination, dynamic TDD without sensing/coordination, and static TDD. Table 1-3 give evaluation results of DL and UL performance for above different TDD cases in high load, medium load and low load.  Evaluation assumptions can refer to appendix in the contribution.

Evaluations show that dynamic TDD with and without sensing/coordination scheme  provides better UPT compared to static TDD. Evaluations also show that dynamic TDD with sensing/coordination scheme provides better UPT compared to static TDD and dynamic TDD without sensing/coordination scheme.

Observation 4: Dynamic TDD with and without sensing/coordination scheme  provides better UPT compared to static TDD.

Observation 5: Dynamic TDD with sensing/coordination scheme provides better UPT compared to static TDD and dynamic TDD without sensing/coordination scheme.

Proposal 6: The CLI measurement, exchange of intended DL/UL transmission direction, and sensing scheme can be combined for CLI mitigation in dynamic TDD.

----------------------------------------------------------------------------------------------------------------------------------------------------------

Schemes:
	- Static TDD ： The DL:UL ratio for the allocated slot is fixed and the same DL:UL ratio is used by all nodes in the network. The scheme is the baseline.
	- Dynamic TDD without sensing/coordination：The change of transmission direction/transmission direction is dependent on the incoming traffic and the scheduler decisions and any slot can transmit DL or UL traffic.
	- Dynamic TDD with sensing/coordination：The method of dynamic TDD is used along with a sensing operation at the gNB or UE before DL transmission (e.g. the UE  performs sensing on the DL slot, if successful the UE can transmit its UL traffic on the DL slot) or UL transmission (e.g. the gNB performs sensing on the UL slot, if successful the gNB can transmit its DL traffic on the UL slot).

Table1. DL and UL Performance for different TDD cases in high load

| TDD Cases |  | Static TDD | Dynamic TDD | Dynamic TDD with sensing |
| --- | --- | --- | --- | --- |
| DL UPT CDF [Mbps] | 5% | 13.430 | 8.806 | 23.615 |
|  | 50% | 44.318 | 60.978 | 74.204 |
|  | 95% | 79.367 | 113.135 | 129.204 |
|  | Mean | 45.525 | 59.525 | 74.784 |
|  | Served / Offered(%) | 99.123 | 98.596 | 100.000 |
|  | RU (%) | 10.693 | 14.723 | 8.119 |
| UL UPT CDF [Mbps] | 5% | 1.605 | 15.315 | 36.352 |
|  | 50% | 5.860 | 53.333 | 78.895 |
|  | 95% | 19.331 | 98.819 | 134.771 |
|  | Mean | 7.376 | 53.661 | 81.947 |
|  | Served/Offered (%) | 87.543 | 98.464 | 100.000 |
|  | UL RU (%) | 26.722 | 12.959 | 9.788 |
| 𝜆 (files/s) |  | 0.24 |  |  |

Table2. DL and UL Performance for different TDD cases in medium load

| TDD Cases |  | Static TDD | Dynamic TDD | Dynamic TDD with sensing |
| --- | --- | --- | --- | --- |
| DL UPT CDF [Mbps] | 5% | 17.621 | 19.663 | 32.754 |
|  | 50% | 54.549 | 95.879 | 97.088 |
|  | 95% | 100.000 | 166.667 | 166.667 |
|  | Mean | 54.557 | 96.737 | 96.188 |
|  | Served / Offered(%) | 99.376 | 98.961 | 100.000 |
|  | RU (%) | 7.249 | 6.369 | 5.115 |
| UL UPT CDF [Mbps] | 5% | 2.886 | 17.671 | 39.137 |
|  | 50% | 13.234 | 74.922 | 107.303 |
|  | 95% | 41.222 | 123.574 | 165.079 |
|  | Mean | 16.303 | 78.263 | 107.651 |
|  | Served/Offered (%) | 98.973 | 99.384 | 100.000 |
|  | UL RU (%) | 16.384 | 6.743 | 5.722 |
| 𝜆 (files/s) |  | 0.20 |  |  |

Table3. DL and UL Performance for different TDD cases in low load

| TDD Cases |  | Static TDD | Dynamic TDD | Dynamic TDD with sensing |
| --- | --- | --- | --- | --- |
| DL UPT CDF [Mbps] | 5% | 6.221 | 7.962 | 15.504 |
|  | 50% | 81.854 | 141.939 | 134.615 |
|  | 95% | 102.632 | 166.667 | 166.667 |
|  | Mean | 76.839 | 127.497 | 126.801 |
|  | Served / Offered(%) | 100.000 | 100.000 | 100.000 |
|  | RU (%) | 2.099 | 1.961 | 1.714 |
| UL UPT CDF [Mbps] | 5% | 8.969 | 13.245 | 37.735 |
|  | 50% | 54.323 | 108.108 | 155.844 |
|  | 95% | 76.952 | 140.741 | 190.476 |
|  | Mean | 49.195 | 95.170 | 138.589 |
|  | Served/Offered (%) | 99.026 | 99.026 | 100.000 |
|  | UL RU (%) | 2.693 | 2.846 | 1.898 |
| 𝜆 (files/s) |  | 0.12 |  |  |

- NOTE 1:	- RU for a link direction (DL or UL) is defined as the amount of occupied resources for the given link direction divided by the total number of resources  (irrespective of link directions).
-  DL and UL Performance for different TDD cases

Conclusion

In this contribution, we discuss remaining issues of network coordination schemes for CLI mitigation, and have the following observations and proposals:

Observation 1: Exchange of a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated can at least help to solve the cross-link interference caused by the direction misalignment in the DL/UL part configured by above semi-static configuration.

Observation 2: If only a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated, as an intended UL/DL configuration, is exchanged between the gNBs, it is hardly helpful in resolving the cross-link interference to support flexible resource adaptation in the flexible slots/symbols configured by above high layer parameters.

Observation 3: If a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated and an intended UL/DL configuration on the flexible slots/symbols, composing a whole intended UL/DL configuration, is exchanged between the gNBs by a unified set of parameters, the receiving gNB can not distinguish which part of the transmission gNB is semi-statically configured and which part can be dynamically configured.

Observation 4: Dynamic TDD with and without sensing/coordination scheme  provides better UPT compared to static TDD.

Observation 5: Dynamic TDD with sensing/coordination scheme provides better UPT compared to static TDD and dynamic TDD without sensing/coordination scheme.

Proposal 1: Network coordination mechanism should support exchange of SRS configurations or measurement configurations for CLI measurement.

Proposal 2: A whole intended UL/DL configuration should include two parts:

- Part I: a semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated

- Part II: an intended UL/DL configuration on the flexible slots/symbols

Proposal 3: The parameters exchanged between the gNBs to indicate the whole intended UL/DL configuration should include two different set of parameters:

- One set for the semi-static UL/DL configuration configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated

- The other set for  the intended UL/DL configuration on the flexible slots/symbols

Proposal 4: The remaining slot(s)/symbol(s) in a configuration period which is not indicated as DL or UL by exchanged parameters are flexible slot(s)/symbol(s).

Proposal 5: Two alternatives can be considered to reduce the impacts of CLI on SS/PBCH, CORESET0, and PRACH.

- Alt.1: Exchange of time/frequency resource configuration of SS/PBCH, CORESET0, and PRACH

- Alt.2: Set the directions of flexible slots/symbols on which SS/PBCH block, CORESET0, and PRACH will be transmitted as intended direction “DL” or “UL”

Proposal 6: The CLI measurement, exchange of intended DL/UL transmission direction, and sensing scheme can be combined for CLI mitigation in dynamic TDD.

Reference

- RP-182864, Revised WID on CLI handling and RIM for NR, LG, Dec. 10 - 13, 2018, RAN#82

- Draft_Minutes_report_RAN1#AH_1901_v010

- R1-1900409, Discussion on backhaul information exchange for CLI management, CMCC

- R1-1901969, Discussion on remaining issues of CLI measurement and reporting, ZTE

- Appendix

Table I. Simulation assumptions

| Parameters | Indoor scenario |
| --- | --- |
| Layout | Indoor floor: (12 BSs per 120m X 50m) |
| Inter-BS distance | 20m |
| Minimum BS-UE (2D) distance | 0m |
| Minimum UE-UE (2D) distance | 3m |
| System bandwidth | 20MHz per CC |
| Carrier frequency | 4.0GHz |
| Number of carriers | 1 |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| Channel model | TRP-to-UE: ITU InH TRP-to-TRP: ITU InH UE-to-UE: A.2.1.2 in TR36.843 |
| BS antenna | Omni antenna model; (M,N,P,Mg,Ng) = (1, 1, 2, 1, 1); 2Tx, 2Rx |
| BS antenna height: | 3m |
| UE antenna | Omni; 2Tx, 2Rx |
| UE antenna height | 1.5m |
| eNB antenna element gain | 5dBi |
| Antenna gain of UE | 0 dBi |
| UE distribution | 10 users per TRP; 100% indoor (3km/h) |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE power control | Full power |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes |
| Traffic load | Downlink and uplink traffic ratios = {1:1} |
| Static TDD configuration | Configuration 1(DL:UL= 6:4) |