3GPP TSG RAN WG1 Meeting #96	                                                                          R1-1902983

Athens, Greece

Feb. 25th – Mar. 1st, 2019

Agenda item:    7.2.2.1.2

Source:             Qualcomm Incorporated

Title:                  DL signals and channels for NR-U

Document for:  Discussion and Decision

# Introduction

The objectives of the NR-U WI [1] include specifying the following for DL signals and channels for NR-U:

- Subcarrier spacing for control and data channels supporting 15kHz, 30kHz, and 60kHz (air-interface perspective; optionality to be discussed separately).

- For DL data channel, support of multiple PDSCH starting positions.

- Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption.

- DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

- Data multiplexing aspects (for both UL and DL) considering LBT and channel access priorities. (RAN1/RAN2)

The agreements and conclusions from the SI phase are captured in [2]. In this contribution, we present our views on some of the remaining aspects of DL signals and channels.

# Previous Agreements

RAN1#92bis – 7.6.2 Frame structure for NR-U operation

Agreement:

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- For sub-7 GHz, NR-U study the SCSs, 15/30/60KHz

  - Study performance difference between different SCS

  - Study if changes to UL design are needed to meet the PSD and OCB requirements

  - Study if an SS block design/RMSI/OSI with 60KHz SCS is needed

  - Impact on MIB and SIB1 content

  - Need for use of ECP for 60KHz

  - RACH design with 60KHz SCS in addition to options currently part of NR

  - Other considerations are not precluded.

  - Impact on support of different BWs with different SCS

- Study supporting more than one switching points within a TxOP

  - FFS the LBT requirement for each DL/UL data/control burst in the TxOP

RAN1#92bis – 7.6.3 Potential physical layer channel design

Agreements:

- Study the design changes needed to support the following channels /signals in NR-U

  - PDCCH/PDSCH

  - PUCCH/PUSCH

  - PSS/SSS/PBCH

  - PRACH

  - DL and UL reference signals applicable to the operational frequency range

RAN1#93 - 7.6.2 Frame structure for NR-U operation

- Agreement:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

RAN1#93 - 7.6.3.1	DL Signals and Channels

Agreement:

- NR-U should have a signal that contains at least SS/PBCH block burst set transmission

  - FFS: Other channels and signals transmitted together as part of the signal

- The design of this signal should consider the following characteristics specific to unlicensed band operation

  - There are no gaps within the time span the signal is transmitted at least within a beam

    - FFS: Whether any gaps are needed for beam switching and, if needed, their duration

  - The occupied channel bandwidth is satisfied (although this may not be a requirement)

  - Strive to minimize the channel occupancy time of the signal

  - Characteristics that may facilitate fast channel access

RAN1#94 - 7.2.2.3.1 DL Signals and Channels

Agreement:

- Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for

- Meeting OCB requirement

- Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

- Support of stand-alone NR-U deployments

- Support of automatic neighbour relations (ANR) functionality in an NR-U deployment

- Resolution of PCI confusion in an NR-U deployment

- Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals

- The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial

RAN1#94bis – 7.2.2.2 Frame structure for NR-U operation

Agreements:

- It has been identified to be beneficial for the NR-U design to not require the gNB to change a pre-determined TBS for a PDSCH transmission depending on the LBT outcome, at least when the PDSCH is transmitted at the beginning of the gNB’s COT.

- The following options have been identified as possible candidates for PDSCH transmission in the partial slot at least for the first PDSCH(s) transmitted in the DL transmission burst.

  - Option 1: PDSCH(s) as in Rel-15 NR

  - Option 2: Punctured PDSCH depending on LBT outcome

  - Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols

  - Option 4: PDSCH across slot boundary

  - FFS for signalling details, specification impact, implementation complexity

  - Note: Above options are not mutually exclusive.

- In addition to the functionalities provided by DCI format 2_0 in Rel-15 NR, indication of the COT structure in the time domain has been identified as being beneficial.

RAN1#95 - 7.2.2.3.1 DL Signals and Channels

Agreements:

- Support of Pattern 1 is recommended for multiplexing of SS/PBCH block(s) and CORESET(s)#0 in NR-U.

- As one element to facilitate a NR-U DRS design without gaps in the time domain, the CORESET#0 configuration(s) and/or Type0-PDCCH common search space configuration(s) may need enhancements compared to NR Rel-15, such as additional time domain configurations of the common search space.

Note: Pattern 1 is understood as CORESET#0 and SS/PBCH block(s) occur in different time instances, and CORESET#0 bandwidth overlaps with the transmission bandwidth of the SS/PBCH block.

- Adopt the following text proposal to reflect the above

  - “Support of Pattern 1 is recommended for multiplexing of SS/PBCH block(s) and CORESET(s)#0 in NR-U, where Pattern 1 is understood as CORESET#0 and SS/PBCH block occur in different time instances, and CORESET#0 bandwidth overlaps with the transmission bandwidth of the SS/PBCH block.

As one element to facilitate a NR-U DRS design without gaps in the time domain, the CORESET#0 configuration(s) and/or Type0-PDCCH common search space configuration(s) may need enhancements compared to NR Rel-15, such as additional time domain configurations of the common search space.”

- The detection of a gNB’s transmission burst by the UE has been studied, and concerns on the UE power consumption required for Tx burst detection e.g. if the UE needs to frequently detect/monitor the PDCCH have been raised. The proposals that have been made by contributions regarding these topics include existing NR signal(s) with potential enhancement(s), a channel such as PDCCH with potential enhancement(s), and the 802.11a/802.11ax preamble with potential enhancement(s); consensus was not achieved on any of these proposals. The detection/decoding reliability of each of the proposals has not been sufficiently evaluated for a complete evaluation of the proposals against each other. The power consumption and detection/decoding complexity of each of the proposals have not been sufficiently evaluated for a complete evaluation of the proposals against each other. The relation of a proposal with C-DRX and/or measurement gap(s) may need further consideration when specifications are being developed.

- Compared to NR Rel-15, it has been identified to be beneficial if the time domain instances in which the UE is expected to receive PDCCH can change dynamically, e.g. by implicit determination related to the gNB’s COT, or explicitly signalled by the gNB.

AH#1901 - 7.2.2.1.2 DL Signals and Channels

Agreements:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

- Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

- FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

# Initial Signal

The previous agreements mentioned above, support PDCCH DMRS based downlink burst detection by a UE. A UE looking for the start of a downlink transmission, may first try to detect a PDCCH DMRS from the serving base station and then attempt PDCCH decoding after successfully detecting the presence of a downlink burst. This was mainly from the perspective of UE power saving. However, it was noted in [3] that a performance relaxation of PDCCH decoding is not needed, hence the detection has to be robust. The misdetection probability should be comparable to the PDCCH probability of error. There is also a robustness issue where if a UE misses detection of these initial signals at the start, then it will miss the full gNB COT.

Any signal used for the purpose of detecting a transmission burst should have the following properties:

- UE should be able to detect at very low geometries with high reliability

- It should be possible to send this signal inside of a COT in case the UE missed detecting it at start of COT

- Less false alarms in a congested environment

A wideband DMRS has the listed desirable properties. Figure 1 below shows the misdetection probability for a PDCCH wideband DMRS. The detection threshold is set for a 1% false alarm probability with thermal noise. It can be seen that the wideband DMRS can be detected with good reliability even at low SNRs.

The detector assumes knowledge of symbol boundary and the transmitted reference DMRS sequence in frequency domain. The detector first estimates the frequency domain channel on the DMRS tones using the regular OFDM channel estimation method. The time domain channel estimate is then obtained by a simple IDFT. The detector then estimates a signal to noise ratio from the time domain taps, assuming a maximum delay spread that is less than the cyclic prefix duration. This SNR is then compared with a detection threshold. The threshold is set using a given false alarm probability with only thermal noise input.

The wideband DMRS can be sent multiple times in a COT, in case UE missed detection at the beginning of the COT. The misdetection probability will increase if we reduce the bandwidth of DMRS signal, due to lower channel estimate SNR. The GC-PDCCH DMRS, having smaller transmission bandwidth, will have lower channel estimate SNR due to PSD regulation and less frequency diversity. Thus the GC-PDCCH DMRS is not preferable. The detection is FFT based and hence timing is needed for detection.

There were some proposals to support WB-DMRS with precoder cycling. However, with precoder cycling, a simple FFT based receiver for DMRS detection may not be feasible. Also, the performance shown below seems sufficient to meet the performance requirements expected of the initial signal. We hence propose that WB-DMRS uses the same precoder.

Figure 1: WB DMRS Misdetection Probability

Table 1: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| SCS | 30 KHz |
| Number of RBs | 50 |
| Number of DMRS REs per RB | 3 |
| False alarm probability | 1% |
| Number of Rx antennas | 2 |
| Precoder cycling | No |
| TDL Channel RMS delay spread | 100ns |

Proposal : PDCCH wideband DMRS without precoder cycling is used for initial signal

Proposal : NR-U design should not preclude transmitting wideband DMRS inside COT

# Dynamic PDCCH monitoring

The RAN1#95 agreement includes the following with respect to dynamic PDCCH monitoring periodicity:

- Compared to NR Rel-15, it has been identified to be beneficial if the time domain instances in which the UE is expected to receive PDCCH can change dynamically, e.g. by implicit determination related to the gNB’s COT, or explicitly signalled by the gNB.

To reduce the LBT related overhead, it is desirable for gNB to have a flexible COT starting point (such as every symbol). However, this would mean the UE has to monitor for the DL burst start continuously. Any design that provides flexible COT starting time in a slot should consider the following:

- Robust detection of the DL transmission

- Ability for UE that missed detecting the start of transmission to detect it at a later time

- UE PDCCH monitoring complexity and power consumption

- Less PDCCH overhead

There are proposals to have gNB schedule data with mini-slot granularity in the beginning of the COT. The current NR PDSCH transmission type B with 2/4/7 symbol duration supports this. The UE has to monitor the PDCCH at mini-slot granularity before COT start and for some time after COT start. To reduce the PDCCH overhead and UE monitoring complexity, the mini-slot based scheduling is applied only in the beginning of the COT. After the first or second slot boundary, the gNB can switch the PDCCH monitoring and scheduling to slot level, as shown in Figure 2. The UE thus monitors PDCCH at mini-slot granularity before COT start and at least till the first slot boundary after COT start, and then switch to slot level or multiple of slot level monitoring granularity. The scheduled frequency resources may also be different before and after COT start. The UE monitoring the PDCCH will switch both the time occasions and the frequency resources monitored. To enable this and to reduce the PDCCH overhead, the CORESET for mini-slot monitoring (e.g. before the COT start and in initial portion of the COT) may be different compared to the CORESET the slot level monitoring (e.g. inside the COT).

Figure 2: Dynamic PDCCH monitoring

Proposal : The current NR PDSCH transmission type B scheduling with 2/4/7 symbol duration (mini-slot based scheduling) can be reused for NR-U, to allow flexible gNB COT starting points.

Proposal : The UEs that support mini-slot level PDCCH monitoring, may be configured with mini-slot based scheduling before and for an initial duration after the COT start, and with slot based scheduling for rest of the COT duration.

Proposal : A UE may be configured with different CORESET configuration for an initial duration after COT start where mini-slot based scheduling is applied, compared to the CORESET configuration for the rest of the COT duration where slot based scheduling is applied.

Proposal : The frequency resources UE monitors for PDCCH may change dynamically, e.g., based on explicit signalling or determined implicitly through the gNB COT start detection.

In case the UE missed detection of the initial mini-slot level PDCCH, there should be a way to recover after the slot boundary where slot level PDCCH starts. One way to achieve this is the following (Figure 3). There is one set of decoding candidates (set A for example), that is common between before and after the COT start. This is in a CORESET which is at the start of a slot (slot level granularity). There is another set of decoding candidates (set B), that is different for the monitoring occasions before and after the COT start. Set B for monitoring occasions before the COT start (say B2) and after the COT start (say B1) may be of different size. The set B1 is hashed in coreset(s) at the start of a slot (slot level granularity), and the set B2 is hashed in mini-slot coreset(s). The coreset(s) for A and B1 can be the same. A UE monitors for PDCCH candidates (set A, set B2) before the COT start and (set A, set B1) after the COT start. The total search candidates before and after the COT start is within the UE blind decode capability.

Note: The trigger for UE to switch the PDCCH monitoring pattern (example: A,B2 vs A,B1) may be an explicit (for example, a bit field in DCI) or implicit (for example, the COT start detection) indication as discussed below.

Figure 3: Nested PDCCH structure

Proposal : In case the PDCCH monitoring configuration is changed dynamically based on COT, there should at least be a set of common PDCCH search space candidates in the PDCCH search space inside and outside the COT to enable UE to recover from missed detection of COT start

Trigger to indicate the UE to switch PDCCH monitoring:

The UE can switch monitoring between the candidate sets before and after COT start, based on one of the following triggers:

- Implicitly based on detection of COT start and the next slot boundary.

- Explicit indication to switch in a common or UE specific PDCCH.

- Implicitly based on past traffic pattern. For example, the UE can monitor the mini-slot level PDCCH candidate only if it was scheduled in the past within a specified time window.

- In NSA case, the licensed carrier may explicitly trigger one of the two PDCCH configurations for UE monitoring. This way the gNB may selectively choose only UEs which have traffic to switch to mini-slot level monitoring.

Proposal : The UE can switch PDCCH monitoring pattern dynamically based on any one of the following triggers:

- Implicitly based on COT start detection and the slot boundary

- Implicitly based on whether it was scheduled within a specific time window

- Explicit indication in the common or UE specific PDCCH

Proposal : In NSA mode of operation, gNB can explicitly indicate/trigger PDCCH monitoring switch for the unlicensed carrier, using the licensed carrier.

# PDSCH transmission

The previous RAN1#92bis to RAN1#95 agreements contains the following with respect to PDSCH transmission

- NR-U supports both Type-A and Type-B mapping already supported in NR

  - Additional starting positions and durations are not precluded

- The following options have been identified as possible candidates for PDSCH transmission in the partial slot at least for the first PDSCH(s) transmitted in the DL transmission burst.

  - Option 1: PDSCH(s) as in Rel-15 NR

  - Option 2: Punctured PDSCH depending on LBT outcome

  - Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols

  - Option 4: PDSCH across slot boundary

  - FFS for signalling details, specification impact, implementation complexity

  - Note: Above options are not mutually exclusive.

- It has been identified to be beneficial for the NR-U design to not require the gNB to change a pre-determined TBS for a PDSCH transmission depending on the LBT outcome, at least when the PDSCH is transmitted at the beginning of the gNB’s COT.

Consider the non-trivial standardization effort needed to support new options (other than option 1 already supported in NR Rel.15), and consider the limited time available, we propose the deprioritize the discussion of other options.

Proposal : PDSCH(s) as in Rel.15 NR is used to support multiple DL starting positions and other options are discussed only if there is time.

# CSI-RS

In NR, CSI-RS transmission can be periodic, semi-persistent, and aperiodic. In NR-U, due to the LBT requirement for DL transmission, aperiodic CSI-RS fits the opportunistic transmission of a DL burst. On the other hand, NR periodic CSI-RS and semi-persistent CSI-RS cannot carry over to NR-U directly, considering the fact that the configured transmission opportunity may not be useable due to LBT failure.

Simple changes to periodic and semi-persistent CSI-RS can be introduced to make it more LBT friendly. For example, if the periodic and semi-persistent CSI-RS are configured, they can be only transmitted when the configured transmission opportunity falls in the DL portion of a COT. On the UE side, if the processing of periodic and semi-persistent CSI-RS is conditioned on the detection of COT start, these CSI-RS types can still be supported in NR-U, as a scheduler choice.

Proposal : Periodic, semi-persistent, and aperiodic CSI-RS can be supported in NR-U. The transmission and processing of periodic and semi-persistent CSI-RS should be conditioned on the detection that the transmission opportunity falls in the DL portion of a COT.

# COT structure indication

The work item description includes the following with respect to COT structure indication (COT-SI):

- DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

The RAN1#94bis agreement includes the following with respect to COT structure indication:

- In addition to the functionalities provided by DCI format 2_0 in Rel-15 NR, indication of the COT structure in the time domain has been identified as being beneficial.

SFI feature was introduced in Rel.15 NR to indicating to the UE on the format of slot(s). The UE configured to monitor the SFI will be informed the OFDM symbols in a slot or a sequence of slots to be DL, UL, or flexible.

In LTE-LAA, C-PDCCH was introduced to indicate to the UE the structure of COT, including the length of DL portion of COT, the length of gap, and the length of UL portion.

As we can see, the key functionality of C-PDCCH can be achieved by using SFI. However, the Rel.15 NR SFI functionality is limited as the slot formats that can be indicated, and the monitoring of the GC-PDCCH carrying SFI were optimized for licensed operation. For NR-U, it is possible some enhancements to the SFI are needed to make it more unlicensed operation friendly.

Some features already in LTE-LAA C-PDCCH that can be considered, includes:

- 2-stage PUSCH trigger

- COT sharing indicator for AUL

Some additional features to be considered for SFI includes

- Using GC-PDCCH carrying SFI to indicate the subband usage is subband LBT is applied

- Triggered RACH occasions

- RRC configured RACH

It is beneficial to transmit some COT information like the COT duration and DL/UL/pause durations, multiple times within a COT to aid the features listed above.

Proposal : NR SFI can be enhanced to support the functionality of LTE-LAA C-PDCCH and support sub-band based channel access.

Proposal : At least some COT information, like the DL/UL/pause durations can be transmitted more than once within the COT.

# Summary

In this section, we summarize the proposals for DL signals and channels for NR-U in SUB-7GHZ BAND.

Proposal 1: PDCCH wideband DMRS without precoder cycling is used for initial signal

Proposal 2: NR-U design should not preclude transmitting wideband DMRS inside COT

Proposal 3: The current NR PDSCH transmission type B scheduling with 2/4/7 symbol duration (mini-slot based scheduling) can be reused for NR-U, to allow flexible gNB COT starting points.

Proposal 4: The UEs that support mini-slot level PDCCH monitoring, may be configured with mini-slot based scheduling before and for an initial duration after the COT start, and with slot based scheduling for rest of the COT duration.

Proposal 5: A UE may be configured with different CORESET configuration for an initial duration after COT start where mini-slot based scheduling is applied, compared to the CORESET configuration for the rest of the COT duration where slot based scheduling is applied.

Proposal 6: The frequency resources UE monitors for PDCCH may change dynamically, e.g., based on explicit signalling or determined implicitly through the gNB COT start detection.

Proposal 7: In case the PDCCH monitoring configuration is changed dynamically based on COT, there should at least be a set of common PDCCH search space candidates in the PDCCH search space inside and outside the COT to enable UE to recover from missed detection of COT start

Proposal 8: The UE can switch PDCCH monitoring pattern dynamically based on any one of the following triggers:

- Implicitly based on COT start detection and the slot boundary

- Implicitly based on whether it was scheduled within a specific time window

- Explicit indication in the common or UE specific PDCCH

Proposal 9: In NSA mode of operation, gNB can explicitly indicate/trigger PDCCH monitoring switch for the unlicensed carrier, using the licensed carrier.

Proposal 10: PDSCH(s) as in Rel.15 NR is used to support multiple DL starting positions and other options are discussed only if there is time.

Proposal 11: Periodic, semi-persistent, and aperiodic CSI-RS can be supported in NR-U. The transmission and processing of periodic and semi-persistent CSI-RS should be conditioned on the detection that the transmission opportunity falls in the DL portion of a COT.

Proposal 12: NR SFI can be enhanced to support the functionality of LTE-LAA C-PDCCH and support sub-band based channel access.

Proposal 13: At least some COT information, like the DL/UL/pause durations can be transmitted more than once within the COT.

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#82, Sorrento, Italy, Dec. 10-13, 2018

[2] TR 38.889, “Study on NR-based Access to Unlicensed Spectrum”, RAN#82, Dec. 2018.

[3] R1-1901419, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, AH-1901, Taipei, Taiwan Jan. 21st – Jan. 25th, 2019