3GPP TSG RAN WG1 Meeting #96	R1-1903387

Athens, Greece, February 25th – March 1st, 2019

Source:	Ericsson

Title:	Maintenance of PT-RS

Agenda Item:	7.1.2

Document for:	Discussion and Decision

# Correction of antenna port indexing for TS 38.214

These two tdocs have the same proposal,

- R1-1901614, “Correction on port indexing in 38.214”, Huawei, HiSilicon

- R1-1901660, “Draft TS 38.214 CR on association between SRS port and PTRS port”, vivo

These CR aligns the port numbering of 38.214 with 38.214, where antenna ports used for SRS are numbered using the “1000-series”.

Please provide your view on this issue below:

| Company | Views |
| --- | --- |
| Ericsson | This is a necessary CR which aligns port numbering between specs, an editorial issue with no functional change. R1-1901660 has the most descriptive cover sheet and is preferred to be agreed as a CR. Note that the text can be improved, since we don’t have “SRS ports” in specifications, but there are “antenna ports” and SRS are transmitted on the antenna ports of the “1000-series”. Hence a better wording would be Antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and antenna port 1000 and 1003 in indicated TPMI share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer [x] of layers which are transmitted with antenna port 1000 and antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer [y] of layers which are transmitted with antenna port 1001 and antenna port 1003 in indicated TPMI, where [x] and/or [y] are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 described in Subclause 7.3.1 of [5, TS38.212]. However, “SRS ports” is used throughout the 38.214 spec and it needs to be cleaned up with a bigger CR and not necessarily the CR provided here. In addition, “PT-RS port” is used but undefined in Section 6.2 of 38.211. |
| Qualcomm | I don’t see the need to update the “SRS port” to “antenna port”. It is used in 214 already and does not create confusion. We are OK with the initial CR of just updating the “i” to “1000+i”. |
| Huawei/HiSilicon | The isolated impact analysis in R1-1901660 says there can be confusion in some cases. Similar to FL, we think this is rather an editorial issue with no functional change, as summarized in R1-1901614. For the further change suggested by FL (from ‘SRS port’ to ‘antenna port’), we prefer to keep the current specs, as the SRS port has been defined in Section 6.2 of 38.211 and is used throughout 38.214. In addition, about PT-RS port indexing, we suggest to add PT-RS under 1000 serious in Section 6.2/7.2 of 38.211 (as PT-RS is embedded in PDSCH/PUSCH), but are also open to discuss other options. |
| Ericsson | As a response to Huawei, there is no definition of “SRS port’” in Section 6.2 of 38.211. Only antenna ports are defined, some are used for transmitting PUSCH others for SRS, but the name “SRS port” is not defined. In 38.214, SRS port is used but there is no definition. The correct name should be “antenna port used for SRS transmission” in 214. A CR adding antenna ports used for PT-RS to 38.211 is a good suggestion. |
| Intel | Our understanding is that there would be no functionality change for the CR. We think FL’s wording the best one. This is not critical, or even may not be an issue. “SRS port 0” = “the first SRS port” = “antenna port 1000”. Since there are many CRs for other issues with functionality change, we suggest deprioritizing such kind of CRs. If time permits, we can discuss it. |
| Samsung | Agree in principle and fine with Ericsson’s updates on SRS ports. Regarding similar issue with PT-RS, should the port number for PT-SR start from 1000 to be aligned with antenna ports for PUSCH? |
| vivo | Fine with Ericsson’ proposal to add PTRS port in 38.211. |

FL proposal:

- Agree on one of the CRs

- Work during the week on a CR for 38.211 to introduce “antenna ports for PT-RS” in section 6.2

- Discuss further whether to change the confusing terminology of “SRS port” and “PT-RS port” to “antenna port X used for SRS” etc