3GPP TSG RAN WG1 Meeting #96bis	 R1-1903979

Xi’an, China, April 8th – 12th, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Evaluation results of DFT-based compression codebook

Document for:	Discussion and Decision

Introduction

It has been agreed at RAN1 #95 [1] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

It has been agreed at RAN1 AH1901 [2-3] that:

Agreement

On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary

- The value of R is fixed to 2

- FFS: Whether secondary implies a separate UE capability or restricted use cases

  - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint

- FFS: Whether FD compression unit is higher-layer configured or reported by the UE

Working Assumption

On the choice of oversampling factor O3, O3 = 4 is supported

Agreement

On the values of L, support L={2,4}

- Decide whether to support L=3 and/or L=6 in the future meetings considering the performance-overhead trade-off for different RI values and/or different number of antenna ports

Agreement

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19,

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96

- Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is  ≥NSB×R

- Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

Agreement

Decide in RAN1#96bis whether to support L=6 (in addition to L=2 and 4) taking into account RI{3,4} and/or 32 ports

- Also consider possible restriction(s) on the use case for supporting the additional value(s) of L

In this contribution, some additional evaluation results are provided as the supplement of the main contribution [4].

Remaining issues for rank 1-2 design

Support of L=6 for 32 ports

For the DFT-based compression codebook, L=2 and 4 have been supported as in R15. However, the spatial information of real channel is rather rich in some scenarios such as 32 CSI-RS ports and the value of L should be increased to represent the channel. In this section, the performance gain as well as feedback overhead of L=6 are analysed to show the necessity of supporting L=6.

The trade-off curves for (L, p)=(4, 1/2) and (6, 1/4) are illustrated in Fig. 1 and Fig. 2, for rank 1 and rank adaptation with maximum rank 2, respectively. It can be observed that (L, p)=(6, 1/4) has a better performance over (L, p)=(4, 1/2) with less overhead. For same overhead, L=6 has around 3% performance gain over L=4. In fact, there exist not many strong frequency components for each spatial beam. The selected frequency basis vectors for different spatial beams always have relatively concentrated location, especially after the phase correction between subbands [5]. With a good phase correction, only a relative small set of frequency basis vectors is enough to cover almost all strong frequency components for all the spatial beams. Therefore, compared with increasing the number of M, increasing the number of L may be more efficient for the compression.

Figure 1. The performance-overhead curves for Type II and DFT-based compression codebook (L=4 and 6).

Figure 2. The performance-overhead curves for Type II and DFT-based compression codebook (L=4 and 6, rank adaptation with maximum rank 2).

Observation 1: For 13-SB case, (L,M)=(6,4) has a better performance over (L,M)=(4,7) with less overhead. For same overhead, L=6 has around 3% performance gain over L=4.

Values of N3

When , the following alternatives are listed in the agreements of RAN1 AH1901, where NSB is the number of CQI subbands.

- Alt1: N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R

- Alt2: N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

For Alt1, since the maximum candidate of NSB is 19 for R=1, the UE implementation may be different for 13 subbands and 14 subband cases. Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

The padding scheme is the UE implementation, however it may have influence on the performance. Supposing m columns are needed to pad in total, three typical padding schemes are listed and compared as follows.

- Padding Scheme 1: Padding m columns of zeros to the end.

- Padding Scheme 2: Padding m columns copied from the last column to the end.

- Padding Scheme 3: Padding ceil(m/2) columns copied from the first column to the beginning, and m-ceil(m/2) columns copied from the last column to the end.

For the determination of N3 when NSB×R>13, the optional value of (NSB×R) are summarized in Table I. To minimize the range of optional N3 values for Alt2, (NSB×R) FD units are divided into two segments as equally as possible, and N3 for each segment is selected as the smallest multiple of 2/3/5 which is equal or larger than NSB×R2.

Table I. Optional values of NSB×R and N3

| NSB×R | R | Alt 1: Optional value of N3 (multiple of 2, 3, 5) | Alt 2: Optional value of N3 for each segment |
| --- | --- | --- | --- |
| 14 | 1, 2 | 15 | 8 |
| 15 | 1 | 15 | 8 |
| 16 | 1, 2 | 16 | 8 |
| 17 | 1 | 18 | 9 |
| 18 | 1, 2 | 18 | 9 |
| 19 | 1 | 20 | 10 |
| 20 | 2 | 20 | 10 |
| 22 | 2 | 24 | 12 |
| 24 | 2 | 24 | 12 |
| 26 | 2 | 27 | 15 |
| 28 | 2 | 30 | 15 |
| 30 | 2 | 30 | 15 |
| 32 | 2 | 32 | 16 |
| 34 | 2 | 36 | 18 |
| 36 | 2 | 36 | 18 |
| 38 | 2 | 40 | 20 |

From Table I, it can be observed that the candidate number of N3 values for Alt2 is smaller than that for Alt1, because the maximum number of FD units for each segment can be limited within 20. Less candidate number of N3 means lower implementation complexity. However, the segmentation introduces additional specification impact. For example, although it is claimed that no padding is needed to align the DFT size with the multiple of 2, 3, or 5, the segmentation rules still need to be defined to avoid the misalignment between reported PMI and actual FD units. Moreover, the UCI design for NSB×R>13 is different from that for NSB×R≤13, because different FD basis subset and non-zero coefficients should be indicated for different segments.

The performance-overhead trade-off curves for different padding and segment schemes are illustrated in Fig. 3. It can be observed that different padding schemes as well as the scheme without padding have almost same performance. However, the segment scheme performs worse.

Figure 3. The performance-overhead curves for different padding and segment schemes.

Another system level simulation result is shown in Fig. 4. In our simulation, BW=50MHz, SCS=30kHz and NSB=17. For Alt1, N3 is selected as 18 and length-17 actual FD units are allocated in FD units with index 0~16. No channel estimation samples is assumed for FD unit 17. As for Alt2, two segments has the same number of FD units. Segment 1 contains subband 0~8, and segment 2 contains subband 8~16. To ensure similar overhead, the number of FD basis for Alt1 and Alt2 are M=6 and M=3, respectively. β=3/4 is assumed for Size-K0 subset. As shown in Fig. 4, Alt1 and Alt2 almost have the same performance. Based on the above analysis and simulation results, no obvious benefit is exhibited through segmentation.

Figure 4. The performance of Alt 1 and Alt 2 for N3 value determination, BW=50 MHz, SCS=30 kHz, NSB=17, R=1.

Observation 2: Although the difference among padding schemes is not significant, the variance of performance gain can be up to 2%. And segmentation shows performance loss.

Proposal 1: For the value of N3 when NSB*R > 13, Alt2 with two segments is not preferred. Also Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observations and proposals are made.

Observation 1: For 13-SB case, (L,M)=(6,4) has a better performance over (L,M)=(4,7) with less overhead. For same overhead, L=6 has around 3% performance gain over L=4.

Observation 2: Although the difference among padding schemes is not significant, the variance of performance gain can be up to 2%. And segmentation shows performance loss.

Proposal 1: For the value of N3 when NSB*R > 13, Alt2 with two segments is not preferred. Also Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

- References

“RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

“RAN1 Chairman’s Notes”, Taipei, January 21-25, 2019.

“RAN1 Chairman’s Notes”, Athens, Greece, February 25-March 1, 2019.

R1-1903969, Huawei, HiSilicon, “Discussion on CSI enhancement”, Xi’an, China, April 8th – 12th, 2019.

R1-1903980, Huawei, HiSilicon, “Phase randomization and correction for CSI quantization in frequency domain”, Xi’an, China, April 8th – 12th, 2019.

- Appendix I

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz for 10MHz |
| System bandwidth | 10MHz (13 subbands, 4 PRBs for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1) / (1,2,2,1,1,1,2); the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |