3GPP TSG RAN WG1 #96bis                                              R1-1904026

Xi’an, China, 8th - 12th April, 2019

Source:         ZTE

Title:            Details and SLS evaluation on L1-SINR measurement and reporting

Agenda item:    7.2.8.6

Document for:	Discussion and Decision

# Introduction

In RAN#80 meeting, a new WID for enhancement of NR-MIMO has been approved for Rel-16 [1]. Regarding multi-beam operation as one of the objectives in the WID, our recommended potential enhancements on multi-beam operation including UL simultaneous transmission, group based beam reporting, UE panel activation/de-activation, spatial relation update for multiple PUCCH resources, MAC-CE based spatial relation update for AP-SRS, beam failure recovery for SCell and L1-SINR reporting have been discussed in our companion contribution [2].

In this contribution, we further discuss and provide details and SLS evaluation results on L1-SINR measurement and reporting, involving L1-RSRP, L1-SINR without IMR index(es) to be reporting and L1-SINR with IMR index(es) to be reported.

# Evaluation assumptions

In this section, we evaluate DL transmission performance according to L1-SINR/L1-RSRP reporting in one single TRP case through system-level simulation in the indoor hotpot scenario. In the setup, we have 4 panel/subarrays at TRP side and two panels (back to back) at the UE side. Each panel contains two TXRUs for dual-polarization. TRP and UE sweep all Tx-Rx beam pairs with oversampling factor of (O1, O2) = (1, 1). One fixed TRP panel with dual polarization is used at TRP side for Tx analog beam sweeping, while both two UE panels with dual-polarization are used for Rx analog beam sweeping at UE side. In this case, UE speed is set as 3km/h. This evaluation scenario for group based reporting/transmission is the same as single-TRP LLS simulation, as shown in our companion contribution [3].

- Regarding definition of metric of beam reporting,

  - L1-RSRP: Linear average over the power contribution (in [W]) of the resource elements carrying RS;

    - In this case, interference beams are not considered for beam selection for reporting.

  - L1-SINR without IMR index(es) to be reported: Linear average over the power contribution (in [W]) of the resource elements carrying RS divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying RS.

    - In this case, interference beams are considered for channel beam selection for reporting based on this metric.

  - L1-SINR with IMR index(es) to be reported: Linear average over the power contribution (in [W]) of the resource elements carrying RS divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying RS and the resource elements of interference measurement resource (IMR).

    - In this case, interference beams are considered for both channel and interference beam selection and the corresponding CRI reporting based on this metric.

    - The IMR is associated with the RS for CMR in the reporting configuration.

- Regarding format of beam reporting

  - Reporting without IMR index(es) to be reported under group based reporting

    - In UE side, one best Rx beams per UE panel is selected with the objective of maximizing L1-RSRP/L1-SINR according to DL beam measurement.

    - In gNB side, two best TX beams, each of which is determined per UE-panel, are reported and subsequently are used for different TRP panel set, each of which comprises one independent TRP panels, i.e., first or second TRP panel in this simulation.

  - Reporting with IMR index(es) to be reported under group based reporting

    - In UE side, one best Rx beams per UE panel is selected with the objective of maximizing L1-RSRP/L1-SINR according to DL beam measurement.

    - In gNB side, two best TX beams, each of which is determined per UE-panel, are reported and subsequently are used for different TRP panel set, each of which comprises one independent TRP panel, i.e., first or second TRP panel in this simulation.

      - Low-interference beam list, i.e., the indexes of IMRs, is reported for the two best Tx beams, where, for each UE panel, the received power of low interference beam(s) should be below 15-dB over its corresponding best Tx beam.

- Regarding interference beam

  - In the case of reporting without IMR index(es)

    - SINR measurements in beam management is based on intra-cell and inter-cell interference within a measurement window.

    - For data transmission, MU-MIMO scheduling is performed with the objective of maximizing system throughput.

  - In the case of reporting with IMR index(es)

    - For SINR measurement in beam management, each Tx beam of the serving TRP is generated to NZP-CSI-RS based IMR for intra-cell interference measurement, besides the measurement of intra-cell interference within a window. A list of low interference beams is obtained after SINR measurement

    - For data transmission, up to 2 UEs in the MU-MIMO scheduling can be paired according to each of corresponding results of low-interference beams to be reported, besides maximizing the system throughput.

Some more details on SLS evaluation assumption can be found in Annex.

# Discussion

In this section, we evaluate system-level performance in the indoor hotpot scenario for the beam reporting of L1-RSRP and SINR with/without IMR index(es) to be reported, under group based approach. The baseline is L1-RSRP scheme. The results are shown in Table 1. From the evaluation results, it is observed that

- L1-SINR reporting without reporting of low-interference beam offers similar performance to legacy L1-RSRP (Even some performance degradation can be observed in terms of mean throughput and 50-ile UE).

- L1-SINR reporting with IMR index(es) to be reported, i.e., reporting of low interference beams have significant performance gain over legacy L1-RSRP (+17.28% and +151.834% in terms of mean throughput and cell-edge throughput, respectively). Through finding the low-interference beams corresponding to the best Tx beam, gNB can schedule the MU Tx beams from the list of low-interference beams and prevent from some strong interference.

Table 1 Throughput performance for L1-RSRP, L1-SINR without/with IMR index(es) to be reported in Indoor Hotpot

|  | RU | Mean Tput. (Mbps) | 5% Tput. (Mbps) | 50% Tput. (Mbps) | 95% Tput. (Mbps) |
| --- | --- | --- | --- | --- | --- |
| L1-RSRP | 47.47% | 427.95 | 69.04 | 414.25 | 860.37 |
| L1-SINR without IMR index(es) to be reported | 46.62% | 425.12 (-0.66%↓) | 75.92 (+9.97%↑) | 404.27 (-2.41%↓) | 883.01 (2.63%↑) |
| L1-SINR with IMR index(es) to be reported | 36.83% | 502.52 (+17.42%↑) | 173.86 (+151.83%↑) | 486.3 (+17.39%↑) | 932.07 (+8.33%↑) |

Subsequently, we consider the system performance as the function of the number of IMR index(es) to be reported. The simulation results can be found in the Table 2, where the maximum number of reporting low-interference beam is limited to X, where X can be 4,8,12 and 16. Note that L1-SINR without upper bound for IMR reporting refers to the L1-SINR with IMR to be reported in Table 1.

It can be observed that L1-SINR up to X IMR indexes still obtains remarkable performance gain than L1-RSRP if appropriate value is chosen for X. For instance, when up to 4 IMR indexes is reported (same overhead as Rel-15 RSRP reporting), the performance gains over the baseline solution of L1-RSRP is +19.34%↑ and +146.71%↑ in terms of mean UPT and 5%-edge UPT, respectively. Besides, some performance degradation of using a larger X over the best one with X=8, can be observed, which is due to the fact that the larger interference for MU-MIMO is experienced through reporting more IMRs with insufficient high SINR.

Table 2 SLS results for L1-RSRP, L1-SINR with up to X IMR indexes and L1-SINR without upper bound for IMR reporting

|  | RU | Mean Tput. (Mbps) | 5% Tput. (Mbps) | 50% Tput. (Mbps) | 95% Tput. (Mbps) |
| --- | --- | --- | --- | --- | --- |
| L1-RSRP | 47.47% | 427.95 | 69.04 | 414.25 | 860.37 |
| L1-SINR with up to 4 IMR indexes | 36.58% | 510.7 (+19.34%↑) | 170.33 (+146.71%↑) | 500.81 (+20.90%↑) | 932.07 (+8.33%↑) |
| L1-SINR with up to 8 IMR indexes | 36.49% | 511.6 (+19.55%↑) | 171.2 (+147.97%↑) | 500.81 (+20.90%↑) | 932.07 (+8.33%↑) |
| L1-SINR with up to 12 IMR indexes | 36.56% | 509.72 (+19.11%↑) | 172.07 (+149.23%↑) | 493.45 (+19.12%↑) | 932.07 (+8.33%↑) |
| L1-SINR with up to 16 IMR indexes | 36.56% | 508.43 (+18.81%↑) | 177.54 (+157.16%↑) | 493.45 (+19.12%↑) | 932.07 (+8.33%↑) |
| L1-SINR without upper bound for IMR reporting | 36.83% | 502.52 (+17.42%↑) | 173.86 (+151.83%↑) | 486.3 (+17.39%↑) | 932.07 (+8.33%↑) |

Observations: From the system-level evaluation results, it can be observed that:

- Under the case without IMR resources for reporting, L1-SINR reporting can NOT obtain any performance gains over L1-RSRP.

- By contrast, L1-SINR reporting with IMR can obtain significant performance gains over L1-RSRP and L1-SINR without IMR, through reporting low-interference beam information using indexes of IMR.

- In practice, L1-SINR with limited IMR index(es) to be reported, e.g., up to 4 IMRs, obtains the similar (slightly better) performance gain as non-upper bound for IMR to be reported.

Proposal: In L1-SINR reporting, information of low-interference beams, i.e., index(es) of IMR, should be reported along with L1-SINR.

# Conclusions

In this contribution, beam reporting for L1-RSRP, L1-SINR without IMR and L1-SINR with IMR are discussed and analyzed here. The observations from evaluation results and the proposals for L1-RSRP, L1-SINR without IMR and L1-SINR with IMR measurement and reporting are summarized as below.

Observations: From the system-level evaluation results, it can be observed that:

- Under the case without IMR resources for reporting, L1-SINR reporting can NOT obtain any performance gains over L1-RSRP.

- By contrast, L1-SINR reporting with IMR can obtain significant performance gains over L1-RSRP and L1-SINR without IMR, through reporting low-interference beam information using indexes of IMR.

- In practice, L1-SINR with limited IMR index(es) to be reported, e.g., up to 4 IMRs, obtains the similar (slightly better) performance gain as non-upper bound for IMR to be reported.

Proposal: In L1-SINR reporting, information of low-interference beams, i.e., index(es) of IMR, should be reported along with L1-SINR.

# References

[1]RP-181453, WI Proposal on NR MIMO Enhancements, Samsung

[2]R1-1904014, Enhancements on Multi-beam Operation, ZTE

[3]R1-1904035, Details and LLS evaluation on L1-SINR measurement and reporting, ZTE

# Appendix:

Table 3 Simulation assumptions for indoor-hotpot system-level evaluation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 120kHz |
| Channel Model | Indoor in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | All panels are attached with one TRP - Step-1: Single TX/RX subelement - without TX/RX beamforming - The first sub-element per panel is used with single-pol. - The best TRP is selected cross all UE panels, in terms of coupling loss; - Step-2: With analog TX/RX beamforming, one best Tx-Rx beam pair is selected per UE panel according to the TRP identified in step-a. |
| ISD | 20m |
| BS Tx power | 23dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1), (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| Transmission scheme | MU, Rank adaptation |

Notes: Any other parameters not specified here remain the same as those in NR evaluation assumption for beam management in eNR-MIMO.