3GPP TSG RAN WG1 Meeting #96bis							R1-1904262

Xi’an, China, April 8 - 12, 2019

Title:	[DRAFT] Reply LS on clarification about CSI-RS measurement

Response to:	R1-1903856 (R2-1902730)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Intel [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Daewon Lee

E-mail Address:	daewon.lee (at) intel (dot) com

Attachments:

1. Overall Description:

RAN2 has asked questions regarding maximum number of CSI-RS resources that can be configured. In this regard, RAN1 has previously agreed to put maximum number of CSI-RS resource restriction per frequency layer and has captured the restriction in TS38.214 section 5.1.6.1.3 (See below).

5.1.6.1.3 CSI-RS for mobility --- omitted --- A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured - with no more than 96 CSI-RS resources when all CSI-RS resources per frequency layer have been configured with associatedSSB, or, - with no more than 64 CSI-RS resources per frequency layer when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB - For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resources per frequency layer. --- omitted ---

Therefore, even though RRC specification allows configuration of 9216 CSI-RS resources, the RAN1 specification does not allow such configuration to be done per frequency layer.

However, RAN1 does recognize that terminology of ‘frequency layer’ is not used in RAN2 specification and clearly not defined in RAN1 specification. Therefore, RAN1 suggest the following:

- RAN1 recommends RAN2 to clarify to appropriate RAN2 specification (e.g. TS 38.331) that “any CSI-RS resource configured by different measurement object shall not overlap or partially overlap in frequency.”

- RAN1 will update the TS38.214 “per frequency layer” text with the following text changes

A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured - with no more than 96 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources configured by the same higher layer parameter MeasObjectNRper frequency layer have been configured with associatedSSB, or, - with no more than 64 CSI-RS resources per higher layer parameter MeasObjectNR frequency layer when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB by the same higher layer parameter MeasObjectNR - For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource

RAN1 would recommend to RAN2 to implement the above suggestion to ensure UE complexity for performing CSI-RS measurements is not over-burdened and previous RAN1 agreements can be honoured.

2. Actions:

To RAN2: 	RAN1 would recommend to RAN2 to implement the above suggestion.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #97 	13 – 17 May 2019			Reno, U.S.A.

TSG-RAN WG1 Meeting #98 	26 – 30 Aug 2019			Prague, Czech Republic