3GPP TSG RAN WG1 Meeting #96bis															R1-1904264

Xi’an, China, 8th – 12th April, 2019

Source: 	Intel Corporation

Title:	UE-group wake-up signal for eMTC

Agenda item:	6.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN#80 a new work item (WI) on Rel-16 MTC enhancements for LTE was agreed. One of the WI objectives is to improve DL transmission efficiency and/or UE power consumption [1]. In particular, specify support for UE-group wake-up signal (WUS).

In the previous RAN1#96 meeting, the following agreements regarding UE-group WUS were made [2]:

Agreement UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs. Agreement - A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location. Agreement - Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource. - Select one among the following: - Up to 2 orthogonal WUS resources may be configured in time domain - Up to 2 orthogonal WUS resources may be configured in frequency domain - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total) - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured) - FFS: whether one of the WUS resources is configured to be shared with legacy WUS Agreement If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. - FFS: Whether the above is also applicable for Rel-15 WUS UEs FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

In this contribution, we express our views on UE-group WUS for eMTC devices in Rel-16.

# Multiplexing Considerations

There are two factors which should be taken into account while choosing among WUS multiplexing options: network (NW) flexibility and UE complexity.

From the network flexibility perspective, the option with up to 2 orthogonal WUS resources per dimension (time/frequency), which results in up to 4 orthogonal WUS resources total, seems to be the most preferable one. It potentially allows the NW to do TDM between the WUS resources, e.g., to provide additional power boosting to the WUS. On other hand, it does not preclude FDM in deployments where the boosting is not needed, and the NW just prefers to increase the number of available WUS resources to serve a larger number of UE groups. From the same perspective, it should be up to the NW configuration to share or not one of the WUS resources between UE-group WUS and the legacy WUS. For example, in deployments with limited number of WUS resources it would be beneficial to share a resource between UE-group and legacy WUSes especially when the relative number of legacy UEs is not high. However, it is still beneficial to configure a WUS resource only to legacy UEs and not to configure it to Rel-16 UEs when the relative number of legacy UEs is large [3]. Because the legacy WUS and new UE-group WUS may share the same WUS resource, it’s proposed to use the legacy WUS as a common WUS for Rel-16 UEs.

Proposal 1:

Up to 2 orthogonal WUS resources per dimension (time/frequency), which results in up to 4 orthogonal WUS resources total, can be configured.

  - This accounts the legacy WUS resource

One of the WUS resources can be configured to be shared with legacy WUS

  - If not configured, the WUS resource is used only for legacy WUS transmission

Legacy WUS is used as a common WUS to wake up all UEs monitoring the same WUS resource.

From the UE complexity perspective, the number of WUS sequences the UE detects within the monitored WUS resources should be minimized. However, in order to distinguish a particular WUS, the UE should be able to detect all WUSes which potentially can be transmitted within the WUS resource. In this case, a minimal number of UE groups sharing the same WUS resource is equal to 3: UE group-0, UE group-1 and the common UE group. Obviously, the single sequence (SS) CDM is used to multiplex theses sequences.

Proposal 2:

For SS-CDM, the max number of WUS sequences within the same WUS resource is equal to 3.

  - This corresponds to UE group-0, UE-group-1 and the common UE group.

An illustration of WUS multiplexing concept described above can be found in Figure 1.

Figure 1. Multiplexing of WUS resources in Rel-16.

# UE-group WUS Sequence Generation Considerations

The WUS sequence generation for LTE eMTC is defined in Section 6.11B.1 of [4]. In order to keep backwards compatibility with Rel-15 and, at the same time, minimize the impact on the existing RAN1 specification, the following procedure for WUS sequence generation is proposed for Rel-16. The Gold RE-level scrambling sequence  is extended  times, , where  is the total number of UE groups using the same initialization value  as defined in Section 6.11B.1 [4]. For WUS scrambling, a shorter subsequence , , from the new long Gold sequence  is selected according to the UE group index , , as follows:

,

where an example correspondence between the index  and UE groups is given in Table 1.

Table 1.

| UE group index, | UE group |
| --- | --- |
| 0 | Legacy Rel-15/ common group |
| 1 | UE group-0 |
| 2 | UE group-1 |

The procedure is illustrated in Figure 2.

Figure 2. Illustration of WUS sequence generation in Rel-16.

It can be easily seen that for the group of legacy Rel-15 UEs (), the proposed procedure produces the same WUS as in Rel-15, thus, providing the required backwards compatibility.

There are other ways to generate multiple WUSes for different UE groups based on the legacy WUS generation such as scrambling initialization, Zadoff-Chu phase shift, and orthogonal cover codes (OCCs) in the frequency domain. According to [5], the most promising approach is OCC in the frequency domain. Below the cross-correlation result analysis is provided for the proposed UE-group WUS sequence generation (also referred as phase shifted scrambling code, i.e., PSSC) and OCC (Figure 3). The ratio of the max cross-correlation value in the time domain within a timing window t to the max autocorrelation peak at zero timing offset is considered. Although we confirm that the max cross-correlation value is better (i.e., lower) in OCC case, it only corresponds to a very small portion of cases, while the rest part of CDF curves corresponding to most cases demonstrates lower cross-correlations for the proposed design.

Figure 3. Cross-correlation results analysis for different timing windows t (samples @ 1.92MHz sampling rate), no frequency offset.

Based on that, the following proposal can be made:

Proposal 3:

UE-group WUS sequence generation in Rel-16 is based on  times longer RE-level scrambling Gold sequence, where  is the total number of UE groups (including legacy Rel-15 UEs).

The initialization value for the long Gold scrambling sequence is reused from Rel-15.

The UE-group WUS sequence is a shorter subsequence from the long Gold scrambling sequence defined by the UE group index .

# Conclusions

In this contribution, we discussed different considerations on UE-group WUS for eMTC devices in Rel-16. Our proposals are summarized as below:

Proposal 1:

Up to 2 orthogonal WUS resources per dimension (time/frequency), which results in up to 4 orthogonal WUS resources total, can be configured.

  - This accounts the legacy WUS resource

One of the WUS resources can be configured to be shared with legacy WUS

  - If not configured, the WUS resource is used only for legacy WUS transmission

Legacy WUS is used as a common WUS to wake up all UEs monitoring the same WUS resource.

Proposal 2:

For SS-CDM, the max number of WUS sequences within the same WUS resource is equal to 3.

  - This corresponds to UE group-0, UE-group-1 and the common UE group.

Proposal 3:

UE-group WUS sequence generation in Rel-16 is based on  times longer RE-level scrambling Gold sequence, where  is the total number of UE groups (including legacy Rel-15 UEs).

The initialization value for the long Gold scrambling sequence is reused from Rel-15.

The UE-group WUS sequence is a shorter subsequence from the long Gold scrambling sequence defined by the UE group index .

# Reference

RP-181450, “Rel-16 MTC enhancements for LTE”, Ericsson, 3GPP TSG RAN meeting #80, La Jolla, USA, June 2018.

RAN1 Chairman’s Notes. 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019.

R1-1902366, “UE-group wake-up signal for MTC”, Qualcomm Inc., 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019.

3GPP TS 36.211. V15.2.0. Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 15). 2018-06.

R1-1812119, “UE-group wake-up signal in LTE-MTC ”, Ericsson, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 12th-16th, 2018.