3GPP TSG RAN WG1 Meeting #96b                                                         	          R1-1904285

Xi'an, China, April 8th – April 12th, 2019

Agenda Item:	7.2.2.2.1

Source: 	Intel Corporation

Title:	Channel access mechanism for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #82, a new WID on NR-based access to Unlicensed Spectrum (RP-182878) [1] was approved. As part of the objectives of the work item, the following physical layer aspects related to the design of initial access signals and channels for unlicensed operation were suggested to be specified as per the outcome of the study item (TR38.889) [2]:

Physical layer procedure(s) including [RAN1, RAN2]: - For LBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). Specification work to be performed by RAN1. - For FBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). FBE is intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc. Further, the targeted scenario is limited to a single NR-U network (i.e. single operator) in the operating band and geographic area. Specification work to be performed by RAN1.

In this context, the following agreements related to channel access mechanism were made during the related study item (SI), and during the previous RAN1 meetings [3-4]:

Agreement: - LTE-LAA channel access mechanism is adopted as baseline for 5GHz - Further enhancements not precluded - LTE-LAA channel access mechanism is adopted as starting point of the design for 6GHz - Further enhancements not precluded - For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation. - Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence. - No-LBT option can be applied to 6GHz band if allowed by regulation - Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band Note: Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges. Agreement: In addition to aspects considered in LTE LAA, CWS (Contention Window Size) adjustment procedure in NR-U may additionally consider at least the following aspects: - CBG based HARQ-ACK operation, - NR scheduling and HARQ-feedback delays and processing times - wideband (>20 MHz) operation including BWPs - Configured grant operation Agreement: - For initiation of a COT by the UE, following LBT schemes are used Channels / signals initiating the COT; Cat 2 LBT; Cat 4 LBT. UL; PUSCH (including at least UL-SCH with user plane data); N/A except for the cases discussed in Note 2 below; Channel access priority class is selected according to the data. SRS-only; N/A; Cat4 with lowest channel access priority class value (as in LTE eLAA). RACH -only; (see Note 2); Cat4 with lowest channel access priority class value. PUCCH -only; (see Note 2); Cat4 with lowest channel access priority class value. Note 1: If the COT includes multiple signals/channels with different channel access categories / priority classes, the highest channel access priority class value and highest channel access category among the channel access priority classes and channel access categories corresponding to the multiple signals/channels applies. Note 2: Applicability of an LBT scheme other than Cat 4 for the following signals / channels have been discussed. - UL control information including UCI only on PUSCH, e.g.: - HARQ-ACK - Scheduling Request - Channel State Information - Random Access - Further discuss the aspects related to Note 2 in the work item Agreement: - It has been identified that FBE operation for the scenario where it is guaranteed that LBE nodes are absent on a long term basis (e.g., by level of regulation) and FBE gNBs are synchronized can achieve the following. - Ability to use frequency reuse factor 1 - Lower complexity for channel access due to lack of necessity to perform random backoff - FFS requirement of synchronization accuracy - FFS specification impact - Note: This does not imply that LBE does not have benefits in similar scenarios although there are differences between the two modes of operation Note: FBE may also have some disadvantages compared to other modes of operation such as LBE, e.g., a fixed overhead for idle time during a frame Agreement: - Study FBE (as in the ETSI BRAN specifications) based frame structure - Identify the changes needed to support FBE operation of NR-U - Restrictions/conditions on when FBE option can be used will be further identified, in consideration of fair coexistence. - Strive to minimize the change from current NR design Agreement: Capture the following in the TR: For FBE: - gNB acquires COT with Cat2 immediately prior to the fixed frame period - within the gNB acquired COT, if a gap is <= 16 us, Cat 1 can be used by the gNB and associated UEs - within the gNB acquired COT, if a gap is > 16 us, Cat 2 should be used by the gNB and associated UEs - Note: This is intended to be aligned with any regulations for FBE operation Agreement (during #AH 1901 [3]): - A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of: - Timing Advance - CP extension - max value of not more than one OFDM symbol - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching - Note: the mechanisms applied in each case may be different for different SCSs - FFS: how to signal the way of creating the gap to the UEs Agreement: For initiation of a gNB transmission: - LBT other than Cat 4 is not used for DRS multiplexed with unicast data - LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS. Note: - This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths Agreement: LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy - Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

In this contribution, the following aspects related to the channel access mechanism are addressed:

- CWS adjustment procedure for both DL and UL, including some related details (e.g., definition of reference burst, and timing between reference burst and scheduling request/feedback information) to handle both TB-based and CBG-based HARQ-ACK operation.

- Channel access details for the FBE design.

- Details related to MCOT sharing between gNB and UE, and vice versa.

# CWS Adjustment procedure

During Rel. 13/14/15, a great effort was devoted in designing a contention window size (CWS) adjustment mechanism for both DL and UL (for ether scheduled or grant-free UL transmissions), which is in par with other incumbent technologies. For either DL or UL, the legacy LTE LAA CWS adjustment procedure is performed based on the HARQ-ACK feedback related to a reference subframe (SF), which is defined as:

- DL: the reference SF is the first DL SF of the latest DL data burst for which HARQ-ACK feedback is available.

- UL: the reference SF set is the first SF with UL-SCH that was transmitted at least 4 ms prior to the UL grant reception in the most recent transmitted burst of contiguous SF that is transmitted after performing a category 4 LBT procedure is defined as the reference SF.

Considering that NR-U needs to fairly coexist with all the incumbent technologies, including LAA, and great effort was already spent during previous WIs for LAA to define a CWS adjustment mechanism, it is advisable to reuse the legacy CWS adjustment mechanism as a baseline. However, some modifications are needed since for NR-U together with TB-based transmissions, CBG-based re-transmissions can be also configured. Furthermore, the concept of SF has been replaced with the concept of slot, whose length depends on the subcarrier spacing (SS):  the duration of the slot is 1 ms, 500 us, 250 us, for 15 kHz, 30 kHz, and 60 kHz, respectively.

## Reference Burst

In order to mimic the legacy mechanism, the concept of reference burst can be introduced. Similarly as LAA, since the initial part of the burst is that which may be more impacted by interference from other nodes, the reference burst is counted from the beginning of the latest DL or UL burst for which HARQ-ACK feedback or an UL grant is available. Given the flexibility of NR-U, and in order to maintain coexistence with LAA, and capture within the reference burst the same amount of interference that is captured in LTE within a reference SF, the reference burst is one slot long, which for 15 KHz would coincide with the definition of reference SF in legacy LTE LAA design. In case either the latest DL or the UL burst for which HARQ-ACK feedback or an UL grant is available starts within a slot, creating a so called partial slot, and the burst prolongs further, the reference burst includes both the partial slot and the following slot. In case the latest DL or UL burst for which HARQ-ACK feedback or an UL grant is available is only composed by a partial burst, the reference burst only contains that partial burst. An illustration of this concept is provided in Fig. 1, which depicts the DL reference burst in two cases: i) case A: the latest DL burst for which HARQ-ACK feedback is available starts at the slot boundary, and the reference burst is constituted by the first slot; ii) case B:  the latest DL burst for which HARQ-ACK feedback is available starts within a slot, and the reference burst is constituted by the first partial slot plus the following slot.

#### Proposal 1: The reference burst is defined as the first slot of the latest DL or UL burst for which HARQ-ACK feedback or an UL grant is available.

#### Proposal 2: If the latest DL or the UL burst for which HARQ-ACK feedback or an UL grant is available starts with a partial slot:

#### and if this burst prolongs further, the reference burst includes both the partial slot and the following slot;

- and if this burst is only composed by a partial slot, the reference burst only includes the partial slot.

Fig. 1 – Illustration of the concept of DL reference burst.

In the context of the reference burst, it is also important to define the minimum time N between the beginning of UL reference burst, and the beginning of the CORESET containing the following UL grant or a DFI DCI in case the configured-grant (GC) UL transmission is configured, which is illustrated in Fig. 2. In legacy LTE LAA, the minimum time between the beginning of reference SF and the beginning of the following UL grant or DFI-DCI is fixed and related to the processing time for PUSCH decoding, which was established to be 4 ms. Under the consideration that NR operates based on a symbol level processing time, and SS may be configured among a set of choices, in NR-U the minimum time N between the start of an UL reference burst and the last symbol of the CORESET containing the following UL grant or a DFI-DCI could be configured.

Fig. 2 – Illustration of timing between an UL reference burst and the following COSESET containing UL grant or DFI DCI.

Proposal 3: A reference burst starts at least N symbols prior to the end of the CORESET containing the following UL grant or a DFI-DCI, where N is configurable.

## 2. 2 Handling of CBG-Based Transmissions

As mentioned at the beginning of this section, in NR-U CBG-based re-transmissions may be configured. However, the legacy LTE LAA CWS adjustment mechanism was designed to account only for TB-based transmissions.  In particular, for DL the legacy CWS procedure consists of the following 2 steps:

1)	for every priority class set

2)	if at least  of HARQ-ACK values corresponding to PDSCH transmission(s) in reference SF are determined as NACK, increase  for every priority class to the next higher allowed value and remain in step 2; otherwise, go to step 1.

While this legacy procedure can be reused in NR-U when either CBG-based or TB-based re-transmissions are configured, the way how Z is determined must be changed to account for CBG-based HARQ-ACK feedbacks. In fact, if per-CBG retransmission is configured, feedback is provided per CBG and the CBG information is encoded in the CBG transmission information (CBGTI), which contains 0, 2, 4, 6, or 8 bits for both DL and UL. This bitmap, with one bit per CBG is fed back instead of a single bit representing the whole transport block (TB).

Along the ETSI BRAN [5], the following rules on how to adjust the CWS are provided:

“An Initiating Device shall operate at least one and no more than four different Channel Access Engines each with a different Priority Class as defined in clause 4.2.7.3.2.4:

- The Channel Access Engine shall set CW to CWmin.

- The Channel Access Engine shall select a random number q from a uniform distribution over the range 0 to CW. Note 2 in table 7 defines an alternative range for q when the previous or next Channel Occupancy Time is greater than the maximum Channel Occupancy Time specified in table 7.

……

- When the Channel Occupancy has completed, and it has been confirmed that at least one transmission that started at the beginning of the Channel Occupancy was successful, the Initiating Device proceeds with step 1) otherwise the Initiating Device proceeds with step 8).

The Initiating Device may retransmit. If the Initiating Device does not retransmit the Channel Access Engine shall discard all data packets associated with the unsuccessful Channel Occupancy and the Channel Access Engine shall proceed with step 1). Otherwise, the Channel Access Engine shall adjust CW to ((CW + 1) × m) - 1 with m ≥ 2. If the adjusted value of CW is greater than CWmax the Channel Access Engine may set CW equal to CWmax. The Channel Access Engine shall proceed with step 2). “

According to this set of rules, the ETSI BRAN mandates that if at least one “transmission” that started at the beginning of the channel occupancy time was successful, the Initiating Device may reset the contention window. However, it is important to remark that the ETSI BRAN does not provide any clear guidance on how to interpret the term “transmission”, and therefore to reduce any possible risk of non-compliance, it is advisable to be cautious and conservative in the interpretation of the regulatory requirements.  In LAA Rel. 13, the term “transmission” was mapped to the concept of TB transmission, and the CWS adjustment was tailored based on this assumption. For an NR-U system, CBG based transmissions may be enabled, however a CBG can be only seen as a portion or part of a TB, and cannot be interpreted as a transmission unit.

With this in mind, even if CBG based transmission is configured, a TB would be considered as a NACK, and counted for the evaluation of Z if at least one of the currently transmitted CBGs of the TB is NACK’ed, similarly as in LTE legacy LAA.

#### Proposal 4: If CBG based transmission is configured, a TB would be considered as a NACK, and counted for the evaluation of Z if at least one of the currently transmitted CBGs of the TB is NACK’ed.

Assuming that some UEs may be configured with CBG-based transmissions, while others may perform TB-based transmissions, and in this case the percentage of NACKs Z should be handled jointly. In this matter, Z can be evaluate as follows:

#### Z = (u*NADLCBG + (1-u)*NADLTB) / (u*NDLCBG + (1-u)*NDLTB)      (2)

where

- NADLCBG is the number of NACKs that are counted within the reference burst for the CBG-based transmissions, where a NACK is counted if at least one of the currently transmitted CBGs of the TB is NACK’ed;

- NADLTB is the number of NACKs that are counted wper TB within the reference burst for the TB-based transmissions;

- NDLCBG is the total number of feedbacks (counted per TB) for the CBG-based transmissions performed in the DL reference burst;

- NDLTB is the total number of feedbacks for the TB-based transmissions performed in the DL reference burst.

The value u can instead have values between 0 and 1, which can be configured based on the configuration or can be chosen as a fixed value.

Note that equation (2) is generic, flexible, and backwards compatible with the legacy LTE LAA behaviour: in fact if u=0, then only TB-based transmissions are counted toward the CWS adjustment in the same way as in legacy LTE LAA. If u=1/2, CWS update is based on ACK/NACKs of transport blocks by putting same weights to both CBG based transmission and TB based transmission. Also note, that (2) can be applied whether gNB shares the COT or not.

If the gNB shares the COT, and there is no PDSCH transmission, the formula above applies using the UL data burst instead of the DL data burst.

#### Proposal 5: For NR-U the DL CWS adjustment procedure from Legacy-LTE is reused. However, the adjustment metric Z is evaluated as

Z = (u*NADLCBG + (1-u)*NADLTB) / (u*NDLCBG + (1-u)*NDLTB)

where NADLCBG is the number of NACKs that are counted within the reference burst for the CBG-based transmissions, where a NACK is counted if at least one of the currently transmitted CBGs of the TB is NACK’ed; NADLTB is the number of NACKs that are counted per TB within the reference burst for the TB-based transmissions; NDLCBG is the total number of feedbacks (counted per TB) for the CBG-based transmissions performed in the DL reference burst; and NDLTB is the total number of feedbacks for the TB-based transmissions performed in the DL reference burst. The value u can instead have values between 0 and 1, which can be configured based on the configuration or can be chosen as a fixed value.

Similarly as legacy LTE LAA, for the CA deployment based on how scheduling is performed (e.g. self-scheduling or cross-carrier scheduling) the way how the ‘DTX’ feedback would be interpreted toward the CWS adjustment may be different. In particular, in the case that the PDCCH is transmitted in a separate channel, ‘DTX’ should be ignored, and when PUCCH is transmitted in the same channel, it is an indication that there may be a collision.  Thus, ‘DTX’ should be treated as a NACK.

Proposal 6: For CA deployment, if the HARQ-ACK value corresponding to a PDSCH transmission is scheduled by gNB on the same channel, ‘DTX’ is counted as a NACK.  Otherwise, it is ignored.

Unlike LTE LAA, several new deployment scenarios have been envisioned for NR-U, including some cases where all UL channels will be transmitted on an unlicensed carrier including PUSCH, PUCCH, and PRACH (e.g. standalone scenario and dual connectivity scenario). In this specific cases, due to the LBT failures at the UE side, it is not guaranteed that the UE’s HARQ-ACK feedback is received by the gNB before the next DL transmission. In this case, some rules may need to be defined how the CWS should be adjusted in absence or late reception of the HARQ-ACK feedback information. In principle, the problem described here is very similar to what is specified for the UL CWS adjustment for autonomous UL transmissions in LAA, and similar rules could be defined for DL CWS case in NR-U. For instance, a new timer can be introduced, and the following high level procedure can be used:

- When DL data is transmitted, the timer is started;

- If no HARQ-ACK feedback is received and the timer has not yet elapsed, CWS may remain unchanged;

- If the timer elapses and no HARQ-ACK feedback is received, the CWS is increased to the next higher value;

- If the feedback for one or more previous Cat-4 LBT DL transmission is received after the timer expired without reception of any feedback, then

  - Step 1: revert the CWS to the value used to transmit the first burst of previous Cat-4 LBT transmission(s);

  - Step 2: update the CWS sequentially in order of the transmission:

    - if the feedback indicates ACK for the reference PDSCH(s) of that burst, CWS is reset;

    - else (if the feedback indicates NACK or there is no feedback), the CWS is doubled.

Proposal 7: When NR-U operates so that the UL HARQ-ACK feedback transmissions are provided over an unlicensed carrier, similar rules as that used for the UL CWS adjustment for AUL in feLAA are adopted for the DL CWS adjustment.

Now for UL in FeLAA, if a UE receives an UL grant or an AUL-DFI, the contention window size for all the priority classes is adjusted as following:

-	If the NDI value for at least one HARQ process associated with HARQ_ID_ref is toggled, or if the HARQ-ACK value(s) for at least one of the HARQ processes associated with HARQ_ID_ref received in the earliest AUL-DFI after +3 indicates ACK.

-	For every priority class set

-	Otherwise, increase  for every priority class to the next higher allowed value;

In NR-U, the general procedure described above can be reused, but it is important to consider that when CBG-based transmissions are configured, the CWS update should account for the following:

- If a configured grant (CG)-DFI is received, and if CBG-based configuration is used, the CWS will be reset to its minimum value if all of the currently scheduled CBGs of the TB are ACK’ed. Otherwise, the CWS should be increased.

- If an UL grant is received, and CBG-based transmissions is configured, the UE knows the status of each CBG via the CBG Transmit Information (CBGTI). If the NDI bit is not toggled (i.e. retransmission), it should consider a NACK if any of the CBGTI bits is set to 1.

Proposal 8: If CBG-based transmission is configured and AUL-DFI is received, the UL CWS will be reset to its minimum value if all of the currently transmitted CBGs of the TB are ACK’ed. Otherwise, the CWS should be increased.

Proposal 9: If CBG-based transmission is configured and UL grant is received, if any individual bits of the CBGTI is set to 1, this will be interpreted as a failure, when the NDI is not toggled (i.e. retransmission) for the same HARQ process, i.e. NACK; otherwise it is considered as successful, i.e. ACK.

# Channel access for FBE

According to the ETSI BRAN [5], a wireless device can operate on the 5 GHz unlicensed band either following the load based equipment (LBE) or the frame based equipment (FBE) framework. While the legacy LTE LAA design is solely based on the LBE framework, during the NR-U SI it was agreed to design NR-U also on the base of the FBE framework striving to minimize changes from current NR design.

When operating a system using the FBE framework, the channel access is based on the concept of fixed frame periods (FFP) as shown in Fig. 3. A device can operate as an initiating or responding device. An initiating device acquires the channel by performing a Cat-2 LBT, and initiates transmission at the beginning of a FFP. Within a FFP, an initiating device can perform multiple transmissions or grant an authorization to one or more associated responding devices to transmit on the current operating channel within the current FFP. The operation through a FBE framework has a periodic timing with a periodicity equal to the FFP itself. According to the regulatory requirements, the FFP can be within a range of 1 to 10 ms, and it can be changed no more than once every 200 ms.

In NR-U SI TR [2], we have captured following operation for FBE:

For FBE mode of operation, gNB acquires COT with Cat2 immediately prior to the fixed frame period. Within the gNB acquired COT, if a gap is <= 16 s, Cat 1 channel access scheme can be used by the gNB and associated UEs. Within the gNB acquired COT, if a gap is > 16 s, Cat 2 channel access scheme should be used by the gNB and associated UEs. Note this channel access mechanisms are intended to be aligned with any regulations for FBE operation.

From this agreement and ETSI BRAN regulation, it can be simply assumed that a gNB is considered as an initiating device and UEs are considered as responding devices. Correspondingly, only gNB can acquire the COT by performing CAT-2 LBT and all DL and UL transmissions can be transmitted inside the COT acquired by the gNB.

Fig. 3 – Fixed frame period illustration [5].

The regulatory requirements [5] mandate that within each FFP, the channel occupancy time acquired by the initiating device shall not be greater than 95% of the fixed frame period, and at the tail of each FFP an idle period of at least 5% of the COT should be left with a minimum of 100 μs. Due to this last constrain, if the FFP is shorter than 2 ms, then the spectral efficiency is highly impacted. In order to illustrate this concept, in Fig. 4 it is shown the maximum percentage of the FFP, which can be used as a COT under the limitation that an idle period has to be at least 100 μs long, in function of the FFP length. Fig. 4 highlights that if the FFP is shorter than 2 ms, then the percentage of FFP which can be used as a COT becomes lesser than 95 %.

That said, the FFP length should be decided according to the rules:

- A FFP shall be larger than 2 ms to guarantees the highest spectral efficiency,

- the regulatory requirements impose a strict rule on how the FFP should be changed in time (i.e., once every 200 ms),

- FBE aims to minimize the changes from the Rel. 15 design, and therefore the length of the FFP should be chosen such that to avoid cross-FFP scheduling and feedback transmissions.

With the aforementioned rules in mind, the baseline for the selection of the length of the FFP can be 10 ms, and the FFP aligns with the radio frame.

Fig. 4 – Maximum percentage of the FFP which can be used as a COT as function of the FFP length.

Observation 1: Considering that the regulatory requirements for FBE imposes a minimum idle period of 100 μs, if the FFP is less than 2 ms, then the spectral efficiency is significantly affected.

Proposal 10: As a baseline, the FFP aligns with the radio frame boundary.

According to the regulatory requirements [5, Sec. 4.2.7.3.1.4 and Sec. 4.2.7.3.1.5], an initiating device is mandated to grant an authorization to one or more associated responding devices to transmit on the current operating channel within the current FFP:

- Sec. 4.2.7.3.1.4 : “(3) An Initiating Device is allowed to grant an authorization to one or more associated Responding Devices to transmit on the current Operating Channel within the current Channel Occupancy Time. A Responding Device that receives such a grant shall follow the procedure described in clause 4.2.7.3.1.5.”

- Sec. 4.2.7.3.1.5 : “Clause 4.2.7.3.1.4, point 3) describes the possibility whereby an Initiating Device grants an authorization to one or more associated Responding Devices to transmit on the current Operating Channel within the current Fixed Frame Period.”

Above clauses can be simply interpreted that transmissions from responding devices are only allowed if any authorization is granted by an initiating device within the current channel occupancy time or FFP. As discussed above, the gNB is considered as the initiating device, which initiates the transmission and provides a scheduled grant to its associated UE within the current FFP, and a UE is considered as the responding device, which receives a scheduled grant to transmit within a specific FFP from the gNB.

However, on this matter a configured grant transmission cannot be enabled inside the COT acquired by the gNB, since it is not compliant with the aforementioned regulatory requirement: in fact [5] mandates a UE to receive a transmission grant from the gNB in all FFP where CG would be allowed, before transmission can be performed, which would limit greatly the autonomy of CG UL transmissions, and defeat the purpose of this feature.

And also, for the same reason as above, this regulatory constraint does not allow in principle to transmit the PRACH preamble inside the COT acquired by the gNB.

Observation 2: For NR-U in FBE mode, the above ETSI BRAN requirement on FBE does not allow configured grant transmissions and PRACH transmission within a gNB-acquired COT due to the lack of gNB’s grant.

However, both configured grant and PRACH are important features in NR-U. At least the PRACH is one of the most essential channels for the NR-U system design even for FBE. Therefore, we may need to discuss further how to enhance PRACH in order to support it for NR-U in FBE mode.

Proposal 11: RAN1 needs to study how to support PRACH for NR-U FBE mode in accordance with ETSI BRAN regulation

The aforementioned regulatory requirement also forbids any cross-FFP scheduling of UL transmissions and any cross-FFP HARQ scheduling, since it clearly and specifically mandates that a responding device is allowed to transmit within a FFP only if the transmission grant for that transmission is provided within the same FFP. Therefore, the length and configurability of a FFP is impacted, and a FFP should be configured to be long enough to allow intra-FFP scheduling and HARQ report, and also to allow the UE to cope with the related processing times.

Proposal 12: In FBE, in accordance with ETSI BRAN regulation only intra-FFP scheduling and HARQ report are allowed.

Based upon the regulatory requirements, even if a responding device (a.k.a a UE) is scheduled to transmit on a certain FFP, it is only allowed to transmit if the initiating device (a.k.a. gNB) succeeds to perform Cat-2 LBT on that FFP, which is indicated as a valid FFP. Therefore, before a UE performs a transmission it needs to assess whether or not a FFP is valid or not. In this matter, a UE can perform the downlink burst detection procedure which is discussed in the DL channel agenda.

Proposal 13: For FBE, to assess whether a COT is acquired by the gNB, a UE performs the DL burst detection mechanism as discussed in the DL channel agenda.

It was agreed that FBE design should try to minimize the change from current NR design. Achieving synchronization accuracy between a single operator’s devices is already a hard problem.  Adding a second operator would add significant overhead to the specification.

Proposal 14: To reduce complexity for FBE operation, single operator scenario is prioritized.

# Shared COT

In order to maximize channel usage, it is of interest to allow sharing from either a UE or a gNB of their COT in case this is not fully utilized.

In IEEE 802.11ac, Wi-Fi devices can have a DL Multiuser (MU) packet, denoted as VHT MU PPDU below, with either immediate acknowledgement (i.e. Block Ack, or BA,  (32 bytes) or Ack (14 bytes) after VHT MU PPDU) or no immediate acknowledgement (i.e. Block Ack Request, or BAR, after VHT MU PPDU), as shows in the Figures 5 and 6 [6], respectively.  In both Figures below, simultaneous transmission to 3 users is performed and 3 DL/UL switching points occur within a shared COT from an AP. However, in general, given N the maximum number of users, IEEE 802.1 ac allows N•2 switching points, where N is equal to 4.  Note that the typical duration of an exchange, is around 2 ms, and given that a single MCOT has a duration of 6-8 ms, this translates to 3 or 4 DL/UL switching points that occur in a sequential fashion. What is important to note is that the duration of the BA/Ack frames are short in duration (around 100us) and the interference they may cause to other nodes is very limited.

Figure 5 – 802.11ac Downlink Multiuser Sequence with Immediate Ack

Figure 6 – 802.11ac Downlink Multiuser Sequence without Immediate Ack

In IEEE 802.11ax, Uplink and Downlink OFDMA along with Uplink MU-MIMO are introduced, and the maximum number of users in DL MU-MIMO is increased up to 8.   In IEEE 802.11ax, Uplink OFDMA improves the medium usage of the Acks so that the TXOP has at least a single switching point, as shown in Figure 7 [7]. This is similar to having an UL transmission following a DL transmission in a gNB acquired COT.

Figure 7 – 802.11ax Downlink Multiuser Sequence with Aggregated Block Acks

Furthermore, in IEEE 802.11ax, an uplink OFDMA MU transmission, with maximum packet duration of 5.484ms, is preceded by a trigger frame so that the MCOT has at least 2 switching points, as shown in Figure 8. To draw a parallel with NR-U, this is similar to have an UL burst following a DL transmission in the gNB acquired COT.  Note that the UE is not limited to uplink OFDMA transmissions, and can still acquire a COT without its transmissions having to be preceded by a DL transmission, in the same manner it had done in IEEE 802.11ac.

Figure 8 – 802.11ax Uplink Multiuser Sequence with Aggregated Block Acks

In IEEE 802.11ax, the concept of MU cascading has been introduced as well, where multiple switching points are allowed in a single MCOT.  The frames contained therein are either control, data, or management frames in both directions.  Please see an example exchange in Figure 9. The MU cascading sequence may have a different set of transmitters in high-efficiency (HE) TB PPDUs, as compared to the receivers of the HE MU PPDU that immediately follows the HE TB PPDUs within the same TXOP. The MU cascading sequence may have a different set of receivers in the HE MU PPDU, as compared to the set of transmitters of HE TB PPDUs that immediately follow the HE MU PPDU within the same TXOP. Note that if the AP does not receive immediate response from any of the STAs, then the AP follows standard enhanced distributed channel access (ECDA) backoff procedure.

Figure 9 – An example of cascading 802.11ax Multiuser Sequence

Though not shown in the Figure 8 above, after the AP captures the medium, Wi-Fi STAs that have been given permission to transmit an UL MIMO by an AP’s trigger frame may be required to perform additional sensing of the medium before transmission occurs. Note that the maximum UL MIMO (i.e. HE TB PPDU) duration, which is allowed without performing additional channel sensing, is 584us. Note that UL MIMO packet duration could be on the order of 4-5 ms, but in those cases, additional channel sensing would be required.   When sensing is required, the UE, or STA, shall perform the energy-detect (ED) at least in the subchannel that contains the STA’s UL allocation, where the sensed subchannel consists of one or more 20 MHz channels. Further details can be found in Sections 27.5.3.3 and 27.5.3.5 in [7].

Given the procedure followed by IEEE 802.11ac, and IEEE 802.11ax, which are described above, and with the scope of mimicking their behaviour, while maintaining far co-existence with these technologies, in NR-U the acquired COT should be allowed to be shared, and multiple switching points should be supported. However, the interference from/to other devices that multiple switching points may cause should be further studies, together with methodologies to limit their impact.

Proposal 15: COT with multiple switching points is supported, and the interference from/to other devices should be studied further.

# Directional LBT

Beamformed transmissions have the benefits to allow for spatial multiplexing and limit interference level throughout directional narrow beam transmissions. However, the regulatory requirements [5] impose constraint on the total transmit power in case of multi-antenna transmissions preventing us from getting the full benefit from beamforming transmissions due to the imposed coverage limitations. During NR-U SI, it was agreed that omni-directional LBT should be supported. Using directional LBT for beamforming transmissions, i.e. LBT performed in the direction of the transmitted beam, was also a subject of the discussion.  However, since the scope of the Rel-16 NR-U WI is now limited up to sub-7 GHz band (FR1), having narrow beams at the UE does not seem feasible.  Therefore, directional LBT should not be supported in Rel-16.

Proposal 16: Directional LBT is not supported in Rel-16.

# Conclusion

In this contribution, we discussed several aspects related to the channel access mechanism, and made the following proposals:

#### Proposal 1: The reference burst is defined as the first slot of the latest DL or UL burst for which HARQ-ACK feedback or an UL grant is available.

#### Proposal 2: If the latest DL or the UL burst for which HARQ-ACK feedback or an UL grant is available starts with a partial slot:

#### and if this burst prolongs further, the reference burst includes both the partial slot and the following slot;

- and if this burst is only composed by a partial slot, the reference burst only includes the partial slot.

Proposal 3: A reference burst starts at least N symbols prior to the beginning of the CORESET containing the following UL grant or a DFI-DCI, where N is configurable.

#### Proposal 4: If CBG based transmission is configured, a TB would be considered as a NACK, and counted for the evaluation of Z if at least one of the currently transmitted CBGs of the TB is NACK’ed.

#### Proposal 5: For NR-U the DL CWS adjustment procedure from Legacy-LTE is reused. However, the adjustment metric Z is evaluated as

Z = (u*NADLCBG + (1-u)*NADLTB) / (u*NDLCBG + (1-u)*NDLTB)

where NADLCBG is the number of NACKs that are counted within the reference burst for the CBG-based transmissions, where a NACK is counted if at least one of the currently transmitted CBGs of the TB is NACK’ed; NADLTB is the number of NACKs that are counted per TB within the reference burst for the TB-based transmissions; NDLCBG is the total number of feedbacks (counted per TB) for the CBG-based transmissions performed in the DL reference burst; and NDLTB is the total number of feedbacks for the TB-based transmissions performed in the DL reference burst. The value u can instead have values between 0 and 1, which can be configured based on the configuration or can be chosen as a fixed value.

#### Proposal 6: For CA deployment, if the HARQ-ACK value corresponding to a PDSCH transmission is scheduled by gNB on the same channel, ‘DTX’ is counted as a NACK.  Otherwise, it is ignored.

Proposal 7: When NR-U operates so that the UL HARQ-ACK feedback transmissions are provided over an unlicensed carrier, similar rules as that used for the UL CWS adjustment for AUL in feLAA are adopted for the DL CWS adjustment.

#### Proposal 8: If CBG-based transmission is configured and CG-DFI is received, the UL CWS will be reset to its minimum value if all of the currently transmitted CBGs of the TB are ACK’ed. Otherwise, the CWS should be increased.

#### Proposal 9: If CBG-based transmission is configured and UL grant is received, if any individual bits of the CBGTI is set to 1, this will be interpreted as a failure, when the NDI is not toggled (i.e. retransmission) for the same HARQ process, i.e. NACK; otherwise it is considered as successful, i.e. ACK.

Observation 1: Considering that the regulatory requirements for FBE imposes a minimum idle period of 100 μs, if the FFP is less than 2 ms, then the spectral efficiency is significantly affected.

Proposal 10: As a baseline, the FFP aligns with the radio frame boundary.

Observation 2: For NR-U in FBE mode, the above ETSI BRAN requirement on FBE does not allow configured grant transmissions and PRACH transmission within a gNB-acquired COT due to the lack of gNB’s grant.

Proposal 11: RAN1 needs to study how to support PRACH for NR-U FBE mode in accordance with ETSI BRAN regulation

Proposal 12: In FBE, in accordance with ETSI BRAN regulation only intra-FFP scheduling and HARQ report are allowed.

Proposal 13: For FBE, to assess whether a COT is acquired by the gNB, a UE performs the DL burst detection mechanism as discussed in the DL channel agenda.

Proposal 14: To reduce complexity for FBE operation, single operator scenario is prioritized.

Proposal 15: COT with multiple switching points is supported, and the interference from/to other devices should be studied further.

Proposal 16: Directional LBT is not supported in Rel-16.

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc, 3GPP TSG RAN Meeting #82, Dec. 2018.

[2] 3GPP TR 38.889, “Study on NR-based Access to Unlicensed Spectrum (Rel-16),” V 16.0.0, Nov. 2018.

[3] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #AH 1901, January, 2019.

[4] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96, February, 2019.

[5] ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, (2017-05).

[6] IEEE 802.11-2016, “IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”.

[7] IEEE Draft P802.11ax D3.3, “High Efficiency WLAN”.