3GPP TSG RAN WG1 Meeting #96bis R1-1904471 Xi’an, China, 8th – 12th April, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; DRAFT; rev; Current version:; 15.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on the determination of the minimum number of PRBs for PUCCH transmission. Source to WG:; MediaTek Inc.. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-4-8. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; To determine the number of PRBs for PUCCH transmission, a minimum number of PRBs is selected and satisfies the max code rate conditions described in 38.213. However, There is additional constraint of PRB number for PUCCH format 3 described in 38.211, as shown below It is possible that UE cannot find an appropriate number of PRBs, fulfilling all the conditions. For example, to satisfy the max code rate conditions, i.e. - Actual code rate with MRB,minPUCCH <= the configured max code rate - Actual code rate with (MRB,minPUCCH-1) > the configured max code rate MRB,minPUCCH may be 11 under some configuration. Since 11 is not allowed for PUCCH Format 3, MRB,minPUCCH=12 has to be used. However, this will conflict to the second max code rate condition as shown above.. Summary of change:; Add description that the determination of the minimum number of PRBs also takes into acount the constraint specified in 38.211 for clarification and remove the condition which may not be satisfied by the determined minimum number of PRBs. Impact Analysis: Impacted functionality: the determination of PUCCH minimum number of PRBs The change is to resolve an bug in the specification. UE may not find an appropriate number of PRBs to transmit PUCCH with satisfying all the constraints in the original version. If the UE is implemented according to the change but the NW is not, - NW may not be able to find an appropriate number of PRBs If the NW is implemented according to the change but the UE is not, - UE may not be able to find an appropriate number of PRBs. Consequences if not approved:; UE may not find an appropriate number of PRBs to transmit PUCCH with satisfying all the constraints in the specification. Clauses affected:; 9.2.3 and 9.2.5.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

----------------------------------------------- Start of Text Proposal ---------------------------------------------

9.2.3	UE procedure for reporting HARQ-ACK

< Unchanged parts are omitted >

If a UE transmits a PUCCH with  HARQ-ACK information bits and  bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs according to [4, TS 38.211], that is smaller than or equal to a number of PRBs  provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to MRBPUCCH>1OACK+OCRC>MRB,minPUCCH-1∙Nsc,ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r, where , , , and  are defined in Subclause 9.2.5.2. If , the UE transmits the PUCCH over  PRBs.

< Unchanged parts are omitted >

9.2.5.1	UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH

< Unchanged parts are omitted >

If a UE transmits a PUCCH with  HARQ-ACK information bits,  SR bits, and  CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes  PRBs, the UE determines a number of PRBs  for the PUCCH transmission to be the minimum number of PRBs according to [4, TS 38.211], that is smaller than or equal to a number of PRBs provided respectively by nrofPRBs in PUCCH-format2  or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to MRBPUCCH>1OACK+OSR+OCRC>MRB,minPUCCH-1∙Nsc,ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r, where , , , and  are defined in Subclause 9.2.5.2. If , the UE transmits the PUCCH over the  PRBs.

< Unchanged parts are omitted >

----------------------------------------------- End of Text Proposal ---------------------------------------------