3GPP TSG RAN WG1 Meeting # 96b                                                           R1-1904673

# Xi'an, China,  8th April - 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | xxxx | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on resource mapping related to BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According the following agreements reached in RAN1#94b, it was agreed that the initial DL BWP in PCell can be configured in SIB1 to be the same as or different with the initial DL BWP as initially defined by CORESET#0. That’s to say, the bandwidth of CORESET#0 can be different from the bandwidth of initial DL BWP. As CORESET#0 is mandatory to be configured in PCell of MCG, for DCI format 1_0 in CSS with P-RNTI, SI-RNTI and TC-RNTI, the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in and the maximum number of RBs possible to indicate in the DCI is given by the size of CORESET#0. For DCI format 1_0 in CSS with RA-RNTI, C-RNTI and CS-RNTI, if CORESET#0 is not configured in this cell, then the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in and the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWP. Agreements: - Modify the agreements in RAN1#94 as follows. - If CORESET#0 is configured for a cell, - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET#0 - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET#0 - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI: - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET#0. - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS - If CORESET#0 is not configured for a cell, - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI: - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWP. - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 in CSS However in TS38.211, for PDSCH transmission scheduled with DCI format 1_0 in CSS, the current spec doesn’t fully reflect the above agreements regarding PDSCH mapping from virtual to physical resource blocks. For example, if the initial DL BWP of PCell has been reconfigured by SIB1. In this case, the size of CORESET#0 is not the same as that of initial DL BWP. If the mapping rule for PDSCH containing SIB1 is still based on the size of initial DL BWP, then IDLE UEs can’t successfully receive the SIB1 since they don’t know the size of initial DL BWP configured by SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For PDSCH transmissions scheduled with DCI format 1_0 with the CRC scrambled by SI-RNTI in Type0-PDCCH common search space in CORESET 0, NBWP,initsize should be the size of CORESET#0. - For PDSCH transmissions scheduled with DCI format 1_0 in any common search space other than Type0-PDCCH common search space, NBWP,initsize should be the size of CORESET#0 if CORESET#0 is configured, otherwise it should be the size of the initial DL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For PDSCH transmissions scheduled with DCI format 1_0 in CSS, the PDSCH mapping from virtual to physical resource blocks is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is to correct PDSCH mapping rule from VRB to PRB for PDSCH transmissions scheduled with DCI format 1_0 in CSS. If the initial DL BWP is not reconfigured by SIB1, i.e., initial DL BWP is defined by CORESET#0, there is no impact. If the initial DL BWP is reconfigured by SIB1, i.e., the size of initial DL BWP is not the same as that of CORESET#0, the impact analysis is as below. If UE implements the change but NW does not Since the PDSCH mapping rule from VRB to PRB is not aligned between network and UE, UE can’t successfully decode PDSCH scheduled with DCI format 1_0 in CSS. If NW implements the change but UE does not Since the PDSCH mapping rule from VRB to PRB is not aligned between network and UE, UE can’t successfully decode PDSCH scheduled with DCI format 1_0 in CSS. |  |  |  |  |  |  |  |  |

7.3.1.6	Mapping from virtual to physical resource blocks

The UE shall assume the virtual resource blocks are mapped to physical resource blocks according to the indicated mapping scheme, non-interleaved or interleaved mapping. If no mapping scheme is indicated, the UE shall assume non-interleaved mapping.

For non-interleaved VRB-to-PRB mapping, virtual resource block  is mapped to physical resource block , except for PDSCH transmissions scheduled with DCI format 1_0 in a common search space in which case virtual resource block n is mapped to physical resource block n+NstartCORESET where NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received.

For interleaved VRB-to-PRB mapping, the mapping process is defined by:

-	Resource block bundles are defined as

-	for PDSCH transmissions scheduled with DCI format 1_0 with the CRC scrambled by SI-RNTI in Type0-PDCCH common search space in CORESET 0, the set of NBWP,initsize resource blocks in CORESET 0 are divided into Nbundle=NBWP,initsizeL resource-block bundles in increasing order of the resource-block number and bundle number where L=2 is the bundle size and

-	resource block bundle Nbundle-1 consists of NBWP,initsize mod L resource blocks if NBWP,initsize mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for PDSCH transmissions scheduled with DCI format 1_0 in any common search space other than Type0-PDCCH common search space in bandwidth part i with starting position NBWP,istart, the set of NBWP,initsize virtual resource blocks 0,1,…,NBWP,initsize-1, where NBWP,initsize is the size of CORESET 0 if CORESET 0 is configured and otherwise NBWP,initsize is the size of the initial DL BWP, are divided into Nbundle virtual resource-block bundles in increasing order of the virtual resource-block number and virtual bundle number and the set of NBWP,initsize physical resource blocks NstartCORESET, NstartCORESET+1,…,NstartCORESET+NBWP,initsize-1 are divided into Nbundle physical resource-block bundles in increasing order of the physical resource-block number and physical bundle number, where Nbundle=NBWP,initsize+NBWP,istart+NstartCORESET mod LL, L=2 is the bundle size, and NstartCORESET is the lowest-numbered physical resource block in the control resource set where the corresponding DCI was received.

-	resource block bundle 0 consists of L-NBWP,istart+NstartCORESET mod L resource blocks,

-	resource block bundle Nbundle-1 consists of NBWP,initsize+NBWP,istart+NstartCORESET mod L resource blocks if NBWP,initsize+NBWP,istart+NstartCORESET mod L>0 and L resource blocks otherwise,

-	all other resource block bundles consists of L resource blocks.

-	for all other PDSCH transmissions, the set of NBWP,isize resource blocks in bandwidth part  with starting position NBWP,istart are divided into Nbundle=NBWP,isize+NBWP,istart mod LiLi resource-block bundles in increasing order of the resource-block number and bundle number where  is the bundle size for bandwidth part  provided by the higher-layer parameter vrb-ToPRB-Interleaver and

-	resource block bundle 0 consists of  resource blocks,

-	resource block bundle  consists of  resource blocks if  and  resource blocks otherwise,

-	all other resource block bundles consists of  resource blocks.

-	Virtual resource blocks in the interval  are mapped to physical resource blocks according to

-	virtual resource block bundle  is mapped to physical resource block bundle

-	virtual resource block bundle  is mapped to physical resource block bundle  where

-	The UE is not expected to be configured with Li=2 simultaneously with a PRG size of 4 as defined in [6, TS 38.214]

-	If no bundle size is configured, the UE shall assume Li=2.

The UE may assume that the same precoding in the frequency domain is used within a PRB bundle and the bundle size is determined by clause 5.1.2.3 in [6, TS 38.214]. The UE shall not make any assumption that the same precoding is used for different bundles of common resource blocks.