3GPP TSG RAN WG1 meeting #96b	                                                  	                      R1-1905270

Xi’an, China，8th – 12th April, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Reliability enhancement on PUCCH&PUSCH with multi-TRP/panel

Document for:	Discussion and Decision

# Introduction

In RAN1#95, the following agreement on multi-TRP transmission was achieved [1].

Agreement:

Study for URLLC reliability/robustness enhancement with multi-TRP/panel/beam, including the case of ideal backhaul

- For PDSCH/PUSCH where the same TB is transmitted including

  - #1: the number of TRP/panel/beams

  - #2: Configuration/indication mechanism of TB repetition

  - Other enhancements are not excluded.

- For PDCCH/PUCCH

  - #1: the number of TRP/panel/beams

  - #2: Repetition/Diversity of DCI/UCI

  - Other enhancements are not excluded.

FFS: Non-ideal backhaul case

In this contribution, we provide our views on PUSCH/PUCCH enhancements using multi-TRP transmission.

# Reliability/robustness enhancement on PUSCH&PUCCH with multi-TRP/panel/beam transmission

For the URLLC application, reliability and robustness can be significantly improved with M-TRP/panel/beams for all NR channels. The main focus of this paper is for reliability enhancement over PUSCH&PUCCH.

## PUSCH

In Rel-15, PUSCH repetition based on slot aggregation has already been supported, where the same time and frequency resource allocation indicated in DCI is used in consecutive slots, as shown in Fig.1.

Figure 1: Precoder cycling based PUSCH slot aggregation in Rel-15

Besides, the same precoder is applied across the repetitions. To further improve the reliability/robustness of PUSCH repetition, multiple precoders could be supported for both codebook based and non-codebook based PUSCH slot aggregation as shown in Figure 2.

Figure 2: Precoder cycling based PUSCH slot aggregation

Besides, the PUSCH precoder cycling pattern could be predefined to improve the flexibility as shown in Fig. 3. The switch of pre-defined precoder cycling pattern can be jointly indicated by RRC and DCI.

Figure 3: Example of pre-defined Precoder cycling pattern

Proposal 1: Rel-16 supports PUSCH repetition over multiple TRPs/panels/beams using multiple precoders

## PUCCH

Similar to PUSCH, a spatial diversity offered by multiple TRPs/beams/panels may be benefits for PUCCH as well. Further study on how to indicate spatial information for PUCCH repetitions can be considered.

# Conclusion

This contribution provides our views on reliability/robustness transmission of PUSCH/PUCCH using multiple TRP transmission. In summary, the following proposals are made.

Proposal 1: Rel-16 supports PUSCH repetition over multiple TRPs/panels/beams using multiple precoders

# References

[1] “3GPP RAN1 #95 Chairman’s Notes”, Spokane, USA, 12th -16th November, 2018.