3GPP TSG RAN WG1 #96bis	R1-1905488

Xi’an, China, 8th – 12th April, 2019

Agenda Item:	7.2.6.5

Source: 	Fujitsu

Title: 	Discussion on UL preemption indication

Document for:	Discussion/Decision

Introduction

In RAN1 #96, it was agreed to support both UL cancelation scheme and enhanced power control scheme to be specified.

Agreements in RAN1 #96 - Recommend both UL cancelation scheme and enhanced UL power control scheme to be specified.

In this contribution, we focus on the UL cancelation scheme. Specifically, to have minimal spec impact, we will discuss the design of UL inter-UE preemption indication based on that for DL.

Discussion

Fig.1 illustration of UL resource preemption.

Considering the 10-5 or 10-6 reliability requirement for URLLC, the corresponding transmission of eMBB UE will be cancelled when a UL preemption indication (PI) is detected. Fig. 1 illustrates the main concept of the UL preemption indication, and the main procedures for the UL preemption indication are stated as follows:

- When a UE has UL URLLC data, it sends a URLLC scheduling request (SR) to the gNB by a dedicated channel.

- The gNB generates the UL preemption indicator signal which is sent to the eMBB UE using a DL control channel.

- The UL preemption indicator signal includes the information of the preempted UL resource. This signal enables eMBB UEs to timely stop any ongoing transmission scheduled in the preempted UL resource.

- The related eMBB UEs will not transmit their UL data in the preempted resource if the UL pre-emption indicator is successfully decoded.

UE monitoring periodicity

To avoid interference to URLLC data, the eMBB UE needs to cancel the corresponding UL transmission before the URLLC UE starts to transmit its UL data. In view of the 1ms latency requirement of the sporadic URLLC traffic, the slot-level monitoring for the UL preemption indicator may not be feasible to enable eMBB UE stop its transmission in a timely way, especially when the subcarrier spacing is small. Thus, to avoid collision of URLLC traffic and eMBB traffic, the non-slot level monitoring of potential UL PI would be required for eMBB UE. This is feasible [1]. Non-slot level monitoring of PDCCH has already been supported by high-layer parameter monitoringSymbolsWithinSlot.

Proposal 1. Support non-slot level monitoring of the UL preemption indication for eMBB UEs.

UE processing timeline for cancellation

Fig.2 The processing timeline of eMBB UE and URLLC UE when uplink preemption occurs.

Fig.2 illustrates the processing timeline of eMBB UE and URLLC UE when the uplink preemption occurs. After receiving the SR from the URLLC UE, gNB decodes the SR and prepares the UL grant for the URLLC UE. Then, gNB generates the UL PI for the eMBB UE. The time of the UL PI transmission is earlier or the same as that of the UL grant transmission. To satisfy the low latency requirement of the URLLC, the slot offset  between the UL grant and the scheduled PUSCH of the URLLC UE should be smaller than that of the eMBB UE , i.e., <. Due to the impact of the propagation time Tp of UL PI and the timing advance (TA) of the canceled PUSCH, the time of processing UL grant and preparing for  PUSCH  is smaller than, i.e., < . To avoid the collision between the URLLC PUSCH and the eMBB PUSCH and satisfy the extremely low latency requirement in Rel-16, the time of eMBB UE processing UL PI and preparing for PUSCH cancellation should be smaller than  , i.e.,<.

Similar to , due to the impact of TA, the actual processing time for PUSCH cancellation is smaller than . Moreover, gNB should consider the largest TA in the cell range, when calculating  or determining the UE ability  of PUSCH cancellation. Furthermore, similar to the equation of calculating  in [2],  will be equal to , since the canceled PUSCH will not carry DMRS and HARQ-ACK and there will be no BWP switch. Thus, a more strict condition is that <, where  is the UE ability of URLLC PUSCH preparation time.

In summary, the relationship among the above parameters are<<<.

Observation 1. The time  of the eMBB UE processing the UL PI and preparing for the PUSCH cancellation should be smaller than or equal to the time  of the URLLC UE processing UL grant and preparing for the PUSCH transmission, i.e., <.

UL PI monitoring window

According to the relationship is < , i.e., the UL scheduling slot offset is larger than the necessary time of an UL PI taking effect, an eMBB UE only needs to start monitoring UL PI when there is a granted UL transmission (dynamically grant or configured grant). Besides, due to a processing time of  is required to cancel an UL symbol, it is meaningless to monitoring UL PI when the time until the end of the UL transmission is less than , because UE is unable to cancel any symbol of the UL transmission. Accordingly, an UE only needs to monitor UL PI in a period of time, which may start from successful received an UL grant and end at a  time before the last symbol of the UL transmission. Additionally, since < , the starting point of the monitoring window can further later than the time an UL grant is received. The monitoring window can be semi-statically or dynamically informed by gNB so as to decrease the UE power consumption for monitoring UL PI.

Observation 2.

- eMBB UEs don’t need to always monitor UL PI;

- eMBB UEs only need to monitor UL PI when there is a dynamically/configured granted UL transmission.

Proposal 2. The time period for monitoring UL PI should be clarified/defined. Further study is necessary.

Group-common signaling

To achieve low latency, the URLLC traffic usually occupies a very large frequency band, which may affect multiple eMBB UEs. Thus, to reduce the signaling overhead of the UL PI, a group-common signaling is recommended for transmitting the UL PI.

Proposal 3. Support group-common UL PI to reduce the signaling overhead.

Reliability of UL PI

In comparison with the DL PI, the UL PI requires much higher reliability. Because the DL PI is used to indicate the eMBB UE the already canceled PDSCH from gNB, while the UL PI is designed to indicate the eMBB UE to cancel the ongoing or impending PUSCH before the URLLC UE starts to transmit its PUSCH. Therefore, to avoid collision with the eMBB UE and achieve the 10-5 overall reliability of the URLLC, the reliability of the UL PI should also be in the order of 10-5 or even higher. The following two kinds of methods can be considered to improve the reliability of the UL PI.

- More time-frequency resource

Allocating more time-frequency resource for the UL PI is a direct method to improve its reliability. For example, it can adopt a higher aggregation level for the PDCCH carried the UL PI. Nevertheless, allocating more rime-frequency resource may cause a high blocking probability of PDCCH that sharing the same search space with the PDCCH carried the UL PI.

- Smaller payload size

Another method is to reduce the payload size of UL PI, for example, to utilize a coarser granularity to indicate the preempted time-frequency resource. As an instance, each UL PI only includes 7 bits, where each bit represents the occupancy of the whole BWP in one slot or several slots, even if URLLC UE only occupies a part of BWP in a few symbols. Specifically, after decoding the UL PI, eMBB UE will stop the PUSCH transmission in the whole BWP in the rest symbols of the indicated period of time if it has started the PUSCH transmission. On the other hand, if eMBB UE has not started to transmit its PUSCH, the eMBB UE will cancel the scheduled PUSCH transmission.

Proposal 4. Higher aggregation level and coarser granularity can be considered to enhance the reliability of the UL PI.

Conclusions

In this contribution, the following observations and proposals are summarized:

Observation 1. The time  of the eMBB UE processing the UL PI and preparing for the PUSCH cancellation should be smaller than or equal to the time  of the URLLC UE processing UL grant and preparing for the PUSCH transmission, i.e., <.

Observation 2.

- eMBB UEs don’t need to always monitor UL PI;

- eMBB UEs only need to monitor UL PI when there is a dynamically/configured granted UL transmission.

Proposal 1. Support non-slot level monitoring of the UL preemption indication for eMBB UEs.

Proposal 2. The time period for monitoring UL PI should be clarified/defined. Further study is necessary.

Proposal 3. Support group-common UL PI to reduce the signaling overhead.

Proposal 4. Higher aggregation level and coarser granularity can be considered to enhance the reliability of the UL PI.

- References

[1] 3GPP TS 38.213, NR; Physical layer procedures for control.

[2] 3GPP TS 38.214 NR; Physical layer procedures for data.