3GPP TSG-RAN WG1 Meeting #96bis	R1-1905855

# Xi’an, China, 8th – 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | x | Radio Access Network |  |  |  |  | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Correction on one port PUSCH power scaling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson, Verizon, T-Mobile USA, Sprint, NTT DOCOMO, Orange, Nokia, Intel, Qualcomm, Samsung, ZTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-04-10 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-15 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | According to the current specification, when codebook based PUSCH of single antenna port is scheduled by DCI format 0_1, the PUSCH transmission power is scaled down by the maximum number of SRS antenna ports supported by a UE in an SRS resource. Consequently, for UEs supporting more than one SRS antenna port in an SRS resource, full power PUSCH transmission is not possible for codebook based PUSCH when the UE is configured for single antenna port operation and scheduled by DCI format 0_1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that for codebook based PUSCH transmission of single antenna port scheduled by DCI format 0_1, PUSCH transmission is not scaled down. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A two Tx or a 4 Tx UE can transmit at most 3 or 6 dB less power, respectively, than its power class when configured for single antenna port codebook based operation and when scheduled by DCI format 0_1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  | This CR has isolated impact on power control for codebook based PUSCH of UEs configured for single antenna port transmission scheduled by DCI format 0_1 and modifies the corresponding behavior as currently specified. Interoperability analysis: - If the network is implemented in accordance to the CR and the UE is not, the network expects higher transmission power from the UE than the UE actually uses. - If the UE is implemented in accordance to the CR and the network is not, the network expects lower transmission power from the UE than the UE actually uses. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Subclause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Subclause 7.1.1. If the PUSCH transmission is scheduled by a DCI format 0_1 and when txConfig in PUSCH-Config is set to 'codebook' and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource. The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.