3GPP TSG RAN WG1 #96	bis                                                                                R1-1904086

Xi’an, China, 8th – 12th April, 2019

Source:	vivo

Title:	Enhanced UL configured grant transmissions for URLLC

Agenda Item:	7.2.6.6

Document for:	Discussion and Decision

- Introduction

In RAN #83 meeting, a new WI regarding “Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)” was approved [1]. The objective about UL configured grant transmission is as follows. In this contribution, we discuss and present our views on the design of multiple resource configuration, including configuration method, activation/deactivation for multiple resource configurations, construction of repetitions aiming at ensuring K repetitions, HARQ process ID determination.

- Specification of enhanced UL configured grant transmission [RAN1, RAN2] - Multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell - Note: V2X use cases are also considered

- Discussion

  - Configuration of multiple resource configuration

In RAN#95, multiple active configured grant configurations for a BWP is agreed for the purpose of support different services or enhancing reliability and reducing latency. For different resource configurations, different starting offsets to enable flexible starting position can be applied. In addition to that, different resources or parameters to adapt to different services, or different TBS can also be allocated for different configurations. At least the following parameters for a resource configuration can be configured separately.

  - Time resource allocation

  - Frequency resource allocation

  - MCS table and/or MCS level

  - Periodicity

  - DMRS configuration/sequence

  - RV sequence

  - Power control parameters

Another issue is that gNB needs to detect the resource configuration on which UE is using to transmit the data if multiple resource configurations are enabled for a UE. Following methods can be adopted.

- Option 1: DMRS based method

- Option 2: UCI based method

For option 1, Different DMRS parameters/sequences are configured for different resource configurations, i.e. each configuration is associated with an individual DMRS sequence or parameter.

For option 2, each transmission includes a UCI carrying information of actual transmitted resource configuration. Note that UCI transmission for UL configured grant is supported for NR-U. Similar to NR-U, HARQ ID can be determined and selected by UE and be transmitted via control information, e.g. UCI, which is separately encoded from data. Therefore, gNB can decode the UCI and acquire HARQ ID before decode the data part.

Due to flexible starting occasion, it may be difficult for gNB to identify the initial transmission of K repetitions. Especially when part of the K repetitions could be missed at gNB side, gNB may not be aware that which repetitions need to be combined. Therefore, indication of initial transmission and repetitions could be useful. NDI is used to differentiate initial transmission and retransmission for dynamic grant. Similarly, NDI can be used for grant-free transmission and indicated via UCI such that gNB can identify the initial transmission of K repetitions from UCI.

UE can be configured with RV sequences for K repetitions. In Rel. 15, RV is associated with the transmission occasions within period. Thus, gNB can identify RV from the transmission occasion where UL data are transmitted. As discussed above, flexible starting occasion is needed for URLLC. Therefore, it is not necessary to associate RV sequences with transmission occasion any more. There may be issue that gNB may need to blindly detect the UL data if RV information is not acquired at gNB. To reduce the detection complexity, RV can be indicated by UCI.

For UCI on grant-free transmission, either puncturing or rate-matching can be used for UCI mapping on PUSCH. Considering the payload of UCI, rate-matching with PUSCH is preferred. When K repetitions are adopted, UCI needs to be mapped to each repetition. Since gNB needs to combine the K repetitions, the code rate of K repetitions should be kept the same.

Proposal 1: Regarding multiple resource configurations, following options can be adopted.

- Option 1: Different DMRS parameters/sequences are configured for different resource configurations

- Option 2: UCI carrying information of UE selected resource configuration is transmitted with data.

  - UCI including HARQ ID is transmitted together with data.

  - NDI is included in grant-free UCI to differentiate initial transmission and repetitions.

  - RV sequence is not associated with the transmission occasion, and RV is included in grant-free UCI.

  - UCI is multiplexed onto each repetition.

As discussed in the previous meetings, multiple resource configurations can be configured with different starting offsets to enable flexible starting position for URLLC traffic, i.e. ensuring K repetitions. Since the number of repetitions are configurable by network, the number of resource configurations of UL configured grant needs to be also configurable. In addition, multiple configured grant configurations with different types can be used for different service types. E.g. a type1 configured grant is configured for the periodic traffic without the need for link adaptation while a type2 configured grant can be used for providing the flexibility to adapt to the channel. Therefore, it should be allowed to configure a UE multiple resource configurations with different configured grant types. To be specific, the total number of resource configurations including type 1 and type 2 UL configured grant is configured by RRC.

Proposal 2: The total number of resource configurations including type 1 and type 2 UL configured grant is configured by RRC.

Whether to have active type 1 and type 2 configurations simultaneously in an active BWP

In case of multiple resource configurations, whether to have active type 1 and type 2 configurations simultaneously in a BWP should be discussed. One use case for multiple resource configurations is to support different service types.

For these services, either type 1 or type 2 UL configured grant can be adopted. There could be a case that there are both type 1 UL configured grant and active type 2 UL configured grant in a BWP, each of which is targeted for a certain service type. For example, in TSN network, a UE could be a switcher such that it may have different types of services. Therefore, multiple active configured grant configurations can be enabled for both Type 1 and Type 2 configured grant transmission.

Proposal 3: Multiple active configured grant configurations can be enabled for both Type 1 and Type 2 configured grant transmission.

  - Activation/deactivation for multiple resource configurations

For type 1 UL configured grant, all the transmission parameters of a resource configuration are configured by RRC. Therefore, when multiple type 1 configurations are enabled, each resource configuration is configured by RRC. Thus, there is no new behavior for UE to support multiple type 1 configurations.

Regarding type 2 UL configured grant, UE activates/deactivates the type 2 UL configured grant when receiving the activation/deactivation DCI. When multiple type2 UL configured grant are enabled, it is straightforward that each type 2 UL configured grant configuration is activated/deactivated by a separate activation/deactivation DCI. However, it may cause increased latency and increased signaling overhead especially when multiple resource configurations are applied for reducing latency. In this case, multiple resource configurations may share most the same transmission parameters but have different starting offsets. Therefore, a DCI activating/deactivating one or multiple type 2 resource configurations can be adopted to reduce the signaling overhead. To be specific, an activation/deactivation field indicating the activated/deactivated configurations can be included in the activation/deactivation DCI, e.g. using the existing fields or new introduced field. An example is shown in the table. Each type 2 UL configured grant configurations are configured with a ID by RRC. The mapping between activation/deactivation state field and activated/deactivated ID(s) is configured by RRC.

If a DCI activating multiple type 2 configured grant configuration can be supported, further discussion is needed on how to indicate the scheduling information, e.g. time/frequency resource or MCS, for different resource configuration.

Table 1: Mapping between activation/deactivation field in DCI and corresponding configurations

| Codepoint | Activation/deactivation configurations |
| --- | --- |
| 0 | UL configured grant configuration ID0 |
| 1 | UL configured grant configuration ID1 |
| 2 | UL configured grant configuration ID2 |
| 3 | UL configured grant configuration ID3 |
| 4 | Set of UL configured grant configuration ID={0,1,2,3} |
| … | … |

Proposal 4: For a UE configured with multiple type 2 UL configured grant configurations, a DCI activating/deactivating one or multiple resource configuration can be supported.

- The activated/deactivated configuration(s) is indicated by the activation/deactivation state field in the activation/deactivation DCI

- The mapping between activation/deactivation state field and activated/deactivated configuration(s) is configured by RRC

  - HARQ process ID determination

In Rel.15 configured grant, number of HARQ processes is configured by RRC and HARQ ID is determined by the timing of transmission occasion. With multiple resource configurations, HARQ process ID determination could be different. There are following options to determine HARQ ID for UL configured grant.

- Opt. 1: For each configuration, HARQ ID is determined on the timing of the first transmission occasion within a period.

- Opt. 2: HARQ ID determination is dependent on timing of the actual initial transmission

- Opt. 3: HARQ ID determination is independent on timing of the transmission or transmission occasion

For option 1, it is almost Rel.15 behavior for HARQ ID determination. Very limited modification may be needed for HARQ ID determination in case of multiple resource configuration. To be more specific, HARQ process pool is separated for different resource configurations. A number of HARQ process can be allocated to a resource configuration, while different resource configurations may have different HARQ processes pool, e.g. HARQ ID offset is configured for each UL configured grant configuration. HARQ ID of a transmission with a certain resource configuration can be determined by the timing of transmission occasion, i.e. similar to Rel.15 configured grant.

For option 2, HARQ ID is determined based on the actual transmission timing of an initial transmission. Since flexible starting position could be adopted, there will be more flexibility if HARQ ID is determined by initial transmission timing. However, initial transmission should be reliably identified by gNB, using distinguishable information e.g. DMRS or UCI.

For option 3, HARQ ID is no longer dependent on the timing of transmission occasion or actual transmission, that is, HARQ ID is selected by UE. Therefore, HARQ ID determination is flexible such that UE can select any available HARQ process for a UL configured grant transmission unless all of HARQ processes are occupied. In this case, HARQ ID needs to be reported by UE by UCI transmitted with data.

Proposal 5: For HARQ process ID determination in case of multiple resource configurations, following options can be adopted

- Opt. 1: For each configuration, HARQ ID is determined on the timing of the first transmission occasion within a period.

- Opt. 2: HARQ ID determination is dependent on timing of the actual initial transmission

- Opt. 3: HARQ ID determination is independent on timing of the transmission or transmission occasion. HARQ ID

  - Multiple resource configurations for ensuring K repetitions

In Rel.15 configured grant, a UE transmits a TB with repetitions with the resources of a resource configuration. It is simple while the starting position or ending position of repetitions need to be restricted to differentiate initial transmission and repetitions at gNB side. This leads to either large latency or lower reliability for repetitions of a TB.

For URLLC traffic, both low latency and high reliability need to be satisfied. Therefore, it is important for a UE to start the transmission for a TB at any transmission occasion and ensure K repetitions regardless of the starting transmission occasion. This means flexible starting occasion for a TB should be supported. Based on the framework of multiple resource configuration, following method is proposed to ensure K repetitions regardless of starting timing of transmission occasion.

Multiple resource configurations with different starting offsets

With multiple resource configurations, it is possible to differentiate initial transmission and repetitions without sacrificing latency or reliability. More specifically, K repetitions are ensured by using multiple resource configurations, where different resource configurations are configured with different starting offsets, as described in Figure 1. For example, UE starts an initial transmission at the first transmission occasion of a resource configuration and K repetitions are ensured within a period P, where the resource configuration with shortest latency is selected, according to the packet arrival timing. For repetitions with multiple resource configurations, repetitions of a TB are transmitted with a resource configuration. Besides, HARQ ID determination based on Rel.15 configured grant can be reused.

Figure 1: Example of repetitions with multiple resource configurations

Proposal 6: For repetitions with multiple resource configurations,

- Multiple resource configurations with different starting offsets can be configured to enable flexible starting time and ensure K repetitions

- Conclusion

In this contribution, we provide our views on enhancements of UL grant-free transmission to meet the latency and reliability requirements of URLLC. The observations and proposals are summarized below.

Proposal 1: Regarding multiple resource configurations, following options can be adopted.

- Option 1: Different DMRS parameters/sequences are configured for different resource configurations

- Option 2: UCI carrying information of UE selected resource configuration is transmitted with data.

  - UCI including HARQ ID is transmitted together with data.

  - NDI is included in grant-free UCI to differentiate initial transmission and repetitions.

  - RV sequence is not associated with the transmission occasion, and RV is included in grant-free UCI.

  - UCI is multiplexed onto each repetition.

Proposal 2: The total number of resource configurations including type 1 and type 2 UL configured grant is configured by RRC.

Proposal 3: Multiple active configured grant configurations can be enabled for both Type 1 and Type 2 configured grant transmission.

Proposal 4: For a UE configured with multiple type 2 UL configured grant configurations, a DCI activating/deactivating one or multiple resource configuration can be supported.

- The activated/deactivated configuration(s) is indicated by the activation/deactivation state field in the activation/deactivation DCI

- The mapping between activation/deactivation state field and activated/deactivated configuration(s) is configured by RRC

Proposal 5: For HARQ process ID determination in case of multiple resource configurations, following options can be adopted

- Opt. 1: For each configuration, HARQ ID is determined on the timing of the first transmission occasion within a period.

- Opt. 2: HARQ ID determination is dependent on timing of the actual initial transmission

- Opt. 3: HARQ ID determination is independent on timing of the transmission or transmission occasion. HARQ ID

Proposal 6: For repetitions with multiple resource configurations,

- Multiple resource configurations with different starting offsets can be configured to enable flexible starting time and ensure K repetitions

- References

- RP-190726,	“New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, Huawei, RAN #83.