3GPP TSG RAN WG1 Ad-Hoc Meeting #96bis	          R1-1904183

Xi’an, China, 8th – 12th April, 2019

Agenda item:		7.2.2.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	HARQ scheduling and feedback for NR-U

Document for:		Discussion and Decision

# 1. Introduction

NR Work Item [1] on NR-based access to unlicensed spectrum has been approved in RAN plenary #82. To maximize the applicability of NR-based access, it is beneficial to specify solutions applicable to unlicensed bands scenarios as part of the NR development. In this contribution, we consider the changes and enhancements needed for HARQ feedback and scheduling for NR unlicensed scenarios. On these topics, following agreements were reached in the RAN1 AH 1901 meeting [3], [6]:

Agreement [3]:

RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

Conclusion [3]:

- No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR 38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT

    - If additional values are introduced these can also be used for requesting feedback outside the COT

  - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested

Agreement [3]:

For enabling multiple opportunities for HARQ A/N transmission and for cross-COT HARQ-ACK feedback, at least the following is supported: gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT)

Agreement [3]: Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16.

Agreement [6]: Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs

- Each TB is mapped to at most one slot or one mini-slot

# 2. UL scheduling

## 2.1 Two-stage scheduling

To take advantage of shared COT more efficiently, eLAA (Rel-14) supports triggered (aka 2-stage) PUSCH scheduling.

- The first (initial) grant denoted as Trigger A grant uses the eLAA DCI formats and provides all the needed information except the exact timing of the scheduled PUSCH

  - The PUSCH trigger A bit in the eLAA UL grant provides the UE with the information if this is a normal grant with given timing, or a triggered grant

  - The Trigger A grant has a certain validity time in terms of number of subframes indicated in the Trigger A grant

- The PUSCH transmission is triggered with the Trigger B field in the eLAA common PDCCH (DCI 1C scrambled with CC-RNTI), and is relative in time to indicated UL configuration for LAA

  - The UL configuration for LAA contains information about the start of the UL burst (‘UL offset’ l) and the length of the UL burst (‘UL duration’, d)

The benefits of 2-stage scheduling come mainly from the reduced latency when triggering an UL transmission at the start of the UL burst. In eLAA, the scheduling latency of 4 ms imposed significant restrictions to the practically feasible COT structures. While in NR-U the processing time as significantly faster, the possibility for multiple switching point makes it even more important to be able to make use of starts of UL burst without unnecessary scheduling restrictions. Therefore, we see that 2-stage scheduling should also be supported in NR-U.

Proposal 1: 2-stage scheduling is supported for NR-U uplink.

## 2.2 Multiple uplink starting points

One of the key challenges in unlicensed band scheduled uplink is the uncertainty of uplink channel access. The channel access probability may be improved by allowing multiple uplink starting points as we discuss in [2] together with the possible mechanisms to support the multiple uplink starting points within a slot. One option is to use short PUSCH allocations with Type B mapping at beginning of a UL burst of multiple PUSCHs. Of course, such solution increases also the number of required HARQ processes – especially if short PUSCH allocations used over multiple slots. Furthermore, to increase transmission capability of UE with larger bandwidth, a UL BWP for a UE may span over multiple sub-bands. When accessing large bandwidth of e.g. 160 MHz (i.e. 8 sub-bands), having HARQ process per sub-band will result in a large number of HARQ processes. In this case, the number of required HARQ process will increase with the number of scheduled slots, the number of mini-slots per slot or/and the number of sub-bands and will greatly exceed the maximal number of HARQ processes (i.e. up to 16 HARQ processes) in NR Rel-15. However, significant increase in the number of HARQ processes has difficult software and hardware implementation implications. On the other hand, although it would be beneficial to schedule one TB over the sub-bands available based on the outcome of LBT, it will be a big challenge for UE to prepare data right after LBT due to long preparation time including e.g. getting data from high layer, TBS determination, channel coding, modulation, IFFT, digital filtering or/and RF retuning. Therefore, further enhancement for multi-TTI PUSCH scheduling is needed to cope with the above challenges while achieving the benefits of the flexible BWP operation & flexible starting points.

Proposal 2: To support flexible BWP operation and flexible starting points with acceptable number of HARQ processes, further enhancements are needed for multi-TTI PUSCH scheduling.

When considering multi-TTI scheduling with allocation of multiple contiguous TTIs, Type B PUSCH mapping is needed only at the beginning of transmission to support flexible PUSCH starting points within a slot. On the other hand, use of short PUSCH allocations causes a significant increase in the number of required HARQ processes. The use of short PUSCH allocations throughout multiple scheduled slots increases also DMRS overhead unnecessarily. The drawbacks from the use of short PUSCH allocations would remain acceptable if the short PUSCH allocations were used only at the beginning of transmission where they are needed. However, the actual transmission starting time is not known beforehand due to uncertain channel access.

One potential approach to solve the problem is to introduce partially floating HARQ processes for multi-slot PUSCH scheduling as illustrated in Figure 1:

- For a multi-slot UL grant with N slots, a floating block would be defined for M slots (e.g. 2 slots), with a floating HARQ process allocated for each PUSCH in the floating block.

- For the later (N-M) scheduled slots, a normal HARQ process per slot would be allocated.

- When the UL LBT operation fails for a slot, the floating block is shifted forward into the next scheduled slot, and the floating HARQ processes are mapped into the corresponding time-frequency resources.

- When a later normally scheduled slot is overridden by the floating block shifted forward, the floating HARQ processes replace the normal HARQ process (with fixed timing); otherwise, the normal HARQ processes are used for transmission after the floating block.

The approach will significantly reduce the number of required HARQ processes when compared to a case where short PUSCH allocations are used in all scheduled slots, as there is only one slot prepared with short PUSCH allocations. For example, with one additional HARQ process, flexible starting points at every ½ slot can be supported as shown in Figure 1. Additionally, UE does not need prepare TBs again and again for each slot on which UE tries unsuccessfully access the channel. It can prepare TBs for the floating block and transmit the prepared TBs once it obtains the channel access.

Proposal 3: A floating HARQ process block is introduced for multi-TTI PUSCH scheduling to support flexible starting points with acceptable number of HARQ processes.

Figure 1. Operation of floating HARQ block in multi-TTI scheduling

## 2.3 Cross-carrier scheduling

When considering the scheduling of NR-U uplink in the Scenario A (Carrier aggregation between licensed band NR (PCell) and NR-U (SCell)), cross-carrier scheduling of NR-U PUSCH from a licensed-band NR cell offers clear benefits. In particular, the inherent uncertainties of unlicensed access do not affect the scheduling PDCCH on licensed band but are limited only to actual NR-U PUSCH transmission and reception. Hence, we see that UL cross-carrier scheduling from a licensed-band NR cell should be supported in NR-U. In the case of Scenario A, the licensed band scheduling carrier may use lower SCS than the unlicensed band scheduled carrier. It was identified during Rel-15 NR WI that multi-TTI scheduling would be beneficial for cross-numerology scheduling from low SCS to high SCS. Hence, we propose that NR-U multi-TTI scheduling supports also cross-numerology scheduling across carriers. We see that the solution should be based on the PUSCH cross-carrier scheduling mechanism across different numerologies that will be completed for (licensed) NR in Rel-16 “DC and CA enhancements” WI [4].

Observation 1: Cross-numerology scheduling is beneficial for NR-U PUSCH scheduling especially in the case of Scenario A (CA with NR cells) deployment

Proposal 4: NR-U PUSCH multi-TTI scheduling supports cross-numerology scheduling based on Rel-16 mechanism.

# 3. PDSCH HARQ feedback over unlicensed band

When HARQ feedback is transmitted over an unlicensed band, channel contention creates additional challenges: channel contention may delay HARQ feedback unpredictably, and hidden node problem and bursty interference may cause occasional HARQ feedback detection failures.

Flexible HARQ feedback determination & timing of NR Release 15 forms the natural baseline for HARQ feedback in NR unlicensed, as it has inbuilt support for highly flexible TDD configurations and support for self-contained slot operation (depending on the UE capability) This is pointed also in NR-U WID [1], determining objectives for HARQ operation as “NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT”. In the following, we consider both cases in more detail.

## 3.1 HARQ feedback in self-contained COT

Immediate transmission of HARQ feedback for the DL data in the same shared COT minimizes the HARQ feedback latency as well as jitter in the feedback latency. This kind of self-contained COT is well supported by Rel-15 PDSCH processing capability 2, having PDSCH processing times as short as 3, 4.5, and 9 symbols for SCS of 15 kHz, 30 kHz and 60 kHz, respectively.

In unlicensed band operation, a gap of even few symbols may be undesirable, as it opens up a possibility for other systems to contend and access the channel. Correspondingly, in TR 38.889 [5], it is identified that a gap of up to 16 µs should be allowed between the end of DL transmission and the immediate feedback transmission. It is also noted that it is beneficial to support useful transmissions in the time between DL data transmission and corresponding immediate HARQ feedback transmission. Such UL transmissions may be e.g. PUSCH, PUCCH used for CSI reporting, or even SRS. In our view, it is important that triggering or scheduling of these transmissions does not require separate DCIs increasing the downlink control overhead but can be triggered by the same DL assignment scheduling PDSCH as shown in Figure 2.

Proposal 5: In self-contained COT operation, DL assignment can trigger a pre-configured UL transmission (short PUSCH, CSI report, or SRS) preceding the HARQ ACK transmission associated to the DL assignment.

Figure 2. HARQ feedback in self-contained COT with UL transmission filling the PDSCH processing time gap.

## 3.2 HARQ feedback across COTs

In addition to immediate HARQ feedback within a self-contained COT, also UEs with more modest processing capabilities shall be supported in NR-U with HARQ feedback outside the COT. In TR38.889 [5], 3 candidate solutions are considered:

-	Alt1: gNB requests/triggers feedback for PDSCH from earlier COT(s)

-	Alt2: UE is configured to report HARQ feedback for PDSCH from earlier COT(s) without an explicit request/trigger

-	Alt3: by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH

In RAN1 AH 1901 [3], it was agreed that at least (a variant of) Alternative 1 is supported while it was concluded that no additional K1 value is needed for Alternative 3. However, further discussions on the LBT type determination is needed for Alternative 3. Alternative 3 complements other alternatives for the case where gNB does not initiate any immediate next COT(s). In such a case, gNB can indicate to UE a HARQ feedback time after the COT.

Alternative 1 is a suitable mechanism for HARQ feedback across COTs. A simple solution for Alternative 1 with efficient HARQ feedback size is shown in Figure 3. UE identifies that dynamic codebook (determined by DAI) is extended across COTs by the K1 value indicating that HARQ feedback time is determined later. In the next COT, the transmission for the pending HARQ feedback is triggered by the incremented DAI. UE continues with the DAI based codebook determination, with the actual transmission time and resource determined by K1 and PRI indicated in the next COT.

Proposal 6: gNB triggers the pending HARQ feedback from earlier COT(s) with the incremented DAI in the DL assignment on the following COT (in Alt 1a of TR38.889).

Figure 3. Extending dynamic codebook across COTs.

In Alternative 2, UE autonomously reports HARQ feedback for PDSCH from earlier COT(s) without an explicit request/trigger. The baseline solution is prone to ambiguity in the transmitted codebook size: e.g. UE may transmit HARQ feedback but gNB may fail to detect it. In the following COT, UE transmits HARQ feedback for that COT only, while gNB expects a combined feedback for both current and previous COT. This causes ambiguity in the codebook size, resulting in failed detection. It may cause ambiguity also on the PUCCH resource selection (based on payload size), resulting in worst case in PUCCH resource collision between different UEs. It has been proposed that UE indicates together with HARQ feedback whether HARQ feedback from earlier COT(s) is combined to the transmission. However, this does not solve the potential ambiguity in the PUCCH resource selection. Further, we do not see any particular benefits that Alternative 2 can offer over Alternatives 1 and 3.

Observation 2: Alt 2 in TR38.889 is prone to ambiguity in the codebook size and/or PUCCH resource selection. There is no need to consider Alt 2 further.

## 3.3 Multiple HARQ feedback transmission opportunities

#### Additional reporting of earlier HARQ feedback based on gNB request/trigger

Although the UE can transmit the HARQ feedback in most of the cases (e.g. with Cat 1 channel access), there is also the risk that HARQ feedback transmission is blocked by UE LBT (if UE LBT is required for HARQ feedback) or corrupted by hidden node transmission starting during PUCCH. Hence, it was agreed in RAN1 AH 1901 [3] that at least Alternative 1 is supported for additional HARQ feedback transmissions.

Requesting additional HARQ feedback reporting offers a robust mechanism to retrieve HARQ feedback in case of corrupted or blocked earlier feedback as well as after a longer disruption on channel access. One design issue to consider is whether the additional HARQ feedback request/trigger is incorporated as part of baseline HARQ feedback reporting and included in the PDSCH scheduling DCI or triggered via a separate DCI.

Requesting additional HARQ feedback reporting via PDSCH scheduling DCI means that the additional DL overhead per requested additional feedback reporting is small, as it is just a field in a DCI transmitted anyway. On other hand, the DCI field causes constant overhead, also when no additional reporting is needed. For a separate triggering DCI, the viewpoints are opposite: there is no overhead when no additional reporting is needed, but considerable overhead (e.g. in comparison to HARQ feedback size) when the additional reporting is triggered. The overhead trade-off depends on the frequency of additional reporting. One can expect that the benefit from additional HARQ feedback reporting (over PDSCH retransmission) is important especially in situations of relatively high load and relatively frequent interference/channel occupancy from other systems. In those situations, the triggering via a PDSCH scheduling DCI appears more attractive.

As the triggering of additional HARQ feedback reporting occurs with ambiguous timing relation to the associated PDSCHs, we see that additional HARQ-ACK feedback reporting can be based on HARQ feedback windows/codebooks or PDSCH groups for which the feedback is requested. HARQ feedback windows/codebooks can be assigned with an ID given in the PDSCH scheduling DCI. The HARQ feedback trigger can indicate the additional codebooks to be included into the transmission. Such dynamic labelling is an efficient way to reduce the size of the additional HARQ feedback reporting when compared e.g. to reporting HARQ feedback for all HARQ processes. It can be further noted:

  - 2-bit codebook ID may be enough, hence, keeping the DL overhead acceptable.

  - The codebook ID in the PDSCH scheduling DCI is kept constant over the HARQ feedback window (or HARQ feedback codebook) and incremented between the HARQ feedback windows. With this, the codebook ID can be used as a group DAI ensuring correct feedback size (with minimum codebook size assumption discussed later) even when a HARQ feedback window is missed.

  - Additional HARQ feedback transmission can be triggered for previous HARQ feedback window, or a number of previous HARQ feedback windows, hence supporting multiple additional transmission opportunities.

Proposal 7: In explicitly triggered HARQ feedback mechanism,

- HARQ feedback codebook (or HARQ feedback window) is assigned with a codebook ID in PDSCH scheduling DCI

- A separate field in PDSCH scheduling DCI can control HARQ feedback transmission for earlier COTs.

One aspect to be considered is sufficiently reliable determination of the transmitted dynamic codebooks. In Rel-15 NR, separate codebooks for TB based feedback and CBG based feedback are transmitted together. During Rel-15, separate determination of dynamic codebooks was seen reliable enough and no need was identified for mechanisms improving the reliability further.

Observation 3: In Rel-15 NR, joint transmission of separately determined dynamic codebooks is considered to be sufficiently reliable.

Based on the Rel-15 HARQ feedback design, we see separate determination of jointly transmitted codebooks as a feasible baseline design. If further reliability improvement is assessed to be necessary for NR-U, we see that the mechanism should support at least:

- HARQ feedback remains correct (in terms of size as well as A/N bit location in the codebook) even if UE misses a whole HARQ feedback window.

- Several retransmission opportunities can be provided for an HARQ-ACK when needed. We see this as an important design aspect, as it improves the robustness of the HARQ feedback reporting.

- Typically, HARQ feedback can be transmitted correctly already on the first transmission opportunity (e.g. with Cat 1 channel access). Additionally, the HARQ feedback for the last PDSCH of the DL burst will frequently be reported in the next COT. Hence, it is important that the HARQ feedback from earlier COT can be limited only to the pending HARQ feedback for the last PDSCH of COT. Unnecessary increase in the HARQ feedback overhead by incorporating the HARQ feedback for the whole COT instead of ACK/NACK only for the last PDSCH shall be avoided. Unnecessary repetition of HARQ feedback would lead to significant increase in the HARQ feedback overhead, especially with CBG-based feedback.

Proposal 8: HARQ feedback mechanism shall be able to provide several retransmission opportunities for a PDSCH HARQ-ACK.

Proposal 9: HARQ feedback transmission for the last PDSCH of the previous COT in a current COT should not cause unnecessary re-transmission of HARQ feedback for other PDSCHs in the previous COT.

We consider codebook determination in various situations in Appendix. Based on the considerations, we see that determining the Type-2 codebook separately for each HARQ feedback window:

- allows efficient use of Rel.15 Type 2 codebook mechanism

- is reliable if the total DAI can indicate a larger value than the total number of PDSCHs scheduled up to the current PDCCH monitoring occasion. The specification change is marginal, especially when compared to other possibilities to improve codebook determination reliability. To improve the reliability further, increasing total DAI size may be considered.

- can support triggering of retransmission opportunity multiple times for an HARQ-ACK.

Further, if the codebook ID operates as a counter DAI for codebooks, it can help to detect a missing HARQ feedback window/codebook and can maintain correct feedback if a minimum codebook size is defined. When the minimum codebook size is defined, and UE detects a missed feedback window/codebook, UE will insert NACKs corresponding to a minimum codebook size to the HARQ feedback report. However, when defining the minimum codebook size, one can note a trade-off with respect to different minimum codebook sizes:

- too large minimum codebook size increases considerably HARQ feedback overhead e.g. in the case when single PDSCH is scheduled for the UE.

- too small minimum codebook size is not sufficiently robust e.g. when a hidden node causes UE to miss a burst of PDCCHs.

To avoid the need for such trade-off, two (a smaller and a larger) minimum codebook sizes can be predetermined, and a 1-bit flag in the DCI triggering the HARQ feedback indicates (based on PDSCH scheduling) which minimum codebook size will be assumed for the triggered HARQ feedback.

Proposal 10: To improve dynamic codebook reliability, total DAI can be allowed to indicate a larger number of PDSCHs than scheduled up to the current PDCCH monitoring occasion. A minimum size for a reported codebook can be determined.

Observation 3: Codebook ID assignment to HARQ feedback codebooks can operate as a codebook counter DAI.

In previous meetings, several proposals for additional HARQ feedback reporting based on gNB request has been made, e.g., in [7]-[11]. In following, we discuss these proposals briefly.

- In [7], a HARQ-ACK codebook contains the HARQ feedback that was (originally) scheduled to be reported in the same PUCCH. In the mechanisms considered in [7], [8], DL assignment contains HARQ-ACK codebook index as well as an indication of HARQ-ACK codebook set (a number of previous codebooks) to be reported. DAI values are accumulated continuously over the concatenated codebooks: “C-DAI/T-DAI should count for PDCCHs of all concatenated HARQ-ACK codebooks, which is basically the same as legacy single dynamic HARQ-ACK codebook” [7]. It is further noted for this mechanism in [12]: “[…] HARQ-ACK retransmission can work properly without ambiguity, if there is only one previous HARQ-ACK codebook and the triggering of retransmission is from the first PDCCH of current HARQ-ACK codebook.”

Based on this observation, justified by examples in [12], the mechanism cannot work properly for the scheduling example shown in Figure 4a: Let’s say that HARQ feedback for PDSCH set#1 fails. However, retransmission for PDSCH set#1 cannot be triggered during the PDSCH set#2 as the triggering would not be from the first PDCCH of current HARQ-ACK codebook. The mechanism cannot support multiple retransmission opportunities for a HARQ-ACK either, as that would mean that multiple previous HARQ-ACK codebooks are reported.

- In mechanism proposed in [9], DAI is accumulated for “the HARQ process(es) in previous COT(s) which is not acknowledged as a mean to request retransmission of unacknowledged or pending feedback” [9]. New feedback indicator (NFI) is introduced, which is toggled when HARQ feedback is correctly received.

Proper operation of the mechanism is not clear for the scheduling example shown in Figure 4b. Let’s say that all PUCCHs are received correctly. However,

  - NFI cannot be toggled at beginning of PDSCH set #2, as the corresponding PUCCH is not yet transmitted.

  - NFI cannot be toggled and DAI counters cannot be re-initialized at beginning of COT #2, as UE may have missed the first PDCCH of the PDSCH set #2.

  - if NFI is not toggled during the PDSCH set #2, the already transmitted & received HARQ feedback for the PDSCH set #1 is unnecessarily transmitted together with PDSCH set #2 feedback, leading to excessive overhead.

  - The same problem is faced also during the PDSCH set #3.

- In mechanism proposed in [10], PDSCH group index and a request (RQ) field are introduced to the scheduling DCI. DAI is continuously accumulated within the PDSCH group, which may contain PDSCHs in multiple consecutive COTs. Request field controls the HARQ feedback type: for RQ = 0, HARQ feedback based on the PDSCH-to-HARQ-timing-indicator values is reported. This feedback may e.g. cover only a portion of PDSCHs in the PDSCH group. For RQ = 1, HARQ feedback for the whole PDSCH group is reported and, hence, can provide HARQ feedback retransmission for the earlier portion of PDSCH group. PDSCH group is not changed before the HARQ feedback for the whole PDSCH group is correctly received.

The mechanism can lead to unnecessary retransmission of correctly received HARQ feedback in scheduling example shown in Figure 4c. An example in [10] shows that PDSCH group is not changed during this kind of scheduling before the HARQ feedback for the whole PDSCH group is correctly received. Hence, PDSCH group remains the same throughout the scheduling shown in Figure 4c, as there is always a PDSCH with pending HARQ feedback. Let’s say that PUCCH detection for 2nd PUCCH fails, and gNB sets RQ = 1 in later part of PDSCH set #3. In this case, UE reports HARQ feedback also for PDSCH set #1, although the corresponding HARQ feedback was correctly received in the 1st PUCCH. The scheduling example shown in Figure 4 is rather common and can be extended in time well beyond the 3 PDSCH sets shown in here. In such cases, patching up a single PUCCH failure with RQ = 1 can cause excessive HARQ feedback overhead (as HARQ feedback for the whole continuous scheduling is reported.)

- In mechanism proposed in [11], PDSCH group index and a HARQ-ACK reset indicator are introduced to the scheduling DCI. DAI is continuously accumulated within the PDSCH group. A PDSCH group may contain PDSCHs in multiple COTs, and the COTs do not need to be consecutive. The operation of the reset indicator resembles the NFI operation in [9] but is associated with the corresponding PDSCH group.

For the scheduling example in Figure 4b, gNB may alternate the indicated PDSCH groups between the consecutive PDSCH sets (e.g. PDSCH group #1 is indicated for PDSCH sets #1 and 3, and PDSCH group #2 is indicated for PDSCH set #2). This way gNB has time to receive pending HARQ feedback before continuing the scheduling of the same PDSCH group. Hence it can determine appropriate value for the reset indicator and can avoid the problems faced with the mechanism in [9].

Observation 4: Dynamic codebook determination mechanisms relying on continuous accumulation of DAI (e.g. in [7]-[10]) cannot operate properly when HARQ-ACK for the last PDSCH of a COT is reported in the next COT.

a)

b)

c)

Figure 4. Considered scheduling examples. Colours indicate PDSCH sets and original HARQ feedback timing based on the PDSCH-to-HARQ-timing-indicators.

#### Multiple HARQ feedback opportunities signalled in advance

The mechanisms for multiple HARQ feedback opportunities listed in TR38.889 include also the signaling of multiple opportunities in advance (Alt 4 and Alt 5 in [5]). Determining multiple HARQ feedback transmission opportunities in advance can reduce the additional latency caused by LBT blocking the original HARQ feedback opportunity, as the transmission for the missing HARQ feedback does not need to be triggered by a separate DCI. Reduced latency facilitates more efficient HARQ operation as some of HARQ processes are not suspended by the missing HARQ feedback.

The drawback is the UL overhead, as multiple PUCCH resources are reserved for single HARQ feedback. Hence, multiple PUCCH resources should be allocated only to the UEs frequently suffering from blocked channel access. The allocation should be dynamic and highly flexible to support fast adaptation to changing load in channel contention. We see that extending the configuration of HARQ feedback timing indication (K1) in DCI to indicate multiple timings is a sufficiently flexible approach: some K1 values may be configured to indicate multiple transmission opportunities while other K1 values may indicate single transmission opportunity. Correspondingly, the PRI may be configured to indicate a single PUCCH resource, or a sequence of PUCCH resources corresponding to the different HARQ feedback timings indicated by K1. The sequence of PUCCH resources may be configured e.g. to cover different LBT sub-bands of the BWP to provide diversity in frequency.

On the other hand, allocating PUCCH resources on multiple carriers would essentially establish a new PUCCH cell. That would require substantially larger specification and implementation impact, as changes would be needed on resource configurations, resource allocation mechanisms, power control, UL transmission priorization rules, UCI multiplexing rules, etc. We do not see that such large effort should be taken to provide cross-carrier diversity for channel access, especially as rather similar diversity can be achieved – if really needed – by PUSCH scheduling and UCI on PUSCH multiplexing. Additionally, similar channel access diversity can be obtained within a wideband carrier extending over multiple LBT sub-bands.

Based on discussion above, we see that HARQ feedback mechanism providing multiple pre-indicated transmission opportunities (Alt 4 and Alt 5) can be considered as a complementary mechanism for Alternative 1.

Proposal 11: Complementary to alternative 1, consider support for mechanism where PDSCH-to-HARQ-timing-indicator and PUCCH resource indicator can be configured to indicate multiple timings and multiple PUCCH resources, respectively, for HARQ feedback reporting within the PUCCH cell.

Observation 5: There is not sufficient need for PUCCH resource allocation on multiple cells justifying the required standardization efforts.

# 4. Conclusions

In this contribution, we discussed the changes and enhancements needed for HARQ scheduling and feedback for NR unlicensed scenarios. Based on the discussion, we make the following observations and proposals:

UL scheduling

Proposal 1: 2-stage scheduling is supported for NR-U uplink.

Proposal 2: To support flexible BWP operation and flexible starting points with acceptable number of HARQ processes, further enhancements are needed for multi-TTI PUSCH scheduling.

Proposal 3: A floating HARQ process block is introduced for multi-TTI PUSCH scheduling to support flexible starting points with acceptable number of HARQ processes.

Observation 1: Cross-numerology scheduling is beneficial for NR-U PUSCH scheduling especially in the case of Scenario A (CA with NR cells) deployment

Proposal 4: NR-U PUSCH multi-TTI scheduling supports cross-numerology scheduling based on Rel-16 mechanism.

HARQ feedback for self-contained COT

Proposal 5: In self-contained COT operation, DL assignment can trigger a pre-configured UL transmission (short PUSCH, CSI report, or SRS) preceding the HARQ ACK transmission associated to the DL assignment.

HARQ feedback across COTs

Observation 2: In HARQ feedback mechanism across the shared COTs, unnecessary transmission of HARQ feedback for multiple times shall be avoided.

Proposal 6: gNB triggers the pending HARQ feedback from earlier COT(s) with the incremented DAI in the DL assignment on the following COT (in Alt 1a of TR38.889).

Observation 3: Alt 2 in TR38.889 is prone to ambiguity in the codebook size and/or PUCCH resource selection. There is no need to consider Alt 2 further

Multiple HARQ feedback opportunities

Proposal 7: In explicitly triggered HARQ feedback mechanism,

- HARQ feedback codebook (or HARQ feedback window) is assigned with a codebook ID in PDSCH scheduling DCI

- A separate field in PDSCH scheduling DCI can control HARQ feedback transmission for earlier COTs

Proposal 8: HARQ feedback mechanism shall be able to provide several retransmission opportunities for a PDSCH HARQ-ACK.

Proposal 9: HARQ feedback transmission for the last PDSCH of the previous COT in a current COT should not cause unnecessary re-transmission of HARQ feedback for other PDSCHs in the previous COT.

Proposal 10: To improve dynamic codebook reliability, total DAI can be allowed to indicate a larger number of PDSCHs than scheduled up to the current PDCCH monitoring occasion. A minimum size for a reported codebook can be determined.

Observation 3: Codebook ID assignment to HARQ feedback codebooks can operate as a codebook counter DAI.

Observation 4: Dynamic codebook determination mechanisms relying on continuous accumulation of DAI (e.g. in [7]-[10]) cannot operate properly when HARQ-ACK for the last PDSCH of a COT is reported in the next COT.

Proposal 11: Complementary to alternative 1, consider support for mechanism where PDSCH-to-HARQ-timing-indicator and PUCCH resource indicator can be configured to indicate multiple timings and multiple PUCCH resources, respectively, for HARQ feedback reporting within the PUCCH cell.

Observation 5: There is not sufficient need for PUCCH resource allocation on multiple cells justifying the required standardization efforts.

# References

- RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- R1-1902108, “On uplink signal and channel structures for NR-U”, Nokia, Nokia Shanghai Bell Nokia

- Draft RAN Meeting Report, 3GPP TSG RAN1 Meeting #AH 1901, 3GPP

- RP-182014, “Way Forward for Cross-Carrier Scheduling with Different Numerologies”, Nokia

- 3GPP TR38.889 v16.0.0

- Draft RAN Meeting Report, 3GPP TSG RAN Meeting #96, 3GPP

- R1-1902259, “HARQ enhancement for NR-U”, Samsung

- R1-1901527, “HARQ enhancement in NR unlicensed”, Huawei, Hi-Silicon

- R1-1902885, “Potential HARQ enhancements for NR-U”, Ericsson

- R1-1901800, “Enhancements to HARQ for NR-U operation”, MediaTek

- R1-1902473, “Enhancements to HARQ for NR-unlicensed”, Intel

- R1-1902265, “Discussion on HARQ-ACK codebook design”, Samsung

# Appendix

In this appendix, we consider Type-2 codebook determination and the combining of multiple codebooks in various situations. In Figure 5, the baseline operation for additional HARQ feedback reporting is illustrated. In the figure, the first codebook (CB 00) is missed by gNB due to detection error. In the following COT, gNB triggers additional transmission for the CB 00, in addition to the codebook CB 01 corresponding to the current HARQ feedback window (marked with dashed line).

Figure 5. Baseline operation for additional HARQ feedback reporting with codebook ID noted as CB-ID, total DAI as T-DAI, and counter DAI as C-DAI.

In Figure 6, the HARQ feedback reporting in the case that UL LBT prevents PUCCH transmission in multiple consecutive transmission opportunities before a successful feedback transmission. We see it important that additional transmission opportunity can be indicated multiple times for a codebook, to improve the robustness of the feedback mechanism. Whether the trigger can indicate additional transmission opportunity for a specific codebook or indicate additional transmission opportunity for multiple codebooks (as in Figure 6) is a further design issue.

Figure 6. HARQ feedback reporting in the case that UL LBT prevents PUCCH transmission multiple times.

In Figure 7, the HARQ feedback reporting is illustrated when UE misses two PDSCHs. As there are no other PDSCHs in the HARQ feedback window, the UE misses the codebook #01 altogether. gNB request an additional transmission for the codebook #01 when scheduling PDSCHs for codebook #10. As the codebook IDs are sequentially incremented, it can detect that a codebook was missed. However, UE does not know the size of the missed codebook, but both UE and gNB should assume the same codebook size. The problem can be solved by defining a minimum size (in terms of PDSCHs) for each codebook that UE reports. In this case, a minimum codebook size of 2 is assumed, and UE reports 2 NACKs for codebook #01.

Figure 7. HARQ feedback reporting when UE misses a HARQ feedback window of 2 PDSCHs.

One issue to consider is whether UE missing the last PDSCH(s) of a codebook results in a codebook size error, and hence, to erroneous transmitted codebook as shown in Figure 8a. We see that the codebook determination reliability can be improved by allowing the total DAI to indicate a larger value than the total number of PDSCHs scheduled up to the current PDCCH monitoring occasion, as shown in Figure 8b. From the Figure 8b it is seen that the last detected total DAI is used in the determination of the codebook size (and hence defining codebook space for the missed last PDSCHs), while the counter DAI is used to determine the ACK/NACK position within the codebook.

By keeping the counter DAI and total DAI operation specific for each codebook, there is no need for additional restrictions on the number of triggered codebooks, or how many times additional transmission can be triggered for a codebook, or in the PDCCH where the additional transmission can be triggered. (There are of course restrictions caused by the size of codebook ID field and the size/definition of the trigger field). If further improvement on the codebook determination is needed, increasing the field size for counter DAI and total DAI may be considered.

a)

b)

Figure 8. HARQ feedback reporting when UE misses PDCCHs at the end of codebook, a) with Rel-15 total DAI b) with predictive total DAI