3GPP TSG RAN WG1 Meeting #97	R1-1905982

Reno, USA, May 13 – 17, 2019

Agenda Item:	6.2.1.6

Source:	Huawei, HiSilicon

Title:	On MPDCCH performance improvement

Document for:	Discussion and decision

# Introduction

In RAN1 #96bis meeting, the following agreements were concluded on MPDCCH performance improvement [1].

Agreement

When CSI-based precoding is configured:

- The UE monitors a set of MPDCCH candidates assuming the mapping between DMRS and CRS ports is based on a previously reported PMI.

  - FFS details, including delay for applicability of PMI.

- A fallback mechanism is supported. The details of the fallback mechanism are FFS.

Agreement

For precoding cycling for 2Tx antennas, predefine one precoder set which consists of 2 existing precoders.  FFS: Which of the two precoder among the existing 2TX precoders are used.

Agreement

For precoding cycling for 4Tx antennas, predefine one precoder set which consists of 4 existing precoders.  FFS: Which of the four precoder among the existing 4TX precoders are used.

Agreement

The precoding cycling in time domain has a granularity of YCH (granularity of frequency hopping) only.

- Precoder cycling can be used even in case frequency hopping is not used as long as YCH is configured

In this contribution, further considerations on MPDCCH performance improvement are provided.

# Discussion

## Precoding based on CSI report

The eNB may miss the CSI reported by the UE or not successfully obtain the CSI reporting. Therefore, the fallback mechanism should be supported. There are many different schemes on the fallback mechanism.

In [2], it is proposed some MPDCCH candidates associated with a part of the UE-specific search space and/or particular time/frequency location(s) are reserved for precoder cycling, if CSI-based precoding is configured. In [3], it is proposed that UE can attempt multiple hypothesis tests for the different precoders. Moreover, for UEs in connected mode, if the UE is configured to use mapping based on CSI report, UE uses the latest reported CSI for mapping. In [4], it is proposed that when the UE is configured with a transmission mode assuming PMI report, the UE may assume that the PMI from its latest CSI feedback is used. Moreover, there are fallback MPDCCH candidates which do not have mapping of CRS and MPDCCH DMRS ports. In [5], it is supported the precoder cycling as a fallback mechanism and it is provided an example of configuring 2 MPDCCH PRB sets, one for mapping based on CSI report and another for precoder cycling. In [6], it is proposed a subset of the MPDCCH ports follows the latest reported CSI report, and a subset of the MPDCCH ports follows a preconfigured mapping with respect to CRS ports. In [7], it is considered indicating the precoder via the existing DCI signaling or determining the precoder based on PMI delay.

The following table summarizes the view of different sources. Several companies proposed to use the precoding cycling as the fallback mechanism. In our view, the fallback mechanism based on precoding cycling is feasible. However, a simple rule should be applied. On the other hand, considering that the probability of eNB missing the CSI reported by the UE is very low, hypothesis test for the different precoders can also be considered.

Table 1: Methods for fallback mechanism

| Fallback mechanism | Details |
| --- | --- |
| Precoder cycling | Partial UE-specific search space and/or particular time/frequency location |
|  | MPDCCH PRB set |
|  | The delay of PMI reporting is more than or equal to Ndelay ms |
| MPDCCH ports | Preconfigured mapping with respect to CRS ports |
| MPDCCH candidates | No mapping of CRS and MPDCCH DMRS ports |
| Hypothesis tests | Test for the different precoders |
| DCI signaling |  |

Proposal 1: When CSI-based precoding is configured, the following fallback mechanisms can be considered:

- based on precoding cycling

- based on hypothesis test for the different precoders

## Precoders set

In [2], it is showed that for the case of four CRS ports, the subset of Rel-8 codebook precoders with indices 12, 13, 14, and 15 show the BLER performance fairly close to that of the best. Therefore, the precoders with indices 12, 13, 14, and 15 can consist of the precoder set for 4Tx.

For 2Tx antennas, with two existing precoders with the opposite vector directions, the 2 precoders cycling and 4 precoders cycling have the same performance [7]. Based on these results, precoders with indices 0, 1 can consist of the precoder set for 2Tx.

Proposal 2: Precoders with indices 12, 13, 14, and 15 can consist of the precoder set for 4Tx.

Proposal 3: Precoders with indices 0, 1 can consist of the precoder set for 2Tx.

# Conclusions

In this contribution, based on the agreement in previous meeting, further considerations on MPDCCH performance improvement are provided and following proposals are given:

Proposal 1: When CSI-based precoding is configured, the following fallback mechanisms can be considered:

- based on precoding cycling

- based on hypothesis test for the different precoders

Proposal 2: Precoders with indices 12, 13, 14, and 15 can consist of the precoder set for 4Tx.

Proposal 3: Precoders with indices 0, 1 can consist of the precoder set for 2Tx.

# References

- RAN1 Chairman notes, RAN1 #96bis, Xi’an, China, 8th – 12th April, 2019

- R1-1903387	MPDCCH performance improvement in LTE-MTC	Ericsson

- R1-1904172	MPDCCH performance improvement	Nokia, Nokia Shanghai Bell

- R1-1904268	MPDCCH performance improvement	Intel Corporation

- R1-1904610	Discussion on MPDCCH performance improvement	 LG Electronics

- R1-1904522	Usage of CRS for MPDCCH	Qualcomm Incorporated

- R1-1904347	Discussion on MPDCCH performance improvement	 ZTE