3GPP TSG RAN WG1 #97									R1-1906184

Reno, USA, May 13th – 17th, 2019

Source:	vivo

Title:	Efficient Scell activation

Agenda Item:	7.2.13.4

Document for:	Discussion and Decision

- Introduction

Similar as in LTE, MAC-CE based Scell activation/deactivation is supported in NR Rel-15. For power saving purpose, the Scell is activated only when there is large amount of traffic for the UE.

As specified in TS38.133 (copied below for reference), the Scell activation delay is up to [THARQ + Tactivation_time + TCSI_Reporting], where Tactivation_time = [TSMTC_SCell + 5ms] for FR1 and FR2 respectively in the best case (corresponding to the highlighted text in the table), i.e. the corresponding Scell is known by the UE. In an optimistic assumption with THARQ =1ms, TCSI_reporting=1ms, the overall activation delay for a known cell is around 27ms for 20ms SMTC periodicity and 12ms for 5ms SMTC periodicity, respectively. Such activation delay is not optimal for burst traffic and may cause traffic delay and unnecessary UE power consumption.

- 8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this section shall apply for the UE configured with one downlink SCell in SCG in EN-DC, or in standalone NR carrier aggregation and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting], where: THARQ is the timing between DL data transmission and acknowledgement as specified in [7]. Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is: - [TSMTC_SCell + 5ms], if the SCell measurement cycle is equal to or smaller than [160ms]. - [TSMTC_MAX + TSMTC_SCell + 5ms], if the SCell measurement cycle is larger than [160ms]. If the SCell is unknown and belongs to FR1, Tactivation_time is: - [2*TSMTC_MAX + 2*TSMTC_SCell + 5ms] provided the SCell can be successfully detected on the first attempt. If the SCell being activated belongs to FR2, Tactivation_time is: - [TSMTC_SCell + 5ms] if there is at least one active serving cell on that FR2 band, provided that the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in section 3.6.3. - [TBD* TSMTC_SCell + 5ms] if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1. Where, TSMTC_MAX: - In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. - In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA. - TSMTC_MAX is bounded to a minimum value of 10ms. TSMTC_SCell: SMTC periodicity of SCell being activated and the minimum value is 10ms. TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in [2].

In this contribution, we discuss the potential enhancements to support more efficient Scell activation.

- Discussion

In general, there can be two ways to enhance the efficiency of Scell activation, as the following

- Reduce the Scell activation delay (from deactivated state to activated state)

In this method, the Scell is activated only when necessary (burst traffic arrival), and the delay from deactivated state to activated is minimized

- Keep the Scell activated, but reduce the UE power consumption during Scell activated state

In this method, the Scell remains activated regardless of traffic condition, such that the Scell can be used for DL transmission immediately after traffic arrival without activation delay. However, the UE power consumption during Scell activation time should be minimized as much as possible.

In the following section, we discuss the above two potential methods in more detail.

  - Reducing Scell activation delay

As analyzed in section 1, the components of Scell activation delay is as the following

- Tactivation_time = [TSMTC_SCell + 5ms], where the fixed 5ms processing time may include the RF retuning time and higher layer processing time (MAC CE), and TSMTC_SCell is the time that UE has to wait until the first available SSB for AGC and time/frequency synchronization.

  - The TSMTC_SCell is determined by the configured SMTC periodicity on the Scell frequency, and can be down to 5ms if the network configures SSB transmission per 5ms. Reducing SMTC periodicity can be helpful for Scell activation, e.g. the activation delay can be reduced from 27ms to 12ms if the SMTC periodicity is reduced from 20ms to 5ms. However, this will cause significant network overhead, as SSB has to be transmitted on the Scell once per 5ms periodically regardless whether there is UE requiring Scell activation or not. Alternatively, to avoid the overhead issue caused by periodic SSB transmission, network can trigger on demand RS to facilitate UE preparation (AGC and time/frequency sync) for Scell activation. The on demand RS can be SSB, or TRS, etc. The triggering of on demand RS can be MAC CE based, or DCI based. The TSMTC_SCell can potentially be reduced to 1~2ms if the on demand RS can be available immediately after the Scell activation command is applied at the UE.

  - The 5ms processing time may include the RF retuning time and higher layer processing time due to MAC CE based activation command. If DCI based Scell activation command is introduced, the processing time can potentially be reduced to 3ms at the maximum assuming 1ms for DCI decoding and 2ms for RF retuning.

- THARQ, If DCI based Scell activation command is introduced, this component can be removed as there is no HARQ-ACK for DCI.

- TCSI_reporting can be assumed 1ms unless faster CSI processing time is introduced.

Based on the above discussion, it is feasible to reduce the Scell activation time to around 5~6ms, if DCI can be used for activating the Scell and triggering the on-demand RS for UE preparation, which is a significant latency reduction compared to the current 27ms delay assuming typical 20ms SMTC periodicity.

Proposal 1: NR supports a DCI based Scell activation command and on-demand RS for UE Scell preparation triggered by the DCI command.

  - Minimizing UE power consumption during Scell activation time

As discussed above, a 5~6ms Scell activation delay still exits even when DCI based activation command is supported. If a tighter latency requirement after DL data arrival is targeted for Scell transmission, the UE may have to keep Scell always active such that the DL traffic can be served on Scell as soon as possible. However, the continuous PDCCH monitoring would cause significant power consumption especially during the time when less or no traffic is scheduled on Scell. Thus, it would be necessary to reduce PDCCH monitoring on the activated Scell for power saving.

In LTE eUCA, a Scell dormant state was introduced. UE is triggered by MAC CE for state transition among the activated state, deactivated state and dormant state. On a dormant Scell, UE does not monitor PDCCH but keep the time/frequency sync and CQI measurement and reporting, thus the power consumption can be significantly reduced. When traffic arrives, network can trigger the UE transition from dormant state to active state with very small delay (mostly MAC CE signalling delay) such that traffic can be served as soon as possible.

In NR, similar concept as dormant Scell state can be used to minimize the UE power consumption. However, as the PDCCH monitoring periodicity can be configured with various values (from 1slot to 2560slots), the power saving gain can be obtained by using long PDCCH monitoring periodicity. As a result, there is no strong need to introduce explicit Scell dormant state in NR if the PDCCH monitoring periodicity can be dynamically switched based on the traffic demand. To achieve the dynamic PDCCH monitoring periodicity on Scell, two options can be considered.

- Dynamic switching of PDCCH monitoring periodicity based on DCI indication

This scheme has been studied in TR38.840 [2] as one candidate solution for UE power saving. Based on the evaluation it is observed above 50% power saving gain for the sporadic traffic. As one example, a long PDCCH monitoring periodicity (e.g. with periodicity of hundreds of slots) can be used for an activated Scell when there is no traffic to serve, while a per-slot PDCCH monitoring periodicity can be used immediately after traffic arrives. To minimize the delay due to switching from a long monitoring periodicity to a short monitoring periodicity, a DCI indication from Pcell can be used to switch the PDCCH monitoring periodicity on Scell.

- “Dormant” BWP

In Rel-15, UE can be configured with a DL BWP without PDCCH search space. Such a DL BWP can be considered as a “dormant” BWP. If UE is configured with a dormant BWP and another regular BWP (with PDCCH search space), the UE can stay on the dormant BWP for power saving and can be dynamically switched to the regular BWP by gNB when traffic arrives, where only the BWP switching delay is needed before UE is able to receive DL traffic on the Scell. Since there is no PDCCH search space on the dormant BWP, a DCI indication from Pcell for UE BWP switching from the dormant BWP to the regular BWP is necessary.

Proposal 2: Do not introduce the explicit Scell dormant state in NR.

Proposal 3: Support the following enhanced schemes to minimize the UE power consumption on an activated Scell:

- Dynamic switching of PDCCH monitoring periodicity for an activated Scell based on a DCI indication from another serving cell, e.g. Pcell

- Dynamic BWP switching from a ‘dormant’ DL BWP (DL BWP without PDCCH search space) to a regular DL BWP for an activated Scell, based on a DCI indication from another serving cell, e.g. Pcell

- Conclusion

In this contribution, we discuss the potential enhancements to support more efficient Scell activation and have following proposals:

Proposal 1: NR supports a DCI based Scell activation command and on-demand RS for UE Scell preparation triggered by the DCI command.

Proposal 2: Do not introduce the explicit Scell dormant state in NR.

Proposal 3: Support the following enhanced schemes to minimize the UE power consumption on an activated Scell:

- Dynamic switching of PDCCH monitoring periodicity for an activated Scell based on a DCI indication from another serving cell, e.g. Pcell

- Dynamic BWP switching from a ‘dormant’ DL BWP (DL BWP without PDCCH search space) to a regular DL BWP for an activated Scell, based on a DCI indication from another serving cell, e.g. Pcell

- Reference

- RP-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, Nokia Shanghai Bell, Huawei

- TR 38.840 v1.0.0, Study on UE Power Saving  (Release 16)