3GPP TSG RAN WG1 Meeting #97			R1-1906220

Reno, USA, May 13th – 17th, 2019

Source:	NTT DOCOMO, INC.

Title:	Updated evaluation results of Indoor Hotspot-eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for: 	Discussion

- Introduction

At the RAN#81 meeting, the evaluation results of eMBB collected from 3GPP companies are discussed and captured into the interim report of ITU self-evaluation which was later submitted to ITU meeting in Oct. 2018. In the report, we provided the evaluation results of eMBB with ideal channel estimation. In this contribution, we present the updated evaluation results under non-ideal channel estimation.

- ITU Requirements

The minimum requirement for the 5th percentile user spectral efficiency and average spectral efficiency for Indoor Hotspot-eMBB sceanrio is provided as follow [1].

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |

The defination of area traffic capacity is also defined in [1]. Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is given by:

Carea = ρ × W × SEavg                                                               (1)

The minimum requirement of area traffic capacity is listed as follow:

TABLE 3

Area traffic capacity

| Test environment | Downlink (Mbit/s) |
| --- | --- |
| Indoor Hotspot – eMBB | 10 |

- Evaluation Results of Spectral Efficiency

3.1 Downlink Spectral Efficiency

In this section, we update the downlink results of Indoor Hotspot–eMBB Config A/B/C under non-ideal channel estimation. The detailed evaluation assumptions can be found in Annex.

The evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency are summarized in Table 4. The area traffic capacity results are provided based on results of the DL average spectral efficiency.

TABLE 4

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config A/B/C

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | SEavg (bit/s/Hz/TRxP) | ITU Requirement of SEavg (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement of SEuser (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| Config A (12TRxP) | 32T (4,4,2,1,1;4,4) 4R (1,2,2,1,1;1,2) | DSUUD | 10.694 | 9 | 0.386 | 0.3 |
| Config B (12TRxP) | 32T (4,8,2,1,1;4,4) 8R (2,4,2,1,2;1,2) | DSUUD | 7.707 | 9 | 0.369 | 0.3 |
| Config C (12TRxP) | 32T (8,16,2,1,1;4,4) 8R (2,4,2,1,2;1,2) | DSUUD | 7.427 | 9 | 0.377 | 0.3 |

Based on above evaluation results, we have following observation.

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A.

Observation 2: The minimum requirement for DL 5th percentile user spectral efficiency can be met for Indoor Hotspot-eMBB configuration B and C, while the DL average spectral efficiency can not be met.

3.2 Uplink Spectral Efficiency

In this section, we provide the uplink results of Indoor Hotspot–eMBB Config A/B/C under non-ideal channel estimation. The detailed evaluation assumptions can be found in Annex.

The evaluation results of UL average spectral efficiency and UL 5th percentile spectral efficiency are summarized in Table 5. It is noted that, in some antenna configurations, there are 8 TxRU for UE in two panels. UE only utilizes 4 TxRU in one panel for data transmission after panel selection.

TABLE 5

Average spectral efficiency and 5th percentile user spectral efficiency for Indoor Hotspot–eMBB Config A/B/C

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | SEavg (bit/s/Hz/TRxP) | ITU Requirement of SEavg (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) | ITU Requirement of SEuser (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| Config A (12TRxP) | 32R (4,4,2,1,1;4,4) 4T (1,2,2,1,1;1,2) | DSUUD | 9.420 | 6.75 | 0.445 | 0.21 |
| Config B (12TRxP) | 32R (4,8,2,1,1;4,4) 8T (2,4,2,1,2;1,2) | DSUUD | 7.043 | 6.75 | 0.372 | 0.21 |
| Config C (12TRxP) | 32R (8,16,2,1,1;4,4) 8T (2,4,2,1,2;1,2) | DSUUD | 6.799 | 6.75 | 0.362 | 0.21 |

Based on above evaluation results, we have following observation.

Observation 3: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

- Evaluation Results of Area Traffic Capacity

In this section, we will analyze the minimum bandwidth to meet the ITU requirement on area traffic capacity based on initial results of the DL average spectral efficiency in Table 6, Table 7 and Table 8.

TABLE 6

Area traffic capacity for Config A

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- | --- |
| Config A (12TRxP) | 32T (4,4,2,1,1;4,4) 4R (1,2,2,1,1;1,2) | DSUUD | 450 | 9.625 | 10 |
|  |  |  | 500 | 10.694 | 10 |

TABLE 7

Area traffic capacity for Config B

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- | --- |
| Config B (12TRxP) | 32T (4,8,2,1,1;4,4) 8R (2,4,2,1,2;1,2) | DSUUD | 600 | 9.248 | 10 |
|  |  |  | 650 | 10.019 | 10 |

TABLE 8

Area traffic capacity for Config C

| Test environment | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs within a panel | Frame structure | Bandwidth, W (MHz) | Area traffic capacity (Mbit/s) | ITU requirement (Mbit/s) |
| --- | --- | --- | --- | --- | --- |
| Config C (12TRxP) | 32T (8,16,2,1,1;4,4) 8R (2,4,2,1,2;1,2) | DSUUD | 650 | 9.655 | 10 |
|  |  |  | 700 | 10.398 | 10 |

Based on above results, we have following observations.

Observation 4: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration A, when the bandwidth is larger than 500MHz.

Observation 5: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration B, when the bandwidth is larger than 650MHz.Observation 6: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration C, when the bandwidth is larger than 700MHz.

- Summary

In this contribution, we provide both the downlink and uplink results of Indoor Hotspot-eMBB Config B and C, including the DL/UL average spectral efficiency, the DL/UL 5th percentile user spectral efficiency and the area traffic capacity. The observations are summarized as follow:

Observation 1: The minimum requirement for DL 5th percentile user spectral efficiency and DL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A.

Observation 2: The minimum requirement for DL 5th percentile user spectral efficiency can be met for Indoor Hotspot-eMBB configuration B and C, while the DL average spectral efficiency can not be met.

Observation 3: The minimum requirement for UL 5th percentile user spectral efficiency and UL average spectral efficiency can be met for Indoor Hotspot-eMBB configuration A, B and C.

Observation 4: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration A, when the bandwidth is larger than 500MHz.

Observation 5: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration B, when the bandwidth is larger than 650MHz.

Observation 6: The requirement on area traffic capacity can be met for Indoor Hotspot-eMBB configuration C, when the bandwidth is larger than 700MHz.

References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

Annex: Evaluation assumptions

| Indoor Hotspot – eMBB |  | Config A | Config B | Config C |
| --- | --- | --- | --- | --- |
| Carrier frequency for evaluation |  | 4 GHz | 30GHz | 70GHz |
| Total transmit power per TRxP |  | 24 dBm for 20MHz bandwdith | 23 dBm for 80 MHz bandwidth | 21 dBm for 80 MHz bandwidth |
| TRxP number per site |  | 1 | 1 | 1 |
| Number of antenna elements per TRxP |  | 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 64Tx/Rx= (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 256Tx/Rx = (M,N,P,Mg,Ng) = (8,16,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Number of TXRU per TRxP |  | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) (1-to-1 mapping) | 32TXRU,(Mp,Np,P,Mg,Ng) = (4,4,2,1,1) | 32TXRU,(Mp,Np,P,Mg,Ng) = (4,4,2,1,1) |
| Number of UE antenna elements |  | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization | 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Number of TXRU per UE |  | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 8TXRU,(Mp,Np,P,Mg,Ng) = (1,2,2,1,2) | 8TXRU,(Mp,Np,P,Mg,Ng) = (1,2,2,1,2) |
| Traffic model |  | Full buffer | Full buffer | Full buffer |
| UE density |  | 10 UEs per TRxP |  |  |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. | Based on RSRP (formula) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method |  | Geographical distance based wrapping |  |  |
| Polarized antenna model |  | Model-2 in TR36.873 |  |  |
| Scheduler |  | UL: SU PF (Max 12 layer) DL: MU PF (Max 12 layer) | UL: SU PF (Max 4 layer) DL: SU PF (Max 4 layer) | UL:SU PF (Max 4 layer) DL:SU PF (Max 4 layer) |
| Subband number |  | 20 | 10 | 10 |
| Overhead uplink | PUCCH | 14 OS, 2PRB | 14 OS, 2PRB | 14 OS, 2PRB |
|  | DMRS (Type II) | 6, 12 ports |  |  |
|  | SRS(expect 4RBs for PUCCH) | 2 OFDM symbols per 5 slots |  |  |
|  | PT-RS | N.A | time density is 4 OS, frequency density is 4 PRB | time density is 4 OS, frequency density is 4 PRB |
| Overhead downlink | SS/PBCH block | 1 SS/PBCH block per 20ms | 24 SS/PBCH block per 20ms | 24 SS/PBCH block per 20ms |
|  | PDCCH | 2 OFDM symbols per slot | 2 OFDM symbols per slot | 2 OFDM symbols per slot |
|  | DMRS (Type II) | 6, 12 ports |  |  |
|  | CSI-RS | 4, 8, 16, 24, 32 ports with periodicity of 5ms |  |  |
|  | TRS | 4 CSI-RS resource in 2 consecutive slots per 20ms, 50PRB |  |  |
|  | PT-RS | N.A. | time desity is 4OS, frequency density is 4PRB | time desity is 4OS, frequency density is 4PRB |
| Receiver |  | MMSE-IRC |  |  |
| CSI feedback |  | periodically 5 slot feedback with 4 slot delay |  |  |
| Channel estimation |  | non-ideal |  |  |