3GPP TSG-RAN WG1 97                                                    	 R1-1906286

Reno, USA, May 13th – 17th, 2019

Source:	OPPO

Title:	Enhancements on overhead reduction and rank extension for type II CSI feedback

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

Introduction

The new WID [1] for NR MIMO was agreed in RAN #80 meeting. The enhancement of type II codebook can be considered in Rel-16 from the following aspects:

- Extend specification support in the following areas [RAN1]

  - Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2

In RAN1 #96bis meeting, the compression scheme for the above enhancements was discussed with following agreements:

Agreement: Table 1 of R1-1905629 is agreed for the support of UCI parameters for MU-CSI

Agreement

On RI=3-4 extension:

- K0 setting: agree on supporting Alt1, i.e. total max # NZ coefficients across all layers ≤ 2K0 where the K0 value set for RI{1,2}

- FD basis subset selection: agree on layer-specific subset selection

- Coefficient subset selection: agree on layer-specific subset selection

Agreement

SD basis subset selection is layer-common

Agreement

On RI=3-4 extension:

- (L,p) setting: In RAN1#97 (Reno), down select and decide from the following alternatives:

  - Alt2B, Alt3C, Alt6E (see Table 9 from R1-1905629)

Agreement

The scheme for indicating the number of NZ coefficients (NZC) will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0}

- Alt1.2: Per-layer # NZC without RI where # NZC = {0, 1, 2, …, K0}

- Alt1.3: RI + differential of # NZC summed across layers

  - Differential means fraction of 2K0 with smaller number of possible values compared to the regular # NZC (in Alt1.1)

- Alt1.4: RI + per-layer differential # NZC

  - Differential means fraction of K0 with smaller number of possible values compared to the regular # NZC (in Alt1.2)

Agreement

For RI=3-4, the bitmap design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt2.1: 2LMi bits per layer, i=0, 1, …, (RI-1)

- Alt2.2: One joint bitmap 1 for all layers, where an indicator bit is 1 if at least one of the RI layers has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.2B: Bitmaps 1 for each layer, where an indicator bit is 1 if at least one of the RI beams has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.3: LMi bits for the layer in which the weaker polarization is dropped (else 2LMi bits) + up to 4-bit bitmap to indicate the layer where the weaker polarization is dropped (UCI part 1); i=0, 1, …, (RI-1)

Agreement

For RI=1, strongest coefficient indicator (SCI) is a -bit indicator. For RI>1, SCI design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt3.1 (applicable to Alt1.2): Per-layer SCI, where SCIi is a –bit indicator (i=0, 1, …, (RI-1))

- Alt3.2 (applicable to Alt1.1): Per-layer SCI, where SCIi is a –bit indicator

- Alt3.3: Per-layer SCI, where SCIi is a –bit or  indicator (i=0, 1, …, (RI-1))

- Alt3.4: Per-layer SCI, where SCIi is a –bit (i=0, 1, …, (RI-1))

Agreement

SD basis subset selection indicator is a -bit indicator.

Agreement

On FD basis subset selection indicator, the design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt5.1: FD basis subset selection indicator is per layer where it is a -bit indicator or -bit indicator or size-N3 bitmap, (i=0, 1, …, (RI-1))

- Alt5.2: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and N3’ is either reported in UCI part 1 or fixed in specification or higher-layer configured, and the intermediate set in UCI part 2

    - Minitial indicated by  (or other values) bits indicates starting point of the intermediate FD basis set. The FD basis in this intermediate set is given by mod(Minitial+n,N3), n=0,1,..,N3’-1

  - The 2nd step uses either N3’-bit bitmap or -bit indicator to indicate the FD basis used for each layer

- Alt5.3: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) selected from multiple higher-layer configured intermediate sets and the value of N3’ is indicated in UCI part 1

  - The 2nd step uses -bit indicator to indicate the FD basis used for each layer

- Alt5.4: FD basis subset is selected as mod(Mi_initial + n,N3), n=0,1,..,Mi–1

  - The subset selection is done per layer

- Alt5.5: The intermediate FD basis subset of size is higher layer configured per rank, and  is not reported in UCI part 1.

  - FFS: FD basis subset of size  per rank

  - The UE reports -bit bitmap or or  bit indicator to indicate the FD basis used for each layer

- Alt5.6: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is either fixed or higher-layer configured

    - The FD basis in this intermediate set is reported either by N3-bit bitmap or  bit indicator

  - The 2nd step uses either N3’-bit bitmap or -bit indicator to indicate the FD basis used for each layer

- Alt5.7: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is indicated in UCI part 1

    - The FD basis in this intermediate set is the union of FD basis for all layers, and is reported bybit indicator

  - The 2nd step uses either N3’-bit bitmap or -bit indicator to indicate the FD basis used for each layer

- Alt5.8:

  - For RI > 2, two-step FD basis subset selection

    - The 1st (intermediate) step uses an intermediate set of size-N3’ (N3’=)

      - Intermediate set is the union of FD basis for all layers, and is reported by size-N3 bitmap

    - The 2nd step uses size-N3’ bitmap to indicate the FD basis used for each layer

  - For RI < 3, FD basis subset selection indicator is per layer where it is a size- N3 bitmap

Agreement

On RI=3-4 extension, with the agreed total max # NZ coefficients across all layers ≤ 2K0 where the K0 value (hence β) set for RI{1,2}, the scheme for determining the # NZC per layer will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt0. KNZ,i is unrestricted as long as

- Alt1. KNZ,i≤K0 as long as

In this contribution, we will provide analysis on overhead reduction for type II CSI feedback along with system-level evaluation results.

Discussion

(L, p) parameters setting

We have concluded that L is to be RI-common and layer-common for Rank3/4 codebook. The configuration mechanism for parameter p will be down-selected among Alt 3C, Alt 2B and Alt 6E according the agreement of RAN1#96bis.

| RI | Layer | Alt3C | Alt2B | Alt6E |
| --- | --- | --- | --- | --- |
| 1 | 0 | y0 | y0 | y0 |
| 2 | 0 |  |  |  |
|  | 1 |  |  |  |
| 3 | 0 | v0 | y0 | v3,0 |
|  | 1 |  |  | v3,1 |
|  | 2 |  | y2 | v3,2 |
| 4 | 0 |  | y0 | v4,0 |
|  | 1 |  |  | v4,1 |
|  | 2 |  | y2 | v4,2 |
|  | 3 |  |  | v4,3 |

Alt 3C: layer-common and RI-common for RI 3/4

- It is the simplest way to extend Rank1/2 codebook

- v0 could be in fixed relationship with y0 or configured by a single RRC parameter

- Overhead compression can be achieved by selecting a smaller subset of FD basis (equivalently, a smaller M) to ensure comparable overhead with Rank1/2;

Alt 2B: layer-group-specific and RI-common for RI 1~4

- An additional parameter y2 could be configured for layer 3/4

- Overhead of RI 3/4 will exceed that of RI 1/2 (generally tens of bits, for example M0=7, M2=4 results in more 4*2*4*2=64 bit for bitmap in L=4 for rank4). It’s hard to regard it as a “comparable” overhead to RI 1/2 (assuming RI1/2 bitmap design directly extend to RI3/4).

- RI-common y0 is not flexible for layer 1/2 because #FD basis of layer 1/2 for RI 3/4 may be reduced.

- It can provide better performance at the cost of larger overhead compared with Alt3C/Alt6E.

Alt 6E: layer-specific and RI-specific for RI 3/4

- It can provide a flexible way to configure p for each RI and each layer

- Alt 3C is a special case of Alt 6E

- From the perspective of simplifying implementation, it’s not desired to have more than one high layer parameters. It is preferred to have a fixed relation with y0 or use a single high layer configured parameter.

The Alts 2B, 3C and 6E were evaluated with some values of p parameter through SLS. In the simulation, a a length-2LM bitmap per layer was assumed. The evaluation results are provided in the following table:

| Alt-p | 13SB,M=7,L=4,beta=0.5,2K0 |  |  |  | Rank4 overhead | Gain over rank2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | layer1 | layer2 | layer3 | layer4 |  |  |
| 2B | 1/2 | 1/2 | 1/4 | 1/4 | 581 | 24.3% |
| 3C | 1/4 | 1/4 | 1/4 | 1/4 | 531 | 22.2% |
| 6E-RI3 | 2/3 | 2/3 | 2/3 |  |  | 22.4% |
| 6E-RI4 | 1/4 | 1/4 | 1/4 | 1/4 |  |  |

Based on the evaluation results, we can conclude the following observation:

Observation 1: Alt 2B outperforms Alt 3C/Alt 6E about 2% with additional 50 bits; Alt 3C/ Alt 6E show similar performance, and the gain of further optimization offered by Alt6E seems marginal.

Alt.2B provides very limited extra gain over Alt.3C but with the cost of additional overhead, 50bits. Alt.3C and Alt.6E offers very similar performance and same overhead reduction. However, Alt.3C is simpler and more friendly to UE implementation. Therefore, in our view, Alt.3C is the first preference and shall be supported. The following proposals are made:

Proposal 1: For p parameter setting, support Alt3C

Alt6E is also acceptable as a comprise for better progress (second preference).

Proposal 2: Support fixed relation for RI3/4 of p setting if Alt 6E is supported.

NZC and SCI

Currently two types of  #NZC indicator are discussed, i.e., a joint indicator representing the sum of #NZC across layers and separate indicator representing #NZC for each layer.  It was agreed the maximum number of all NZC across layers is 2K0 for rank3/4 extension. #NZC indicator in UCI part1 should be desired to be as effective as possible. It can be seen Alt 1.1 outperform Alt 1.2 significantly. For example, in the case of K0=28 for rank4 reporting, Alt 1.1 requires 2+log228×2=8  bits and Alt 1.2 requires 4×log229=20 bits with maximum #NZC per layer less than K0. In terms of signaling overhead, Alt 1.1 is much better than Alt. 1.2. However, Alt1.1 require a new SCI for RI >1 (Alt3.2 Alt3.3 or Alt 3.4), because directly extending conventional RI=1 SCI causes ambiguity of UCI part 2 for Alt 1.1.

Length of SCI in Alt 3.2 is variable while it is constant in Alt 3.3 and Alt 3.4. At extremely condition (e.g LoS) Alt 3.2 is better. In general, Alt 3.2 and Alt3.3 tend to be similar if the UE is configured with a proper K0  because  KNZ should not be too much smaller in unit of bit than K0. Alt 3.4 can further reduce log2βM bits for each layer in comparison with Alt 3.3. The bit reduction depends on the bandwidth and is generally 1 or 2 bits per layer.

Observation 2: For NZC indicator, Alt 1.1 reduce overhead obviously (e.g 12 bits); For new SCI indicator, Alt 3.2 and Alt 3.3 are expected to be similar, and Alt 3.4 can save 1~2 bit per layer in comparison with Alt 3.3;

Proposal 3: Support joint NZC indicator (Alt1.1) and a new   SCI indicator with log22L bits per layer (Alt 3.4).

FD basis selection indicator

In RAN1#96bis meeting, 8 alternatives were discussed for the design of FD basis selection indicator.  In the corresponding e-mail discussions, FL’s proposals are summarized as:

Proposal 1: The two-step FD basis subset selection is described as follows: · The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) o The value of N3’ is RI- and layer-common o The intermediate FD basis subset is RI- and layer-common · The 2nd step uses combinatorial indicator to indicate the FD basis used for each layer o FFS: exact bitwidth · FFS (to be resolved in RAN1#97): select one of the following alternatives on N3’ setting mechanism: o 1) reported in UCI part 1 o 2) higher-layer configured o 3) fixed · FFS (to be resolved in RAN1#97): select one of the following alternatives on size- N3’ intermediate subset (IntS) o 1) IntS is adjacent and fully parameterized with Minitial, § FFS (to be resolved in RAN1#97): whether is reported in UCI part 2, higher-layer configured, or fixed o 2) IntS is selected freely from N3 FD bases, log2N3N3'-bit combinatorial indicator is reported in UCI part 2 Proposal 2: In RAN1#97, decide on FD basis subset selection scheme from the following alternatives: · Free selection (Alt 5.1 in RAN1#96b) · Fixed selection (Alt 5.4 in RAN1#96b) · Two-step selection (cf. outcome of proposal 1)

For two-step selection, UE reports or can be configured with a length-N3’ intermediate subset which is a union FD basis set (or subset) of all layer in the first step, and encode FD basis of each layer in the intermediate set for the second step. Regarding N3’ setting mechanism, the method of reporting N3’ in UCI can provide better overhead reduction with no performance loss when FD-basis of multi-layer are highly overlapped. The drawback of reporting N3’ is that log2N3or log2minN3,lMl-minlMl bits in UCI part1 are required. Higher-layer configured N3’ may be more flexible than the method of a fixed N3’ setting for overhead reduction. However, for the same reason as p parameter setting, a fixed relationship between N3’ and the same single high-layer configured p parameter is preferred.

Proposal 4: Support fixed N3’ setting, a fixed relationship between N3’ and the same single higher layer configured p parameter is preferred.

Another issue is whether the intermediate set is adjacent or not. The motivation for free Intermediate Set (InS) selection is adjacent selection would decrease T’put performance. The adjacent IntS selection considers the fact that CIR is typically low frequency concentrated, or tradeoff between T’put and overhead through window-based union set. In our evaluation, N3’ is fixed for each case:

1) UE finds the best window with length of N3’ and starting point of M_inital;

2) A fixed length-N3’ restricted window corresponding to FD basis mod(-N3’/2:N3’/2,N3) ;

3) Length-N3’ IntS are selected freely;

4) Length-Ml FD basis are selected with per layer M_initial;

The method of free FD basis selection was used as performance reference in our evaluation. The evaluation results, as shown in following chart, show that adjacent IntS is only slight worse than (less than 1%) freely IntS selection. Reporting M_initial does not bring any benefit to throughput. Fixed selection shows obvious performance loss (about 2%).

On overhead, given a N3’ value,  the method of free IntS selection requires additional log2N3N3' bits to indicate IntS in comparison with fixed IntS selection, which ranges 0~10 bits for N3=13 and it is monotone decreasing as N3' increase to N3.

Hence we have the following observations and proposals:

Observation 3: For throughput, adjacent IntS selection slightly decrease performance (less than 1%); reporting M_initial does not bring benefit to performance; fixed selection sacrifice performance obviously (about 2%).

Observation 4: For overhead, free IntS selection require additional (less than 10) bits overhead and bring 1% gain compared with fixed IntS selection.

Proposal 5: Do not support fixed selection and do not support M_initial reporting.

Proposal 6: Support free IntS for two-step selection.

Another issue is to determine the exact bit width of FD basis indicator. For free selection: 1) log2N3Ml and 2) log2N3-1Ml-1 ; for two step selection 1) log2N3N3' or 2) log2N3-1N3'-1. Alternative 2) rely on cyclic shifting of the selected FD basis (union selected set) by UE implementation. log2N3-1M-1 reduced at most 1 bit compared with log2N3M. If the equivalence of selected FD basis set is built by cyclic shift operation, with the greatest effort, approximately log21N3N3M bits is needed result in log2N3  bits (3~4 bits in generally) reduction, it’s impractical for high complexity and low benefit. However, it still could do better than log2N3-1M-1. For example, in two step selection UE desire a highly overlapped FD basis across layer and may rotate FD basis of each layer, a simple implementation would be UE find the “necklace” of each layer by N3 rotations and comparisons. In this manner, LSB would always be 1 while MSB would always be 0 (more than one 0s is possible which is dependent on parameter p), hence at least  log2N3-2M-1 is achieved (2bits reduced).

Anyway, these shorter bitwidth highly depends on UE implementation and overhead reduction is marginal, preference for alternatives is not strong.

Proposal 7: For intermediate set indicator, support eitherlog2N3-1N3'-1 or size-N3-1 bitmap; for FD basis indicator, support either  log2N3'-1Ml-1 or size- N3'-1 bitmap.

Maximum NZC restriction per layer

It’s agreed maximum NZC across layers is limited by 2K0 and if there is a per-layer restriction is to be decided. Except that per-layer restriction would save 1 bit for SCI Alt 3.3,  we don’t see any reason to restrict maximum  K0 for KNZ,i .

Proposal 8: For # NZC per layer, support unrestricted KNZ,i as long as  (Alt0).

Conclusion

In this contribution, we analyze the overhead reduction of Type II CSI feedback with some system level evaluation results. Based on the analysis and evaluation, we have following observations and proposals:

Observation 1: Alt 2B outperforms Alt 3C/Alt 6E about 2% with additional 50 bits; Alt 3C/ Alt 6E show similar performance, and the gain of further optimization offered by Alt6E seems marginal.

Observation 2: For NZC indicator, Alt 1.1 reduce overhead obviously (e.g 12 bits); For new SCI indicator, Alt 3.2 and Alt 3.3 are expected to be similar, and Alt 3.4 can save 1~2 bit per layer in comparison with Alt 3.3;

Observation 3: For throughput, adjacent IntS selection slightly decrease performance (less than 1%); reporting M_initial does not bring benefit to performance; fixed selection sacrifice performance obviously (about 2%).

Observation 4: For overhead, free IntS selection require additional (less than 10) bits overhead and bring 1% gain compared with fixed IntS selection.

Proposal 1: For p parameter setting, support Alt3C

Alt6E is also acceptable as a comprise for better progress (second preference).

Proposal 2: Support fixed relation for RI3/4 of p setting if Alt 6E is supported.

Observation 2: For NZC indicator, Alt 1.1 reduce overhead obviously (e.g 12 bits); For new SCI indicator, Alt 3.2 and Alt 3.3 are expected to be similar, and Alt 3.4 can save 1~2 bit per layer in comparison with Alt 3.3;

Proposal 3: Support joint NZC indicator (Alt1.1) and a new   SCI indicator with log22L bits per layer (Alt 3.4).

Proposal 4: Support fixed N3’ setting, a fixed relationship between N3’ and the same single higher layer configured p parameter is preferred.

Proposal 5: Do not support fixed selection and do not support M_initial reporting.

Proposal 6: Support free IntS for two-step selection.

Proposal 7: For intermediate set indicator, support eitherlog2N3-1N3'-1 or size-N3-1 bitmap; for FD basis indicator, support either  log2N3'-1Ml-1 or size- N3'-1 bitmap.

Proposal 8: For # NZC per layer, support unrestricted KNZ,i as long as  (Alt0).

References

- RP-181453, WI Proposal on NR MIMO Enhancements, Samsung, RAN#80, June 2018

Appendix

| Parameter |  | Value |
| --- | --- | --- |
| Duplex |  | FDD |
| Waveform |  | OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Frequency Range |  | 4GHz. |
| Inter-BS distance |  | 200m |
| Layout |  | Hexagonal grid, 3 sectors per site, 19 macro sites, 570 UEs |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ for overhead reduction |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for overhead reduction 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank extension |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO with rank adaptation |
| maximum MU layers |  | 12 |
| CSI feedback |  | CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms Codebook coeff. quantization (Amplitude, phase )= (3bits,3bits) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 20% |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook for overhead reduction. |
| Overhead |  | 2 PDCCH symbols DMRS overhead: up to actually scheduled total layers 1 SSB per 20ms CSI-RS: 32ports, 5ms period, 1RE/port/RB CSI-IM: 4 REs/PRB, 5ms period TRS: 12 REs/PRB, 20ms period, maximal bandwidth with 52 PRB Total overhead: 24.24% |