3GPP TSG RAN WG1 Meeting #97                                                                       R1-1906340

Reno, USA, May 13th – 17th, 2019

Source:	CATT

Title:	Self-evaluation on User Experienced Data Rate in Dense Urban-eMBB for LTE and NR

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. At RAN#81 meeting, the preliminary self evaluation results of 5G are provided [2] and the evaluation results are captured in TR 37.910[3]. Under Dense Urban-eMBB, user experienced data rate evaluation is provided only for NR. Remaining open issues include [4]:

The evaluation results for eMBB spectral efficiency with ideal channel estimation need to be updated with realistic channel estimation. The LTE part evaluation is to be provided in the test environments not yet evaluated (i.e. other than Rural – eMBB and Urban Macro – mMTC). Further clarification of evaluation assumptions e.g. interference error modeling is needed.

In this document, the evaluation results of channel model B on the user experienced data rate in Dense Urban-eMBB based on Rel-15 LTE are supplemented based on our contribution [5] of meeting #96bis.

Evaluation methodology and assumption

According to the evaluation methodology of Report ITU-R M.2412 [6], the user experienced data rate can be calculated by 5th percentile user spectral efficiency SEuser through equation in [7] (this KPI is only applied to Dense Urban-eMBB).

Ruser = W × SEuser

According to the discussion in RNA1#94 meeting, for TDD mode, the bandwidth W for the calculation of user experienced data rate is effective bandwidth considering the TDD subframe configuration. For TDD subframe configuration in our evaluation, which is DSUUD, S (11D: 1GP: 2UL), the ratio of DL effective bandwidth to system bandwidth is 0.5643, the ratio of UL effective bandwidth to system bandwidth is 0.4357.

Evaluation results

Considering the various bandwidth at 4GHz, according to the evaluation results of the DL and UL 5th percentile user spectral efficiency of the Dense Urban-eMBB in our companion contribution [8], the preliminary evaluation results of user experienced data rate for LTE are provided in Table 1 and Table 2. According to the updated downlink evaluation results of 5th percentile user spectral efficiency for NR, the updated downlink user experienced data rate for NR are provided in Table 3.

In the perspective of RAN1, the maximum supported aggregation bandwidth for LTE is 640MHz, which is aggregated by 32 component carriers with 20MHz bandwidth. So for the preliminary evaluation of user experienced data rate in this contribution, the maximum bandwidth is 640MHz for both DL and UL for LTE.

Table 1 DL user experienced data rate for Dense Urban-eMBB for LTE

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment (Mbit/s) | SEuser (bit/s/Hz) | Band-width [MHz] | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) | 100 | 0.282 | 360 | 101.52 |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.293 | 360 | 105.48 |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.379 | 480 | 102.66 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.386 | 460 | 100.20 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.519 | 360 | 105.43 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.524 | 340 | 100.54 |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) |  | 0.282 | 360 | 101.52 |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.288 | 360 | 103.68 |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.378 | 480 | 102.39 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.380 | 480 | 102.93 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.504 | 360 | 102.39 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.523 | 340 | 100.34 |

Table 2 UL user experienced data rate for Dense Urban-eMBB for LTE

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment (Mbit/s) | SEuser (bit/s/Hz) | Band-width [MHz] | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) | 50 | 0.389 | 140 | 54.46 |
|  | TDD DSUUD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 0.302 | 380 | 50.00 |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 0.407 | 140 | 56.98 |
|  | TDD DSUUD | Codebook-based | 4T/32R, (16,8,2,1,1;2,8) |  | 0.320 | 360 | 50.19 |

Observation 1: The ITU requirement on DL and UL user experienced data rate can be met by LTE.

For NR, refer to TR 37.910[3], it is assumed that FDD DL and TDD DL with 15 kHz SCS, a component carrier with 40 MHz is used; for TDD DL with 30 kHz SCS, a component carrier with 100 MHz is used; for FDD UL with 15 kHz SCS, a component carrier with 40 MHz is used; for TDD UL with 15 kHz SCS, a component carrier with 50 MHz is used; and for TDD UL with 30 kHz SCS, a component carrier with 100 MHz is used. Multiple component carriers are aggregated to achieve the DL and UL target user experienced data rate. The evaluation results of DL user experienced data rate are based on the evaluation results of larger BW considering the overhead of PDCCH.

Table 3 Updated DL user experienced data rate for Dense Urban-eMBB for NR

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment (Mbit/s) | SEuser (bit/s/Hz) | Band-width [MHz] | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) | 100 | 0.517 | 200 | 103.40 |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.535 | 200 | 107.00 |
|  | TDD DSUUD 15KHz SCS | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.476 | 400 | 107.44 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.474 | 400 | 106.99 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.680 | 280 | 107.44 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.683 | 280 | 107.92 |
|  | TDD DSUUD 30KHz SCS | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.547 | 400 | 123.47 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.555 | 400 | 125.27 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.746 | 300 | 126.29 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.785 | 300 | 132.89 |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | Non-precoded CSI-RS based | 32T/4R, (8,8,2,1,1;2,8) |  | 0.523 | 200 | 104.60 |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.529 | 200 | 105.80 |
|  | TDD DSUUD 15KHz SCS | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.463 | 400 | 104.51 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.468 | 400 | 105.64 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.653 | 280 | 103.18 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.701 | 280 | 110.76 |
|  | TDD DSUUD 30KHz SCS | SRS and Non-precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.515 | 400 | 116.25 |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (16,8,2,1,1;2,8) |  | 0.515 | 400 | 116.25 |
|  |  |  | 64T/4R, (12,8,2,1,1;4,8) |  | 0.749 | 300 | 126.80 |
|  |  |  | 64T/4R, (16,8,2,1,1;4,8) |  | 0.799 | 300 | 135.26 |

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR.

Conclusion

In this contribution, the preliminary LTE evaluation results and updated NR downlink evaluation results on user experienced data rate of Dense Urban-eMBB are provided. There are the following observations:

Observation 1: The ITU requirement on DL and UL user experienced data rate can be met by LTE.

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson, Dubrovnik, Croatia, March 6 - 9, 2017.

  - RP-182097, “Draft LTI on 3GPP 5G Preliminary Description Template and Self-Evaluation”, 3GPP TSG RAN, Gold Coast, Australia, September 10 - 13, 2018.

  - 3GPP TR 37.910, “Study on Self Evaluation towards IMT-2020 Submission”, V1.0.0, 2018-09.

  - RP-182539，“Status Report to TSG”, Huawei, Sorrento, Italy, December 10-13, 2018.

  - R1-1904556, “Self-evaluation on User Experienced Data Rate in Dense Urban-eMBB for LTE and NR”, CATT, Xi’an, China, 8th – 12th April, 2019.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - R1-1906338, “Self-evaluation on Spectrum Efficiency in Dense Urban-eMBB for LTE and NR”, CATT, CATT, Reno, USA, May 13th – 17th, 2019.