3GPP TSG RAN WG1 Meeting #97		                                                   R1-1906348

Reno, USA, May 13th – 17th, 2019

Source:	CATT

Title:	Phase preprocessing for Type II CSI enhancement

Agenda Item:	7.2.8.5

Document for:	Discussion and Decision

Introduction

DFT-based compression in frequency domain (FD) has been agreed to reduce overhead of Type II CSI feedback [1], and been extended to higher rank [2]. In this contribution, in order to effectively compress FD subband coefficients, we propose a phase preprocessing algorithm before performing compression.

Phase preprocessing for subband coefficients

Calculation of subband coefficients and FD compression

In Rel-15 Type II codebook, the precoder can be written as W=W1W2 for each layer, where W1 denotes the wideband beam which is represented by a set of DFT basis, W2 denotes subband coefficients. W2 could be calculated in the following way. Let en denote the channel eigenvector of nth subband which is obtained by eigenvalue decomposition of channel covariance matrix. Then, the nth subband coefficients wn is the projection of en onto wideband beam W1, i.e., wn=W1Hen. Similarly, the other subband coefficients are also obtained, and these coefficients are cascaded to construct the coefficients matrix W2for all subbands.

In [1], it was agree that subband coefficients can be further compressed based on DFT basis by employing the correlation in frequency domain. The compression coefficients  W2 is calculated as  W2=W2Wf, where Wf denotes the FD compression basis consisting of M DFT basis vectors with length N, and N is the number of subbands. At gNB side,  W2 can be reconstructed as W2WfH. Then, the precoder can be written as W=W1 W2WfH for each layer. The reported coefficient is W2 instead of W2. Comparing with W2, W2 has less number of columns  in FD as M<N. Therefore, feedback overhead could be reduced.

Phase preprocessing algorithm

According to the description in section 2.1, the channel eigenvectors are calculated through eigenvalue decomposition, which may results in non-continuous phase coefficients among adjacent subbands. Therefore, the correlation of phase is destroyed that they cannot be efficiently compressed by utilizing FD compression basis. In order to improve the correlation of FD, a suitable phase preprocessing is necessary before performing the compression. If all phase coefficients of a subband are rotated with the same angle, the system performance will not be changed. Motivated by this, we make all phase coefficients of the nth subband minus an angle ∅n to do phase preprocessing for better compression.

Due to the propagation characteristic of wireless channel, there is at least a strongest beam for the whole wideband. In order to minimize performance loss, the compression energy associated with the strongest beam should be remained as much as possible after compression. This requires that the correlation of among subbands corresponding to the strongest beam should be kept with the highest priority. Therefore, ∅n should be equal to the phase coefficient corresponding to the strongest beam. Then, after performing the similar calculation for the other subbands, the phases associated with the strongest beam are equal to zero. The correlation of phase coefficients corresponding to the strongest beam is actually improved. In mathematics, the phase preprocessing algorithm can be represented as follows.

Let phases of all subband coefficients be denoted by a matrix  which is represented as

D=d1,1⋯d1,N⋮⋱⋮dl,1⋯dl,N⋮⋱⋮d2L,1⋯d2L,N,

where L is the number of beam.  Assume the lth beam is the strongest beam. All phases minus the lth row phase for each subband, i.e, ∅n=dl,n. We obtain an another phase matrix  which is written as

D'=d1,1-dl,1⋯d1,N-dl,N⋮⋱⋮0⋯0⋮⋱⋮d2L,1-dl,1⋯d2L,N-dl,N.

Then, subband coefficients  is calculated by using amplitude of W2 and , and FD compression are performed for .

In order to verify whether the phase preprocessing algorithm is feasible, we adopt the similar method as proposed in [3] to calculate a power ratio. In this contribution, the power ratio is defined as the preserved power after FD compression and the power of without FD compression. A larger power ratio means less energy loss after compression, and FD compression is effective after phase preprocessing as well. Otherwise, it cannot compress FD coefficients effectively. Fig.1 shows the CDF curves of power ratio with and without phase preprocessing. In this figure, ρ  denotes the power ratio, P and Pc denotes the power of without FD compression and the preserved power after FD compression, respectively. Comparing the two cases, most energy has been remained with phase preprocessing. This verifies that the FD coefficients could be compressed effectively by exploiting the proposed phase preprocessing before compression.

Figure 1: CDF curves of power ratio of the case with and without phase preprocessing

Observation-1: In order to compress FD coefficients effectively, it is necessary to perform phase preprocessing before compression.

Simulation results

In this section,  system-level simulation are adopted to evaluate the performance with phase preprocessing and without phase preprocessing. The number of beam L and the number of subband N are set to 4 and 13, respectively. The other simulation parameters are given in Table AI in Appendix. For the reported compression coefficients, both amplitude coefficients and phase coefficients are quantized with 3 bits.  As a performance baseline, Rel-15 Type II codebook using L=4 beams with SB amplitude and 8-PSK phase quantization is also evaluated.

Cell average and edge performance comparison of the case with and without phase preprocessing for different compression factors is shown in Fig.2. When compression factor M=13, the best performance is achieved since all subband coefficients are kept without any energy loss. As shown in the figure, when M=4, the performance with phase preprocessing is close to the uncompressed case. This further verifies that the proposed phase preprocessing can be used to effectively compress FD coefficients. Note that the reported coefficients can be reduced by 69.2% when M=4. Therefore, feedback overhead is significantly reduced. Comparing with the case without phase preprocessing, the performance of the case with phase preprocessing can be improved more than 10% when M=4. In addition, when M≥4, the performance is always better than that of Rel-15Type II codebook. While the performance of the case without phase preprocessing is always worse than that of Rel-15 Type II codebook except M=13.

Figure 2: Cell average and edge performance with different compression factors

Observation-2: The performance of the compressed codebook can be improved significantly with phase preprocessing. When compression factor M=4, the performance almost achieves the best one through using the proposed phasing preprocessing algorithm, and is even better than that of Rel-15 Type II codebook.

Conclusions

In this contribution we discussed the phase preprocessing before performing FD compression. Base on analysis and simulation results, our observations are summarized as follows.

Observations:

Observation-1: In order to compress FD coefficients effectively, it is necessary to perform phase preprocessing before compression.

Observation-2: The performance of the compressed codebook can be improved significantly with phase preprocessing. When compression factor M=4, the performance almost achieves the best one through using the proposed phasing preprocessing algorithm, and is even better than that of Rel-15 Type II codebook.

References

- Chairman's Notes RAN1#95, Spokane, USA, 12th November– 16th November 2018.

- Chairman's Notes RAN1#96, Athens, Greece, 25th February – 1st March 2019.

- R1-1903980, Huawei, HiSilicon, “Phase Randomization and Correction for CSI quantization in frequency domain”, Xi’an, China, April 8-12, 2019.

Appendix

Table AI: Evaluation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| Scenario | Dense Urban (Macro) |
| Carrier frequency | 4GHz |
| Channel model | According to the TR 38.901 |
| BS Tx power | 41dBm |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| BS antenna height | 25m |
| UE antenna configurations | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE receiver noise figure | 9 dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size=8448bits |
| Number of RBs | 52 RBs for 15 kHz SCS |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| UE distribution | 80% Indoor, 3km/h, 20% Outdoor, 30km/h |
| UE receiver type | MMSE and IRC |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| MIMO mode | MU-MIMO with rank adaption |
| CSI feedback period | 5ms |
| Feedback delay | 4ms |