3GPP TSG RAN WG1 #97                                            R1-1906428

Reno, USA, May 13th - 17th, 2019

Source:          ZTE, Fudan University

Title:             New sequence for NR PRS

Agenda item:     7.2.10.5

Document for:    Discussion and Decision

Introduction

In RAN#83, NR positioning WID was approved [1] to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning techniques, an agreement on sequence for NR PRS was made:

- “DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1

- FFS: cinit value for DL PRS

- QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM

- FFS: Whether a DL PRS sequence generated using a different mechanism is also specified.”

While agree on Gold sequence for Rel-16 PRS, we think the possibility for new sequence was not fully discussed, in this contribution we provide a new sequence that may have better performance for positioning usage for future study.

Discussion

Traditional way for PRS sequence is that it can be generated with the pseudo-random sequence from a length−31 Gold sequence. The output sequence c(n) of length MPN,where n=0,1,…,MPN-1, is defined by

cn=x1n+Nc+x2n+Ncmod2

x1n+31=x1n+3+x1nmod2

x2n+31=x1n+3+x2n+2 +x2n+1+x2nmod2

where Nc=1600 and the first m-sequence shall be initialized with x10=1,x1n=0, n=1,2,…,30. The initialization of second m-sequence is denoted by cinit=i=030x2i∙2i with the value depending on the application of the sequence.

cinit=228∙NIDPRS512+210∙7∙NS+1+L+1∙2∙NIDPRSmod512+1+2∙NIDPRSmod512+NCP

NIDPRS∈{0,1,…,4095} equals NIDcell and NCP=1 for normal CP0 for extended CP

There are various other methods of constructing the Gold sequence family. One of the typical constructions is to make use of the trace function. The idea works as follows. Let q=2n and let γ be a generator of the multiplicative group Fq*.

Then the Gold sequences are {-1Tra+b,-1Traγ+bγ3,…,-1Tr(aγq-2+bγq-23|a,b∈Fq}, where Tr is the trace function from Fq to F2.

To generalize the above idea to finite fields of odd characters, we consider the sequence {η1+a+b,ηγ2+aγ+b,ηα4+aγ2+b,…,ηα2q-2+aγq-2+ba,b∈Fq, where η is a quadratic multiplicative character from Fq to C*.

The pre-defined steps of the construction of Gold-like sequence are as follows:

- Let q be an odd prime power. Let η be a map from Fq to C* defined as

- ηα=1    if α is a nonzero square-1    if α is a non-square 0                                   if α=0

- Consider the set of monic quadratic polynomials

- S1≔x2+ax+b,a∈Fq*,b∈Fq\{x-a2:a∈Fq*}

- Define an equivalence relation in S1 as follows:

- x2+a1x+b1~x2+a2x+b2, if there exists β∈Fq* that a1=βa2 and b1=β2b2

- Define T1=x-1∪x2+ax+b∈S1 : x2+ax+b are pairwise distinct

Construction of Gold-like sequence:

- Fix a generator γ∈Fq* of Fq*;

- For each polynomial f(x) in T1 defined above, define

sf≔{ ηf1,ηfγ,…,ηfγq-2}

- The sequence family is defined as

F1≔{sf :f∈T1}

Properties of Gold-like sequence:

- The length of our Gold-like sequence is q-1, where q is an odd prime or odd prime power.

- The number of Gold-like sequence is q-1.

The PRS sequence based on Gold-like sequences is like the PRS sequence used in LTE, truncated from a very long Gold-like sequence.

Similar with LTE, to maximally randomize the inter-cell interference, or equivalently, minimize the cross-correlation among the PRSs from different cells, the initial value of the sequence should depend on the OFDM symbol index in a slot, the slot number within a radio frame, as well as the PRS ID.

Proposal 1.  The initialization of the random sequences for PRS generation should depend, at least, on the OFDM symbol index in a slot, the slot number within a radio frame, as well as the PRS ID.

Consider the maximum number of PRS ID in LTE is 4096, the symbol index in a slot and the slot number within a radio frame, we recommend to use q=573451 Gold-like sequence to cover all the three parameters to initialize the random sequences for PRS generation.

Proposal 2.  The initialization of the random sequences for PRS generation should use the Gold-like sequences which generated from F573451.

Because every polynomial fx=x2+ax+b decides a Gold-like sequence family, so we can use the method below to initialize the PRS signal.

a=1

b=NIDPRS*180+100⋅ ns/10+10⋅l+(ns mod 10)

NIDPRS is the PRS ID

l is the OFDM symbol number in the slot nsf within a radio frame

ns is the slot number within a radio frame

Compared with Gold sequence, our Gold-like sequence has the following two advantages:

- The length of Gold-like sequence is q-1 where q can be any odd prime or odd prime power. But the length of Gold sequence can only be in the form of 2n. This means that our Gold-like sequence provides more flexibility on the length. This fact has great significance when we need another specific length for generating PRS. For example, if we need 400 sequences of length of 400, we can just generate 400 PRS sequence based on Gold-like sequences of length 400 generated from F401.It has a better auto-correlation and cross-correlation property than PRS signal truncated from Gold sequence of length 29=512.

We randomly choose two PRS sequences based on Gold sequence and two PRS sequences based on Gold-like sequence.

|  | Auto-correlation of seq1 | Auto-correlation of seq1 | Cross-correlation |
| --- | --- | --- | --- |
| Gold sequence | 56 | 60 | 66 |
| Gold-like sequence | 40 | 40 | 38 |

Table 1 Correlation properties of Gold sequences and Gold-like sequences

- PRS sequence based on Gold-like sequences have better auto-correlation and better cross-correlation than PRS sequence based on Gold sequences. For example, to generate the required PRS in LTE. We randomly choose 3 groups of PRS sequence based on Gold sequences and 3 groups of PRS sequence based on Gold-like sequences. By simulation, we can see that both of the auto-correlation and cross-correlation of PRS sequences from Gold-like sequences is better than that from Gold sequences.

PRS sequences based on Gold-like sequences have much better max correlation property than that of PRS sequences based on Gold sequence.

Below we give two figures to show the auto-correlation and cross-correlation of PRS sequence of length 400 truncated from a long Gold-like sequence of length 573451.

Figure 1 auto-correlation of the Gold-like PRS signals

Figure 2 cross-correlation of the Gold-like PRS signals

Conclusion

Here is the list of proposals for this contribution:

Proposal 1: The initialization of the random sequences for PRS generation should depend, at least, on the OFDM symbol index in a slot, the slot number within a radio frame, as well as the PRS ID.

Proposal 2: The initialization of the random sequences for PRS generation should use the Gold-like sequences which generated from F573451.

References

3GPP RP-190752, “New WID: NR positioning support,” Shenzhen, China, 2019.”