3GPP TSG RAN WG1 meeting #97						 	R1-1906563

Reno, Nevada, USA, May 13 - 17, 2019

Agenda Item: 7.2.10.3

Source: MediaTek

Title: Measurement and reporting design for NR positioning

Document for:	Discussion and decision

# Introduction

Agreement:

- NR should support angle (DL-AoD) based DL-only positioning techniques with at least beam sweeping at least at the gNB in FR1 and FR2

  - Note: This does not necessarily need different reference signals from the timing based DL-only positioning techniques

In RAN1 AH-1901 meeting [1], it is agreed that DL-AoD based positioning techniques should be supported in NR:

This agreement has been also captured in TR 38.855 [2].

Agreement:

- A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID

  - The DL PRS Resources in a DL PRS Resource set are associated with the same TRP

- A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams)

- Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE.

In RAN#96b meeting [3], the following agreement is achieved:

In DL-AoD positioning, an UE measures multiple PRS beams transmitted from TRPs and report the measurement results to the network. The network estimates the UE’s position based on the measurement results reported by the UE.  In this contribution, we share our views on how UE performs measurement, and how UE may reduce the overhead of reporting measurement results.

# DL-AoD positioning framework and methods for reducing UE RSRP reporting overhead

The DL-AoD positioning may work as follows (see Figure 1 for an illustration):

Step 1: The network configures an UE to measure RSRP for several PRS resource sets

- Note: A PRS resource set is associated with a TRP

Step 2: Each TRP transmits PRS on time/frequency resources specified by PRS resources of a PRS resource set

- Note: A PRS resource is associated with a Tx beam and a PRS resource ID

Step 3: The UE measures PRS transmitted from TRPs and reports measurement results to the network

Step 4: The network estimates the AoDs based on the UE’s RSRP report.

Step 5: The location server estimates the UE’s position by using the estimated AoDs

Figure 1: an illustration of DL-AoD positioning framework

For limited resolution use cases (i.e., the requirement of positioning precision is relatively low), for a PRS resource set, it may be sufficient that the UE only reports the PRS resource ID corresponding to the PRS resource with the strongest measured RSRP.

Proposal 1: For high resolution DL-AoD based positioning, an UE may select a portion of all measured PRS resource sets and report only RSRPs measured from the selected PRS resource sets. A method for UE to select PRS resource sets is given as follows:

- The network configures the parameters T,kmax, kmin to an UE

- For an UE, let m be the number of cells with measured RSRP greater than T

  - If m≥kmax, then UE selects best (maximum power) kmax cells

  - If m<kmax, then UE selects best kmin cells

FFS: values of T,kmax, kmin

For high resolution use cases, to reduce the UE reporting overhead, we have the following proposals:

Note that under this proposal:

- A RSRP threshold T is configured by the network to a UE in order to help UE to rule out TRPs with poor measured RSRP values

- An UE is configured to select at most kmax or at least kmin PRS resource sets, and only report RSRP measured from theses selected PRS resource sets

- Furthermore, if kmax=kmin, then an UE simply select kmax best PRS resource sets for RSRP reporting. Thus this method also allows an UE to perform a simple selection of PRS resource sets.

Proposal 2: For high resolution DL-AoD based positioning, an UE may select a portion of PRS resources from a PRS resource set and only report RSRP measured from the selected PRS resources. A method for UE to select PRS resources is given as follows:

- The network configures a parameter k to the UE

- For a PRS resource set, the UE selects k PRS resources such that the RSRPs measured from these k PRS resources are larger than that measured from the other PRS resources

FFS: value range of k

Simulation results regarding proposal 1 is given in Appendix 1.

In other words, method in proposal 2 allows UE to select beams from all PRS beams transmitted from a cell. The idea of this method is that UE may select best k beams for RSRP reporting. Simulation results regarding proposal 2 is given in Appendix 2.

Proposal 3: For high resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured PRS resources of a PRS resource set. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

- Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

- Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

- Then UE reports the PRS resource ID m, but UE does not report pm

- For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

FFS: value range and quantization level of RSRP ratios or differential RSRPs

To further reduce UE RSRP reporting overhead and unify the reporting format, we have the following proposal:

In other words, for a cell, the method in proposal 3 allows an UE to report the beam index (PRS resource ID) of beam with maximum measured RSRP without reporting the maximum measured RSRP. For the other beams, the UE reports beam indices and RSRP ratios (or differential RSRPs) with respect to the beam with maximum measured RSRP.

# Other aspects on UE measurement

In the early stage of NR commercial deployment, it is likely that a deployment may contain mixed E-UTRAN and NR nodes. The positioning performance should not be compromised in such scenarios. Thus, we have the following proposal:

Proposal 4: NR supports RSTD measurements with mixed E-UTRAN and NR nodes

Furthermore, we propose the following definition of TOA:

Proposal 5: For a cell, TOA is the first TOA among the TOAs obtained from the individual DL beams within a PRS occasion

As such, for a cell, UE may combine measurement results from several DL beams to conclude a TOA, and there is no need to introduce beam specific RSTD.

Regarding number of ports for PRS-RSRP measurement, since RSRP measurement may be improved by TRP transmitting PRS on 2 ports corresponding to 2 different polarizations, we propose that

Proposal 6: Two ports can be used for PRS-RSRP measurement

Regarding requirements and parameters related to the measurement reporting, based on our simulation results on DL-AoD positioning (see Appendix 1 and 2), we have the following proposal:

Proposal 7: For DL-AoD, it is sufficient to use the following configurations:

- UE measures at most 8 cells

- For each cell, UE measures at most 8 beams

- For each cell, UE reports at most 8 beam RSRPs

Regarding RTT measurement, we note that in general measuring RTT for multiple cells may improve positioning accuracy. If there is only one serving cell, the positioning error may be large in the case UE-cell link is NLOS.

Considering the TA information, serving cells should be used for Rx-Tx time difference measurement with higher priority. Neighbour cells can also be supported, and the network may configure UE to do Rx-Tx time difference measurement for neighbour cells if the number of serving cells is not enough. Thus, we have the following proposal:

Proposal 8: NR supports UE Rx – Tx time difference measurements for serving and neighbour cell(s) in Rel-16

# Conclusions

The proposals are summarized below:

Proposal 1: For high resolution DL-AoD based positioning, an UE may select a portion of all measured PRS resource sets and report only RSRPs measured from the selected PRS resource sets. A method for UE to select PRS resource sets is given as follows:

- The network configures the parameters T,kmax, kmin to an UE

- For an UE, let m be the number of cells with measured RSRP greater than T

  - If m≥kmax, then UE selects best (maximum power) kmax cells

  - If m<kmax, then UE selects best kmin cells

FFS: values of T,kmax, kmin

Proposal 2: For high resolution DL-AoD based positioning, an UE may select a portion of PRS resources from a PRS resource set and only report RSRP measured from the selected PRS resources. A method for UE to select PRS resources is given as follows:

- The network configures a parameter k to the UE

- For a PRS resource set, the UE selects k PRS resources such that the RSRPs measured from these k PRS resources are larger than that measured from the other PRS resources

FFS: value range of k

Proposal 3: For high resolution DL-AoD based positioning, an UE may report RSRP ratios or differential RSRPs derived from measured PRS resources of a PRS resource set. A method for UE to report RSRP ratios or differential RSRPs is given as follows:

- Suppose pk is the RSRP measured from PRS resource with ID k, k = 1,2,…,N

- Suppose pm is a maximum RSRP, i.e.,  pm≥pk  for k = 1,2,…,N

- Then UE reports the PRS resource ID m, but UE does not report pm

- For PRS resource with ID j, where j≠m, UE reports j and xj to the network, where xj=pj/pm or xj=10logpm-10logpj

FFS: value range and quantization level of RSRP ratios or differential RSRPs

Proposal 4: NR supports RSTD measurements with mixed E-UTRAN and NR nodes

Proposal 5: For a cell, TOA is the first TOA among the TOAs obtained from the individual DL beams within a PRS occasion

Proposal 6: Two ports can be used for PRS-RSRP measurement

Proposal 7: For DL-AoD, it is sufficient to use the following configurations:

- UE measures at most 8 cells

- For each cell, UE measures at most 8 beams

- For each cell, UE reports at most 8 beam RSRPs

Proposal 8: NR supports UE Rx – Tx time difference measurements for serving and neighbour cell(s) in Rel-16

# Reference

- Chairman's Notes RAN1 AdHoc 1901

- R1-1903824 TR 38.855

- Chairman's Notes RAN1#96bis

# Appendix 1

Consider the UMi FR1 scenario. In our simulation, UEs are configured to measure 7 cells (a cell is associated with a PRS resource set) for which UEs are in their 1200 cell coverage. See Figure 2 for an illustration, in which UEs dropped in cell 0 are configured to measure signals transmitted from cell 0, 3, 8, 11, 13, 16, 18.

Figure 2: An illustration for the setting that UEs are configured to measure 7 cells for which UEs are in their 1200 cell coverage

Suppose all RSRP measurement results reported by an UE are used for estimating the UE position. In Figure 3, we compare the performance obtained by assuming two different UE behaviour (detailed simulation settings and positioning techniques are given in the appendix 3):

- UE selects all 7 PRS resource sets

- UE selects best 4 PRS resource sets (PRS resource sets with maximum measured beam RSRPs)

Figure 3: For UMI FR1, performance comparison between UE selecting 4 or 7 PRS resource sets

From Figure 3 we see that UE selecting 4 best PRS resource sets for RSRP reporting dose not degrade positioning performance.

Now consider the office FR2 scenario. Again in our simulation, UEs are configured to measure 7 nearest cells (a cell is associated with a PRS resource set) for which UEs are in their 1200 cell coverage.

In Figure 4, we compare the performance by assuming two different UE behaviour as mentioned above:

Figure 4: For office FR2, performance comparison between UE selecting 4 or 7 PRS resource sets

From figure 4, we see that UE selecting 4 best PRS resource sets for RSRP reporting achieves much better performance. This is because by UE’s selection, the PRS resource sets corresponding to cells that are more close to UEs are selected with higher probability.

Consider again the office FR2 scenario, but assume each UE is equipped with 1 non-omni-direction Rx antenna element. In the following figure, we compare the positioning performance obtained by using the following 3 methods for UE to select cells for RSRP reporting:

- M1: UE selects best (maximum RSRP) 4 cells

- M2: UE selects best 2 cells

- M3: For an UE, let m be the number of cells with measured RSRP greater than T=0 dB

  - If m≥4, then UE selects best 4 cells

  - If m<4, then UE selects best 3 cells

For a selected cell, an UE selects all PRS resources (beams) and reports RSRPs measured from the selected PRS resources to the network.

Figure 5: Office FR2, performance obtained by using the different methods for UE to select cells for RSRP reporting under the assumption that UE is equipped with one omni-directional Rx AE

Note that the reporting overhead of M3 is not greater than M1. From Figure 5, we see that M3 has better performance than the other two methods, and the positioning accuracy is less than 4.4m for 80% of UEs.

# Appendix 2

Consider the UMi FR1 scenario. Suppose a cell is associated with a PRS resource set consisting of 8 PRS resources, where a PRS resource corresponds to a Tx beam (i.e., every cell transmits PRS with 8 beams). Assume that an UE selects 4 best PRS resource sets for RSRP reporting as described in Appendix 1.

In Figure 5, we compare the performance obtained by assuming the following two different UE behaviour:

- UE selects all PRS resources of a PRS resource set

- UE selects a portion of measured PRS resources of a PRS resources set

Figure 6: For UMI FR1, performance comparison between UE selecting 8 and 6 PRS resources of a PRS resource set

From Figure 6, we see that by UE selecting 6 from 8 PRS resources, the performance is degraded but still meet the regulatory requirement.

Now consider office FR2 scenario. The simulation result is shown in Figure 7:

Figure 7: For office FR2, performance comparison between UE selecting 8 and 4 PRS resources of a PRS resource set

From Figure 7, we see that by UE selecting 4 from 8 PRS resources, the performance is degraded but still meet the commercial requirement.

# Appendix 3

Simulation settings for UMa, UMi and office scenario are given below:

| Parameter | UMi & UMa, FR1 | Office, FR2 |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |
| Carrier frequency | 4Gz | 30GHz |
| Subcarrier spacing | 15kHz | 60kHz |
| Reference Signal Transmission Bandwidth | 20MHz | 36MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 | In a two-symbol PRS occasion: 1st symbol is comb-4 starting at RE 0 2nd symbol is comb-4 starting at RE 2 |
| Reference signal (type of sequence, number of ports, …) | PN sequence, 1 port | PN sequence, 1 port |
| Number of sites | 7 | 12 |
| Number of symbols used per occasion | 2 | 2 |
| number of occasions used per positioning estimate | 10 | 10 |
| Power-boosting level | 6.02 dB | 6.02 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | RSRP = average power per RE | RSRP = average power per RE |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured RSRP ratios of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator | AoD estimation: Cell beam pattern is known in advance, so a look-up table (LUT) saving the AoD and power of each beam can be constructed. UE reports the measured RSRP ratios of beams of a TP to the network AoDs are estimated by using the LUT and RSRP reported from UE Transforming AoD estimation results to UE location: Least-square estimator |
| Network synchronization assumptions | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero | Each cell performs beamsweeping with LCS α angle = [-52.5 -37.5 -22.5 -7.5 7.5 22.5 37.5 52.5] degrees. Note these angles are equally 15-degree spaced and are centered at zero |
| Precoding assumptions | N/A | N/A |
| Additional notes, if any | UE is equipped with one omni-directional antenna element | Unless otherwise specified, UE is equipped with one omni-directional antenna element |