3GPP TSG RAN WG1 #97	R1-1906793

May 13 – May 17, 2019

Reno, USA

Agenda item:	7.2.3.4

Source: 	Intel Corporation

Title: 	Mechanisms to Support the Case #1 OTA Timing Alignment

Document for:	Discussion and decision

# Introduction

RAN1#AH1901 and RAN1#96 have made the following agreements for IAB Timing.

  - An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

- T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

- TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

RAN1#96bis has further made the following agreements.

- In order to align the DL TX timing of the IAB node with the DL TX timing of the parent node by setting DL TX timing of the IAB node (TA/2 + T_delta) ahead of its DL Rx timing, T_delta should be set to the (-1/2) of time interval at the parent node between the start of UL RX frame i for the IAB node and the start of DL TX frame i.

- The setting of T_delta is not necessarily specified.

- Note: The above setting of T_delta assumes that, for the same purpose, TA should be the time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i.

- Send LS to RAN4 for timing clarification. (Xinghua, Huawei) final LS in R1-1905842.

- In case the calculated TA/2 + T_delta at IAB node is negative, the IAB node should not adjust its DL-Tx timing.

In this contribution, based on those agreements and the WI scope for derivation of DL timing at a given IAB node based on OTA synchronization to parent node when operating with Case #1 timing, we further elaborate on the timing and synchronization for NR IAB Case #1.

# Observations/Clarification on DL TX/UL RX Misalignment in an IAB Parent Node

We re-draw the figure regarding the alignment of DL TX and UL RX timing at a parent IAB in LS R1-1905842 [1].

Figure 1: add one notation Tx to the figure in LS R1-1905842 [1]

We add a notation of Tx in Figure 1 to describe the extra/floating offset at the parent node, i.e. when the switching gap Tg is not equal to the fixed offset NTA_offset×Tc and

Tg=NTA_offset×Tc+Tx                                   (1)

Then, the TA at the IAB node which is the timing gap between UL TX timing and DL Rx timing, can be expressed as

TA=2×Tp+Tg

TA=2×Tp+NTA_offset×Tc+Tx                   (2)

It can be directly observed from Eq. (2) that TA is related to propagation delay (Tp), fixed offset (NTA_offset×Tc) and floating offset (Tx).

In the meantime, since  Tp=TA/2+Tdelta and Tp=(TA-Tg)/2, we can derive that

Tdelta=-Tg2=-12(NTA_offset×Tc+Tx)           (3)

It can be directly observed from Eq. (3) that Tdelta is related to fixed offset (NTA_offset×Tc) and floating offset (Tx).

Hence, since both TA and  Tdelta have the Tx component, they will change their values whenever there is  Tx update. In the meantime, whenever there is an update in Tdelta, which indicates there is actually an update in Tx, there will also be an update in TA since TA also has the Tx component. On the other hand, if there is an update in TA, which can be caused by Tp update or  Tx update, there may be  Tdelta change (if TA update is caused by Tx) or not (if TA update is caused by Tp).

Observation 1: TA is related to propagation delay (Tp), fixed offset (NTA_offset×Tc) and floating offset (Tx).

Observation 2: Tdelta is related to fixed offset (NTA_offset×Tc) and floating offset (Tx).

Observation 3: The relation of TA change and  Tdelta change is as follows:

- Whenever there is an update in the floating offset Tx, both TA and Tdelta will change their values.

- Whenever there is an update in Tdelta, there will also be an update in TA.

- Whenever there is an update in TA, there may be  Tdelta change (if TA update is caused by Tx) or not (if TA update is caused by Tp).

# Discussion on inconsistent TA and T_delta

Since both TA and  Tdelta have the Tx component, they will change their values whenever there is  Tx update. However, if the parent node does not signal the new TA value or the new  Tdelta value in a timely manner, or there exists delay between receiving Tdelta and application of Tdelta at an IAB node, there can be the scenario that the TA value and the Tdelta value at an IAB node are inconsistent with each other, which means they are corresponding to different  Tx values (old  Tx and new  Tx at the parent node).

For example, at an IAB node, the TA corresponds to a  Tx value and the  Tdelta corresponds to a different  Tx' value. Then with TA=2×Tp+NTA_offset×Tc+Tx and Tdelta=-12(NTA_offset×Tc+Tx')  we have the corresponding DL TX timing at the IAB node as

TA/2+Tdelta=Tp+(Tx-Tx')2

=Tp+∆Tx2                       (4)

We can see that with inconsistent TA and Tdelta, the DL TX timing at an IAB node will shift ∆Tx2 offset from the exact DL TX timing Tp, where ∆Tx is the difference between the Tx values in TA and Tdelta.

There can be several alternative solutions to avoid the DL TX timing offset due to the inconsistent TA and Tdelta. Firstly, TA and Tdelta can be considered to be signaled together, for example, within the same MAC CE as the TA signaling (new fields extended to accommodate Tdelta), or with different MAC CEs but within the same MAC PDU. Secondly, we can confine the TA and Tdeltatransmission within a limited timing window. Thirdly, we can also add a non- Tdelta-update indication together with the TA signaling to easily decide whether  Tdelta update should be addressed or not.

Then, whenever a TA update is received, before the IAB node re-calculates/updates its DL TX timing, it needs to make sure there is further update on  Tdelta or not with the alternative solutions discussed above. Whenever a Tdelta update received, there must be an update in TA, and the IAB node needs to make sure to re-calculate/updates it DL TX timing with both updated  Tdelta and updated TA.

If the ∆Tx value is very small and the effect of DL TX timing offset may be negligible, then the inconsistent issue can also be left to implementation.

Observation 4: With inconsistent TA and Tdelta, the DL TX timing at an IAB node will shift ∆Tx2 offset from the exact DL TX timing Tp, where ∆Tx is the difference between the Tx values in TA and Tdelta.

Proposal 1: The inconsistent TA and Tdelta issue (which means they are corresponding to different  Tx values) needs to be addressed.

Proposal 2: There can be several alternative solutions for the inconsistent TA and Tdelta issue:

- Alt 1: TA and Tdelta are signaled together.

- Alt 2: confine the TA and Tdeltatransmission within a limited timing window.

- Alt 3: add a non- Tdelta-update indication together with the TA signaling.

- Alt 4: Left to implementation.

# Signalling of T_delta

As it has been agreed that  Tdelta needs to be indicated from the parent node, similar methods can be applied as the current TA signalling, i.e. carried over MAC RAR during initial access and MAC CE in connected mode. Those methods have the benefit that the  Tdelta information is transmitted together with the TA commands in current specifications, so that it is easy to define and locate. Note that those methods are UE-specific, either during the random access of a child IAB node, or MAC CE TA command to individual child IAB node in connected mode.

In the case of the  Tdelta is common to all child nodes, i.e. the UL Rx timing at an IAB node is aligned for all its children, the Tdelta information can be broadcast to the child IAB nodes/child UEs, through MIB/SIB1 or group common PDCCH.

Proposal 3: The Tdelta information can be signalled from an IAB node to its child nodes in the following manner:

- The Tdelta information can be signalled with similar methods as the current TA signaling, i.e. carried over MAC RAR during initial access and MAC CE in connected mode.

- If the Tdelta information is common for all the child IAB nodes, it can be broadcast to the child IAB nodes.

# Conclusion

In this contribution, we discussed timing and synchronization for NR IAB Case #1. It is summarized by the following observations and proposals.

Observation 1: TA is related to propagation delay (Tp), fixed offset (NTA_offset×Tc) and floating offset (Tx).

Observation 2: Tdelta is related to fixed offset (NTA_offset×Tc) and floating offset (Tx).

Observation 3: The relation of TA change and  Tdelta change is as follows:

- Whenever there is an update in the floating offset Tx, both TA and Tdelta will change their values.

- Whenever there is an update in Tdelta, there will also be an update in TA.

- Whenever there is an update in TA, there may be  Tdelta change (if TA update is caused by Tx) or not (if TA update is caused by Tp).

Observation 4: With inconsistent TA and Tdelta, the DL TX timing at an IAB node will shift ∆Tx2 offset from the exact DL TX timing Tp, where ∆Tx is the difference between the Tx values in TA and Tdelta.

Proposal 1: The inconsistent TA and Tdelta issue (which means they are corresponding to different  Tx values) needs to be addressed.

Proposal 2: There can be several alternative solutions for the inconsistent TA and Tdelta issue:

- Alt 1: TA and Tdelta are signaled together.

- Alt 2: confine the TA and Tdeltatransmission within a limited timing window.

- Alt 3: add a non- Tdelta-update indication together with the TA signaling.

- Alt 4: Left to implementation.

Proposal 3: The Tdelta information can be signalled from an IAB node to its child nodes in the following manner:

- The Tdelta information can be signalled with similar methods as the current TA signaling, i.e. carried over MAC RAR during initial access and MAC CE in connected mode.

- If the Tdelta information is common for all the child IAB nodes, it can be broadcast to the child IAB nodes.

# References

[1] R1-1905842, "LS on clarification of OTA timing alignment for IAB".