3GPP TSG RAN WG1 #97	R1-1906853

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.10.2

Source: 	Sony

Title:	Considerations on UL reference signals for NR positioning

Document for:	Discussion / Decision

# Introduction

In RAN plenary #83, a new work item “NR positioning support” was approved [1] . One of the work item objectives are [1] :

Specify NR DL and UL reference signals to facilitate support of NR positioning techniques (DL-TDOA, DL-AoD, UL-TDOA, UL-AoA, multi-cell RTT and E-CID) [RAN1]

  - Support E-CID downlink measurements based on at least RRM measurements defined in NR Rel. 15

  - Identify whether and which Rel-15 NR reference signals can be used for different NR positioning techniques

  - Define new DL positioning reference signals applicable at least for DL-TDOA, DL-AoD, RTT

  - Define UL SRS with possible enhancements for positioning which is applicable at least for RTT, UL-TDOA, UL-AoA

# In RAN1 meeting #96bis, the following agreements were made:

Agreement:

- Support the following configurations of SRS for positioning

- Semi-persistent configuration

- Periodic configuration

- Aperiodic configuration

- For positioning, regarding the number of SRS symbols per resource, decide by RAN1#97 whether to increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4}.

- If it is not decided to increase the number of SRS symbols per resource by RAN1#97, increasing the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4} will not be considered further after RAN1#97.

- For positioning, regarding the possible SRS symbol locations per slot decide by RAN1#97 whether the SRS can be configured only in the last N symbols of a slot with N>6

- If the above decision is not made by RAN1#97, configuring SRS only in the last N symbols of a slot with N>6 will not be discussed further after RAN1#97.

- Select one or both of the following options to support staggered SRS transmissions for UL SRS positioning

- Staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource

- Configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set.

# In this contribution, we discuss our view on uplink reference signals to support NR positioning, particularly on SRS enhancements for uplink-based positioning.

# Uplink SRS Enhancements

It is known that in 5G NR, especially in FR2, both gNB and UE use a directional (narrow) beam. In legacy NR, the UE and its serving gNB typically could maintain a proper beam direction. Uplink based positioning in NR would still require multi-lateration of received gNBs for UE positioning estimation. Furthermore, the obtained angular information can be used to improve the positioning accuracy. However, the existing 5G NR only specify serving gNB to receive the SRS transmission. The specification has not defined the controlling of neighbor gNB Rx beam for SRS reception of a selected UE. Thus, the neighbor gNBs may not be able to perform positioning measurement. Furthermore, UTDOA with multi-lateration may not be performed at LS as the LS may only receive the positioning measurement from the serving gNB. In FR1, the UE may be equipped with an omnidirectional antenna and thus the beam-alignment requirement may not be as stringent as in FR2. However, the gNB may still be equipped with directional antenna. Thus, the beam coordination in both FR1 and FR2 are required.

Observation 1: Beam coordination between UE transmit beam and one or more gNB(s) receive beam is essential in uplink-based NR positioning.

The beam coordination for uplink-based positioning can be facilitated based on the downlink measurement operation at the UE. NR downlink measurement mechanism has been established/defined in NR rel.15. It is typically performed by the UE to measure mainly the channel quality (e.g. RSRP) of serving gNB. The RSRP measurement of neighbor cell is only performed if the RSRP measurement of serving cell is deteriorated. For positioning purpose, the UE needs to perform measurement based on the received downlink reference signals from multiple gNBs. This measurement may affect the uplink SRS configuration, such as SRS transmit power. In case the UE needs to reach at least 3 gNBs, the maximum transmit power can be based on the selected gNB where the measured RSRP is as the third-largest according to the list of measured RSRP values from the neighbor gNBs.

Proposal 1: Define a new power control procedure of UL SRS for positioning.

Proposal 2: The transmit power of uplink positioning reference signal (e.g. SRS) is determined based on the downlink measurement information of one or more gNBs.

The UE measures RSRP from each surrounding gNBs within a predefined interval and duration. Moreover, the UE can also collect information on the best gNB transmit beam. Based on the downlink measurement in NR rel.15, the UE is basically has collected information related to the surrounding cells, such as channel quality and beam direction. Furthermore, this information can be used by the UE with minimum effort for the uplink-based positioning, particularly for beam coordination purpose. The only additional effort is the UE transmits the measurement report in preparation for configuring the SRS transmission for positioning purpose.

Proposal 3: The uplink positioning reference signal (e.g. SRS) beam configuration are determined based on the UE reports on selected downlink measurement information.

NR UE may not always support beam correspondence. In this case, the uplink beam configuration may not completely rely on the received beam configuration. In such condition, the transmission of SRS with beam sweeping operation can be supported.

Proposal 4: SRS transmission with UL beam sweeping is supported, particularly when the UE does not support beam correspondence.

One of the supported SRS configurations for positioning is the aperiodic configuration. Here, we consider the aperiodic SRS transmission can be triggered in conjunction with periodic SRS transmission or periodic DL PRS transmission. This is basically to support on-demand positioning where it may require UE positioning estimation with higher accuracy.

Proposal 5: Aperiodic SRS transmission can be used to support on-demand positioning.

# Conclusion

In this contribution, we discussed our view on UL SRS enhancements for NR positioning. Our proposals are listed below:

Observation 1: Beam coordination between UE transmit beam and one or more gNB(s) receive beam is essential in uplink-based NR positioning.

Proposal 1: Define a new power control procedure of UL SRS for positioning.

Proposal 2: SRS transmission with UL beam sweeping is supported, particularly when the UE does not support beam correspondence.

Proposal 3: The transmit power of UL SRS is determined based on the downlink measurement information of one or more gNBs.

Proposal 4: The beam configuration of UL SRS is determined based on the UE reports on selected downlink measurement information.

Proposal 5: Aperiodic SRS transmission can be used to support on-demand positioning.

# References

- RP-190752, “New WID: NR Positioning Support”

- Chairman notes RAN1 Meeting 96bis.