3GPP TSG RAN WG1 #97         		          R1-1906885

Reno, USA, 13th-17th May 2019

Source:	China Telecom.

Title:	Updated results of rural-eMBB for NR and LTE in TDD

Agenda Item:	7.2.7.2

Document for: 	Discussion

Introduction

At RAN#81 meeting in September 2018, the updated 3GPP 5G preliminary description template with self-evaluation results was agreed to be sent to ITU-R WP5D#31. According to 3GPP work plan on IMT-2020 self-evaluation, the study will continue after June 2018 to aim for the final submission at RAN#84 that is before ITU-R WP 5D#32 (July 2019), the deadline of IMT-2020 proposal submission in ITU-R [1].

The minimum requirement for 5th percentile user spectral efficiency and average spectral efficiency for Rural-eMBB scenario is provided as follow [2].

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Rural – eMBB | 0.12 | 0.045 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Rural – eMBB | 3.3 | 1.6 |

Since the realistic channel estimation is required from ITU-R evaluation guideline, the updated evaluation results on DL spectral efficiency for both NR and LTE in TDD were provided based on the agreed technical features and system configurations in [3].

# TDD downlink evaluation results

NR downlink evaluation results:

In this section, we provide our updated downlink results of NR Rural-eMBB Config A/B/C in TDD. The detailed evaluation assumptions can be found in Annex.

The evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency are summarized in Table 3 and Table 4

TABLE 3

NR average spectral efficiency

| Test environment | TxRU configuration | Element configuration | Requirement (bit/s/Hz/TRxP) | Channel model | Average SEavg (bit/s /Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8Tx/2Rx | 64Tx/2Rx | 3.3 | A | 6.05 | 83.4% |
|  |  |  |  | B | 6.59 | 99.8% |
| Rural – eMBB Config B | 32Tx/4Rx | 128Tx/4Rx | 3.3 | A | 13.79 | 317.9% |
|  |  |  |  | B | 14.04 | 325.3% |
| Rural – eMBB Config C | 8Tx/4Rx | 64Tx/4Rx | 3.3 | A | 8.12 | 145.9% |
|  |  |  |  | B | 8.32 | 152.1% |

TABLE 4

NR 5th percentile user spectral efficiency

| Test environment | TxRU configuration | Element configuration | Requirement (bit/s/Hz/TRxP) | Channel model | Average SEavg (bit/s /Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8Tx/2Rx | 64Tx/2Rx | 0.12 | A | 0.16 | 29.3% |
|  |  |  |  | B | 0.20 | 68.7% |
| Rural – eMBB Config B | 32Tx/4Rx | 128Tx/4Rx | 0.12 | A | 0.63 | 427.0% |
|  |  |  |  | B | 0.47 | 292.8% |
| Rural – eMBB Config C | 8Tx/4Rx | 64Tx/4Rx | 0.12 | A | 0.34 | 185.2% |
|  |  |  |  | B | 0.36 | 200.4% |

Based on above evaluation results, we have following observation.

Observation 1: In TDD, the minimum requirement for DL average spectral efficiency and DL 5th percentile user spectral efficiency can be met for NR Rural-eMBB configuration A, B and C.

LTE downlink evaluation results:

In this section, we provide our updated downlink results of LTE Rural-eMBB Config A/B/C in TDD. The detailed evaluation assumptions can be found in Annex.

The evaluation results of DL average spectral efficiency and DL 5th percentile spectral efficiency are summarized in Table 5 and Table 6.

TABLE 5

LTE average spectral efficiency

| Test environment | TxRU configuration | Element configuration | Requirement (bit/s/Hz/TRxP) | Channel model | Average SEavg (bit/s /Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8Tx/2Rx | 64Tx/2Rx | 3.3 | A | 5.10 | 54.5% |
|  |  |  |  | B | 5.63 | 70.6% |
| Rural – eMBB Config B | 32Tx/4Rx | 128Tx/4Rx | 3.3 | A | 11.81 | 257.8% |
|  |  |  |  | B | 12.47 | 277.9% |
| Rural – eMBB Config C | 8Tx/4Rx | 64Tx/4Rx | 3.3 | A | 6.99 | 111.8% |
|  |  |  |  | B | 6.81 | 106.4% |

TABLE 4

LTE 5th percentile user spectral efficiency

| Test environment | TxRU configuration | Element configuration | Requirement (bit/s/Hz/TRxP) | Channel model | Average SEavg (bit/s /Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural – eMBB Config A | 8Tx/2Rx | 64Tx/2Rx | 0.12 | A | 0.18 | 50% |
|  |  |  |  | B | 0.19 | 58.3% |
| Rural – eMBB Config B | 32Tx/4Rx | 128Tx/4Rx | 0.12 | A | 0.46 | 286.1% |
|  |  |  |  | B | 0.39 | 229.1% |
| Rural – eMBB Config C | 8Tx/4Rx | 64Tx/4Rx | 0.12 | A | 0.31 | 158.3% |
|  |  |  |  | B | 0.28 | 133.3% |

Based on above evaluation results, we have following observation.

Observation 2: In TDD, the minimum requirement for DL average spectral efficiency and DL 5th percentile user spectral efficiency can be met for LTE Rural-eMBB configuration A, B and C.

# Conclusion

In this contribution, we provide our updated downlink evaluation results for Rural-eMBB. The observations are summarized as follow:

Observation 1: In TDD, the minimum requirement for DL average spectral efficiency and DL 5th percentile user spectral efficiency can be met for Rural-eMBB configuration A, B and C.

Observation 2: In TDD, the minimum requirement for DL average spectral efficiency and DL 5th percentile user spectral efficiency can be met for LTE Rural-eMBB configuration A, B and C.

# References

- RP-181318, “Revised SID: Study on Self-Evaluation towards IMT-2020 submission”, Huawei, Ericsson, Telecom Italia, June 2018.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, November. 2017.

- R1-1803386, “Summary of offline discussion for IMT-2020 self-evaluation”, Huawei, RAN1#92, March 2018.

# Annex: Evaluation assumptions

In this contribution, the downlink evaluation assumptions of Rural-eMBB test environment for NR and LTE are provided in the following table.

Table A-1 TDD DL evaluation assumption for Rural-eMBB

| Rural - eMBB | Config. A | Config. B | Config. C (LMLC) |
| --- | --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 4 GHz | 700 MHz |
| Slot structure | DDDSU | DDDSU | DDDSU |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 1732 m | 1732 m | 6000 m |
| Number of antenna elements per TRxP | 64 Tx/Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 128Tx/Rx=(M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 64 Tx/Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU=(Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 32TXRU=(Mp,Np,P,Mg,Ng) = (1,16,2,1,1) | 8TXRU=(Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx = (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°,90° polarization | 4Tx/Rx = (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°,90° polarization | 4Tx/Rx = (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, 0.5)λ 0°,90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 4TXRU (1-to-1 mapping) | 4TXRU (1-to-1 mapping) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 40% indoor, 40% outdoor (pedestrian), 20% outdoor (in-car) Randomly and uniformly distributed over the area |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (pedestrian): 3 km/h; Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 | See Table 9 in Report ITU-R M.2412 | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | TDD: 10 MHz | TDD: 10 MHz | TDD: 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channel model variant | Channel model A&B | Channel model A&B | Channel model A&B |
| TRxP number per site | 3 | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [100°] in LCS | [100°] in LCS | [96°] in LCS |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Modulation | For NR: up to 256QAM For LTE: up to 64QAM | For NR: up to 256QAM For LTE: up to 64QAM | For NR: up to 256QAM For LTE: up to 64QAM |
| Coding on PDSCH | For NR: LDPC with BP decoding For LTE: Turbo | For NR: LDPC with BP decoding For LTE: Turbo | For NR: LDPC with BP decoding For LTE: Turbo |
| Scheduling | closed SU/MU-MIMO adaptation | closed SU/MU-MIMO adaptation | closed SU/MU-MIMO adaptation |
| MIMO mode | MU-MIMO For NR: maximum MU layer = 8 For LTE: maximum MU layer = 4 | MU-MIMO For NR: maximum MU layer =12 For LTE: maximum MU layer = 8 | MU-MIMO For NR: maximum MU layer = 8 For LTE: maximum MU layer = 4 |
| Receiver | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI measurement | Non-precoded CSI-RS based | Non-precoded CSI-RS based | Non-precoded CSI-RS based |
| CSI method | TDD: SRS based with 5 slots period | TDD: SRS based with 5 slots period | TDD: SRS based with 5 slots period |
| CSI feedback | Every 5 slots Subband based | Every 5 slots Subband based | Every 5 slots Subband based |
| HARQ ACK delay | For NR: the next available UL slot For LTE: N+4 | For NR: the next available UL slot For LTE: N+4 | For NR: the next available UL slot For LTE: N+4 |
| HARQ retransmission delay | The next available DL slot after receiving NACK | The next available DL slot after receiving NACK | The next available DL slot after receiving NACK |
| Channel estimation | Non-ideal | Non-ideal | Non-ideal |
| Overhead | For NR: SS/PBCH: 1SSB/20ms PDCCH: 2 complete symbols DMRS: Type II, based on MU-layer (dynamic in simulation) CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB TRS: 12 RE/PRB/20ms For LTE: PDCCH: 2 complete symbols DMRS: 12RE/PRB CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB PSS/SSS: 288RE/10ms PBCH: 240RE/10ms CRS: 2ports | For NR: SS/PBCH: 1SSB/20ms PDCCH: 2 complete symbols DMRS: Type II, based on MU-layer (dynamic in simulation) CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB TRS: 12 RE/PRB/20ms For LTE: PDCCH: 2 complete symbols DMRS: 12RE/PRB CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB PSS/SSS: 288RE/10ms PBCH: 240RE/10ms CRS: 2ports | For NR: SS/PBCH: 1SSB/20ms PDCCH: 2 complete symbols DMRS: Type II, based on MU-layer (dynamic in simulation) CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB TRS: 12 RE/PRB/20ms For LTE: PDCCH: 2 complete symbols DMRS: 12RE/PRB CSI-RS for CM: 4 ports per UE per 5 slots CSI-RS for IM: 4 RE/PRB PSS/SSS: 288RE/10ms PBCH: 240RE/10ms CRS: 2ports |