3GPP TSG RAN WG1 Meeting #97	R1-1905xxx

# Reno, USA,  May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XX | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PH Calculation and Report across RAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case where duration of NR slot is different from LTE subframe, which NR slot or LTE subframe is used for PH calculation is unclear as follows: - When NR PH is reported over LTE subframe, multiple NR slots can overlap with a LTE subframe. Also, it is possible multiple PUSCH transmissions in the NR slot that overlaps with the LTE subframe when PUSCH mapping Type B is configured. - When LTE PH is reported over NR slot, multiple LTE subframes can overlap with a NR slot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When a SCS configuration of NR slots is larger than LTE SCS, - If a UE is configured with NR PHR over LTE and the UE provides a Type 1 PHR in a PUSCH transmission in a LTE subframe that overlaps with multiple NR slots, the UE provides a Type 1 PHR for the first PUSCH, if any, on the first slot of the multiple slots that fully overlaps with the LTE subframe. - If a UE is configured with LTE PHR over NR and the UE provides a Type 1 PHR in a PUSCH transmission in a NR slot that overlaps with multiple LTE subframes, the UE provides a Type 1 PHR for the PUSCH, on the first subframe of the multiple LTE subframes that overlaps with the NR slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When duration of NR slot is different from one of LTE subframe, which slot or subframe is used PHR calculation is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR only impacts NR PHR via LTE or LTE PHR via NR when a SCS configuration of NR slots is different from LTE SCS. If the network is implemented based on the CR and the UE is not, the UE might calculate PH in an unlear way. If the UE is implemented based on the CR and the network is not, no inter-operability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

7.7.1	Type 1 PH report

If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where , , , , ,  and  are defined in Subclause 7.1.1.

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration  on active UL BWP  of carrier  of serving cell  is smaller than a SCS configuration  on active UL BWP  of carrier  of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP  that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP  that fully overlaps with the slot on active UL BWP .

If a UE indicates a capability for dynamic power sharing for EN-DC, if dualConnectivityPHR is configured after PHR is triggged, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in an E-UTRA subframe that overlaps with multiple slots, where a SCS configuration on active UL BWP  of carrier  of serving cell  in NR is larger than that in E-UTRA, the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple NR slots on active UL BWP  that fully overlaps with the subframe in which E-UTRA power headroom report is transmitted.

If a UE indicates a capability for dynamic power sharing for NE-DC, if multiplePHR is configured after PHR is triggered, and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP , where a SCS configuration on active UL BWP  of carrier  of serving cell  in NR is larger than that in E-UTRA, the UE provides a Type 1 power headroom report for the PUSCH on the first subframe of the multiple E-UTRA subframes that overlaps with the slot on active UL BWP  in which NR power headroom report is transmitted.

If a UE is configured with LTE PHR over NR and the UE provides a Type 1 PHR in a PUSCH transmission in a NR slot that overlaps with multiple LTE subframes, the UE provides a Type 1 PHR for the PUSCH, on the first subframe of the multiple LTE subframes that overlaps with the NR slot.

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP  of carrier  of serving cell , a second PUSCH transmission on active UL BWP  of carrier  of serving cell  that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format 0_0 or a DCI format 0_1 in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered

or

-	the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered.

*** Unchanged text is omitted ***