3GPP TSG RAN WG1 Meeting #97                                                        R1-1906951

# Reno, USA, 13th – 17th May 2019

Agenda item:	7.2.5.1

Source: 	Samsung

Title: 	Remaining issues on NTN link-level and system-level simulation assumptions

Document for: 	Discussion

- Introduction

New radio (NR) targets a wide range of use cases in 5G. The application of a high altitude platform station (HAPS) and satellite nodes in NR can be important components of 5G. The deployment of non-terrestrial networks (NTNs) is highly different from that of terrestrial networks, which can cause impacts on standard specifications. A study item “Solutions for NR to support Non Terrestrial Network” has been approved [1], and the objectives are as follows.

Physical layer

Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed  [RAN1]:

- Physical layer control procedures (e.g. CSI feedback, power control)

- Uplink Timing advance/RACH procedure including PRACH sequence/format/message

- Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

NTN is particularly essential when long-range connectivity in underserved areas and communication links in emergency and disasters are required [2]. Due to different propagation nature of NTN links compared to ordinary new radio (NR) links, it can be expected that the design of NTN has new considerations. TR 38.811 [3] has summarized potential impacts of NTN on the current NR system. To assist the design of NTN, evaluations via simulations need to be performed. As the NTN SI proceeds, more details are to be confirmed. This contribution is addressing these considerations on link-level and system-level simulations for NTN.

- Discussion for link-level simulation

## 2.1 LLS for synchronization performance

In [4], the necessity of studying the synchronization performance using NR Rel-15 synchronization framework is discussed, including the synchronization procedure and physical layer signals/channels e.g. SS/PBCH blocks.

In this contribution, a discussion on the evaluation assumptions for link-level simulation on synchronization performance is made, and the purpose of this discussion is to trigger the coordination work on the evaluation effort in the study item phase. The following table, similar to the one utilized for NR Rel-15 evaluation, can be used as a starting point for determining the set of parameters for link-level simulation.

Proposal 1: Use the following table as a starting point for link-level simulation evaluation for synchronization performance.

|  | FR1 | FR2 |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | As in TR 38.811 [3] |  |
| Subcarrier Spacing(s) | 15 kHz | 120 kHz |
| SNR Range | > -6 dB | > -6 dB |
| Number of Antenna | 1TX, 1RX | 1TX, 1RX |
| Moving Speed | GEO: 0km/s, LEO: 8 km/s | GEO: 0km/s, LEO: 8 km/s |
| Interference model | Scenario 1: no interference | Scenario 1: no interference |
| Initial Frequency Offset | TX: Uniform within [-0.05, 0.05] ppm RX: Uniform within [-5, 5] ppm |  |

- Discussion for system-level simulation

## Overall discussions

To confine the efforts on performance evaluation, RAN1 should only consider a small number of essential and prioritized scenarios, with a limited set of parameters. For a complex system like NTN, scenarios, frequency bands, and parameter options construct a massive number of simulation cases. Elevation angle values, for instance, have nine difference options, which can be found in [3]. Each elevation angle value corresponds to a unique set of parameters for each scenario. This highly complicates the performance comparison process. As a result, it is important to further reduce the number of simulation cases by considering a typical set of scenarios and parameters.

Proposal 2: Reduce the number of simulation cases by considering a typical set of scenarios and parameters.

## Metrics to evaluate

Metrics to evaluate should involve quantities which have high impact to system design, while keeping the number of metrics minimum.

Coupling loss and geometry SINR are the two metrics highly related to link budget and long-term performance of the system. Also, they closely impact NR features such as handover and power control. Therefore, it is mandatory to consider these two metrics. Another important metric for NTN is the round-trip time (RTT), due to its long propagation delay. The behaviour of RTT can significantly influence the design of TA adjustment, RACH procedures, and synchronization procedures. Hence, of RTT should be regarded as mandatory as well. Characteristics of Doppler shift play an important role in the design of high-speed systems, which impact the design of reference signal densities and measurement report periods. Considering that pre-compensation can be employed, residual Doppler shift should at least be considered.

Proposal 3: The number of metrics to evaluate should be minimized to reduce efforts. Coupling loss, geometry SINR, RTT and residual Doppler shift should be regarded as mandatory metrics.

## Simulation assumptions

Currently, RAN1 is looking for system level simulation assumptions for Scenario A (GEO – transparent), Scenario C2 (LEO – transparent – moving beams), and Scenario D2 (LEO – regenerative – moving beams). Following the email discussion, we propose the following parameters for these scenarios, which are based on existing ITU-R documents and reasonable assumptions.

Table 1: System Level Simulation parameters for configuration scenario A

| Configuration scenario | A (GEO – transparent) (Note 1) |
| --- | --- |
| Frequency band | <6 GHz (e.g. S-band 2 GHz) >6 GHz (e.g. Ka-band DL 20 GHz, UL 30 GHz) |
| Maximum Bandwidth per beam (DL + UL) | S-Band: 10 MHz Ka band: 100 MHz |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | Table A |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Linear polarization |
| Beam layout | Hexagonal beam layout. |
| Number of beams | 57 |
| Frequency re-use factor | 1/4 [5] |
| Deployment scenarios | Base-line: Rural Additional deployment scenario results can be provided |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | X=10 UEs per beam with uniform distribution in all the beams should be considered for NTN evaluation. |
| UE configuration | S-band: - Handheld Ka-band: - VSAT |
| UE orientation | VSAT: Ideal Tracking serving beam; Handheld: Random |
| UE antenna and polarization configurations | S-band: - Handheld (optional): omni-directional antenna with 0dBi [6] , linear polarization Ka-band: - VSAT: Ka band: dish antenna with radius 0.74m [5], linear polarization |
| UE EIRP, noise figure, antenna temperature, G/T | S-band: - Handheld (optional): EIRP: -3 dBW [6], noise figure: 5dB, antenna temperature 225K, G/T –24.6 dB/K [6] Ka-band: - VSAT: EIRP: 42.6 dBW [5], noise figure: 5dB, antenna temperature 225K [5], G/T 18.6 dB/K [5] |
| Handover Margin | 0dB |
| UE attachment | RSRP |
| Metrics for calibration | Base line: Coupling loss, Geometry Optional: RTT and Doppler shift. |

Table 2: System Level Simulation parameters for configuration scenarios C2/D2

| Configuration scenario | C2 (LEO – transparent – moving beams) (Note 1) / D2 (LEO – regenerative – moving beams) |
| --- | --- |
| Frequency - Band | <6 GHz (e.g. S-band 2 GHz) >6 GHz (e.g. Ka-band DL 20 GHz, UL 30 GHz) |
| Maximum Bandwidth per beam (DL + UL) | S-band: 10 MHz Ka band: 100 MHz |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | Table A |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Linear polarization |
| Beam layout | Hexagonal beam layout. |
| Number of beams | 48 |
| Frequency re-use factor | 1/4 |
| Deployment scenarios | Base-line: Rural Additional deployment scenario results can be provided |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | X=10 UEs per beam with uniform distribution in all the beams should be considered for NTN evaluation. |
| UE configuration | S-band: - Handheld: omni-directional antenna with 0dBi, linear polarization Ka-band: - VSAT: dish antenna with radius 0.74m [5], linear polarization |
| UE orientation | VSAT: Ideal Tracking serving beam; Handheld: Random |
| UE antenna and polarization configurations | S-band: - Handheld (optional): omni-directional antenna with 0dBi [6] , linear polarization Ka-band: - VSAT: Ka band: dish antenna with radius 0.74m [5], linear polarization |
| UE EIRP, noise figure, antenna temperature, G/T | S-band: - Handheld (optional): EIRP: -3 dBW [6], noise figure: 5dB, antenna temperature 225K, G/T –24.6 dB/K [6]. Ka-band: - VSAT: EIRP: 42.6 dBW [5], noise figure: 5dB, antenna temperature 225K [5], G/T 18.6 dB/K [5] |
| Handover Margin | 0dB |
| UE attachment | RSRP |
| Metrics for calibration | Base line: Coupling loss, Geometry Optional: RTT and Doppler shift. |

Proposal 4: Use Table 1 and Table 2 for system level simulation.

## Additional settings

A number of additional settings were discussed via emails. We are recapping our views as below.

| System bandwidth | S-band {UL:10MHz, DL:10MHz}, Ka-band {UL:100MHz, DL:100MHz} |
| --- | --- |
| Number of beams | Scenario A {57 fixed spot beams} (State-of-the-art: Inmarsat I-5F4, 89 fixed spot beams and 6 steerable beams) Scenario C2/D2 {48 spot beams} (State-of-the-art: Iridium, 48 spot beams) |
| Beam layout | ‘Constant inter-beam distance on the ground’ can be used. c.f. ITU-R S.2361 |
| Number of UEs per beam | X=10, no preference on additional values |
| UE configurations | Refer to Table 1 and Table 2 |
| UE orientation for others | No preference |
| Handover margin | 0dB |
| UE attachment | RSRP |
| Satellite characteristics (e.g G/T, EIRP density, antenna diameter) | No preference. However, as this a second set of parameters, it should be optional. |
| Satellite polarization antenna configuration | Linear polarization |
| Multi-satellite simulation | No preferred. We should not consider multi-satellite simulations for simplicity |
| Metrics for calibration | Round trip time and Doppler shift as optional metrics. |

- Conclusions

In this contribution, we have discussed link-level and system-level simulation details for NTN. We have the following proposals and observations

Proposal 1: Use the following table as a starting point for link-level simulation evaluation for synchronization performance.

|  | FR1 | FR2 |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | As in TR 38.811 [3] |  |
| Subcarrier Spacing(s) | 15 kHz | 120 kHz |
| SNR Range | > -6 dB | > -6 dB |
| Number of Antenna | 1TX, 1RX | 1TX, 1RX |
| Moving Speed | GEO: 0km/s, LEO: 8 km/s | GEO: 0km/s, LEO: 8 km/s |
| Interference model | Scenario 1: no interference | Scenario 1: no interference |
| Initial Frequency Offset | TX: Uniform within [-0.05, 0.05] ppm RX: Uniform within [-5, 5] ppm |  |

Proposal 2: Reduce the number of simulation cases by considering a typical set of scenarios and parameters.

Proposal 3: The number of metrics to evaluate should be minimized to reduce efforts. Coupling loss, geometry SINR, RTT and residual Doppler shift should be regarded as mandatory metrics.

Proposal 4: Use Table 1 and Table 2 for system level simulation.

- References

[1] RP-190710, "Study on solutions for NR to support non-terrestrial networks (NTN)", Thales, Mar. 2019.

[2] ONE5G D2.1, "Scenarios, KPIs, use cases and baseline system evaluation ", H2020-ICT-2016-2, June 2017.

[3] 3GPP T. R. 38.811, "Study on New Radio (NR) to support non terrestrial networks " V15.0.0, June 2018.

[4] R1-1906953, Uplink timing advance/RACH procedure and initial access for NTN, Samsung, May 2019.

[5] ITU-R S.2361-0, Broadband Access by Fixed-Satellite Service Systems, June 2015.

[6] ITU-R M.2047-0, Detailed specifications of the satellite radio interfaces of International Mobile Telecommunications-Advanced, Dec. 2013.

- Appendix

Table A: Set 1 of satellite parameters and payload characteristics for downlink transmissions and uplink receptions

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| G/T |  | 19 dB K-1 | 11.2 dB K-1 | 11.2 dB K-1 |
| Satellite beam diameter |  | 300 km | 150 km | 70 km |
| Equivalent satellite antenna aperture (Note1) | Ka-band | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| G/T |  | 22 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite beam diameter |  | 130 km | 40 km | 20 km |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR38.811v15.0.0. |  |  |  |  |