3GPP TSG-RAN WG1 Meeting #97	R1-1907138

Reno, US, May 13th – 17th 2019	(Revision of R1-1905492)

Agenda Item:	7.2.4.2.3

Source:	Ericsson

Title:	On Mode-1 HARQ indication to gNB

Document for:	Discussion, Decision

# 1	Introduction

A new WI [1] on 5G V2X has been approved in RAN plenary # 83. One of the objectives of the WID is to specify the L1/L2 HARQ procedures for unicast and groupcast for UEs operating in either mode-1 (i.e. Network controlled) and mode-2 (i.e. autonomous). In this contribution, we focus the discussion on Mode-1 HARQ related procedures based on the SI outcome [2].

Related to mode-1 HARQ indication to gNB, the following agreements were made in RAN1#96.

Agreements: - In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE. - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc. - RAN1 continues discussion on whether to support report from the receiver UE - No inter-BS communication will be considered. Agreements: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

According to the SI agreements mentioned above, it is clear that only transmitter UE sends the HARQ indication to gNB. There has been extensive discussions on the support of HARQ indication from receiver UE, however, it was not agreed. This was mainly because the gains of supporting this feature is not justified given the required specification effort and the lack of time. Furthermore, such receiver UE based indication will require receiver UE to be in RRC connected mode which may not always be the case. For instance, receiver UE can be in RRC idle mode but still a part of PC5 unicast/groupcast connection with a transmitter UE (i.e. RRC connected mode). Therefore, due to scarcity of time, we propose to focus on one common solution that is applicable to all possible V2X scenarios as described in the WID.

The gains of mode-1 HARQ indication via receiver UE could not be justified and it was not agreed during the SI phase.

During the WI, RAN1 focusses on specifying mode-1 HARQ indication via transmitted UE only.

# 2	Discussion

In NR Uu, HARQ ACK/NACK is the only signalling that can indicate whether a DL transmission is correctly received by the UE or not and gNB triggers a retransmission in case the transmission is not correctly received (i.e. HARQ NACK). Also, in contrast to LTE, NR Uu supports asynchronous UL HARQ i.e. the scheduling (re)transmissions belonging to different HARQ processes is totally under gNB control. However, the HARQ related information (e.g. process ID, RV, NDI etc.) needs to be conveyed in the scheduling grant. To keep the flexible design of NR, we also foresee asynchronous HARQ for sidelink.

NR supports asynchronous HARQ for both uplink and downlink.

NR supports asynchronous HARQ for sidelink.

When it comes to resource allocation and timing configuration of HARQ feedback, in NR Uu, it is also done by gNB using DCI. For instance, DCI indicates the timing difference (among the RRC configured timing relations) between PDSCH and corresponding HARQ feedback. Similarly, the PUCCH resources are allocated to a UE and is indicated via DCI. Therefore, gNB is in full control of scheduling HARQ feedbacks in NR Uu.

In NR Uu, gNB is in full control of scheduling (re-)transmissions and the corresponding HARQ feedbacks.

Therefore, following the same principle as in NR Uu, it would have been straightforward to indicate sidelink retransmissions in the form of ACK/NACK in case of mode-1 i.e. gNB schedules HARQ feedback resources on PUCCH for SL HARQ feedback. This option has been discussed extensively during the SI and was supported by many companies except a few. In our view, such solution would have been the most straight forward and simplest from specification perspective, at the same time providing the desired functionality.

Sidelink HARQ ACK/NACK reporting from UE to gNB would have been the most straight-forward solution and at the same time provide the required functionality.

However, for the sake of compromise, it was accepted in RAN1#96 that the indication could be in the form of SR/BSR etc., rather than HARQ ACK/NACK. In RAN1#96bis, this issue was again highlighted since many companies observed the specification efforts and complexity related to support HARQ indication in the form of SR/BSR. It was concluded that this issue would be discussed in RAN1#97 and the following was captured in the chairman’s notes.

- Revisit the issue 2.4 as in x5834 in RAN1#97.

The primary purpose of SR/BSR is to request grant for the new data transmission i.e. SR/BSR is only triggered when there is new data in the buffer. Since retransmissions are not perceived as new data and the information is only limited to HARQ entity, it is necessary that RAN2 specify new triggering conditions for SR/BSR based indication for retransmissions.

New SR/BSR triggering conditions are necessary to indicate sidelink retransmissions.

Furthermore, the legacy SR/BSR design only supports scheduling for new transmission. By receiving the legacy sidelink SR/BSR, gNB does not know whether it is for new data transmission or retransmission or both. Therefore, gNB will be unable to schedule (re)transmissions properly if such information is not available. For instance, it may happen that gNB provides the grant which is not sufficient for retransmission for which the TB has already been constructed and is present in the HARQ buffer.

Therefore, it is important that if SR/BSR based solution is supported for mode-1 HARQ indication, the information related to SL retransmission is known at gNB and is conveyed via SR or BSR.

To support the scheduling for SL retransmission, SR/BSR has to convey SL retransmission related information.

Although such explicit indication of retransmission information to gNB is highly desirable to have full gNB control, however, it incurs quite high overhead for BSR and is not necessary since gNB already have the information for retransmission. Such high overhead could have been naturally avoided if HARQ indication in the form of ACK/NACK has been supported.

Sidelink HARQ ACK/NACK reporting from UE to gNB incurs lower overhead on UL.

Based on the above analysis, we propose the following:

Revert the RAN1 agreement to not support Sidelink HARQ ACK/NACK report from UE to gNB i.e. Sidelink HARQ ACK/NACK report from UE to gNB is supported.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The gains of mode-1 HARQ indication via receiver UE could not be justified and it was not agreed during the SI phase.

Observation 2	NR supports asynchronous HARQ for both uplink and downlink.

Observation 3	In NR Uu, gNB is in full control of scheduling (re-)transmissions and the corresponding HARQ feedbacks.

Observation 4	Sidelink HARQ ACK/NACK reporting from UE to gNB would have been the most straight-forward solution and at the same time provide the required functionality.

Observation 5	New SR/BSR triggering conditions are necessary to indicate sidelink retransmissions.

Observation 6	To support the scheduling for SL retransmission, SR/BSR has to convey SL retransmission related information.

Observation 7	Sidelink HARQ ACK/NACK reporting from UE to gNB incurs lower overhead on UL.

Based on the discussion in the previous sections we propose the following:

Proposal 1	During the WI, RAN1 focusses on specifying mode-1 HARQ indication via transmitted UE only.

Proposal 2	NR supports asynchronous HARQ for sidelink.

Proposal 3	Revert the RAN1 agreement to not support Sidelink HARQ ACK/NACK report from UE to gNB i.e. Sidelink HARQ ACK/NACK report from UE to gNB is supported.

# 4	References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, RAN#83, March 18-21, 2019.

- 3GPP TR 38.885, “Study on Vehicle-to-Everything”, V2.0.0, March 2019.