3GPP TSG RAN WG1 Meeting #97	　	                                 R1- 1907178

Reno, USA, 13th May – 17th May. 2019

Source:	Mitsubishi Electric

Title:	Views on UL reference signal designs for NR positioning

Agenda Item:	7.2.10.2 UL Reference Signals for NR Positioning

Document for: 	Discussion/Decision

Introduction

In RAN1 #96b meeting, parameters and measurements needed DL and UL only positioning techniques were discussed. The agreements related to DL and UL PRS were captured in TR 38.855 [3]. In Section 2, proposals related to UL PRS are described. The contribution is summarized in Section 3.

RS design for UL positioning

In RAN1#96b, the number of SRS symbols used for positioning is discussed. Currently, in Rel. 15, up to 4 NR SRS symbols can be placed in the last 6 symbols in a slot. In RAN1#96b, due to the need for frequent sounding for NR-U, an entire slot, up to 14 symbols, were agreed to be used for Rel. 16 SRS. To increase coverage of SRS for positioning in FR2 and improve positioning accuracy, the number of SRS symbols should be increased to 14. A need for 14-symbol SRS is also discussed in Rel. 16 LTE. Similarly, UL positioning methods such as UTDOA should send SRS with minimum interference. Thus, we make the following proposals.

Proposal 1: Up to N=14 NR SRS symbol is used for positioning

Proposal 2: A slot or multiple slots dedicated for positioning are supported for NR positioning

Similar to the DL PRS setting [4], one SRS resource should support staggered patterns to reduce overhead.

Proposal 3: Support staggered patterns (a collection of SRS symbols from the same antenna port with different vertical frequency offsets for at least some symbols) in a single SRS resource

One of the discussions is to design PRS as an extension of SRS. Although Rel. 15 SRS can be designed flexibility in terms of the number of symbols and location, our recommendation is to create a new set usage, namely positioning, for SRS since SRS for positioning would need dedicated setting for the number of symbols as proposed in this contribution. It should be noted that the current set usages for SRS are “codebook”, “noncodebook”, “antennaSwitching” and “beamManagement” [4].

Proposal 4: Support a new set usage for SRS, “positioning”

Conclusion

In this contribution we make the following proposals:

Proposal 1: Up to N=14 NR SRS symbol is used for positioning

Proposal 2: A slot or multiple slots dedicated for positioning are supported for NR positioning

Proposal 3: Support staggered patterns (a collection of SRS symbols from the same antenna port with different vertical frequency offsets fro at least some symbols) in a single SRS resource

Proposal 4: Support a new set usage for SRS, “positioning”

- References

- [1]	RAN1 Chairman’s Note, RAN1 #96b, Xi’an, China, Apr. 2019.

- [2]	RP-190752, “NR Positioning Support,” Shenzhen, China, March, 2019.

- [3]	3GPP TR 38.855, “Study on NR positioning support (Rel.16)”, ver. 1.0.0., March 2019.

- [4]	R1-1907177, “Views on DL reference signal designs for NR positioning”, Reno, USA, May. 2019.

- [4]	3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”, ver. 15.4.0., Dec. 2019.

Appendix:

Agreement: Support the following configurations of SRS for positioning - Semi-persistent configuration - Periodic configuration - Aperiodic configuration Agreement: For positioning, regarding the number of SRS symbols per resource, decide by RAN1#97 whether to increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4}. - If it is not decided to increase the number of SRS symbols per resource by RAN1#97, increasing the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4} will not be considered further after RAN1#97. Agreement: For positioning, regarding the possible SRS symbol locations per slot decide by RAN1#97 whether the SRS can be configured only in the last N symbols of a slot with N>6 - If the above decision is not made by RAN1#97, configuring SRS only in the last N symbols of a slot with N>6 will not be discussed further after RAN1#97. Agreement: Select one or both of the following options to support staggered SRS transmissions for UL SRS positioning - Staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource - Configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set.