3GPP TSG RAN WG1 Meeting #97				R1-1907197

Reno, USA, May 13th – 17th, 2019

Source 	: CAICT

Title 	: Considerations on Procedure for Two-step RACH

Agenda Item	: 7.2.1.2

Document for	: Discussion and decision

Introduction

In last meeting in RAN1#96b, a bunch of agreements in aspect of physical structure for 2-step RACH have been reached as follows[1]:

Agreements: - For the relation of PRACH resources between 2-step and 4-step RACH, the network has the flexibility to configure the following options: - Option 1: Separate ROs are configured for 2-step and 4-step RACH - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH Agreements: Further study the granularity of the time advance command, if supported in MsgB: - E.g., Based on the subcarrier spacing of MsgA PUSCH using a 12-bit TA command, where the granularity of the TA command is determined according to the following table. Subcarrier Spacing (kHz) of the PUSCH; Unit. 15; 16*64 Tc. 30; 8*64 Tc. 60; 4*64 Tc. 120; 2*64 Tc. - Other options/variations are not precluded Agreements: For 2-step RACH preamble power control parameter configuration, further study and down select from the following options: - Option 1: Power control parameters can be separately configured for 2-step and 4-step RACH. - If a power control parameter is not configured for 2-step RACH, the corresponding power control parameter of 4-step RACH is used instead for 2-step. Option 2: The corresponding power control parameter of 2-step RACH preamble follows that of 4-step RACH preamble.

In this contribution, we will present our further considerations on the MsgA transmission, MsgB content and the fallback scheme based on the reached agreements and the offline feature lead in last meeting. Observations and proposals are provided according to the discussions.

Discussions

MsgA transmission

In order to improve the PUSCH transmission reliability, we propose to adopt the HARQ mechanism for the initial transmission and retransmissions of PUSCH. In case of success preamble detection and failed PUSCH detection in a MsgA, the gNB can allocate a RNTI generated with the position of RO and preamble ID associated with previous PUSCH to the UE for the retransmission of PUSCH. Then when a gNB detects a retransmission of PUSCH, it can realize the position of previous PUSCH and apply the HARQ mechanism.

Observation1: HARQ mechanism for the initial transmission and retransmissions of PUSCH can improve the PUSCH transmission reliability.

Proposal1: It is suggested to apply HARQ mechanism to the PUSCH transmission and set the RNTI generated with the position of RO and preamble ID associated with previous PUSCH for the retransmission of PUSCH in case of success preamble detection and failed PUSCH detection in a MsgA.

MsgB Content

In our observation, the MsgB can be groupcast to the UEs with successful preamble detection and failed PUSCH decoding, or be unicast to the UEs with successful detection of both preamble and PUSCH, since the UE can include the RNTI information for MsgB scrambling into the payload of PUSCH and address the MsgB with the sent RNTI if the payload is successfully detected at gNB. We refer the MsgB in groupcast manner as Msg2-like MsgB. As for the content of Msg2-like MsgB, it encounters the problem of whether the Msg2-like MsgB can be combined with the Msg2 in 4-step RACH as one MAC PDU, and the problem of whether design a new subPDU format to support the retransmission of whole MsgA or PUSCH only.

If the Msg2-like MsgB is not combined with Msg2 in 4-step RACH, if 2-step and 4-step RACH are configured with separate ROs, the UEs can differentiate the Msg2-like MsgB and Msg2 with conventional RA-RNTI. However, if 2-step and 4-step RACH are configured with shared RO and separate preambles, a new RA-RNTI might be necessary for Msg2-like MsgB to avoid UEs detecting both MAC PDU of Msg2-like MsgB and Msg2 to save the UEs’ energy and time cost of Msg2 detection.

If the Msg2-like MsgB is combined with Msg2 in 4-step RACH, it only happens in case of 2-step and 4-step RACH sharing the same RO. If to start a msg3 transmission in case of preamble detection success and PUSCH decoding fail, then conventional MAC PDU of RAR can be adopted. However, if to support the retransmission of whole MsgA or PUSCH only, a new subPDU format might be needed with part of content of conventional RAR, such as just including the preamble ID, TA and RNTI information. Also, the conventional RAR can be reused for Msg2-like MsgB and the exclusive information can be set empty. UEs extract the associated subPDU according to the preamble ID. In the combined MAC PDU, it might include two backoff indicator for 2-step and 4-step RACH, respectively, if to design a seperate backoff indictor for 2-step RACH. An example of MAC PDU format bundling with Msg2-like MsgB f and Msg2 is shown in Fig.1.

FIG 1 Illustration of MAC PDU format bundling with Msg2-like MsgB for 2-step users and Msg2 for 4-step RACH users.

Observation2: As for the content of Msg2-like MsgB, it encounters the problem of whether the Msg2-like MsgB can be combined with the Msg2 in 4-step RACH as one MAC PDU, and the problem of whether design a new subPDU format to support the retransmission of whole MsgA or PUSCH only.

Proposal2: FFS the problem of whether the Msg2-like MsgB can be combined with the Msg2 in 4-step RACH as one MAC PDU, and the problem of whether design a new subPDU format to support the retransmission of whole MsgA or PUSCH only.

Fallback Scheme

Generally, for the fallback scheme with 2-step RACH re-attempt, it shall further consider the MsgA retransmission strategy. For example, the retransmission would be adaptive or non-adaptive and the backoff indicator for 2-step RACH would be adopted or not. In the adaptive retransmission, the RO for MsgA retransmission might be different with the one used in the last transmission. In the non-adaptive retransmission, the same RO would be reused for MsgA retransmission. Since the RO used for MsgA transmission is not associated UE specific ID, the gNB might not recognize the MsgA retransmission by the same UE except the gNB allocates a RNTI for PUSCH retransmission and informs the UE of RO used for MsgA retransmission in case of preamble detection success and PUSCH decoding fail or UCI with PUSCH is adopted. And when the gNB is configured to recognize the MsgA retransmission in case of preamble detection success and PUSCH decoding fail, the PUSCH only retransmission and the corresponding HARQ are then supportable.

Observation3: For 2-step RACH re-attempt, it shall further consider the MsgA retransmission strategy with details about the configuration of adaptive or non-adaptive retransmission, backoff indicator and possible HARQ for PUSCH.

Proposal3: FFS the details for the MsgA retransmission strategy.

In this contribution, we further propose to configure a maximum transmission time of MsgA for 2-step RACH re-attempt and a response window to detect MsgB after sending MsgA. When the UE does not detect any response to its MsgA in the response window, and the transmission time of MsgA does not exceed the configured maximum transmission time, the UE would retransmit MsgA according to the 2-step RACH configuations. Otherwise, if the transmission time of MsgA exceeds the configured maximum transmission time, the UE would switch to 4-step RACH to send Msg1. And after each transmission of MsgA, the counter of preamble transmission time shall also increment by one as the counter of the transmission time of MsgA. Note that the configured maximum transmission time of MsgA shall not be greater than the configured maximum transmission time of preamble in the conventional 4-step RACH. Otherwise, the fallback from 2-step RACH to 4-step RACH is meaningless and the UEs of 2-step RACH can directly inform the failure of RACH to the higher layer after reaching the maximum transmission time of MsgA.

Observation4: The configured maximum transmission time for MsgA if it exists shall be no greater than the configured maximum transmission time of preamble in the conventional 4-step RACH. Otherwise the fallback from 2-step RACH o 4-step RACH is meaningless.

In case of preamble detection success and PUSCH decoding fail, it is equivalent to the situation in 4-step RACH where the Msg1 is sent out successfully. It is determined by the gNB whether to respond Msg2 in 4-step RACH, or a part of Msg2 without the UL grant for Msg3 to let the UE retransmit MsgA in 2-step RACH, or no response. If the system is configured to enable the gNB to be aware of transmission time of MsgA and a maximum transmission time of MsgA is configured, then the gNB shall not allocate the UL grant for Msg3 until the transmission time of MsgA reaches its maximum value. At the UE side, if it detects a response containing the same preamble ID as in MsgA, and if there is a UL grant corresponding to the preamble ID, then the UE shall use the UL grant to transmit Msg3 according to the 4-step RACH procedure. However, if there is no UL grant except the information of TA corresponding to the preamble ID, the UE shall retransmit MsgA if the configured maximum transmission time of MsgA has not been reached, otherwise switch to transmit Msg1 using 4-step RACH. The timing of transmission shall be adjusted according to the received TA.

We further illustrate the procedures of our scheme applied at the UE side and gNB side in FIG2. and FIG3, respectively. Moreover, when a backoff indicator is informed to the UE for MsgA retransmission, then the UE shall select a backoff counter to determine the latency before the next retransmission of MsgA.

FIG 2 The procedure of designed scheme of fallback from 2-step RACH to 4-step RACH at the UE side.

FIG 3 The procedure of designed scheme of fallback from 2-step RACH to 4-step RACH at the gNB side.

Proposal4: It is suggested to configure a maximum transmission time for MsgA in 2-step RACH and adopt the above designed scheme of fallback from 2-step RACH to 4-step RACH. And the maximum transmission time shall be no greater than the configured maximum transmission time of preamble in the conventional 4-step RACH.

In addition, in the case that gNB shall respond more than one MsgBs to different UE groups in separate DL grants, as shown in FIG4, then the gNB shall determine the priority of the MsgB responses based on the latency requirement of UEs and RACH triggers, and also the time of attempts of MsgA. For example, the UE can include an indicator of the transmission time of MsgA in the PUSCH ohf MsgA via MAC CE or UCI. The gNB then shall respond MsgB to the UEs with high latency requirement and large transmission time of MsgA in priority.

FIG 4 An illustration of case that gNB shall respond more than one MsgBs to different UE groups in separate DL grants.

Observation5: In the case that gNB shall respond more than one MsgBs to different UE groups in separate DL grants, the gNB shall determine the priority of the MsgB responses.

Propsal5: It is suggested to FFS the details on how to determine the priority of the MsgB responses at gNB side.

Conclusions

In this contribution, we have present our further considerations on the MsgA transmission, MsgB content and the fallback scheme based on the reached agreements and the offline feature lead in last meeting. We put forward the following observations and proposals:

Observation1: HARQ mechanism for the initial transmission and retransmissions of PUSCH can improve the PUSCH transmission reliability.

Proposal1: It is suggested to apply HARQ mechanism to the PUSCH transmission and set the RNTI generated with the position of RO and preamble ID associated with previous PUSCH for the retransmission of PUSCH in case of success preamble detection and failed PUSCH detection in a MsgA.

Observation2: As for the content of Msg2-like MsgB, it encounters the problem of whether the Msg2-like MsgB can be combined with the Msg2 in 4-step RACH as one MAC PDU, and the problem of whether design a new subPDU format to support the retransmission of whole MsgA or PUSCH only.

Proposal2: FFS the problem of whether the Msg2-like MsgB can be combined with the Msg2 in 4-step RACH as one MAC PDU, and the problem of whether design a new subPDU format to support the retransmission of whole MsgA or PUSCH only.

Observation3: For 2-step RACH re-attempt, it shall further consider the MsgA retransmission strategy with details about the configuration of adaptive or non-adaptive retransmission, backoff indicator and possible HARQ for PUSCH.

Proposal3: FFS the details for the MsgA retransmission strategy.

Observation4: The configured maximum transmission time for MsgA if it exists shall be no greater than the configured maximum transmission time of preamble in the conventional 4-step RACH. Otherwise the fallback from 2-step RACH o 4-step RACH is meaningless.

Proposal4: It is suggested to configure a maximum transmission time for MsgA in 2-step RACH and adopt the above designed scheme of fallback from 2-step RACH to 4-step RACH. And the maximum transmission time shall be no greater than the configured maximum transmission time of preamble in the conventional 4-step RACH.

Observation5: In the case that gNB shall respond more than one MsgBs to different UE groups in separate DL grants, the gNB shall determine the priority of the MsgB responses.

Propsal5: It is suggested to FFS the details on how to determine the priority of the MsgB responses at gNB side.

References

- Chairman's Notes RAN1#96b, Mar. 2019.

- R1-1905126, Considerations on Procedure for Two-step RACH, CAICT, Mar. 2019.